3GPP TSG-RAN5 Meeting #108	R5-254633

# Bengaluru, India, Aug 25th-29th, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on the optimization of Out-of-band Blocking (OOBB) testing

Document for:	Discussion and endorsement

Introduction

In the discussion of OOBB testing, the large number of test frequency points and the long testing time are currently recognized issues. In [1] proposals were made for the analysis of the factors affecting OOBB. In this contribution, we further discuss the proposals.

Discussion

Impacts of regulations and certification

Comments #1：Does the proposed OOBB optimization method have regulations or certifications related impacts?

Multiple regulations and certification including FCC, CE, PTCRB, GCF and so on have been searched, and have reached the following conclusions.

| Regulations | Will it be affected by the optimization of OOBB? | Note | Reference |
| --- | --- | --- | --- |
| FCC | Theoretically, no | The TCB review requires detailed documentation, with no restrictions on actual values; a reasonable explanation and submission will suffice. The regulations are more concerned with the impact of emissions rather than Rx characteristics. | [2] |
| ETSI/CE 301908-25 | no | Its testing refers to 3GPP test specifications. | [3] |
| PTCRB/GCF | no | It is explicitly stated that the testing is based on the 3GPP test specifications. | [4][5] |
| MIC/TELEC | no | Basically, only the requirements which have impact on other radio systems (e.g. Tx characteristics, spurious emission) are mandatorily tested in Japan. OOBB testing is not regulated. | [6][7] |

And for certification, the technical requirements for certification organization such as PTCRB and GCF are based on standards developed by criterion Development Organizations (SDOs) such as 3GPP and Open Mobile Alliance (OMA), and test plans developed by CTIA certification. Therefore, we do not think there is direct impact on regulations.

Observation 1:	Most of the laws and regulations restrict the transmitter characteristics and emission. That is, mandatory regulations mainly care the impact on other Radio systems.

Proposal 1:	Test procedure update is feasible.

Imperfections in RF Chain for the unwanted signal far away from the wanted signal

Comments #2: Does the imperfections in the band filter need to be considered in OOBB testing simplification?

First, when an out-of-band signal arrives at the receiver, it first goes through a band filter for out-of-band suppression. We have researched hundreds of mainstream band filters currently available on the market and have observed that these filters generally have good suppression capabilities, even the worst ones can reach a suppression level of >15dB. So, we believe that most of the filters can effectively suppress far strong unwanted signal (e.g. interference at -15dBm level in range 3). Annex is the absolute attenuation summary of several typical band filters.

Next, the worst case also can be considered, i.e. a bad band filter is used and interference far away has leaked. The signal filtered by band filter will enter the LNA (Low Noise Amplifier), LNA itself is not a full-bandwidth component. it will amplify the signal at wanted carrier and hardly amplify unwanted signals that are far away. The residual interference signal after band filter and the LNA might still be higher than the wanted signal, but it will be further suppressed by the LPF and baseband process therefore will not limit the demodulation performance.

From RF front-end point of view, as long as the total power difference between interference signal and wanted signal is within a reasonable range, we could expect the RF chain will not be “blocked”. In this example the power of interference signal is ~31dB higher than wanted signal at the input of LNA, which is not challenging comparing with many other potential interferences. For example, in FDD band the Tx leakage could be 81dB higher than wanted signal at the input of LNA, but no relaxation will be considered if the Tx leakage is >100MHz away from the Rx band, e.g. band n1.

Calculation of Tx leakage: PA output is 27dBm assuming PC3 UE, duplexer isolation is usually assumed as 50dB in RAN4, the wanted signal is -100dBm/5MHz on band n1 according to REFSENS requirement in TS 38.101-1, and RF insertion loss is 4dB. The resulting Tx Leakage to signal power difference after duplexer is 81 dB, which is manageable by the UE.

Observation 2:	Mainstream band filters on the market can suppress strong unwanted signal far away from the wanted signal. Even band filter with poor performance is used, the LNA's bandwidth characteristics will not amplify this interference, nor will it affect the non-linearity of the LNA and RFIC.

Observation 3:	The impact of strong interference signals (aside from intermodulation products and harmonic products) that are far from the wanted signal bandwidth entering the receiver can be ignored in the RF system.

The number of exceptions for receiver blocking spurious response

Comments #3: Does the proposed OOBB optimization method have impacts on the number of exceptions?

In [8] and [9] the number of exceptions for UE receiver blocking in E-UTRA band was first discussed. The exceptions are allowed for OOBB testing to take account of the intermodulation and various discrete frequency components generated by clocks/oscillators within the UE hardware and these products are related to the wanted channel bandwidth. Therefore, the number of exceptions should scale with NRB, which is the number of resource blocks in the transmission bandwidth configurations for the downlink. As a result, the number of exceptions depends on the step size and channel bandwidth.

The formula to calculate number of exceptions, i.e. a maximum of  exceptions are allowed for spurious response frequencies in each test, shall also apply the same principle in NR band OOBB test. In the optimized OOBB test we proposed, the frequency points selected will still in range1, range2 and range3, therefore the numerator in the formula remains unchanged. And the step size is unchanged. What the optimization do, is just to skip those test points which do not have an impact, so the formula remains unchanged.

Observation 4: The number of exceptions for receiver blocking spurious response shall keep unchanged for each OOBB optimization test.

The number of the test points calculation for the proposed method

The OOBB optimization method3 we proposed are as follows：

- For NR band OOBB, the interferer frequencies near the passband of Band Filter should be tested to check the characteristics of band filter and LNA is from FDL_low-[100MHz] to FDL_low-[15MHz], and from FDL_high+[15MHz] to FDL_high+[100MHz].

- Execute the test when the IM3/IM5 of wanted signal and interferer fall into FDL_low to FDL_high, and when the ≤1/5~5 times harmonic fall into – (FDL_high - FDL_low) /2 to (FDL_high - FDL_low)/2.

The test requirements can be specified as per band. Taking Band n3 as an example, FDL_low = 1805MHz, FDL_high = 1880MHz.

According to the first OOBB optimization method, the scan ranges can be at 1705 to 1790MHz and 1895 to 1980MHz.

According to the second OOBB optimization method, IM3/IM5 and harmonic product of wanted signal and interfering signal：

| product label | Product | funwanted range |
| --- | --- | --- |
| IM3 product1 | 2×funwanted−fwanted | 1823.75 - 1861.25 |
| IM3 product2 | 2×fwanted − funwanted | 1805 - 1880 |
| IM5 product1 | 3×funwanted −2×fwanted | 1830 - 1855 |
| IM5 product2 | 3×fwanted −2×funwanted | 1823.75 - 1861.25 |
| 1/3 harmonic product | funwanted −1/3×fwanted | 576.67 - 651.67 |
| 1/5 harmonic product | funwanted −1/5×fwanted | 331 - 406 |
| 3 harmonic product | funwanted −3×fwanted | 5490 - 5565 |
| 5 harmonic product | funwanted −5×fwanted | 9175 - 9250 |

So the test requirements can be as followed:

Table 7.6.3.5-2: Out of-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n3 | Pinterferer | dBm | -44 | -30 | -15 |
|  | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 331 - 406 576.67 - 651.67 1705 - 1720 1965 - 1980 5490 - 5565 9175 - 9250 |
| NOTE 1: For frequency range 1≤ f < FDL_low – [100] and FDL_high + [100] < f ≤ 12750, execute the test when the IM3/IM5 products of wanted signal and interfering signal fall into [fc – BW/2, fc + BW/2], or the mixer product of interferer and 1/3rd, 1/5th , 3rd, 5th harmonic of wanted signal fall into [-BW/2, BW/2]. |  |  |  |  |  |

Based on the above analysis, we put forward the following proposal:

Proposal 2: For NR band OOBB, the interferer frequencies near the passband of Band Filter should be tested to check the characteristics of band filter and LNA is from FDL_low-[100MHz] to FDL_low-[15MHz], and from FDL_high+[15MHz] to FDL_high+[100MHz].

Proposal 3: Execute the test when the IM3/IM5 of wanted signal and interferer fall into FDL_low to FDL_high, and when the ≤1/5~5 times harmonic fall into – (FDL_high - FDL_low) /2 to (FDL_high - FDL_low)/2.

The OOBB test optimization for CA/DC

For CA\DC case we suggest the similar optimization method. To be more specific,

-	When the intermodulation and harmonic products fall in the frequency band combinations, only ≤5 times intermodulation and ≤1/5~5 times harmonic in range 1, range 2, range 3 test frequency points are tested.

-	Otherwise, for each single NR frequency band only one combination is tested to ensure that the DUT meets OOBB requirements.

Proposal 4: For CA\DC case, when the intermodulation and harmonic products fall in the frequency band combinations, only ≤ 5 times intermodulation and ≤ 1/5~5 times harmonic in range 1, range 2, range 3 test frequency points are tested. Otherwise, for each single NR frequency band only one combination is tested to ensure that the DUT meets OOBB requirements.

Conclusion

Based on the above discussion, following observations can be obtained:

Observation 1:	Most of the laws and regulations restrict the transmitter characteristics and emission. That is, mandatory regulations mainly care the impact on other Radio systems.

Observation 2:	Mainstream band filters on the market can suppress strong unwanted signal far away from the wanted signal. Even band filter with poor performance is used, the LNA's bandwidth characteristics will not amplify this interference, nor will it affect the non-linearity and Gain Gear Switching of the LNA and RFIC.

Observation 3:	The impact of strong interference signals (aside from intermodulation products and harmonic products) that are far from the wanted signal bandwidth entering the receiver can be ignored in the RF system.

Observation 4:	The number of exceptions for receiver blocking spurious response shall keep unchanged for each OOBB optimization test.

Proposal 1:	Test procedure update is feasible.

Proposal 2: For NR band OOBB, the interferer frequencies near the passband of Band Filter should be tested to check the characteristics of band filter and LNA is from FDL_low-[100MHz] to FDL_low-[15MHz], and from FDL_high+[15MHz] to FDL_high+[100MHz].

Proposal 3: Execute the test when the IM3/IM5 of wanted signal and interferer fall into FDL_low to FDL_high, and when the ≤1/5~5 times harmonic fall into – (FDL_high - FDL_low) /2 to (FDL_high - FDL_low)/2.

Proposal 4: For CA\DC case, when the intermodulation and harmonic products fall in the frequency band combinations, only ≤ 5 times intermodulation and ≤ 1/5~5 times harmonic in range 1, range 2, range 3 test frequency points are tested. Otherwise, for each single NR frequency band only one combination is tested to ensure that the DUT meets OOBB requirements.

- 4 References

- R5-253614 “Discussion on how to reduce the measurement complexity of Out-of-Band Blocking in RAN5”, Huawei.

- Code of Federal Regulations Title 47, https://www.ecfr.gov/current/title-47/

- https://cdn.standards.iteh.ai/samples/54786/8e95f89d11204c34babf95c55e860ec1/ETSI-EN-301-908-25-V15-0-0-2024-07-.pdf

- https://www.ptcrb.com/wp-content/uploads/2025/06/NAPRD03-V6_21.pdf

- https://www.globalcertificationforum.org/static/uploaded/219a6372-9e9a-4a01-86745b90adeb6b67.pdf

- https://www.arib.or.jp/english/html/overview/doc/5-STD-T66v2_1-E.pdf

- https://hourei.net/law/325AC0000000131?utm_source

- R4-081436, “Number of exceptions in Rx blocking”, Qualcomm.

- R4-082250, “Discussion of Number of Rx blocking exceptions”, Qualcomm.

- 5 Annex

Band n3 1805~1880MHz Band filter:

Brand 1:

Brand 2:

Brand 3:

Brand 4:

Brand 5:

Brand 6: