3GPP TSG-RAN5 Meeting #108	R5-254726

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5137 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR NTN enhancements test case 9.4.1.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Rel-18 NR NTN test case 9.4.1.5 – " NR NTN Enhancements / Support for Unavailability Period" needs to be added as per NR_NTN_enh_plus_CT-UEConTest WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new Rel-18 NR NTN test case 9.4.1.5 – " NR NTN Enhancements / Support for Unavailability Period" |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_NTN_enh_plus_CT-UEConTest WP will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.1.5 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0627 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

#### 9.4.1.5	NR NTN Enhancements / Support for Unavailability Period

9.4.1.5	Test Purpose (TP)

(1)

with { UE supporting unavailability period on a satellite NG-RAN cell in 5GMM-DEREGISTERED state }

ensure that {

when { UE performs initial registration }

then { UE sets UN-PER bit to "unavailability period supported" in the 5GMM capability IE of the REGISTRATION REQUEST message }

}

(2)

with { UE supporting unavailability period on a satellite NG-RAN cell in 5GMM-IDLE state }

ensure that {

when { UE needs to provide the unavailability information to the AMF }

then { UE provides unavailability information by initiating the mobility registration procedure }

}

(3)

with { UE having activated unavailability period on a satellite NG-RAN cell in 5GMM-REGISTERED.NO-CELL-AVAILABLE state }

ensure that {

when { UE comes out of the unavailability period }

then { UE performs registration procedure }

}

9.4.1.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501 clauses 5.3.26, 5.5.1.2.2, 5.5.1.2.4, 5.5.1.3.2 and 5.5.1.3.4. Unless otherwise stated these are Rel-18 requirements.

[24.501, clause 5.3.26]

…

To activate the unavailability period, either the UE provides unavailability information, including a type of unavailability, an unavailability period duration if known, and the start of the unavailability period if known, during the registration procedure or the UE includes the type of unavailability and the unavailability period duration if known, during the de-registration procedure (see 3GPP TS 23.501 [8] and 3GPP TS 23.502 [9]). The support for the unavailability period is negotiated in the registration procedure. If the UE is registered to a PLMN via a satellite NG-RAN cell, the AMF may for discontinuous coverage provide the unavailability period duration of the UE or the start of the unavailability period or both to the UE during the registration procedure. The AMF may consider the unavailability period duration provided by the UE if available and determine the unavailability period duration of the UE as described in subclause 5.5.1.2.4 and 5.5.1.3.4. The AMF may also consider the start of the unavailability period provided by the UE, if available, and determine the start of the unavailability period of the UE as described in subclause 5.5.1.3.4.

If the UE provided unavailability information in the last registration procedure for mobility registration or the de-registration procedure, the AMF considers the UE unreachable until the UE registers for a normal service. If the UE did not include a start of the unavailability period, the AMF shall consider the start of the unavailability period to be the time at which AMF received the REGISTRATION REQUEST message or the DEREGISTRATION REQUEST message from the UE. During the registration procedure, the AMF may determine the values of the negotiated extended DRX parameters, the timer T3324, and the periodic registration update timer (T3512) to be provided to the UE based on the discontinuous coverage maximum time offset, the unavailability period duration and the start of the unavailability period based on their availability. The AMF should set the value of the mobile reachable timer and implicit de-registration timer based on the unavailability period duration and the start of the unavailability period. The AMF releases the N1 signalling connection after the completion of the registration procedure in which the UE provided unavailability information without providing the start of the unavailability period.

NOTE 2: 	If the UE supports MUSIM and the UE is registered with the support of unavailability period, then the UE can indicate a different unavailability periods to each registered network.

…

If the AMF sets the EUPR bit value in the Unavailability configuration IE to "UE does not need to report end of unavailability period", then upon returning to a TA in the current registration area the UE shall choose an appropriate substate of 5GMM-REGISTERED and the UE is not required to trigger the registration procedure for mobility registration update when the unavailability period duration has ended. If the AMF does not provide the Unavailability configuration IE or the AMF sets the EUPR bit value in the Unavailability configuration IE to "UE needs to report end of unavailability period", the UE should trigger registration procedure for mobility registration update when the unavailability period duration has ended.

…

When the UE activates the unavailability period using registration procedure without providing the start of the unavailability period, then after successful completion of the procedure the UE shall enter the state 5GMM-REGISTERED.NO-CELL-AVAILABLE and may deactivate AS layer. Otherwise, if the UE provided the start of unavailability period in the registration procedure, the UE shall enter the state 5GMM-REGISTERED.NO-CELL-AVAILABLE and may deactivate AS layer only after the UE activates the unavailability period.

…

When the UE comes out of the unavailability period the UE shall activate the AS layer if deactivated and perform registration procedure as described in subclause 5.5.1.2.2 and 5.5.1.3.2.

[24.501, clause 5.5.1.2.2]

The UE in state 5GMM-DEREGISTERED shall initiate the registration procedure for initial registration by sending a REGISTRATION REQUEST message to the AMF,

…

g)	when the UE performs initial registration to come out of unavailability period and resume normal services;

…

If the UE supports the unavailability period, the UE shall set the UN-PER bit to "unavailability period supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

[24.501, clause 5.5.1.2.4]

If the UE set the UN-PER bit to "unavailability period supported" in the 5GMM capability IE in the REGISTRATION REQUEST message and the AMF supports and accepts the use of unavailability period for the UE, then the AMF shall set the UN-PER bit to "unavailability period supported" in the 5GS network feature support IE in the REGISTRATION ACCEPT message. Furthermore, if the AMF is able to determine an unavailability period duration for the UE based on satellite coverage availability information (see 3GPP TS 23.401 [7]), the AMF shall store the determined unavailability period duration and provide the determined unavailability period duration to the UE by including the unavailability period duration in the Unavailability configuration IE in the REGISTRATION ACCEPT message. If the AMF is able to determine the start of unavailability period based on satellite coverage availability information (see 3GPP TS 23.401 [7]), the AMF shall store the determined start of unavailability period and provide the determined start of unavailability period to the UE by including the start of unavailability period in the Unavailability configuration IE in the REGISTRATION ACCEPT message. The AMF may determine the periodic registration update timer value based on the network determined unavailability period duration, a network determined start of the unavailability period or a discontinuous coverage maximum time offset value or any combination of them.

…

If the UE receives the Unavailability configuration IE in the REGISTRATION ACCEPT message, the UE shall consider that the corresponding unavailability type is "unavailability due to discontinuous coverage". A UE that does not implement this unavailability type may ignore the Unavailability configuration IE provided by the AMF. If the EUPR bit is set to "UE does not need to report end of unavailability period", the UE is not required to initiate the registration procedure for mobility registration update when the unavailability period duration has ended.

If the UE receives the Unavailability configuration IE with a value of the unavailability period duration in the REGISTRATION ACCEPT, then the UE may either:

a)	delete a UE determined value and start using the received unavailability period duration value; or

b)	use a UE determined value.If the UE receives the Unavailability configuration IE with a value of the start of the unavailability period in the REGISTRATION ACCEPT message, then the UE may either:

a)	delete a UE determined value and start using the received start of the unavailability period value; or

b)	use a UE determined value.

NOTE 24: The UE can consider the received value from the network when determining the value for unavailability period duration and the start of the unavailability period.

[24.501, clause 5.5.1.3.2]

The UE in state 5GMM-REGISTERED shall initiate the registration procedure for mobility and periodic registration update by sending a REGISTRATION REQUEST message to the AMF,

…

zm1)	when the UE needs to provide the unavailability information or to update the unavailability information;

zm2)	void;

NOTE 3A: How UE determines that it is about to lose satellite coverage is an implementation option.

zn)	when the UE needs to come out of unavailability period and resume normal services;

…

If the UE supports the unavailability period, the UE shall set the UN-PER bit to "unavailability period supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

…

For case zm1, if the network indicated support for the unavailability period in the last registration procedure, the UE shall include the Unavailability information IE in the REGISTRATION REQUEST message. If the UE did not include a start of the unavailability period in the Unavailability information IE, the UE shall set the Follow-on request indicator to "No follow-on request pending" in the REGISTRATION REQUEST message and shall not include the Uplink data status IE or the Allowed PDU session status IE in the REGISTRATION REQUEST message even if the UE has one or more active always-on PDU sessions associated with the 3GPP access. If the UE includes the Unavailability information IE to indicate the type of the unavailability and the UE will be unavailable due to NR satellite access discontinuous coverage, the UE shall set the Unavailability type bit to "unavailability due to discontinuous coverage" in the Unavailability information IE.

For case zm1, the UE should initiate the registration procedure for mobility and periodic registration update only if the UE can determine, based on its implementation, that there is enough time to complete the procedure before the start of the unavailability period.

[24.501, clause 5.5.1.3.4]

If the UE sets the UN-PER bit to "unavailability period supported" in the 5GMM capability IE in the REGISTRATION REQUEST message and the AMF supports and accepts the use of unavailability period for the UE, then the AMF shall set the UN-PER bit to "unavailability period supported" in the 5GS network feature support IE in the REGISTRATION ACCEPT message.

…

If the UE sets the Unavailability type to "unavailability due to discontinuous coverage" in the Unavailability information IE and the UE provides the Unavailability information IE in the REGISTRATION REQUEST message then:

a)	if the AMF is able to determine an unavailability period duration for the UE based on satellite coverage availability information (see 3GPP TS 23.401 [7]) and the value of the Unavailability information IE in the REGISTRATION REQUEST message if available, the AMF shall store the determined unavailability period duration and provide the expected unavailability period duration to the UE by including the Unavailability period duration in the Unavailability configuration IE in the REGISTRATION ACCEPT message. If the AMF is able to determine the start of the unavailability period based on satellite coverage availability information and the value of the Unavailability information IE in the REGISTRATION REQUEST message if available, the AMF shall store the determined start of the unavailability period and provide the expected start of the unavailability period to the UE by including the start of the unavailability period in the Unavailability configuration IE in the REGISTRATION ACCEPT message; and

b1)	the AMF shall determine the unavailability period duration value as:

-	A value that was provided by the UE; or

-	A value that was determined by the AMF based on satellite coverage availability information; and

b2)	the AMF shall determine the start of the unavailability period value as:

-	A value that was provided by the UE; or

-	A value that was determined by the AMF based on satellite coverage availability information; and

the AMF shall store the unavailability period duration and the start of unavailability period value. When the unavailability period starts, the AMF shall consider the UE as unreachable until the UE registers for normal service again;

c)	the AMF shall determine whether the UE is required to perform the registration procedure in NG-RAN satellite access when the unavailability period has ended and set the EURP bit to "UE needs to report end of unavailability period" or "UE does not need to report end of unavailability period" in the Unavailability configuration IE in the REGISTRATION ACCEPT message.

The AMF may determine the periodic registration update timer value based on the stored value of the received unavailability period duration if any, the received Start of unavailability period if any, the network determined unavailability period duration if any and the network determined Start of unavailability period if any. If the UE does not provide the Unavailability information IE in the REGISTRATION REQUEST message, the AMF shall delete any stored value of the Unavailability information IE if exists.

If the 5GS registration type IE in the REGISTRATION REQUEST message indicates "periodic registration updating" the AMF shall not incude the Unavailability configuration IE in the REGISTRATION ACCEPT message.

If the UE receives the Unavailability configuration IE with a value of the unavailability period duration in the REGISTRATION ACCEPT message, then the UE may either:

a)	delete a UE determined value and start using the received value; or

b)	use a UE determined value.

If the UE receives the Unavailability configuration IE with a value of the start of the unavailability period in the REGISTRATION ACCEPT message, then the UE may either:

a)	delete a UE determined value and start using the received value; or

b)	use a UE determined value.

NOTE 20a: The UE can consider the received value from the network when determining the value for unavailability period duration and the start of the unavailability period.

9.4.1.5.3	Test description

9.4.1.5.3.1	Pre-test conditions

System Simulator:

-	NGC Cell A and NGC Cell B belonging to the same PLMN and belongs to different TAI in accordance with TS 38.508-1 [4] Table 6.3.2.2-1

-	System information combination NR-29 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NGC cells.

UE:

-	The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5C.

Preamble:

-	UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [4].

9.4.1.5.3.2	Test procedure sequence

Table 9.4.1.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - NGC Cell A as the "Serving cell " - NGC Cell B as “Non-suitable "Off" cell" | - | - | - | - |
| 2 | The UE is switched ON. | - | - | - | - |
| 3 | Check: Does UE transmit a REGISTRATION REQUEST message including 5GMM capability IE UN-PER bit to "unavailability period supported" on NGC Cell A? | --> | 5GMM: REGISTRATION REQUEST | 1 | P |
| 4-18a1 | Steps 5-19a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 19 | The SS transmits a CONFIGURATION UPDATE COMMAND message including Local time zone, Universal time and local time zone, Network daylight saving time information. (Note 1) | <-- | CONFIGURATION UPDATE COMMAND | - | - |
| 20 | SS transmits an RRCRelease message. | <-- | - | - | - |
| 21 | Check: Does the UE transmit a REGISTRATION REQUEST message with the 5GS registration type IE indicating "mobility registration updating"on NGC Cell A with in 60secs? | --> | 5GMM: REGISTRATION REQUEST | 2 | P |
| 22-24a1 | Steps 4-6a1 of Table 4.9.5.2.2-1 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| - | The SS configures: - NGC cell A as a "Non-suitable "Off” cell" - NGC cell B as the "Serving cell" | - | - | - | - |
| 25 | SS waits for 2mins. | - | - | - | - |
| 26 | Check: Does the UE transmit a REGISTRATION REQUEST message on NGC Cell B with in 60secs? | --> | 5GMM: REGISTRATION REQUEST | 3 | P |
| - | EXCEPTION: Steps 27a1 to 27b3a1 describe behaviour that depends on events happening prior to their execution; the "lower case letter" identifies a step sequence that take place if a specific prior event takes place. | - | - | - | - |
| 27a1-27a16a1 | IF 5GS registration type is set as Initial Registration in step 26, THEN Steps 5 to 20a1 of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed. | - | - | - | - |
| 27b1-27b3a1 | IF 5GS registration type is set as Mobility Registration in step 26, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed. | - | - | - | - |
| Note 1: SS derives Local time zone, Universal time and local time zone, Network daylight saving time based on the time of test execution. Note 2: This could be done by an MMI/AT command. |  |  |  |  |  |

9.4.1.5.3.3	Specific message contents

Table 9.4.1.5.3.3-1: SIB19 for NGC Cell A (Preamble and all steps, Table 9.4.1.5.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| t-service-r17 | Time as applied in Universal time and local time zone IE at step 19 of Table 9.4.1.5.3.2-1 + 1min |  |  |
| satSwitchWithReSync-r18 SEQUENCE { |  |  |  |
| ntn-Config-r18 | FFS |  |  |
| t-ServiceStart-r18 | Time as applied in Universal time and local time zone IE at step 19 of Table 9.4.1.5.3.2-1 + 3mins |  |  |
| ssb-TimeOffset-r18 | FFS |  |  |
| } |  |  |  |
| } |  |  |  |
| Note 1: Value is calculated in multiples of 10 ms after 00:00:00 on Gregorian calendar date 1 January 1900. |  |  |  |

Table 9.4.1.5.3.3-2: REGISTRATION REQUEST (Steps 3 and 26, Table 9.4.1.5.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GMM capability |  |  |  |  |
| UN-PER (octet 9, bit 3) |  | '1'B | Unavailability period supported |  |
| Unavailability information |  | Not present or any allowed value |  |  |

Table 9.4.1.5.3.3-3: REGISTRATION ACCEPT (Steps 13, 22, 27a10 and 27b1, Table 9.4.1.5.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-7. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GS network feature support |  |  |  |
| UN-PER (octet 5, bit 8) | '1'B | Unavailability period supported |  |
| Maximum time offset | '1010 0001'B | 1 minute |  |
| Unavailability configuration | Not present |  | Steps 13, 27a10, 27b1 |
| Unavailability configuration |  |  | Step 22 |
| Unavailability configuration contents | '0000 0110'B | UE needs to report end of unavailability period |  |
| Unavailability period duration | ' 0011 1100 'B | 60secs |  |

Table 9.4.1.5.3.3-4: REGISTRATION REQUEST (Step 26, Table 9.4.1.5.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Unavailability information |  |  |  |
| All octets with the exception of octet 3, bits 1 to 3 | Not checked |  |  |
| Unavailability type (octet 3, bits 1 to 3) | '001'B | unavailability due to discontinuous coverage |  |

<End of modified section>