3GPP TSG-RAN5 Meeting #108 	R5-254851

# Bengaluru, India, 25th August 2025 – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.903 | CR | 1083 | rev | - | Current version: | 18.7.3 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TT analysis separation for NR-U and legacy SCell activation and deactivation tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM Europe Inc. - Spain |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_unlic-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The TT analysis for NR-U SCell activation and deactivation tests 10.3.3.1, 10.3.3.2, 10.3.3.3, 11.4.3.1, 11.4.3.2, 11.4.3.3, 13.2.2.1, 13.2.2.2, 13.2.2.3 is currently shared with legacy tests 4.5.3.1, 4.5.3.2, 4.5.3.3, 6.5.3.1, 6.5.3.2, 6.5.3.3. There have been a correction attempt (R5-252752) to some of the legacy tests, that has caused a conflict with the NR-U equivalent tests. This CR splits the TT analysis between these two WIs, so they can now be individually used. No meaningful technically content has been added, other than what is required to split the analysis. As a result, there is no need to update 38.533 due to this change. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Split the analysis of the above-mentioned NR-U tests from their equivalent legacy tests. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test tolerance analysis would be merged, which could lead to unnecessary conflicts. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Delete file: 38.533 4.5.3.1+4.5.3.2+6.5.3.1+6.5.3.2 TT v6.zip Add file: 38.533 4.5.3.1+4.5.3.2+6.5.3.1+6.5.3.2 TT v7.zip Add file: 38.533 10.3.3.1+10.3.3.2+10.3.3.3+11.4.3.1 TT.zip |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Delete file: 38.533 4.5.3.1+4.5.3.2+6.5.3.1+6.5.3.2 TT v6.zip

Add file: 38.533 4.5.3.1+4.5.3.2+6.5.3.1+6.5.3.2 TT v7.zip

Add file: 38.533 10.3.3.1+10.3.3.2+10.3.3.3+11.4.3.1 TT.zip

<< Start of Changes >>

# 8	Grouping of test cases defined in TS 38.533

Table 8-1: Grouping of FR1 test cases defined in Clauses 4, 4A, 6, 8, 16, 18 and 19 of TS 38.533

| Group | Test Case Numbers | .zip file name | Comments |
| --- | --- | --- | --- |
| SCell_Activation_01 | 4.5.3.1 4.5.3.2 4.5.3.3 6.5.3.1 6.5.3.2 6.5.3.3 | “38.533 4.5.3.1+4.5.3.2+6.5.3.1+6.5.3.2 TT v7.zip” | “2 Inter Frequency NR Cells, 3 time periods, Various number of sub-tests, No fading” |
| SCell_Activation_05 | 4.5.3.4 | “38.533 4.5.3.4 TT.zip” | “3 Cells with 2 Intra frequency and 3rd cell with Inter Frequency NR FR1 Cells, Various number of sub-tests, No fading” multiple unknown SCells in FR1 with single activation/deactivation command |
| SCell_Activation_02 | 4.5.3.5 6.5.3.4 | “38.533 4.5.3.5 +6.5.3.4 TT.zip” | “2 Inter Frequency NR Cells, 2 time periods, Various number of sub-tests, No fading” Direct Scell activation configurations |
| DCI_Based_BWP_Switch_02 | 4.5.6.3.1 6.5.6.3.1 | “38.533 4.5.6.3.1 TT.zip” | “2 NR cells, 3 time periods, No fading” DCI-based and Timer-based |
| RRC_Based_BWP_Switch_02 | 4.5.6.4.1 6.5.6.5.1 | “38.533 4.5.6.4.1 TT.zip” | “3 NR cells, 3 time periods, No fading” RRC-based |
| PSCell_Activation | 4.5.10.1 | “38.533 4.5.10.1 TT.zip” | 1 NR Cell, no fading |
| PSCell_Activation | 5.5.12.1 | “38.533 5.5.12.1 TT.zip” | 1 NR Cell, no fading |
| SCell_Activation_03 | 6.5.3.5 | “38.533 6.5.3.5 TT.zip” | “3 Cells with 2 Intra frequency and 3rd cell with Inter Frequency NR FR1 Cells, Various number of sub-tests, No fading” direct Scell activation configurations |
| SCell_Activation_04 | 4.5.3.6 4.5.3.7 6.5.3.10 6.5.3.11 | “38.533 4.5.3.6+4.5.3.7 +6.5.3.10+6.5.3.11 TT.zip” | “2 Inter Frequency NR Cells, 2 time periods, Various number of sub-tests, No fading” Fast Scell activation configurations |
| FastSCell_Activation | 7.5.3.14 | “38.533 7.5.3.14 TT.zip” | “2 Inter Frequency NR FR2 Cells, 2 time periods, No fading” |
| Intra_Freq_Meas_01 | 4.6.1.1 4.6.1.2 4.6.1.3 4.6.1.4 4.6.1.5 4.6.1.6 4.6.1.7 6.6.1.1 6.6.1.2 6.6.1.3 6.6.1.4 6.6.1.5 6.6.1.6 6.6.1.7 6.6.1.9 6.6.1.13 6.6.19.1 6.6.24.1 6.6.24.5 16.6.1.2 16.6.1.4 16.6.1.6 16.6.1.8 16.6.1.10 16.6.1.12 16.6.5.2 | “38.533 4.6.1.1+4.6.1.2+4.6.1.3+4.6.1.4 TT v6.zip” | “2 Intra Frequency NR Cells, 2 time periods, Various number of sub-tests, No fading” Note: The spreadsheet “16.6.1.4-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |
| Intra_Freq_Meas_02 | 6.6.1.12 | “38.533 6.6.1.12 TT.zip” | “2 Intra Frequency NR Cells, 2 time periods, No fading” |
| Intra_Freq_CGI_identification | 4.6.6.1 6.6.7.1 | “38.533 4.6.6.1 TT.zip” | “2 Intra Frequency NR Cells, 3 time periods, Various number of sub-tests, No fading” |
| Inter_RAT_E-UTRA_CGI_Identification | 16.6.5.4 6.6.7.2 | “38.533 16.6.5.4 TT_v2.zip” | “1 NR cell and 1 E-UTRA cell, 3 time periods, Various number of sub-tests, No fading” |
| Intra_Freq_MG_Enh | 6.6.17.1 6.6.17.2 | “38.533 6.6.17.1+6.6.17.2 TT.zip” | “2 Intra Frequency NR Cells, 3 time periods, Various number of sub-tests, No fading” |
| Intra_Freq_Meas_RedCap_1Rx_01 | 16.6.1.1 16.6.1.3 16.6.1.5 16.6.1.7 16.6.1.9 16.6.1.11 16.6.5.1 | “38.533 16.6.1.1+16.6.1.3+16.6.1.5+16.6.1.7 TT.zip” | “2 Intra Frequency NR Cells, 2 time periods, Various number of sub-tests, No fading” |
| Intra_Freq_HST_Meas_01 | 4.6.1.8 6.6.1.8 6.6.19.4 | “38.533 4.6.1.8+6.6.1.8 TT” | “3 NR cells on 2 channels (2 Intra Frequency NR Cells), 2 time periods, Various number of sub-tests, No fading” |
| Inter_Freq_Meas_01 | 4.6.2.1 4.6.2.2 4.6.2.5 4.6.2.6 4.6.2.9 6.6.2.1 6.6.2.2 6.6.2.5 6.6.2.6 6.6.2.9 6.6.2.10 6.6.2.11 6.6.2.12 6.6.2.13 6.6.2.14 6.6.19.2 6.6.24.2 6.6.24.3 6.6.24.4 16.6.2.2 16.6.2.4 16.6.2.6 16.6.2.8 16.6.2.10 16.6.2.12 | “4.6.2.1+4.6.2.2+4.6.2.5+4.6.2.6 TT v6.zip" | “2 Inter Frequency NR Cells, 2 time periods, Various number of sub-tests, No fading” Note: The spreadsheet “16.6.2.2-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations |
| Inter_Freq_Meas_02 | 6.6.2.15 | “38.533 6.6.2.15 TT.zip" | “2 Inter Frequency NR Cells, 2 time periods, Various number of sub-tests, No fading” |
| Inter_Freq_MG_Enh | 6.6.18.1 6.6.18.2 | “6.6.18.1+6.6.18.2 TT.zip" | “3 Inter Frequency NR Cells, 2 time periods, Various number of sub-tests, No fading” |
| Inter_Freq_MG_Enh_01 | 6.6.18.4 | “38.533 6.6.18.4 TT.zip" | “2 Inter Frequency NR Cells, 1 Intra Frequency NR Cell with PRS 2 time periods, Various number of sub-tests, No fading” |
| Intra_Reselection_01 | 6.1.1.1 16.1.1.2 | 38.533 6.1.1.1 + 16.1.1.2 TT | “2 Intra Frequency NR Cells, 3 time periods, No fading” The spreadsheet “16.1.1.2-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |
| Intra_Reselection_02 | 6.1.1.9 | 38.533 6.1.1.9 TT | “2 Intra Frequency NR Cells, 3 time periods, No fading” |
| Inter_Reselection_01 | 6.1.1.2 6.1.1.8 16.1.1.4 | 38.533 6.1.1.2 + 16.1.1.4 TT | “2 Inter Frequency NR Cells, 3 time periods, No fading” The spreadsheet “16.1.1.4-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |
| InterRAT_Higher_Reselection_01 | 6.1.2.1 16.1.2.2 | 38.533 6.1.2.1+16.1.2.2 | “1 E-UTRAN Cell, 1 NR Cells, 3 time periods, No fading” The spreadsheet “16.1.2.2-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |
| InterRAT_Lower_Reselection_01 | 6.1.2.2 6.1.2.3 6.1.2.5 16.1.2.4 16.1.2.6 | 38.533 6.1.2.2+6.1.2.3+6.1.2.5+16.1.2.4 | “1 E-UTRAN Cell, 1 NR Cells, 2 time periods, No fading” The spreadsheet “16.1.2.4-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |
| InterRAT_Lower_Reselection_02 | 6.1.2.4 | “38.533 6.1.2.4 TT.zip” | “1 E-UTRAN Cell, 1 NR Cells, 2 time periods, No fading” |
| InterRAT_Known_Handover_01_1Rx | 16.3.1.7 | ”38.533 16.3.1.7 TT.zip” | “1 E-UTRAN Cell, 1 NR Cells, 3 time periods, No fading” |
| InterRAT_Known_Handover_01 | 6.3.1.4 16.3.1.8 | ”38.533 6.3.1.4+16.3.1.8 TT v2.zip” | “1 E-UTRAN Cell, 1 NR Cells, 3 time periods, No fading” Note: The spreadsheet “16.3.1.8-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |
| InterRAT_Unknown_Handover_01_1Rx | 16.3.1.9 | ”38.533 16.3.1.9 TT.zip” | “1 E-UTRAN Cell, 1 NR Cells, 2 time periods, No fading” |
| InterRAT_Unknown_Handover_01 | 6.3.1.5 16.3.1.10 | “38.533 6.3.1.5+16.3.1.10 TT.zip” | “1 E-UTRAN Cell, 1 NR Cells, 2 time periods, No fading” Note: The spreadsheet “16.3.1.10-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |
| Intra_RRC_re-establishment_01 | 6.3.2.1.1 16.3.2.1.1 16.3.2.1.2 | “38.533 6.3.2.1.1+16.3.2.1.1+16.3.2.1.2 TT.zip” | “2 Intra Frequency NR Cells, 3 time periods, No fading” Note: The spreadsheet “16.3.2.1.2-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |
| Intra_SS-RSRP_Abs_Acc_01 | 4.7.1.1.1 6.7.1.1.1 16.7.1.2.1 19.6.1.1.1 | “38.533 4.7.1.1.1+6.7.1.1.1+16.7.1.2.1+19.6.1.1.1 TT.zip” | “2 Intra-Frequency NR Cells, 3 Sub-tests, periodic reporting, No fading” |
| Intra_SS-RSRP_Rel_Acc_01 | 4.7.1.1.2 6.7.1.1.2 16.7.1.2.2 19.6.1.1.2 | “38.533 4.7.1.1.2+6.7.1.1.2+16.7.1.2.2+19.6.1.1.2 TT.zip" | “2 Intra-Frequency NR Cells, 3 Sub-tests, periodic reporting, No fading” |
| Intra_CSI-RSRP_Abs_Acc_01 | 4.7.8.1 | “38.533 4.7.8.1 TT.zip” | “2 Intra-Frequency NR Cells, 3 Sub-tests, periodic reporting, No fading” |
| Intra_SS-RSRP_Abs_Acc_01_RedCap_1Rx | 16.7.1.1.1 | “38.533 16.7.1.1.1 TT.zip” | 2 Intra-Frequency NR Cells, 3 Sub-tests, periodic reporting, No fading” |
| Intra_SS-RSRP_Rel_Acc_01_RedCap_1Rx | 16.7.1.1.2 | 38.533 16.7.1.1.2 TT.zip | “2 Intra-Frequency NR Cells, 3 Sub-tests, periodic reporting, No fading” |
| Inter_RRC_re-establishment_01 | 6.3.2.1.2 16.3.2.1.3 16.3.2.1.4 | “38.533 6.3.2.1.2+16.3.2.1.3+16.3.2.1.4 TT.zip” | “2 Inter Frequency NR Cells, 3 time periods, No fading” Note: The spreadsheet “16.3.2.1.4-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |
| Inter_RRC_redirection_01 | 6.3.2.3.1 16.3.2.3.116.3.2.3.2 18.2.2.1 | “38.533 6.3.2.3.1+16.3.2.3.1+16.3.2.3.2+18.2.2.1 TT.zip” | “2 Inter Frequency NR Cells, 2 time periods, No fading” Note: The spreadsheet “16.3.2.3.2-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |
| InterRAT_RRC_redirection_01 | 6.3.2.3.2 16.3.2.3.3 16.3.2.3.4 | “38.533 6.3.2.3.2+16.3.2.3.3+16.3.2.3.4 TT.zip” | “1 E-UTRAN Cell, 1 NR Cells, 2 time periods, No fading” Note: The spreadsheet “16.3.2.3.4-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |
| RLM_InSync_01 | 4.5.1.2 4.5.1.4 6.5.1.2 6.5.1.4 4.5.1.6 4.5.1.8 6.5.1.6 6.5.1.8 16.5.1.3 16.5.1.4 16.5.1.7 16.5.1.8 16.5.1.11 16.5.1.12 16.5.1.15 16.5.1.16 | “38.533 4.5.1.2+4.5.1.4+6.5.1.2+6.5.1.4 TT v4.zip” | “1 NR Cell (1 E-UTRA Cell for NSA case), 1 sub-test, Fading, 5 Time Periods” Note: The spreadsheet “16.5.1.4-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |
| RLM_Out_of_Sync_01 | 4.5.1.1 4.5.1.3 6.5.1.1 6.5.1.3 4.5.1.5 4.5.1.7 6.5.1.5 6.5.1.7 16.5.1.1 16.5.1.2 16.5.1.5 16.5.1.6 16.5.1.9 16.5.1.10 16.5.1.13 16.5.1.14 | “38.533 4.5.1.1+4.5.1.3+6.5.1.1+6.5.1.3 TT v5.zip” | “1 NR Cell (1 E-UTRA Cell for NSA case), 1 sub-test, Fading, 3 Time Periods” Note: The spreadsheet “16.5.1.2-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |
| RLM_InSync_02 | 6.5.1.16 | “38.533 6.5.1.16 TT.zip” | “1 NR Cell, Fading, 5 Time Periods” |
| RLM_Out_of_Sync_02 | 6.5.1.13 | “38.533 6.5.1.13 TT.zip” | “1 NR Cell, Fading, 3 Time Periods” |
| RLM_Out_of_Sync_03 | 6.5.1.14 | “38.533 6.5.1.14 TT.zip” | “1 NR Cell, Fading, 3 Time Periods” |
| UE_Timing_Advance_01 | 4.4.3.1 6.4.3.1 10.2.2.1 11.3.2.1 16.4.3.1 16.4.3.2 | “38.533 4.4.3.1+6.4.3.1+16.4.3.1+16.4.3.2 TT_v2.zip” | “1 NR Cell (1 E-UTRA Cell for NSA case), No Fading” |
| UE Transmit_Timing_01 | 4.4.1.1 6.4.1.1 10.2.1.1 11.3.1.1 16.4.1.1 16.4.1.2 | “38.533 4.4.1.1+6.4.1.1+16.4.1.1+16.4.1.2 TT_v2.zip” | “1 NR Cell (1 E-UTRA Cell for NSA case), 2 sub-tests, No Fading” |
| RRC_reconfiguration_delay_01 | 4.5.4.1 6.5.4.1 | “38.533 4.5.4.1+6.5.4.1 TT_v2.zip” | “1 E-UTRA Cell, 2 NR Cells”, 3 Time Periods, No Fading” |
| Intra_Freq_HO_Known_Target | 6.3.1.1 16.3.1.2 19.2.1.1 | “38.533 6.3.1.1+16.3.1.2 TT.zip” | “2 Intra-Freq NR Cells, 3 Time Periods, No Fading” Note: The spreadsheet “16.3.1.2-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |
| Intra_Freq_HO_Known_Target_1Rx_RedCap | 16.3.1.1 | “38.533 16.3.1.1 TT_v2.zip” | “2 Intra-Freq NR Cells, 3 Time Periods, No Fading” |
| Intra_Freq_HO_Unknown_Target | 6.3.1.2 16.3.1.3 16.3.1.4 | “38.533 6.3.1.2+16.3.1.4 TT.zip” | “2 Intra-Freq NR Cells, 2 Time Periods, No Fading” Note: The spreadsheet “16.3.1.4-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |
| Intra_Freq_HO_Unknown_Target_01 | 6.3.1.18 | “38.533 6.3.1.18 TT.zip” | “2 Intra-Freq NR Cells, 2 Time Periods, No Fading” |
| Inter_Freq_HO | 6.3.1.3 16.3.1.5 16.3.1.6 19.2.1.2 | “38.533 6.3.1.3+16.3.1.6 TT.zip” | “2 Inter-Freq NR Cells, 2 Time Periods, No Fading” Note: The spreadsheet “16.3.1.6-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |
| InterRAT_Meas_01 | 6.6.3.1 6.6.3.2 6.6.19.3 | “38.533 6.6.3.1+6.6.3.2 TT v3.zip” | “1 E-UTRAN Cell, 1 NR Cell, 2 time periods |
| InterRAT_Meas_MG_Enh | 6.6.18.3 | “38.533 6.6.18.3 TT.zip” | “1 E-UTRAN Cell, 2 Inter-Frequency NR Cells, 2 time periods |
| Interruption_Transition_01 | 4.5.2.1 4.5.2.2 | “38.533 4.5.2.1+4.5.2.2 TT.zip” | “1 E-UTRAN Cell, 1 NR Cells, 1 time period, No fading” |
| Interruption_meas_NR_SCC_01 | 4.5.2.3 4.5.2.4 6.5.2.1 | “38.533 4.5.2.3+4.5.2.4+6.5.2.1 TT.zip” | “1 E-UTRAN Cell, 2 NR Cells (2 NR Cells for SA case), 1 time period, No fading” |
| Interruption_meas_NR_SCC_01 | 4.5.2.5 4.5.2.6 | “38.533 4.5.2.5+4.5.2.6 TT.zip” | “2 E-UTRAN Cell, 1 NR Cells, 1 time period, No fading” |
| Interruption_meas_NR_PSCell | 4.5.2.10 | “38.533 4.5.2.10 TT.zip” | “1 E-UTRAN Cell, 1 NR Cells, 1 time period, No fading” |
| Inter_SS-RSRP_Abs_Acc_01 | 4.7.1.2.1 6.7.1.2.1 16.7.1.4.1 19.6.1.2.1 | ”38.533 4.7.1.2.1+6.7.1.2.1+16.7.1.4.1+19.6.1.2.1 TT.zip” | “2 Inter-Frequency NR Cells, periodic reporting, No fading” |
| Inter_SS-RSRP_Rel_Acc_01 | 4.7.1.2.2 6.7.1.2.2 16.7.1.4.2 19.6.1.2.2 | "38.533 4.7.1.2.2+6.7.1.2.2+16.7.1.4.2+19.6.1.2.2 TT.zip” | “2 Inter-Frequency NR Cells, periodic reporting, No fading” |
| Inter_SS-RSRP_Abs_Acc_01_RedCap_1Rx | 16.7.1.3.1 | 38.533 16.7.1.3.1 TT.zip | “2 Inter-Frequency NR Cells, periodic reporting, No fading” |
| Inter_SS-RSRP_Rel_Acc_01_RedCap_1Rx | 16.7.1.3.2 | 38.533 16.7.1.3.2 TT.zip | “2 Inter-Frequency NR Cells, periodic reporting, No fading” |
| Intra_SS-SINR_Acc_01 | 4.7.3.1 6.7.3.1 16.7.3.1 16.7.3.2 19.6.3.1 | “38.533 4.7.3.1+6.7.3.1+16.7.3.1+16.7.3.2 TT_v2.zip” | “2 Intra-Frequency NR Cells, periodic reporting, No fading” |
| Inter_CSI-RSRP_Abs_Acc_01 | 4.7.8.2 | “38.533 4.7.8.2 TT.zip” | “2 Inter-Frequency NR Cells, periodic reporting, No fading” |
| SSB_Based_L1-RSRP-Meas | 4.6.4.1 4.6.4.2 4.6.4.5 6.6.4.1 6.6.4.2 6.6.4.5 6.6.4.6 16.6.4.2 16.6.4.4 | "38.533 4.6.4.1+4.6.4.2+6.6.4.1+6.6.4.2 TT v4.zip" | “1 NR Cell (1 E-UTRA Cell for NSA case), 2 time periods, No fading” Note: The spreadsheet “16.6.4.2-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |
| SSB_Based_L1-RSRP-Meas_RedCap_1Rx | 16.6.4.1 16.6.4.3 | “38.533 16.6.4.1+16.6.4.3 TT.zip” | “1 NR Cell, 2 time periods, No fading” |
| Intra-frequency_SSB_Based_L1-RSRP-Meas | 6.6.26.1 | “38.533 6.6.26.1 TT.zip” | “1 NR serving cell and 1 NR intra frequency cell, various number of sub-tests, 2 time periods, periodic reporting, no fading” |
| Inter-frequency_SSB_Based_L1-RSRP-Meas_01 | 6.6.27.1 | “38.533 6.6.27.1 TT.zip” | “1 NR serving cell and 1 NR inter frequency cell, 2 time periods, periodic reporting, no fading” |
| Inter-frequency_SSB_Based_L1-RSRP-Meas_02 | 6.6.28.1 | “38.533 6.6.28.1 TT.zip” | “1 NR serving cell and 1 NR inter frequency cell, various number of sub-tests, 2 time periods, periodic reporting, no fading” |
| CSI-RS_Based_L1-RSRP-Meas | 4.6.4.3 4.6.4.4 6.6.4.3 6.6.4.4 16.6.4.6 16.6.4.8 | “38.533 4.6.4.3+4.6.4.4+6.6.4.3+6.6.4.4 TT v6.zip” | “1 NR Cell (1 E-UTRA Cell for NSA case), one time period, No fading” Note: The spreadsheet “16.6.4.6-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |
| CSI-RS_Based_L1-RSRP-Meas_RedCap_1Rx | 16.6.4.5 16.6.4.7 | “38.533 16.6.4.5+16.6.4.7 TT.zip” | “1 NR Cell, one time period, No fading” |
| CSI-RS_WithoutIMR_L1-SINR-Meas | 4.6.7.1 6.6.8.1 | “38.533 4.6.7.1+6.6.8.1 TT.zip” | “1 NR Cell (1 E-UTRA Cell for NSA case), one time period, No fading” |
| SSB_WithCSI-IM_L1-SINR-Meas | 4.6.7.2 7.6.3.6 | “38.533 4.6.7.2 TT.zip” | “1 E-UTRA Cell, 1 NR Cell, 2 time periods, No fading” |
| CSI-RS_WithCSI-IM_L1-SINR-Meas | 4.7.7.3.1 6.7.9.3.1 | “38.533 4.7.7.3.1+6.7.9.3.1 TT.zip” | “1 NR Cell (1 E-UTRA Cell for NSA case), one time period, No fading” |
| CSI-RS_WithCSI-IM_L1-SINR-Meas | 4.7.7.3.2 6.7.9.3.2 | “38.533 4.7.7.3.2+6.7.9.3.2 TT.zip” | “1 NR Cell (1 E-UTRA Cell for NSA case), one time period, No fading” |
| CSI-RS_Based_L1-SINR-Meas | 4.7.7.1.1 6.7.7.9.1 | “38.533 4.7.7.1.1+6.7.9.1.1 TT.zip” | “1 NR Cell (1 E-UTRA Cell for NSA case), one time period, No fading” |
| L1-SINR_Accuracy_3 | 7.7.6.2 | “38.533 7.7.6.2 TT.zip” | 1 NR FR2 Cell, 2 SSB and 2 CSI-RS, 1 subtests, 1 AoA in Rx peak and rough beam |
| L1-SINR_Accuracy_4 | 7.7.6.3 | “38.533 7.7.6.3 TT.zip” | 1 NR FR2 Cell, 2 CSI-RS and 2 CSI-IM, 1 subtest, 1 AoA in Rx peak and rough beam |
| CSI-RS_WithNZP_L1-SINR-Meas | 4.6.7.3 | “38.533 4.6.7.3 TT.zip” | “1 E-UTRA Cell, 1 NR Cell, one time period, No fading” |
| Intra_SS-RSRQ_Acc_01 | 4.7.2.1 6.7.2.1 16.7.2.2 19.6.2.1 | “38.533 4.7.2.1+6.7.2.1+16.7.2.2+19.6.2.1 TT.zip” | “2 Intra-Frequency NR Cells, periodic reporting, No fading” |
| Intra_SS-RSRQ_Acc_01_RedCap_1Rx | 16.7.2.1 | “38.533 16.7.2.1 TT.zip” | “2 Intra-Frequency NR Cells, periodic reporting, No fading” |
| Inter_SS-RSRQ_Abs_Acc_01 | 4.7.2.2.1 6.7.2.2.1 19.6.2.2.1 | “38.533 4.7.2.2.1+6.7.2.2.1+16.7.2.4.1+19.6.2.2.1 TT.zip” | “2 Inter-Frequency NR Cells, periodic reporting, No fading” |
| Inter_SS-RSRQ_Rel_Acc_01 | 4.7.2.2.2 6.7.2.2.2 19.6.2.2.2 | “38.533 4.7.2.2.2+6.7.2.2.2+16.7.2.4.2+19.6.2.2.2 TT.zip” | “2 Inter-Frequency NR Cells, periodic reporting, No fading” |
| Inter_SS-SINR_Abs_Acc_01 | 4.7.3.2.1 6.7.3.2.1 16.7.3.4.1 19.6.3.2.1 | “38.533 4.7.3.2.1+6.7.3.2.1+16.7.3.4.1+19.6.3.2.1 TT.zip” | “2 Inter-Frequency NR Cells, periodic reporting, No fading” |
| Inter_SS-SINR_Rel_Acc_01 | 4.7.3.2.2 6.7.3.2.2 16.7.3.4.2 19.6.3.2.2 | “38.533 4.7.3.2.2+6.7.3.2.2+16.7.3.4.2+19.6.3.2.2 TT.zip” | “2 Inter-Frequency NR Cells, periodic reporting, No fading” |
| Inter_RAT_SS-RSRP_LTE_Serving_01 | 8.5.2.1.1.1 | “38.533 8.5.2.1.1.1 TT.zip” | 1 NR Cell, 1 LTE serving cell, periodic SS-RSRP reporting, No fading |
| Inter_RAT_SS-RSRQ_LTE_Serving_01 | 8.5.2.2.1 | “38.533 8.5.2.2.1 TT.zip” | 1 NR Cell, 1 LTE serving cell, periodic SS-RSRQ reporting, No fading |
| Inter_RAT_SS-SINR_LTE_Serving_01 | 8.5.2.3.1 | “38.533 8.5.2.3.1 TT.zip” | 1 NR Cell, 1 LTE serving cell, periodic SS-SINR reporting, No fading |
| L1-RSRP_Abs_Acc_01 | 4.7.4.1.1 6.7.4.1.1 4.7.4.2.1 6.7.4.2.1 16.7.4.2.1 16.7.4.4.1 | “38.533 4.7.4.1.1+4.7.4.2.1+6.7.4.1.1+6.7.4.2.1 TT v3.zip” | 1 NR Cell, periodic L1-RSRP reporting, No fading |
| L1-RSRP_Abs_Acc_01_RedCap_1Rx | 16.7.4.1.1 16.7.4.3.1 | “38.533 16.7.4.1.1 TT.zip” | 1 NR Cell, periodic L1-RSRP reporting, No fading |
| Inter-frequency_L1-RSRP_Abs_Acc | 6.7.17.1.1 | “38.533 6.7.17.1.1 TT.zip” | “1 NR serving cell and 1 NR inter frequency cell, periodic L1-RSRP reporting, no fading” |
| L1-RSRP_Rel_Acc_01 | 4.7.4.1.2 6.7.4.1.2 4.7.4.2.2 6.7.4.2.2 16.7.4.4.2 | “38.533 4.7.4.1.2+4.7.4.2.2+6.7.4.1.2+6.7.4.2.2 TT v3.zip” | 1 NR Cell with 2 Beams, periodic L1-RSRP reporting, No fading |
| L1-RSRP_Rel_Acc_01_RedCap_1Rx | 16.7.4.1.2 16.7.4.3.2 | “38.533 16.7.4.1.2 TT.zip” | 1 NR Cell with 2 Beams, periodic L1-RSRP reporting, No fading |
| SSB_Based_BFR | 4.5.5.1 4.5.5.2 6.5.5.1 6.5.5.2 16.5.2.2 16.5.2.4 | “38.533 4.5.5.1+4.5.5.2+6.5.5.1+6.5.5.2 TT v4.zip” | “1 NR Cell (1 E-UTRA Cell for NSA case), 5 time periods, Fading” Note: The spreadsheet “16.5.2.2-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |
| TRP specific SSB_Based_BFR | 4.5.5.7 | 38.533 4.5.5.7 TT.zip | 1 NR Cell 1 E-UTRA Cell PCell, 5 time periods, Fading” |
| SSB_Based_BFR_RedCap_1Rx_01 | 16.5.2.116.5.2.3 | “38.533 16.5.2.3 TT.zip” | “1 NR Cell (1 E-UTRA Cell for NSA case), 5 time periods, Fading” |
| CSI-RS_Based_BFR | 4.5.5.3 4.5.5.4 6.5.5.3 6.5.5.4 16.5.2.6 16.5.2.8 | “38.533 4.5.5.3+4.5.5.4+6.5.5.3+6.5.5.4 TT v3.zip” | “1 NR Cell (1 E-UTRA Cell for NSA case), 5 time periods, Fading” Note: The spreadsheet “16.5.2.6-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |
| CSI-RS_Based_BFR_RedCap_1Rx | 16.5.2.5 16.5.2.7 | “38.533 16.5.2.5+16.5.2.7 TT.zip” | “1 NR Cell, 5 time periods, Fading” |
| TRP specific CSI-RS_Based_BFR | 6.5.5.7 | “38.533 6.5.5.7 TT.zip” | “1 NR Cell (1 E-UTRA Cell for NSA case), 5 time periods, Fading” |
| CSI-RS_Based_BFD_SSB_Based_FR | 4.5.5.5 4.5.5.6 6.5.5.5 6.5.5.6 | For SSB refer to “38.533 4.5.5.1+4.5.5.2+6.5.5.1+6.5.5.2 TT v2.zip” For CSI-RS refer to “38.533 4.5.5.3+4.5.5.4+6.5.5.3+6.5.5.4 TT v2.zip” | “2 NR Cell (1 E-UTRA Cell for NSA case), 5 time periods, Fading” |
| TRP specific CSI-RS_Based_BFD_SSB_Based_FR | 4.5.5.8 | 38.533 4.5.5.8 TT.zip | “2 NR Cell (1 E-UTRA Cell for NSA case), 5 time periods, Fading” |
| DCI_Based_BWP_Switch | 4.5.6.1.1 4.5.6.1.2 6.5.6.1.1 6.5.6.1.2 16.5.3.1.1 16.5.3.1.2 | “38.533 4.5.6.1.1+4.5.6.1.2+6.5.6.1.1+6.5.6.1.2 TT v2.zip“ | “1 NR Cell (2NR Cells for Scell case, 1 E-UTRA Cell for NSA case), 3 time periods, No fading” |
| RRC_Based_BWP_Switch_01 | 4.5.6.2.1 6.5.6.2.1 16.5.3.2.1 16.5.3.2.2 | “38.533 4.5.6.2.1+6.5.6.2.1+16.5.3.2.1+16.5.3.2.2 TT.zip” | “1 NR Cell (1 E-UTRA Cell for NSA case), 3 time periods, No fading” |
| Intra_RRC_re-establishment_without_timing | 6.3.2.1.3 16.3.2.1.5 16.3.2.1.6 | “38.533 6.3.2.1.3+16.3.2.1.5+16.3.2.1.6 TT.zip” | “2 Intra Frequency NR Cells, 3 time periods, No fading” Note: The spreadsheet “16.3.2.1.6-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |
| InterRAT_re-selection_LTE_Serving | 8.2.1.1 | “38.533 8.2.1.1 TT.zip” | “1 NR Cell, 1 LTE serving cell, 3 time periods No fading” |
| InterRAT_HO_LTE_Serving | 8.3.1.1 18.2.1.1 | “38.533 8.3.1.1+18.2.1.1 TT.zip” | “1 NR Cell, 1 LTE serving cell, 3 time periods No fading” Note: The spreadsheet “18.2.1.1-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |
| InterRAT_SFTD_Meas_LTE_Serving | 8.4.1.1 8.4.1.2 | “38.533 8.4.1.1+8.4.1.2 TT.zip” | “1 NR Cell, 1 LTE serving cell, 1 time period No fading” |
| InterRAT_Meas_LTE_Serving | 8.4.2.1 8.4.2.2 8.4.2.3 8.4.2.4 18.3.1.1 18.3.1.2 18.3.1.3 18.3.1.4 | “38.533 8.4.2.1+8.4.2.2+8.4.2.3+8.4.2.4 TT v3.zip” | “1 NR Cell, 1 LTE serving cell, 2 time periods, No Fading” |
| PSCell_Addition | 4.5.7.1 | “38.533 4.5.7.1 TT v2.zip” | 1 NR Cell, no fading |
| PSCell_Addition_01 | 4.5.11.1 | “38.533 4.5.11.1 TT.zip” | 1 NR Cell, no fading |
| SFTD_Accuracy | 4.7.5.1 | “38.533 4.7.5.1 TT.zip” | 1 E-UTRA Cell, 1 NR Cell, no fading |
| SSB_WithNZP-IMR_L1-SINR-Meas | 6.6.8.2 | “38.533 6.6.8.2 TT.zip” | “1 NR Cell, 2 time periods, No fading” |
| CSI-RS_WithCSI-IM_L1-SINR-Meas | 6.6.8.3 | “38.533 6.6.8.3 TT.zip” | “1 NR Cell, one time period, No fading” |
| RLM_Out_of_Sync_PowerSavingEnh | 4.5.1.9 6.5.1.9 | 38.533 4.5.1.9+6.5.1.9 TT.zip | 1 NR Cell (1 E-UTRA Cell for NSA case), 1 sub-test, Fading, 3 Time Periods |
| iRAT_E-UTRA_RSRP_Accuracy | 6.7.5.1 | “38.533 6.7.5.1 TT.zip” | 1 E-UTRA Cell, 1 NR Cell, no fading |
| iRAT_E-UTRA_RSRQ_Accuracy | 6.7.6.1 | “38.533 6.7.6.1 TT.zip” | 1 E-UTRA Cell, 1 NR Cell, no fading |
| iRAT_E-UTRA_RS-SINR_Accuracy | 6.7.7.1 | “38.533 6.7.7.1 TT.zip” | 1 E-UTRA Cell, 1 NR Cell, no fading |
| InterRAT_SFTD_Meas_Accuracy_LTE_Serving | 8.5.1.1 | “38.533 8.5.1.1 TT.zip” | 1 E-UTRA Cell, 1 NR Cell, no fading |
| Inter_Freq_HO_DAPS | 6.3.1.9 6.3.1.10 | “38.533 6.3.1.9+6.3.1.10 TT.zip” | “2 Inter-Freq NR Cells, 5 Time Periods, No Fading” |
| Intra_Reselection_Low_Mobility | 6.1.1.3 16.1.1.6 | 38.533 6.1.1.3+16.1.1.6 TT | “2 Intra Frequency NR Cells, 2 time periods, No fading” The spreadsheet “16.1.1.6-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |
| Intra_Reselection_Not_cell_edge | 6.1.1.4 | “38.533 6.1.1.4 TT.zip” | “2 Intra Frequency NR Cells, 2 time periods, No fading” |
| Inter_Reselection_Low_mobility | 6.1.1.5 16.1.1.8 | 38.533 6.1.1.5+16.1.1.8 TT | “2 Inter Frequency NR Cells, 2 time periods, No fading” The spreadsheet “16.1.1.8-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |
| Inter_Reselection_Not_cell_edge | 6.1.1.6 | “38.533 6.1.1.6 TT.zip” | “2 Inter Frequency NR Cells, 2 time periods, No fading” |
| Intra_Reselection_HST | 6.1.1.7 | “38.533 6.1.1.7 TT.zip” | “2 Intra Frequency NR Cells, 3 time periods, No fading” |
| InterRAT_HO_UTRA_FDD | 6.3.1.6 | “38.533 6.3.1.6 TT.zip” | “1 UTRA Cell, 1 NR Cell, 3 time periods, No fading” |
| InterRAT_Meas_UTRA_FDD | 6.6.5.1 | “38.533 6.6.5.1 TT.zip” | “1 UTRA Cell, 1 NR Cell, 2 time periods, No fading” |
| InterRAT_Meas_HST | 6.6.3.3 | “38.533 6.6.3.3 TT.zip” | 1 E-UTRA Cell, 1 NR Cell, no fading |
| InterRAT_re-selection_LTE_Serving_HST | 8.2.1.2 | “38.533 8.2.1.2 TT.zip” | 1 E-UTRA Cell, 1 NR Cell, no fading |
| InterRAT_Meas_LTE_Serving_HST | 8.4.2.9 | “38.533 8.4.2.9 TT.zip” | 1 E-UTRA Cell, 1 NR Cell, no fading |
| Intra_Freq_HO_DAPS | 6.3.1.7 6.3.1.8 | “38.533 6.3.1.7+6.3.1.8 TT.zip” | “2 Intra-Freq NR Cells, 5 Time Periods, No Fading” |
| Inter_Freq_HO_DAPS | 6.3.1.9 6.3.1.10 | “38.533 6.3.1.9+6.3.1.10 TT.zip” | “2 Inter-Freq NR Cells, 5 Time Periods, No Fading” |
| Inter_Freq_HO_DAPS | 6.3.1.11 6.3.1.12 | “38.533 6.3.1.11+6.3.1.12 TT.zip” | “2 Inter-band Inter-Freq NR Cells, 5 Time Periods, No Fading” |
| Intra_Freq_CHO | 6.3.3.1 6.3.3.6 | “38.533 6.3.3.1 TT.zip” | “2 Intra-Freq NR Cells, 2 Time Periods, No Fading” |
| Intra_Freq_CHO_01 | 6.3.3.3 | “38.533 6.3.3.3 TT.zip” | “4 Intra-Freq NR Cells, 2 Time Periods, No Fading” |
| Intra_Freq_CHO_02 | 6.3.3.4 | “38.533 6.3.3.4 TT.zip” | “4 Intra-Freq NR Cells, 3 Time Periods, No Fading” |
| Inter_Freq_CHO | 6.3.3.2 | “38.533 6.3.3.2 TT.zip” | “2 Inter-Freq NR Cells, 2 Time Periods, No Fading” |
| Inter_Freq_NES_CHO | 6.3.3.7 | “38.533 6.3.3.7 TT.zip” | “2 Inter-Freq NR Cells, 2 Time Periods, No Fading” |
| Intra_Freq_LTM_cell_switch_RACHbased | 6.3.4.1 | “38.533 6.3.4.1TT.zip” | “2 Intra-Freq NR Cells, 4 time periods, No Fading” |
| Inter_Freq_LTM_cell_switch_RACHbased | 6.3.4.2 | “38.533 6.3.4.2 TT.zip” | “2 Inter-Freq NR Cells, |
| Intra_Freq_LTM_cell_switch_RACHless | 6.3.4.3 6.3.4.4 | “38.533 6.3.4.3 TT_v2.zip” | “2 Intra-Freq NR Cells, 5 Time Periods, No Fading” |
| Intra_Freq_CLI_SRS-RSRP | 4.6.5.1 6.6.6.1 | “38.533 4.6.5.1+6.6.6.1 TT.zip” | “1 E-UTRA Cell (clause 4 test only), 1 NR Cell, 1 Virtual UE, 2 Time Periods, no fading” |
| Intra_Freq_CLI_RSSI | 4.6.5.2 | “38.533 4.6.5.2 TT.zip” | “1 E-UTRA Cell, 1 NR Cell, 2 Time Periods, no fading” |
| Intra_Freq_CLI_SRS-RSRP_Accuracy | 4.7.6.1 6.7.8.1 | “38.533 4.7.6.1+6.7.8.1 TT.zip” | “1 E-UTRA Cell (clause 4 test only), 1 NR Cell, 1 Virtual UE, 1 Time Period, no fading” |
| DL_Interruption_UL_Switching_SA_01 | 6.5.7.1 6.5.7.2 6.5.7A.1 6.5.7A.2 6.5.7B.1 6.5.7B.2 6.5.7C.1 6.5.7C.2 6.5.7D.1 6.5.7D.2 6.5.7D.3 6.5.7D.4 4.5.8.1 | “38.533 6.5.7.1+6.5.7.2+6.5.7A.1+6.5.7A.2 TT_v2.zip” | “2 NR Cells, 1 time period, no fading” |
| PRACH_01_1RX | 16.3.2.2.1 16.3.2.2.3 16.3.2.2.5 16.3.2.2.7 | “38.533 16.3.2.2.1+16.3.2.2.3+16.3.2.2.5+16.3.2.2.7 TT v2” | “1 NR Cell, PRACH measurements, no fading” |
| PRACH_01 | 4.3.2.2.1 4.3.2.2.2 6.3.2.2.1 6.3.2.2.2 16.3.2.2.2 16.3.2.2.4 | “38.533 4.3.2.2.1+4.3.2.2.2+6.3.2.2.1+6.3.2.2.2 v2 TT.zip” | “1 NR Cell, PRACH measurements, no fading” Note: The spreadsheet “16.3.2.2.2-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |
| PRACH_02 | 4.3.2.2.3 4.3.2.2.4 6.3.2.2.3 6.3.2.2.4 16.3.2.2.6 16.3.2.2.8 | “38.533 4.3.2.2.3+4.3.2.2.4+6.3.2.2.3+6.3.2.2.4 TT v4.zip” | “1 NR Cell, PRACH measurements, no fading” Note: The spreadsheet “16.3.2.2.6-3” only applies to 30kHz SCS + 20MHz CBW RedCap test configurations. |
| Idle_CADC_meas | 6.6.9.1 | “38.533 6.6.9.1 TT.zip” | “2 NR Cells, 5 time periods, no fading” |
| CBW_change | 6.5.8.1 16.5.4.1 16.5.4.2 | “38.533 6.5.8.1+16.5.4.1+16.5.4.2 TT.zip” | “1 NR Cell, 1 time period, no fading” |
| NE-DC PSCell addition | 4A.1.1.1 | “38.533 4A.1.1.1 TT.zip” | “1 E-UTRA Cell, no fading” |
| NE-DC SFTD accuracy | 4A.2.1.1 | “38.533 4A.2.1.1 TT.zip” | “1 NR Cell, 1 E-UTRA Cell, no fading” |
| InterRAT_Idle_CADC_meas | 6.6.15.1 | “38.533 6.6.15.1 TT.zip” | “1 NR Cell, 1 E-UTRA Cell, 3 time periods, no fading” |
| InterRAT_LTE_Idle_CADC_meas | 8.2.2.1 | “38.533 8.2.2.1 TT.zip” | “1 E-UTRA Cell, 1 NR Cell, 3 time periods, no fading” |
| UE_Rx_Tx_difference_PDC_01 | 6.6.20.1 6.6.21.1 | “38.533 6.6.20+6.6.21 TT.zip” | “1 NR Cell, 2 time periods, no fading” |
| Inter_Reselection_RedCap_1Rx | 16.1.1.1 | “38.533 16.1.1.1 TT_v2.zip” | “2 Intra Frequency NR Cells, 3 time periods, No fading, 1Rx RedCap UE” |
| Inter_Reselection_RedCap_1Rx | 16.1.1.3 | 38.533 16.1.1.3 TT | “2 Inter Frequency NR Cells, 3 time periods, No fading, 1Rx RedCap UE” |
| Intra_Reselection_Low_Mobility_RedCap_1Rx | 16.1.1.5 | “38.533 16.1.1.5 TT_v2.zip” | “2 Intra Frequency NR Cells, 2 time periods, No fading, 1Rx RedCap UE” |
| Inter_Reselection_Low_mobility_RedCap_1Rx | 16.1.1.7 | 38.533 16.1.1.7 TT | “2 Inter Frequency NR Cells, 2 time periods, No fading, 1Rx RedCap UE” |
| InterRAT_Higher_Reselection_RedCap_1Rx | 16.1.2.1 | 38.533 16.1.2.1 TT | “1 E-UTRAN Cell, 1 NR Cells, 3 time periods, No fading, 1Rx RedCap UE” |
| InterRAT_Lower_Reselection_RedCap_1Rx | 16.1.2.3 | 38.533 16.1.2.3 TT | “1 E-UTRAN Cell, 1 NR Cells, 2 time periods, No fading, 1Rx RedCap UE” |
| InterRAT_Lower_Reselection_RedCap_Relaxed_1Rx | 16.1.2.5 | 38.533 16.1.2.5 TT | “1 E-UTRAN Cell, 1 NR Cells, 2 time periods, No fading, 1Rx RedCap UE” |
| Inter_Freq_Meas_01 | 16.6.2.1 16.6.2.3 16.6.2.5 16.6.2.7 16.6.2.9 16.6.2.11 | 38.533 16.6.2.1+16.6.2.3+16.6.2.5+16.6.2.7 TT | “2 Inter Frequency NR Cells, 2 time periods, Various number of sub-tests, No fading, 1Rx RedCap UE” |
| CG-SDT | 6.2.1 | 38.533 6.2.1+16.6.1.2 TT | “1 NR Cell, 6 time periods, no fading” Note: the spreadsheet "16.2.1.2-3" only applies to RedCap 30kHz+20MHz test configuration |
| CG-SDT_1Rx | 16.2.1.1 | “38.533 16.2.1.1 TT.zip” | “1 NR Cell, 6 time periods, no fading” |
| Inter-RAT_Meas_RedCap_1Rx_01 | 16.6.3.1 16.6.3.3 | “38.533 16.6.3.1+16.6.3.3 TT_v2.zip” | 1 E-UTRA Cell, 1 NR Cell, no fading, 1Rx RedCap UE |
| Inter-RAT_Meas_RedCap_1Rx_02 | 16.6.3.2 16.6.3.4 | “38.533 16.6.3.2+16.6.3.4 TT_v2.zip” | 1 E-UTRA Cell, 1 NR Cell, no fading, 2Rx RedCap UE |
| UE_Specific_CBW_Change | 4.5.9.1 | “38.533 4.5.9.1 TT.zip” | “1 E-UTRA Cell and 1 NR Cell, 1 time period, no fading” |
| Intra-LTM_PDCCH_Order | 6.3.2.4.1 6.3.2.4.3 | “38.533 6.3.2.4.1+6.3.2.4.3 TT.zip” | “2 Intra Frequency NR Cells, 2 time periods, Various number of sub-tests, No fading” |
| Inter-LTM_PDCCH_Order | 6.3.2.4.2 | “38.533 6.3.2.4.2 TT.zip” | “2 Inter Frequency NR Cells, 2 time periods, Various number of sub-tests, No fading” |
| InterRAT_re-selection_LTE_Serving_RedCap | 18.1.1.1 | “38.533 18.1.1.1 TT.zip” | “1 NR Cell, 1 LTE serving cell, 3 time periods, no fading, 2Rx RedCap UE” |
| SSBless_SCell_Activation | 4.5.3.12 6.5.3.16 | “38.533 4.5.3.12+6.5.3.16 TT.zip” | “2 Inter Frequency NR Cells, 2 time periods, No fading” |
| SSBless_SCell_Activation_01 | 4.5.3.11 | “38.533 4.5.3.11 TT.zip” | “2 Inter Frequency NR Cells, 2 time periods, No fading” |

<< Skipped tables >>

Table 8-4: Grouping of FR1 NR shared spectrum test cases defined in Clauses 10, 11, 12 and 13 of TS 38.533

| Group | Test Case Numbers | .zip file name | Comments |
| --- | --- | --- | --- |
| NR-U_EN-DC_HO_01 | 10.1.2 | “38.533 10.1.2 TT.zip” | “2 intra-frequency E-UTRAN Cells, 2 intra-frequency NR Cells under CCA, 1 sub-test, no fading, 3 Time Periods,” |
| NR-U_RLM_Out_of_Sync_01 | 10.3.1.2 11.4.1.2 | “38.533 10.3.1.2+11.4.1.2 TT.zip” | “1 NR Cell under CCA (1 E-UTRA Cell for NSA case), 1 sub-test, Fading, 3 Time Periods,” |
| NR-U_RLM_In_Sync_01 | 10.3.1.3 11.4.1.3 | “38.533 10.3.1.3+11.4.1.3 TT.zip” | “1 NR Cell under CCA (1 E-UTRA Cell for NSA case), 1 sub-test, Fading, 5 Time Periods,” |
| NR-U_SCell_Activation_01 | 10.3.3.1 10.3.3.2 10.3.3.3 11.4.3.1 11.4.3.2 11.4.3.3 13.2.2.1 13.2.2.2 13.2.2.3 | “38.533 10.3.3.1+10.3.3.2+10.3.3.3+11.4.3.1 TT.zip” | “2 Inter Frequency NR Cells under CCA, 3 time periods, Various number of sub-tests, No fading” |
| NR-U_SSB_Based_BFR_01 | 10.3.4.1 10.3.4.2 11.4.4.1 11.4.4.2 | “38.533 10.3.4.1+10.3.4.2+11.4.4.1+11.4.4.2 TT.zip” | “1 NR Cell under CCA (1 E-UTRA Cell for NSA case), 1 sub-test, 5 time periods, Fading” |
| NR-U_BWP_Switch_01 | 10.3.5.1 11.4.5.1 | "38.533 10.3.5.1+11.4.5.1 TT.zip" | "1 NR PCell (1 E-UTRAN Cell for NSA case), 2 time periods, no fading” |
| NR-U_BWP_Switch_02 | 10.3.5.2.1 10.3.5.2.2 11.4.5.2.1 11.4.5.2.2 | “38.533 10.3.5.2.1+10.3.5.2.2+11.4.5.2.1+11.4.5.2.2 TT.zip" | "1 E-UTRAN Cell, 1 NR PCell, (1 NR SCell), 3 time periods, no fading” |
| NR-U_BWP_Switch_03 | 10.3.5.3.1 11.4.5.3.1 | "38.533 10.3.5.3.1+11.4.5.3.1 TT.zip" | "1 NR PCell (1 E-UTRAN Cell for NSA case), 1 time period, no fading” |
| NR-U_Intra_SS-RSRQ_Acc_01 | 10.5.2.1 11.6.2.1 11.6.2.3 13.4.2.1 | “38.533 10.5.2.1+11.6.2.1+11.6.2.3+13.4.2.1 TT.zip” | “2 Intra-Frequency NR Cells, periodic reporting, No fading” |
| NR-U_Inter_SS-SINR_Abs_Acc_01 | 10.5.3.2 11.6.3.2 | “38.533 10.5.3.2+11.6.3.2 TT.zip” | “2 Inter-Frequency NR Cells, periodic reporting, No fading” |
| NR-U_L1-RSRP_Abs_Acc_01 | 10.5.4.1 11.6.4.1 13.4.4.1 | “38.533 10.5.4.1+11.6.4.1+13.4.4.1 TT.zip” | 1 NR Cell, periodic L1-RSRP reporting, No fading |
| NR-U_RRC_Redirection_01 | 11.2.2.3.1 11.2.2.3.2 | "38.533 11.2.2.3.1 + 11.2.2.3.2 TT.zip" | “2 Inter Frequency NR Cells, 2 time periods, No fading” |
| NR-U_PSCell_addition_01 | 10.3.6.1 | "38.533 10.3.6.1 TT.zip" | "1 NR PCell, 1 E-UTRAN Cell, 5 time periods, no fading” |
| NR-U_Inter_Freq_Meas_01 | 10.4.2.3 10.4.2.4 10.4.2.5 10.4.2.6 11.5.2.3 11.5.2.4 11.5.2.5 11.5.2.6 | “38.533 10.4.2.3+10.4.2.4+10.4.2.5+10.4.2.6 TT.zip" | “2 Inter Frequency NR Cells, 2 time periods, Various number of sub-tests, No fading” |
| NR-U_Inter_Freq_Meas_02 | 10.4.2.7 10.4.2.8 10.4.2.9 10.4.2.10 11.5.2.7 11.5.2.8 11.5.2.9 11.5.2.10 | “38.533 10.4.2.7+10.4.2.8+10.4.2.9+10.4.2.10 TT.zip" | “2 Inter Frequency NR Cells, 2 time periods, Various number of sub-tests, No fading” |
| NR-U_Inter_Freq_Meas_03 | 13.3.2.3 13.3.2.4 13.3.2.5 13.3.2.6 | “38.533 13.3.2.3+13.3.2.4+13.3.2.5+13.3.2.6 TT.zip" | “2 Inter Frequency NR Cells, 2 time periods, Various number of sub-tests, No fading” |
| NR-U_meas_intra-freq_01 | 10.4.1.1 10.4.1.4 | "38.533 10.4.1.1+10.4.1.4 TT.zip" | "1 NR PCell (1 E-UTRAN Cell for NSA case), 2 time period, no fading” |
| NR-U_meas_intra-freq_02 | 11.5.1.1 11.5.1.2 | 38.533 11.5.1.1+11.5.1.2 TT.zip | "1 NR PCell, 2 time period, no fading” |
| NR-U_InterRAT_Meas_01 | 11.5.3.1 11.5.3.2 | 38.533 11.5.3.1+11.5.3.2 TT.zip | “1 E-UTRAN Cell, 1 NR Cell, 2 time periods |
| NR-U_meas_intra-freq_02 | 13.3.1.1 13.3.1.2 13.3.1.3 13.3.1.4 | "38.533 13.3.1.1+13.3.1.2+13.3.1.3+13.3.1.4 TT.zip" | "1 licensed carrier NR Cell, 2 unlicensed carrier NR Cells, 2 time periods, no fading” |
| NR-U_Inter-RAT_Meas_01 | 10.4.4.1 10.4.4.2 10.4.4.3 10.4.4.4 12.4.2.1 12.4.2.2 12.4.2.3 12.4.2.4 | “38.533 10.4.4.1+10.4.4.2+12.4.2.1+12.4.2.2 TT.zip" | “1 E-UTRA PCell, 1-2 NR Cell(s), 2 time periods, Various number of sub-tests, No fading” |
| NR-U_L1-RSRP_meas_01 | 10.4.3.1 10.4.3.2 10.4.3.3 10.4.3.4 11.5.4.1 11.5.4.2 11.5.4.3 11.5.4.4 13.3.3.1 13.3.3.2 | "38.533 10.4.3.1+10.4.3.2+11.5.4.1+11.5.4.2 TT.zip" | "1 NR PCell (1 E-UTRAN Cell for NSA case), 2 time periods, no fading” |
| NR-U_Intra_SS-RSRP_Abs_Acc_01 | 10.5.1.1 11.6.1.1 13.4.1.1 | “38.533 10.5.1.1+11.6.1.1+13.4.1.1 TT.zip” | “2 Intra-Frequency NR Cells, 3 Sub-tests, periodic reporting, No fading” |
| NR-U_Inter_SS-RSRP_Abs_Acc_01 | 10.5.1.2 | “38.533 10.5.1.2 TT.zip” | “2 Inter-Frequency NR Cells, periodic reporting, No fading” |
| NR-U_Inter_SS-RSRQ_Abs_Acc_01 | 10.5.2.2 11.6.2.2 | “38.533 10.5.2.2+11.6.2.2 TT.zip” | “2 Inter-Frequency NR Cells, periodic reporting, No fading” |
| NR-U_Intra_SS-SINR_Acc_01 | 10.5.3.1 10.5.3.3 11.6.3.1 11.6.3.3 13.4.3.1 | “38.533 10.5.3.1+10.5.3.3+11.6.3.1+11.6.3.3 TT.zip” | “2 Intra-Frequency NR Cells, periodic reporting, No fading” |
| NR-U_PRACH_01 | 10.1.1.1.1 10.1.1.1.2 11.2.2.2.1 11.2.2.2.2 | “38.533 10.1.1.1.1+10.1.1.1.2+11.2.2.2.1+11.2.2.2.2 TT.zip” | “1 NR Cell under CCA (1 E-UTRA Cell for NSA case), PRACH measurements, no fading” |
| NR-U_PRACH_02 | 10.1.1.1.3 10.1.1.1.4 11.2.2.2.3 11.2.2.2.4 | “38.533 10.1.1.1.3+10.1.1.1.4+11.2.2.2.3+11.2.2.2.4 TT.zip” | “1 NR Cell under CCA (1 E-UTRA Cell for NSA case), PRACH measurements, no fading” |
| NR-U_Intra_RSSI_Abs_Acc_01 | 10.5.5.1 10.5.5.2 11.6.5.1 11.6.5.2 13.4.5.1 | “38.533 10.5.5.1+10.5.5.2+11.6.5.1+11.6.5.2 TT_v2.zip” | “1 NR Cell under CCA (1 E-UTRA Cell for NSA case), periodic reporting, no fading” |
| NR-U_Inter_RSSI _Abs_Acc_01 | 10.5.5.3 11.6.5.3 13.4.5.2 | “38.533 10.5.5.3+11.6.5.3+13.4.5.2 TT.zip” | “2 Inter-Frequency NR Cell under CCA (1 E-UTRA Cell for NSA case), periodic reporting, no fading” |
| NR-U_Intra_SS-RSRP_Abs_Acc_02 | 11.6.1.2 | “38.533 11.6.1.2 TT.zip” | “3 Intra-Frequency NR Cells, 3 Sub-tests, periodic reporting, No fading” |
| NR-U_Intra_channel_occupancy _Abs_Acc_01 | 10.5.6.1 10.5.6.2 11.6.6.1 11.6.6.2 13.4.6.1 | “38.533 10.5.6.1+10.5.6.2+11.6.6.1+11.6.6.2 TT_v2.zip” | “1 NR Cell under CCA (1 E-UTRA Cell for NSA case), periodic reporting, no fading” |
| NR-U_Inter_channel_occupancy _Abs_Acc_01 | 10.5.6.3 11.6.6.3 13.4.6.2 | “38.533 10.5.6.3+11.6.6.3+13.4.6.2 TT.zip” | “2 NR Cell under CCA (1 E-UTRA Cell for NSA case), periodic reporting, no fading” |
| NR-U_Intra_Reselection_01 | 11.1.1.1 | “38.533 11.1.1.1 TT.zip” | “2 Intra Frequency NR Cells under CCA, 3 time periods, No fading” |
| NR-U_Inter_Reselection_01 | 11.1.1.2 | “38.533 11.1.1.2 TT.zip” | “2 Inter Frequency NR Cells under CCA, 3 time periods, No fading” |
| NR-U_Inter_Reselection_02 | 11.1.2.1 | “38.533 11.1.2.1 TT.zip” | “2 Inter Frequency NR Cells with source cell under CCA, 3 time periods, No fading” |
| NR-U_Inter_Reselection_03 | 11.1.3.1 | “38.533 11.1.3.1 TT.zip” | “2 Inter Frequency NR Cells with target cell under CCA, 3 time periods, No fading” |
| NR-U_InterRAT_Higher_Reselection_01 | 11.1.4.1 | “38.533 11.1.4.1 TT.zip” | “1 E-UTRAN Cell, 1 NR Cell under CCA, 3 time periods, No fading” |
| NR-U_InterRAT_lower_Reselection_01 | 11.1.4.2 | “38.533 11.1.4.2 TT.zip” | “1 E-UTRAN Cell, 1 NR Cell under CCA, 3 time periods, No fading” |
| NR-U_Intra_Freq_HO_Known_Target_01 | 11.2.1.1 | “38.533 11.2.1.1 TT.zip” | “2 Intra-Freq NR Cells under CCA, 3 Time Periods, No Fading” |
| NR-U_Intra_Freq_HO_Unknown_Target_01 | 11.2.1.2 | “38.533 11.2.1.2 TT.zip” | “2 Intra-Freq NR Cells under CCA, 2 Time Periods, No Fading” |
| NR-U_Inter_Freq_HO_01 | 11.2.1.3 | “38.533 11.2.1.3 TT.zip” | “2 Inter-Freq NR Cells under CCA, 2 Time Periods, No Fading” |
| NR-U_Inter_Freq_HO_known_01 | 11.2.1.4 | “38.533 11.2.1.4 TT.zip” | “2 Inter-Freq NR Cells with source under CCA, 3 Time Periods, No Fading” |
| NR-U_Inter_Freq_HO_02 | 11.2.1.5 | “38.533 11.2.1.5 TT.zip” | “2 Inter-Freq NR Cells with source cell under CCA, 2 Time Periods, No Fading” |
| NR-U_Inter_Freq_HO_03 | 11.2.1.6 | “38.533 11.2.1.6 TT.zip” | “2 Inter-Freq NR Cells with target cell under CCA, 2 Time Periods, No Fading” |
| NR-U_InterRAT_Known _Handover_01 | 11.2.1.7 | “38.533 11.2.1.7 TT.zip” | “1 E-UTRAN Cell, 1 NR Cell under CCA, 3 time periods, No fading” |
| NR-U_InterRAT_Unknown _Handover_01 | 11.2.1.8 | “38.533 11.2.1.8 TT.zip” | “1 E-UTRAN Cell, 1 NR Cell under CCA, 3 time periods, No fading” |
| NR-U_Intra_RRC_re-establishment_01 | 11.2.2.1.1 | “38.533 11.2.2.1.1 TT.zip” | “2 Intra Frequency NR Cells under CCA, 3 time periods, No fading” |
| NR-U_Inter_RRC_re-establishment_01 | 11.2.2.1.2 | “38.533 11.2.2.1.2 TT.zip” | “2 Inter Frequency NR Cells under CCA, 3 time periods, No fading” |
| NR-U_Intra_RRC_re-establishment_without_timing_01 | 11.2.2.1.3 | “38.533 11.2.2.1.3 TT.zip” | “2 Intra Frequency NR Cells under CCA, 3 time periods, No fading” |
| NR-U_Inter_RRC_re-establishment_02 | 11.2.2.1.4 | “38.533 11.2.2.1.4 TT.zip” | “2 Inter Frequency NR Cells with target cell under CCA, 3 time periods, No fading” |
| NR-U_InterRAT_re-selection_LTE_Serving | 12.1.1.1 | “38.533 12.1.1.1 TT.zip” | “1 NR Cell under CCA, 1 LTE serving cell, 3 time periods No fading” |
| NR-U_InterRAT_HO_LTE_Serving | 12.2.1.1 | “38.533 12.2.1.1 TT.zip” | “1 NR Cell under CCA, 1 LTE serving cell, 3 time periods No fading” |
| NR-U_InterRAT_SFTD_meas | 12.4.1.1 | “38.533 12.4.1.1 TT.zip” | “1 NR Cell under CCA, 1 LTE serving cell, 1 time period No fading” |
| NR-U_InterRAT_SFTD_meas_accuracy | 12.5.1.1 | “38.533 12.5.1.1 TT.zip” | “1 NR Cell under CCA, 1 LTE serving cell, 1 time period No fading” |

<< End of Changes >>