# Title:	Test Tolerances for FR2 Tx Timing Test cases

# Source:	Anritsu, Rohde & Schwarz, Nokia

# 1. Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for RRM transmit timing accuracy tests.

- 5.4.1.1, EN-DC FR2 UE transmit timing accuracy

- 7.4.1.1, SA FR2 UE transmit timing accuracy

The applicability of the analysis is:

- UE Power class 1

- UE Power class 3

- UE Power class 6

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. Test case in Annex A of TS 38.133

The test conditions are defined in the following extracts from TS 38.133 v17.19.0 Annex A.

The standalone test case is chosen to show only the NR-related parameters.

#### A.7.4.1.1	NR UE Transmit Timing Test for FR2

##### A.7.4.1.1.1	Test Purpose and environment

The purpose of this test is to verify that the UE can follow frame timing change of the connected gNodeb and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.1.2.

Supported test configurations are shown in Table 7.4.1.1.1-1.

Table A.7.4.1.1.1-1: Supported test configurations for FR2 PCell

| Configuration | Description |
| --- | --- |
| 1 | NR TDD, SSB SCS 240 kHz, data SCS 120 kHz, BW 100 MHz |

For this test a single NR cell is used. Tables A.7.4.1.1.1-2 and A.7.4.1.1.1-2A define the parameters to be configured and strength of the transmitted signals. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.7.4.1.1.1-3.

Table A.7.4.1.1.1-2: Cell Specific Test Parameters for UL Transmit Timing test

| Parameter | Unit | Config | Test1 |  |  | Test2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SSB ARFCN |  | 1 | Freq1 |  |  | Freq1 |  |
| TDD configuration |  | 1 |  | TDDConf.3.1 |  |  |  |
| BWchannel | MHz | 1 | 100: NRB,c = 66 |  |  |  |  |
| Initial BWP Configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |
| Dedicated BWP Configuration |  | 1 | DLBWP.1.1 ULBWP.1.1 |  |  |  |  |
| TRS Configuration |  | 1 | TRS.2.1 TDD |  |  |  |  |
| PDSCH/PDCCH TCI state |  | 1 | TCI.State.2 |  |  |  |  |
| DRx Cycle | ms | 1 | N/A |  |  |  | DRX.8Note5 |
| PDSCH Reference measurement channel |  | 1 | SR.3.3 TDD |  |  |  |  |
| RMSI CORESET Reference Channel |  | 1 | CR.3.2 TDD |  |  |  |  |
| Dedicated CORESET Reference Channel |  | 1 | CCR.3.7 TDD |  |  |  |  |
| OCNG Patterns |  | 1 | OP.1 |  |  |  |  |
| SSB Configuration |  | 1 | SSB.4 FR2 |  |  |  |  |
| SMTC Configuration |  | 1 | SMTC.1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1 | 120 |  |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1 | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Propagation condition |  | 1 | AWGN |  |  |  |  |
| SRS Config |  | 1 | SRSConf.1Note6 |  | SRSConf.2Note6 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Void Note 4: Void Note 5: DRx related parameters are given in Table A.3.3.8-1 Note 6: SRS configs are given in Table A.7.4.1.1.1-3 |  |  |  |  |  |  |  |

Table A.7.4.1.1.1-2A: OTA related test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| Angle of arrival configuration |  | According to clause A.3.15.1 |  |
| Assumption for UE beamsNote 6 |  | Fine |  |
| Note1 | dBm/15kHzNote4 | -112 |  |
| Note1 | dBm/SCSNote3 | -100 |  |
|  | dB | 4 |  |
| SS-RSRPNote2 | dBm/SCS Note4 | -96 |  |
|  | dB | 4 |  |
| IoNote2 | dBm/95.04 MHz Note4 | -68.5 |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |

Table A.7.4.1.1.1-3: SRS Configuration for Timing Accuracy Test

|  | Field | SRSConf.1 | SRSConf.2 | Comments |
| --- | --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 |  |
|  | srs-ResourceIdList | 0 | 0 |  |
|  | resourceType | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 | 0 |  |
|  | nrofSRS-Ports | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 |  |
|  | freqHopping c-SRS | 17 | 17 | Matches NRB,c |
|  | freqHopping b-SRS | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither |  |
|  | resourceType | Periodic | Periodic |  |
|  | periodicityAndOffset-p | sl1, 0 | sl2560, 4 | Offset to align with DRx periodicity |
|  | sequenceId | 0 | 0 | Any 10 bit number |

Table A.7.4.1.1.1-4: Void

##### A.7.4.1.1.2	Test requirements

The test sequence shall be carried out in RRC_CONNECTED for every test case.

Following will be the test sequence for this test:

1)	Setup NR PCell according to parameters given in Table A.7.4.1.1.1-1.

2)	After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB.

a.	The NTA offset value (in Tc units) is 13792

b.	The Te values depend on the DL and UL SCS for which the test is being run and are given in Table 7.1.2-1

3)	The test system shall adjust the timing of the DL path by values given in Table A.7.4.1.1.2-1

Table A.7.4.1.1.2-1 Adjustment Value for DL Timing

| SCS of SSB signals (kHz) | Adjustment Value |  |
| --- | --- | --- |
|  | Test1 | Test2 |
| 240 | +8*64Tc | +4*64Tc |

4)	The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in clause 7.1.2 Table 7.1.2-3 until the UE transmit timing offset is within (NTA + NTA_offset) ×Tc ± Te respective to the first detected path (in time) of DL SSB.  Skip this step for test 2 with DRX configured.

5)	The test system shall verify that the UE transmit timing offset stays within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB. For Test 2 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

# 3. Discussion

The test consists of two subtests with one cell configured; the difference between the Subtests is related to DRX cycle and SRS configuration. Each Subtest consists of one time period. The downlink timing of Cell 1 is changed and the changes in UE transmit timing are observed. The transmit timing is verified by the UE transmitting SRS used as a measurement reference facilitating the Test system timing estimation.

# 4. Calculation of dB Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet. The purpose of the spreadsheet is to check that the downlink signal reaching the UE is within side conditions. The Test Tolerances for timing requirements are handled in section 5.

a) Original specified key parameters

The key parameters are copied from Tables A.7.4.1.1.1-1, A.7.4.1.1.1-2 and A.7.4.1.1.1-2A in TS 38.133. The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test. The angle of arrival of the radiated signals is also recorded, and the assumption for the UE beam.

Several UE core requirement values and their associated parameters are also entered in this section, taking into account the angle of arrival of the radiated signals. The requirements are entered because they affect the radiated signal received by the UE or affect how the UE sees the received signals at baseband.

The key parameters appear in section a) of the accompanying spreadsheet.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

Some UE core requirements in section a) are converted into UE-related derived parameters, in a form that allows them to be used in later sections of the spreadsheet. Spherical coverage is not applicable in this test case and is greyed out.

c) Uncertainties

The Test system provides 1 cell on 1 frequency, with AWGN. We propose to control the following parameters:

- AWGN Absolute power, Noc ±5.65 dB averaged over BWConfig

- AWGN Absolute power, Noc ±5.65 dB for PRBs #RBJ-RBJ+39

- Ratio of NR cell signal / AWGN, Ês / Noc ±0.3 dB averaged over BWConfig

- Ratio of NR cell signal / AWGN, Ês / Noc ±0.3 dB for PRBs #RBJ-RBJ+39

d) Controlled parameters critical to verdict

In many RRM test cases there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict.

It is therefore essential to identify those parameters determining the test verdict, and also the side conditions that should be met for the UE to be tested correctly. The 3 controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and by careful reading of the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given in the “Requirement” column of section g) in the spreadsheet.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB.

Where there is a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict, the sensitivity factors are derived by inspection as one, or if no effect, as zero.

Where there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict, the calculation of sensitivity factors is done using a method selected from TR 38.903. NR Cell 1 Es/Iot at UE baseband is calculated according to TR 38.903 A.2.4, and applied Io calculated according to TR 38.903 A.2.5.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, taking into account the uncertainties and sensitivity factors for each parameter that can be set by the test equipment. This process follows the superposition principle. More details and explanation can be found in section 4 of TR 38.903.

e) Determine parameters to offset

Clause B.2.6.1 has been deleted from TS 38.133, and no requirements are defined for Es/IotBB, Applied SSB_RP or Applied Io. No offsets are therefore applied.

f) Parameters modified by Test Tolerances

The set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, modified by the Test Tolerances which in this case are all zero.

Re-derived parameters are calculated using the same methods as were used in step b).

g) Check controlled parameters Min/Max

Using a format similar to that in step d), the nominal value of each controlled parameter is recalculated.

The minimum and maximum values, due to variability from uncertainties, of controlled parameters are then calculated (Es/IotBB, Applied SSB_RP and Applied Io). However, clause B.2.6.1 was deleted from TS 38.133 in R4-2016161, and no requirements are defined.

# 5. Calculation of timing Test Tolerances

5.1	Calculations for Transmit Timing error Te

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in TS 38.133 Table 7.1.2-1:

Table 7.1.2-1: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 10*64*Tc |
|  | 30 | 15 | 8*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 7*64*Tc |
| 2 | 120 | 60 | 3.5*64*Tc |
|  |  | 120 | 3.5*64*Tc |
|  | 240 | 60 | 3*64*Tc |
|  |  | 120 | 3*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

The measurement uncertainty of the test system is the only additional effect to be considered. The Test tolerance for uplink signal transmit timing relative to downlink is 0.75*64*Tc, considered to be a reasonable uncertainty value for the Test system measurement of transmit timing. This was derived from taking 25% of the tightest UE core requirement for FR2, which is 3*64*Tc for 240kHz SCS, giving a ±0.75* 64*Tc uncertainty.

For Test 1 and Test 2 (SSB SCS 240 kHz, 100MHz Channel BW) the test requirements are:

Formula: Upper limit + TT, Lower limit – TT:

All conditions (±3*64*Tc ±0.75*64*Tc) = ± 3.75*64*Tc

These values are therefore used in TS 38.533 in the table for Test requirements for test case 7.4.1.1.

5.2	Calculations for Transmit Timing change rules (Tq, Tp)

The UE shall be capable of changing the transmission timing according to the received downlink frame. When the transmission timing error between the UE and the reference timing exceeds ±Te the UE is required to adjust its timing to within ±Te. The reference timing shall be before the downlink timing.

All adjustments made to the UE uplink timing shall follow these rules, from TS 38.133 clause 7.1.2.1:

1)	The maximum amount of the magnitude of the timing change in one adjustment shall be Tq.

2)	The minimum aggregate adjustment rate shall be Tp  per second.

3)	The maximum aggregate adjustment rate shall be Tq per 200ms.

where the maximum autonomous time adjustment step Tq and the aggregate adjustment rate Tp are specified in Table 7.1.2-1.

Table 7.1.2.1-1: Tq Maximum Autonomous Time Adjustment Step and Tp Minimum Aggregate Adjustment rate

| Frequency Range | SCS of uplink signals (kHz) | Tq | Tp |
| --- | --- | --- | --- |
| 1 | 15 | 5.5*64*Tc | 5.5*64*Tc |
|  | 30 | 5.5*64*Tc | 5.5*64*Tc |
|  | 60 | 5.5*64*Tc | 5.5*64*Tc |
| 2 | 60 | 2.5*64*Tc | 2.5*64*Tc |
|  | 120 | 2.5*64*Tc | 2.5*64*Tc |
| NOTE: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

For the time adjustment test, as well as the measurement uncertainty of the test system two additional effects need to be considered. Both the allowed UE frequency error and the UE time adjustment quantisation can affect the result of the test, and it is not possible to test the UE Transmit Timing accuracy in isolation from these core requirements. They are therefore taken into account when setting the Test Tolerances and the Test Requirements.

Test system uncertainty

The maximum test system uncertainty for relative transmit timing during adjustment is 0.625*64*Tc, considered to be a reasonable uncertainty value for the Test system measurement of relative transmit timing during adjustment. This was derived from taking 25% of the tightest UE core requirement for FR2, which is 2.5*64*Tc, giving a ±0.625*64*Tc uncertainty.

Allowed UE frequency error

In addition to the effect of test system uncertainty for relative transmit timing, the effect of allowed UE frequency error specified in TS 38.101-2 clause 6.4.1 should be taken into account.

### 6.4.1	Frequency Error

The UE basic measurement interval of modulated carrier frequency is 1 UL slot. The mean value of basic measurements of UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 msec of cumulated measurement intevals compared to the carrier frequency received from the NR gNB.

The effect of allowed UE frequency error on the timing adjustment rules is calculated as follows for Configuration 1:

Rule 1: The maximum amount of the magnitude of the timing change in one adjustment shall be Tq (2.5*64*Tc).

As the SRS is configured to be transmitted every 1 slot for subtest 1, the UE timing change from frequency error could be (0.000125s x (0.1 x 10-6)) = 0.13ns. This is negligible compared to the core requirement and does not need to be considered for the rule 1 requirement.

Rule 2: The minimum aggregate adjustment rate shall be Tp (2.5*64*Tc) per second.

During 1s, the UE timing change from frequency error could be (1s x (0.1 x 10-6)) = 100ns. From TS 38.211 clause 4.1,  where Δfmax=480∙103 Hz and , giving 1/(480000 x 4096) s or 0.51ns, and 64*Tc = 32.6ns. Expressed in the same ways as the requirement, the contribution from frequency error could be (100/32.6) = 3.1*64*Tc.

Rule 3: The maximum aggregate adjustment rate shall be Tq (2.5*64*Tc) per 200ms.

During 200ms, the UE timing change from frequency error could be (0.2s x (0.1 x 10-6)) = 20ns. From TS 38.211 clause 4.1,  where Δfmax=480∙103 Hz and , giving 1/(480000 x 4096) s or 0.51ns, and 64*Tc = 32.6ns. Expressed in the same ways as the requirement, the contribution from frequency error could be (20/32.6) = 0.6*64*Tc.

Sliding time window duration

For Test 1, Table A.7.4.1.1.1-3: SRS Configuration, SRSConf.1 specifies periodicityAndOffset-p as sl1, 0.

In TS 38.331 SRS-Config field descriptions, sl1 corresponds to a periodicity of 1 slot. To calculate the slot duration, relevant clauses are extracted from TS 38.211:

## 4.1	General

Throughout this specification, unless otherwise noted, the size of various fields in the time domain is expressed in time units  where Δfmax=480∙103 Hz and . The constant  where ,  and .

## 4.2	Numerologies

Multiple OFDM numerologies are supported as given by Table 4.2-1 where μ and the cyclic prefix for a bandwidth part are obtained from the higher-layer parameter subcarrierSpacing and cyclicPrefix, respectively.

Table 4.2-1: Supported transmission numerologies.

|  |  | Cyclic prefix |
| --- | --- | --- |
| 0 | 15 | Normal |
| 1 | 30 | Normal |
| 2 | 60 | Normal, Extended |
| 3 | 120 | Normal |
| 4 | 240 | Normal |

## 4.3	Frame structure

### 4.3.1	Frames and subframes

Downlink and uplink transmissions are organized into frames with  duration, each consisting of ten subframes of  duration. The number of consecutive OFDM symbols per subframe is Nsymbsubframe,μ=NsymbslotNslotsubframe,μ. Each frame is divided into two equally-sized half-frames of five subframes each with half-frame 0 consisting of subframes 0 – 4 and half-frame 1 consisting of subframes 5 – 9.

There is one set of frames in the uplink and one set of frames in the downlink on a carrier.

Uplink frame number  for transmission from the UE shall start  before the start of the corresponding downlink frame at the UE where  is given by [5, TS 38.213].

Figure 4.3.1-1: Uplink-downlink timing relation.

### 4.3.2	Slots

For subcarrier spacing configuration , slots are numbered nsμ∈0, …,Nslotsubframe,μ-1  in increasing order within a subframe and ns,fμ∈0, …,Nslotframe,μ-1  in increasing order within a frame. There are  consecutive OFDM symbols in a slot where  depends on the cyclic prefix as given by Tables 4.3.2-1 and 4.3.2-2. The start of slot nsμ in a subframe is aligned in time with the start of OFDM symbol  in the same subframe.

OFDM symbols in a slot can be classified as 'downlink', 'flexible', or 'uplink'. Signaling of slot formats is described in subclause 11.1 of [5, TS 38.213].

In a slot in a downlink frame, the UE shall assume that downlink transmissions only occur in 'downlink' or 'flexible' symbols.

In a slot in an uplink frame, the UE shall only transmit in 'uplink' or 'flexible' symbols.

A UE not capable of full-duplex communication among a group of cells is not expected to transmit in the uplink in one cell within the group of cells earlier than NRx-TxTc after the end of the last received downlink symbol in the same or different cell within the group of cells where NRx-Tx is given by Table 4.3.2-3.

A UE not capable of full-duplex communication among a group of cells is not expected to receive in the downlink in one cell within the group of cells earlier than NTx-RxTc after the end of the last transmitted uplink symbol in the same or different cell within the group of cells where NTx-Rx is given by Table 4.3.2-3.

Table 4.3.2-1: Number of OFDM symbols per slot, slots per frame, and slots per subframe for normal cyclic prefix.

| μ | Nsymbslot | Nslotframe,μ | Nslotsubframe,μ |
| --- | --- | --- | --- |
| 0 | 14 | 10 | 1 |
| 1 | 14 | 20 | 2 |
| 2 | 14 | 40 | 4 |
| 3 | 14 | 80 | 8 |
| 4 | 14 | 160 | 16 |

For the parameters shown (120kHz data SCS, µ=3), 1 slot = 1/8 subframe giving a duration of 0.125ms.

UE time adjustment quantisation

Although not explicitly stated in TS 38.133, the UE timing adjustment is assumed to be quantised, and a quantisation step size up to Tq should be taken into account.

The effect of quantised UE timing adjustment on the timing adjustment rules is calculated as follows:

Rule 1: The maximum amount of the magnitude of the timing change in one adjustment shall be Tq seconds.

As the magnitude of the timing change in one adjustment is being directly measured, no additional Tq allowance is needed for the rule 1 requirement.

Rule 2: The minimum aggregate adjustment rate shall be Tp  per second, which is 2.5*64*Tc per second for FR2.

If a time window of exactly 1s is used, and the UE makes the largest allowed timing steps with the longest allowed time between changes as shown in the diagram, the Test System could catch one or two UE timing changes depending on the synchronisation between the Test System measurement period of 1s and the UE timing change. As this part of the test aims to check the UE minimum adjustment rate, the sliding time window is increased by the minimum amount of one sample (0.125ms for subtest 1). This ensures that the Test System will not miss any timing changes occurring during the 1s time period specified in the core requirements.

The case shown in the diagram is a UE making the largest allowed timing steps of 2.5*64*Tc with the longest allowed time between changes (1 change in 1s to meet the minimum aggregate adjustment rate of 2.5*64*Tc per second) for FR2.

Rule 3: The maximum aggregate adjustment rate shall be Tq per 200ms.

If a time window of exactly 200ms is used, and the UE makes the largest allowed timing steps with the longest allowed time between changes as shown in the diagram, the Test System could catch zero, one or two UE timing changes depending on the synchronisation between the Test System measurement period of 200ms and the UE timing change. As this part of the test aims to check the UE maximum adjustment rate, the sliding time window is decreased by the minimum amount of one sample (0.125ms for subtest 1). This ensures that the Test System will not include any extra timing changes occurring during the 200ms time period specified in the core requirements.

The case shown in the diagram is a UE making the largest allowed timing steps of 2.5*64*Tc with the longest allowed time between changes (1 change every 200ms to meet the minimum aggregate adjustment rate of 2.5*64*Tc per 200ms).

Combined effect of allowed UE frequency error and UE time adjustment quantisation

The diagram above shows the combined effect of allowed UE frequency error, UE time adjustment quantisation and the use of a sliding measurement window (for simplicity the test system timing measurement uncertainty is not included in the diagram).

The dashed green and blue lines show the specified maximum and minimum aggregate adjustment rates respectively for 100MHz channel bandwidth. The stepped green and blue lines show possible behaviour if the UE uses the largest possible time quantisation step, and changes the timing infrequently, which gives the worst case for measurement. Smaller or more frequent steps are also allowed.

The effect of frequency error is shown by dashed black lines, and can make the UE Tx timing earlier or later than expected. The combined effect is shown by the green and blue lines with sloping steps.

For the maximum adjustment rate measurement, the 199.875ms window “Position A” shows a typical measurement, where the test system would expect to see one timing quantisation step, together with a frequency error which may increase the timing change. In window “Position B” the test system would see no timing quantisation steps, but as the Test requirement is a one-sided limit this is unimportant.

For the minimum adjustment rate measurement, the 1.000125s window “Position C” shows a typical measurement, where the test system would expect to see one timing quantisation step, together with a frequency error which may decrease the timing change. In window “Position D” the test system could see two timing quantisation steps, but as the Test requirement is a one-sided limit this is unimportant.

When calculating the Test requirements, the test system uncertainty, the effect of allowed UE frequency error where relevant, and the effect of UE time adjustment quantisation where relevant, are included.

For Test 1 with Configuration 1 (SSB SCS of 240kHz) the test requirements are:

- The Maximum timing change in one adjustment Tq calculated as shown:

|  | Lower limit | Upper limit |
| --- | --- | --- |
| Core requirement | Not applicable | +2.5*64*Tc |
| Test system uncertainty | Not applicable | +0.625*64*Tc |
| UE frequency error allowance | Not applicable | Not applicable |
| Test requirement | Not applicable | +3.125*64*Tc |

- The Minimum aggregate adjustment rate per second calculated as shown:

|  | Lower limit | Upper limit |
| --- | --- | --- |
| Core requirement | +2.5*64*Tc | Not applicable |
| Test system uncertainty | -0.625*64*Tc | Not applicable |
| UE frequency error allowance | -3.1*64* Tc | Not applicable |
| Test requirement | -1.225*64*Tc | Not applicable |

- The Maximum aggregate adjustment rate per 200 ms calculated as shown:

|  | Lower limit | Upper limit |
| --- | --- | --- |
| Core requirement | Not applicable | +2.5*64*Tc |
| Test system uncertainty | Not applicable | +0.625*64*Tc |
| UE frequency error allowance | Not applicable | +0.6*64*Tc |
| Test requirement | Not applicable | +3.725*64*Tc |

Subtest 2 with DRX does not require checking of the Transmit Timing change rules.

# 6. Treatment of related test cases

TS 38.533 also contains a similar EN-DC test case in clause 5.4.1.1, which has the same NR Cell signal levels, so we propose to apply the same level uncertainties and Test Tolerances. There is no need to provide a tab on the spreadsheet or a separate calculation as all the numeric values are identical.

# 7. Treatment of other UE Power Classes

The initial analysis was done for UE Power class 3, as defined in TS 38.101-2, Table 6.2.1.0-1.

As the calculations use different values for UE Power classes 1 and 6, separate spreadsheet tabs are provided. The following parameters are changed, shown as underlined cells on both tabs of the spreadsheet:

- UE Refsens (Rx Beam peak direction), section a)

- UE Spherical coverage (%ile directions), section a)

- UE rough/fine beam gain ∆ Rx Beam peak, Y, section a)

- UE rough/fine beam gain ∆ Spherical Coverage, Z, section a)