3GPP TSG-RAN5 Meeting #109	R5-255558

# Dallas, US, 17th – 21st November, 2025

Agenda item:	5.4.20

Source:	Keysight Technologies UK Ltd

Title:	FR2 MU - PC3 analysis for OBW UL MIMO test

Document for:	Discussion and Endorsement

# Introduction

This contribution aims to continue the MU analysis for PC3 in OBW UL MIMO initiated in [1] and [2].

# Discussion

Analysis made in [1] concluded that minimum acceptable MBW should be 1.2*CBW. In case of FR2c 400MHz. Based on the information available in [3], OBW meas error vs SNR for MBW = {1.1, 1.3, 1.5}*CBW, the only confirmed values low enough was for MBW=1.1*CBW, about 1.2%*CBW, because for MBW=1.3*CBW the expected error was about 13%*CBW. For that reason, in [1] and resubmitted in [2], it has been proposed not to test OBW UL MIMO for FR2c 400MHz.

However, so far it has not been provided any MU value for MBW = 1.2*CBW nor clarified what could be acceptable MTSU for the OBW measurement.

We had provided no values because from analysis made in [5], our expectation from other TEVs is that for MBW = 1.2*CBW, their MU could be about 5% or higher.  We consider that MTSU = 5%*CBW is an unacceptable measurement error based on how OBW is defined in [7]. MTSU of 5%*CBW means 20MHz potential error for 400MHz CBW, what even divided by 2, it exceeds the one side guardband as defined in Table 5.3.3-1 in [7], making even an ideal UE to fail the test. For that reason, maybe not correctly explained in previous contributions, we have been proposing declare this test as not testable, despite expectation in our own test system was better than that assumption.

However, during offline discussions last meeting, we were encouraged, along with other TEVs, to provide the actual MU value we can commit in our test system in order to better evaluate the testability of the case, because previous assumptions were based on interpretations from graphs not covering explicitly the case of MBW = 1.2*CBW.

We have done such an evaluation in our test system and concluded on an MTSU = 2%*CBW. Currently, the highest OBW MTSU ever accepted is 1.7%*CBW that corresponds to the case of UL MIMO, FR2b, 400MHz. Therefore we consider this new value of 2%*CBW should be acceptable.

Proposal 1. For OBW UL MIMO test in FR2c, for CBW = 400MHz, apply MBW = 1.2*CBW and define MTSU as +/- 2.0%*CBW.

# Conclusion

The following observations and proposals were made in this contribution.

Proposal 1. For OBW UL MIMO test in FR2c, for CBW = 400MHz, apply MBW = 1.2*CBW and define MTSU as +/- 2.0%*CBW.

# References

- R5-253527, “FR2 MU - PC3 analysis for OBW UL MIMO test”, Keysight, RAN5#107

- R5-253804, “FR2 MU - PC3 analysis for OBW UL MIMO test”, Keysight, RAN5#108

- R5-247165, “MU discussion on OBW for FR2 UL MIMO”, Anritsu, RAN5#105

- R5-247337, “On the MU and TT for SISO and MIMO test cases”, R&S, RAN5#105

- R5-242998, “On testability issue of FR2c OBW”, Anritsu, RAN5#103

- R5-244167, “Discussion on MU and TT analysis for FR2c and FR2b”, Keysight, RAN5#104

- TS 38.521-2, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 2 Standalone”, v19.0.0 (2025-09)