3GPP TSG RAN5 Meeting #109	 	R5-255725

Dallas, Texas, USA, 17th - 21st November 2025

Source:			Rohde & Schwarz

Title:	On the MU for RRM NB-IOT HD-FDD UE Timing Tests

Agenda Item:		5.5.1.13

Document for:	Discussion and Endorsement

# 1.	Introduction

In the last meeting the RRM HD-FDD UE Timing Advance Adjustment Accuracy Tests 7.2.9,13.4.2.1 and 13.4.2.2 were made incomplete in [1], [2] with the following reasoning:

“The measurement uncertainty for relative UE timing adjustment is incorrectly derived with no proper justification for the MU value. If we take legacy test case as an example, a much higher MU needs to be considered. More detailed analysis is needed for coming up with MU value.”

In this contribution, we present the result of our assessment for deriving the MTSU for the relative UE timing adjustment.

# Discussion

For LTE RRM, the uncertainty for relative UE timing adjustment was derived by taking 25% of the tightest UE core requirement, which was based on “Maximum Autonomous Time Adjustment Step:”

This MTSU is still in use and defined in B.6 of [3]

Observation1: For LTE the transmit timing relative measurement uncertainty derived based on 25% of the tightest applicable core requirement.

The core requirements for NB-IoT and NB-IoT for Satellite access are defined in

1.Sections 7.20.2 and 7.20A.2 of [4] and [5] are of 58.33*Ts

2. Section 7.22.2.2 and 7.22A.2.2 of [4] and [5] are ±13.33* TS

Observation2: The tightest applicable core requirement for transmit timing relative measurement uncertainty as per 7.22.2.2 and 7.22A.2.2, is 13.33*Ts

Proposal 1: Define new MTSU for Relative UE timing adjustment for NB-IOT and NB-NTN by applying 25% of the tightest UE core requirement, which is 13.33 * Ts, giving a ±3.33 * Ts uncertainty.

Proposal 1a: Update Section B.6 in [3] with new MTSU term

Proposal 1b: Adapt the new MTSU term to the TT analysis of HD-FDD UE Timing Advance Adjustment Accuracy Test 7.2.9,13.4.2.1 and 13.4.2.2 test cases.

Accompanying CRs in [6] and [7]

# Conclusion

Observation1: For LTE the transmit timing relative measurement uncertainty derived based on 25% of the tightest applicable core requirement.

Observation2: The tightest applicable core requirement for transmit timing relative measurement uncertainty as per 7.22.2.2 and 7.22A.2.2, is 13.33*Ts

Proposal 1: Define new MTSU for Relative UE timing adjustment for NB-IOT and NB-NTN by applying 25% of the tightest UE core requirement, which is 13.33 * Ts, giving a ±3.33 * Ts uncertainty.

Proposal 1a: Update Section B.6 in [3] with new MTSU term

Proposal 1b: Adapt the new MTSU term to the TT analysis of HD-FDD UE Timing Advance Adjustment Accuracy Test 7.2.9,13.4.2.1 and 13.4.2.2 test cases.

# References

- R5-255419, Test Tolerance update for HD-FDD UE Timing Advance Adjustment Accuracy Test 7.2.9,13.4.2.1 and 13.4.2.2., Qualcomm Korea, 3GPP TSG RAN WG 5 Meeting #108, August 2025

- 3GPP TS 36.521-3 V18.10.0, (2025-09)

- 3GPP TR 36.903 V18.5.0 (2025-06)

- 3GPP TS 36.133 V17.16.0 (2025-03)

- 3GPP TS 36.133 V13.24.0 (2024-09)

- R5-255726, MU and TT update for RRM HD-FDD UE Timing Advance Adjustment Accuracy Tests, TR 36.903, Rohde & Schwarz, Qualcomm Korea, 3GPP TSG RAN WG 5 Meeting #109, November 2025

- R5-255727, Corrections for RRM HD-FDD UE Timing Advance Adjustment Accuracy Tests including TT, TS 36.521-3, Rohde & Schwarz, Qualcomm Korea, 3GPP TSG RAN WG 5 Meeting #109, November 2025