3GPP TSG-RAN5 Meeting #109	R5-255726

# Dallas, Texas, USA, 17th - 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.903 | CR | 0474 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MU and TT update for RRM HD-FDD UE Timing Advance Adjustment Accuracy Tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz, Qualcomm Korea |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Existing TT analysis of RRM HD-FDD UE Timing Advance Adjustment Accuracy Tests is incomplete, MU for relative UE timing adjustment for NB-IOT and NB-NTN is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test tolerance analysis for test cases NB-IOT test case 7.2.9 and NB-NTN test cases 13.4.2.1 and 13.4.2.2 with correct uncertainty consideration. Add the following attached .zip file to TR 36.903: 7.2.9+13.4.2.1+13.4.2.2 TT v2.zip Remove the following attached .zip file from TR 36.903: 7.2.9+13.4.2.1+13.4.2.2 TT v1.zip |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases implementation will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.6, attachments |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521-3 CR 3108 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## < Unchanged sections omitted >

Add the following attached .zip file to TR 36.903:

7.2.9+13.4.2.1+13.4.2.2 TT v2.zip

Remove the following attached .zip file from  TR 36.903:

7.2.9+13.4.2.1+13.4.2.2 TT v1.zip

## < Unchanged sections omitted >

# B.6	Group F: E-UTRA Transmit timing and Timing advance

The following uncertainties and parameters are suggested for E-UTRA Transmit timing and Timing advance tests:

Table B.6-1: Maximum Test System Uncertainty for E-UTRA Transmit timing and Timing advance

| Downlink signal: |  |
| --- | --- |
| Noc averaged over BWConfig | ±3.0 dB |
| Ês / Noc averaged over BWConfig | ±0.3 dB |
| Uplink signal: |  |
| Uplink signal transmit timing relative to downlink | ±3Ts TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 |
| Relative UE timing adjustment | ±0.5Ts TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 |
| Relative UE timing adjustment for NB-IoT and NB-IoT for Satellite Access | ±3.33Ts TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 |

The downlink uncertainty values are chosen to be the same as the performance tests in section 8 of TS 36.521-1 [4]. For Transmit timing and Timing advance tests, neither the absolute level of Noc nor the signal to noise ratio is critical.

The uncertainty for uplink signal transmit timing relative to downlink measurement was derived by taking 25% of the tightest UE core requirement, which is 12 * Ts for ≥3 MHz Channel bandwidth, giving a ±3*Ts uncertainty.

The uncertainty for relative UE timing adjustment was derived by taking 25% of the tightest UE core requirement, which is 2 * Ts for ≥10 MHz Channel bandwidth, giving a ±0.5 * Ts uncertainty.

The uncertainty for relative UE timing adjustment of NB-IoT and NB-IoT for Satellite Access was derived by taking 25% of the tightest UE core requirement, which is 13.33 * Ts, giving a ±3.33 * Ts uncertainty.

Both timing uncertainties are expressed in units of TS = 1 / (15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [7].

These choices form a minimum set, so the superposition principle can be applied.

< End of changes >