3GPP TSG-RAN5 Meeting #109	R5-255825

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.509 | CR | 0089 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to Test loop mode for ATSSS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, ATSSS_Ph2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As some ATSSS Phase 2 test cases (e.g. 11.9.1/3) need to check IP Packet is loop back via the WLAN cell, but currently the generic procedure Ipsec_SA_Released does not specify the requirement of Test Loop Back function. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Generic procedure Ipsec_SA_Released was updated for TLB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test purpose cann`t be checked. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 5.3.1, 5.3.4.2.5 (new) and 6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

1	Scope

The present document defines for User Equipment (UE) those special functions and their activation/deactivation methods that are required in the UE for conformance testing purposes when the UE is connected to the 5G System (5GS) via its radio interface(s).

The document also describes the operation of these special functions when the 5GS capable UEs are connected via a non-5GS system e.g. E-UTRA FDD or TDD system or WLAN.

Depending on the 5GS system's architecture some relevant for the UE for conformance testing special functions may be defined in TS 36.509 [6].

==============Next change==============

5.3	Test loop functions

5.3.1	General

Before a loop functionality can be exercised, the test loop needs to be closed; this is to be understood as the UE being instructed to start looping back received data packets. When looping back received data packets is not any longer required the loop should be re-opened; opening of a loop does not change the type of bearer being established by the UE test mode activation function (subclause 5.2.2).

To limit the number of special test functions, the concept of closing and opening a loop is also used as instruction to the UE to initiate/terminate other actions. An example of this is counting the received packets and reporting the number of received packets back to the SS; in the context of this utilisation closing a loop is to be understood as the UE being instructed to start counting the packets, whereas opening a loop should be understood as stop counting the packets. Other utilisation of the closing/opening a "test loop" test functionality can be specified if appropriate.

The UE test loop functions for 5GS are intended for:

-	NR receiver and transmitter testing to disable application data transfer in downlink and uplink during SS UE measurements.

-	NR layer 2 (MAC, RLC, PDCP, SDAP) and data radio bearer testing to generate data transfer in downlink and uplink.

-	NR layer 2 (MAC, RLC, PDCP, SDAP) and sidelink data radio bearer testing to generate data transfer in sidelink.

-	NR layer 2 (MAC, RLC, PDCP) and MBS radio bearer testing.

-	5GC and NR layer 3 testing to verify data transfer continuation over RRC and 5GC procedures.

-	5GC testing to verify data transfer continuation over WLAN and 5GC procedures.

-	5GC and NR layer 3 testing to verify data transfer over PC5 interface.

-	5GC NAS user-plane testing to verify uplink QoS flow descriptions handling.

-	PC5-U testing to verify sidelink PC5 QoS flow descriptions handling.

==============Next change==============

5.3.4.2.5	UE test loop mode B operation for WLAN

UE test loop mode B provides loopback of IP PDUs while UE is operated in WLAN mode. Prior to closing the UE test loop mode B, thereby requesting the UE to start looping back the received data packets, at least one MA PDU session shall have been established between SS and UE. This implies that before the procedure for establishing the MA PDU session takes place the SS needs to activate the UE test mode as specified in subclause 5.2.2.

The downlink IP PDU's received by the UE on all MA PDU sessions are returned by the UE without any modification of the IP header for transmission in uplink.

Figure 5.3.4.2.5-1 shows functional block diagrams of UE test loop function for the TMC entity and UE test loop mode B for the case when WLAN is configured.

The 5GS UE test loop mode B operation is the same as the one described in TS 36.509 [6], subclause 5.4.4.

Figure 5.3.4.2.5-1: Model for Test Mode Control and UE Test Loop Mode B on UE side when WLAN s configured

5.3.4.2A	UE test loop mode C operation

==============Next change==============

6.1	General

Clause 6 describes only TMC protocol messages.

When UE under test is operated in EN-DC or NGEN-DC the TMC messages are sent using the E-UTRA RRC DLInformationTransfer and ULInformationTransfer procedures, see TS 36.331 [10], subclauses 5.6.1 and 5.6.2.

When UE under test is operated in NR or NE-DC the TMC messages are sent using the NR RRC DLInformationTransfer and ULInformationTransfer procedures, see TS 38.331 [11], subclauses 5.7.1 and 5.7.2.

When UE under test is operated in WLAN the TMC messages are sent to the WLAN.

NOTE 1:	A message received with skip indicator different from 0 will be ignored.

NOTE 2:	For general definition of Layer 3 message format see TS 24.007 [7], clause 11.

NOTE 3:	5GS use the same protocol discriminator value ("1111") as E-UTRA, UTRA and GSM/GPRS as specified in TS 24.007 [7], subclause 11.2.3.1.1. 5GS test control messages the message type value series 101xxxxx is reserved, where x represents 0 or 1. The message type values 0000xxxx to 1001xxxx are reserved to E-UTRA, UTRA and GSM/GPRS as specified in TS 36.509 [6], TS 34.109 [8] and TS 44.014 [9]. For 5GS test control messages that are common with E-UTRA control messages in TS 36.509 [6] the 5GS test control messages use the same message type values as used for the E-UTRA test control messages in TS 36.509 [6].

All the TMC messages are integrity protected and ciphered according to TS 24.301 [17] subclause 4.4 or TS 24.501 [21] subclause 4.4 depending on whether the TMC message is sent via EPS or 5GC.

==============End of change==============