Title:	Discussion on test point selection for CA_NS_205 A-MPR in FR2

Source:	China Unicom, Huawei, HiSilicon

Introduction

The purpose of this paper is to provide test point selection proposal for 38.101-2 A-MPR requirements when CA_NS_205 is signaled (band n257 and n258). Additional RF emission requirements apply in this scenario in the form of Spurious emissions, meaning this proposal applies for test cases 6.2A.3 (A-MPR for CA) and 6.5A.3.3 (Additional Spurious emission for CA).

Test point selection is considered for PC1, PC2, PC3 and PC4.

Discussion

Since A-MPR for CA_NS_205 in RAN4 is defined together with A-Spurious requirements, a combined analysis is required. In general, the following non-compliant UE behaviors need to be checked:

- UE apply too much A-MPR (more than RAN4 allow)

- UE apply to little A-MPR (causing too much RF emissions)

Case A can be verified in A-MPR test case.

Case B can be verified in A-Spurious test case and does not require check of UE output power if it is ensured that the same test point is tested inside A-MPR test.

A-MPR for CA_NS_205 is defined in 38.101-2 [1] as:

For power class 1, For intra-band contiguous UL CA with single contiguous RB allocation, A-MPR for CA_NS_205 shall be 9.0 dB if offset frequency < BWintraCA, 6.0 dB otherwise.

For intra-band contiguous UL CA with non-contiguous RB allocation or non-contiguous UL CA, A-MPR for CA_NS_205 shall be 13.0 dB if offset frequency < BWintraCA, 6.0 dB otherwise.

Offset frequency is defined as the frequency from the upper edge of the protected frequency range i.e. 24 GHz to the lower edge of the aggregated channel bandwidth of the configured UL CA.

For power class 2, A-MPR for CA_NS_205 specified in sub-clause 6.2A.3.6.3 applies.

For power class 3, For intra-band contiguous UL CA with single contiguous RB allocation, A-MPR for CA_NS_205 shall be 5.0 dB if offset frequency < BWintraCA, 0.0 dB otherwise.

For intra-band contiguous UL CA with non-contiguous RB allocation or non-contiguous UL CA, A-MPR for CA_NS_205 shall be 8.0 dB if offset frequency < BWintraCA, 0.0 dB otherwise.

Offset frequency is defined as the frequency from the upper edge of the protected frequency range i.e. 24 GHz to the lower edge of the aggregated carrier bandwidth of the configured UL CA.

For power class 4, A-MPR for CA_NS_205 specified in sub-clause 6.2A.3.6.3 applies.

For power class 5, A-MPR for CA_NS_205 specified in sub-clause 6.2A.3.6.3 applies.

For power class 6, A-MPR for CA_NS_205 specified in sub-clause 6.2A.3.6.3 applies.

- Spurious for CA_NS_205 is defined in 38.101-2 [1] as:

When "CA_NS_205" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.6-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) as defined in section 6.5A.3.

Table 6.5.3.2.6-1: Additional requirements (NS_205)

| Frequency Range | Maximum Level | Measurement bandwidth |
| --- | --- | --- |
| 23.6 GHz f 24.0 GHz | -5 dBm | 200 MHz |

Test Environment

RF emission requirements for CA_NS_205 are defined as Spurious emissions. Regular spurious emissions for FR2 is tested in Normal conditions, and the same selection is appropriate also for Additional Spurious emission.

Test Frequencies

Protected range for CA_NS_205 is below band n257 and n258. It is judged that the worst case is when the carrier frequency is as close as possible to the protected range, which implies Low frequency range.

The A-MPR requirements is dependent on whether the offset frequency is less than BWintraCA, where the offset frequency is the frequency from the upper edge of the protected frequency range i.e. 24 GHz to the lower edge of the aggregated channel bandwidth of the configured UL CA. For contiguous UL CA with single contiguous RB allocation, such as band CA_n257I, and band CA_n258I with BWintraCA <400MHz, the offset frequency is always larger than BWintraCA. For band CA_n258I with BWintraCA>=400MHz, the offset frequency could be less than BWintraCA when using Low frequency range. To further verify the A-MPR requirements corresponding to offset frequency >= BWintraCA, Mid frequency range could be tested additionally.For intra-band contiguous UL CA with non-contiguous RB allocation or non-contiguous UL CA, due to a lack of industrial demand, further analysis will be conducted in the future.

Test Channel Bandwidth

The A-MPR requirements are the same for a particular power class for any channel bandwidth. Therefore, test channel bandwidth can be decided based on what is most demanding for Spurious emission. Selection can therefore be reused from normal spurious test case which means Highest aggregated channel bandwidth.

  - Test Subcarrier Spacing

RF emission requirements for CA_NS_205 are defined as Spurious emissions. Regular spurious emissions for FR2 is tested in Lowest SCS, and the same selection is appropriate also for Additional Spurious emission.

However, the MPR test is using 120 kHz SCS as a simplification since it is the only supported SCS at 400 MHz channel Bandwidth. Therefore, the proposal is changed to test only 120 kHz SCS.

  - UL Modulation

RF emission requirements for CA_NS_205 are defined as Spurious emissions. Regular spurious emissions for FR2 is tested with QPSK DFT-s-OFDM modulation to maximize UE output power (lowest power backoff). Testing at a high output power is considered more important than testing a more demanding modulation scheme for Spurious testing.

  - Uplink RB allocation

RF emission requirements for CA_NS_205 are defined as Spurious emissions.

For PC2, PC3, PC4, PC5 and PC6:

For power class 2 to PC5, and for all UL modulations and cumulative aggregated bandwidths, MPRC_CA for intra-band contiguous UL CA with contiguous allocations within the cumulative aggregated bandwidth is larger than A-MPR or equal to A-MPR otherwise and hence does not need to be considered in the test point selection.

For PC1:

For intra-band contiguous UL CA with single contiguous RB allocation, A-MPR is 9.0 dB if offset frequency < BWintraCA,  6.0 dB otherwise, which is always larger than MPR with DFT-s-OFDM QPSK modulation (such as band CA_n257I, and band CA_n258I with BWintraCA <=400MHz). A worst case to test will then be an outer allocation closest to the band edge with high order modulation.

Inner_Partial allocation is appropriate due to high MPRnarrow values applicable to Inner_1RB waveform. In order to avoid possible test ability issue caused by large MPR value, it can be considered to use partial allocation of 0 MPR

Check of alignment of test parameters between power measurement test case (A-MPR) and RF emissions test case (A-Spur)

Now that test point proposal is completed, it is required to check that all test points selected in RF emissions test case (Spurious in this case) is covered in A-MPR test case.

Result:

The test points in this paper matches the test points in A-Spurious and A-MPR test cases in TS38.521-2 for all power classes.

Proposal

Following test points are proposed for CA_NS_205 A-MPR and A-Spur test cases (PC1)

- Test environment: Normal Conditions

- Test frequencies: Low range for band CA_n257I and CA_n258I with all test channel bandwidth, and Mid range for band CA_n258I with BWintraCA>=400MHz

- Test Channel Bandwidth: Highest aggregated channel bandwidth

- Test SCS: 120 kHz

- Modulation: DFT-s-OFDM QPSK

- RB allocation and modulation: Inner_Partial, Outer_Full

Number of Test Points

The number of test points for CA_NS_205 A-MPR is 1 for band CA_n257I, 2 for band CA_n258I

The number of test points for CA_NS_205 A-Spurious is 1 for band CA_n257I, 2 for band CA_n258I

References

- TS 38.101-2 v19.2.0 User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone (Release 19)