3GPP TSG-RAN5 Meeting #109	R5-255964

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | 0106 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of band 252 into A-MPR TC for category M1 and NB-IoT NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_FDD_S_band-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The A-MPR requirement for category M1 and NB-IoT NTN band 252 is missing and needs to be added |  |  |  |  |  |  |  |  |
| Summary of change: |  | The A-MPR requirement for category M1 and NB-IoT NTN band 252 have been added into clause 6.2A.1 and 6.2B.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Band 252 A-MPR requirement will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.3, 6.2B.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | WIC belongs to Rel-19 that will trigger the specific upgrade to Rel-19 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

6.2A.3	UE additional maximum output power reduction for category M1 UE

Editor’s note: The following aspects are either missing or not yet determined:

- Mapping of NS values to AdditionalSpectrumEmission information element for NS_03N, NS_04N, NS_ 05N, NS_06N, NS_07N, NS_08N, NS_11N and NS_12N.

- Band 254 Minimum conformance requirements for NS_11N and NS_12N

- Band 254 Test configurations tables for NS_03N, NS_04N, NS_05N, NS_11N and NS_12N

- Band 254 Test requirements for NS_03N, NS_04N, NS_05N, NS_11N and NS_12N

- Band 252 Test configurations tables for NS_06N, NS_07N and NS_08N

- Band 252 Test requirements for NS_06N, NS_07N and NS_08N

6.2A.3.1	Test purpose

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction A-MPR is allowed for the output power as specified in Table 6.2A.1-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

6.2A.3.2	Test applicability

The requirements of this test apply in test case 6.5A.3.3 Additional Spectrum Emission Mask for network signalled values NS_02N and NS-24 to all types of E-UTRA UE release 17 and forward of UE category M1 that support satellite access operation.

The requirements of this test apply in test case 6.5A.4.4 Additional Spurious Emissions for network signalled values NS_02N, NS_03N, NS_04N, NS_05N, NS_06N, NS_07N, NS_08N,  NS_11N, NS_12N, and NS_24 to all types of E-UTRA UE release 17 and forward of UE category M1 that support satellite access operation.

6.2A.3.3	Minimum conformance requirements

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power as specified in Table 6.2A.1-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For UE Power Class 3 and 5 the specific requirements and identified subclauses are specified in Table 6.2A.3-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2A.3-1 are in addition to the allowed MPR requirements specified in subclause 6.2A.2.

Table 6.2A.3-1: Additional Maximum Power Reduction (A-MPR) for category M1 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Resources Blocks (NRB) | A-MPR (dB) |  |
| --- | --- | --- | --- | --- | --- |
| NS_01 | 6.5A.4.2.3 | Table 5.2-1 | Table 5.3A-1 | N/A |  |
| NS_02N | 6.5A.4.4.3.1 | 255 | Table 5.3A-1 | N/A |  |
| NS_03N | 6.5A.4.4.3.3 | 254 | Table 5.3A-1 | N/A |  |
| NS_04N | 6.5A.4.4.3.4 | 254 | Table 5.3A-1 | N/A |  |
| NS_05N | 6.5A.4.4.3.4 | 254 | Table 5.3A-1 | N/A |  |
| NS_06N | 6.5A.3.3.3, 6.5A.4.4.5 | 252 | Table 5.3A-1 | N/A |  |
| NS_07N | 6.5A.4.4.8 6.5A.3.3.1 | 252 | Table 5.3A-1 | N/A |  |
| NS_08N | 6.5A.4.4.9 6.5A.3.3.1 | 252 | Table 5.3A-1 | N/A |  |
| NS_11N | 6.5A.4.4.3.4 | 254 | Table 5.3A-1 | N/A |  |
| NS_12N |  | 254 | Table 5.3A-1 | N/A |  |
| NS_24 | 6.5A.4.4.3.2 | 256 | Table 5.3A-1 | PC3 | PC5 |
|  |  |  |  | ≤3.5 | ≤0.5 |

For subPRB allocation, the allowed A-MPR values specified below in Table 6.2A.3-2 for category M1 UE are in addition to the allowed MPR requirements specified in subclause 6.2A.2.

Table 6.2A.3-2: Additional Maximum Power Reduction (A-MPR) for category M1 UE for subPRB allocation

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5A.4.2.3 | Table 5.2-1 | N/A |  |
| NS_02N | 6.5A.4.4.3.1 | 255 | N/A |  |
| NS_03N | 6.5A.4.4.3.3 | 254 | N/A |  |
| NS_04N | 6.5A.4.4.3.4 | 254 | N/A |  |
| NS_05N | 6.5A.4.4.3.4 | 254 | N/A |  |
| NS_06N | 6.5A.4.4.7 6.5A.3.3.1 | 252 | N/A |  |
| NS_07N | 6.5A.4.4.8 6.5A.3.3.1 | 252 | N/A |  |
| NS_08N | 6.5A.4.4.9 6.5A.3.3.1 | 252 | N/A |  |
| NS_24 | 6.5A.4.4.3.2 | 256 | PC3 | PC5 |
|  |  |  | ≤3.5 | ≤0.5 |

The normative reference for this requirement is TS 36.102 [11] subclause 6.2A.3

6.2A.3.4	Test description

6.2A.3.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in sub-clause 5.2A. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in Table 6.2A.3.4.1-1 to 6.2.A.3.4.1-10a. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.2A.3.4.1-1: Test Configuration Table (network signalled value “NS_02N”)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] subclause 4.3.1 |  |  | 1.4MHz |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |
|  |  |  |  | FDD and HD-FDD |
| 1 | 1.4MHz |  | QPSK | 2 |  |
| 2 | 1.4MHz |  | QPSK | 5 |  |
| 3 | 1.4MHz |  | QPSK | 6 |  |
| 4 | 1.4MHz |  | 16QAM | 2 |  |
| 5 | 1.4MHz |  | 16QAM | 5 |  |
| Note 1: The RBstart of partial RB allocation shall be RB#0 |  |  |  |  |  |

Table 6.2A.3.4.1-1a: Test Configuration Table, subPRB allocation (network signalled value “NS_02N”)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 4.3.1 |  |  | Low Range, Mid range, High Range |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] subclause 4.3.1 |  |  | 1.4MHz |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |
|  |  |  |  | FDD and HD-FDD |
| 1 | 1.4MHz |  | QPSK | ½ |  |
| Note 1: the SCstart shall be 0. |  |  |  |  |  |

Table 6.2A.3.4.1-2: Test Configuration Table (network signalled value "NS_24")

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] subclause 4.3.1 |  |  | 1.4MHz |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |
|  |  |  |  | FDD and HD-FDD |
| 1 | 1.4MHz |  | QPSK | 2 |  |
| 2 | 1.4MHz |  | QPSK | 5 |  |
| 3 | 1.4MHz |  | QPSK | 6 |  |
| 4 | 1.4MHz |  | 16QAM | 2 |  |
| 5 | 1.4MHz |  | 16QAM | 5 |  |
| Note 1: The RBstart of partial RB allocation shall be RB#0 |  |  |  |  |  |

Table 6.2A.3.4.1-2a: Test Configuration Table, subPRB allocation (network signalled value "NS_24")

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 4.3.1 |  |  | Low Range, Mid range, High Range |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] subclause 4.3.1 |  |  | 1.4MHz |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |
|  |  |  |  | FDD and HD-FDD |
| 1 | 1.4MHz |  | QPSK | ½ |  |
| Note 1: the SCstart shall be 0. |  |  |  |  |  |

Table 6.2A.3.4.1-3: Test Configuration Table (network signalled value "NS_03N")

FFS

Table 6.2A.3.4.1-3a: Test Configuration Table, subPRB allocation (network signalled value "NS_03N")

FFS

Table 6.2A.3.4.1-4: Test Configuration Table (network signalled value "NS_04N")

FFS

Table 6.2A.3.4.1-4a: Test Configuration Table, subPRB allocation (network signalled value "NS_04N")

FFS

Table 6.2A.3.4.1-5: Test Configuration Table (network signalled value "NS_05N")

FFS

Table 6.2A.3.4.1-5a: Test Configuration Table, subPRB allocation (network signalled value "NS_05N")

FFS

Table 6.2A.3.4.1-6: Test Configuration Table (network signalled value "NS_11N")

FFS

Table 6.2A.3.4.1-6a: Test Configuration Table, subPRB allocation (network signalled value "NS_11N")

FFS

Table 6.2A.3.4.1-7: Test Configuration Table (network signalled value "NS_12N")

FFS

Table 6.2A.3.4.1-7a: Test Configuration Table, subPRB allocation (network signalled value "NS_12N")

FFS

Table 6.2A.3.4.1-8: Test Configuration Table (network signalled value "NS_06N")

FFS

Table 6.2A.3.4.1-8a: Test Configuration Table, subPRB allocation (network signalled value "NS_06N")

FFS

Table 6.2A.3.4.1-9: Test Configuration Table (network signalled value "NS_07N")

FFS

Table 6.2A.3.4.1-9a: Test Configuration Table, subPRB allocation (network signalled value "NS_07N")

FFS

Table 6.2A.3.4.1-10: Test Configuration Table (network signalled value "NS_08N")

FFS

Table 6.2A.3.4.1-10a: Test Configuration Table, subPRB allocation (network signalled value "NS_08N")

FFS

==============Next change==============

6.2A.3.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 4.6 with the condition CEModeA and with the following exceptions for each network signalled value.

6.2A.3.4.3.1	Message contents exceptions (network signalled value “NS_02N”)

1.	Information element additionalSpectrumEmission is set to NS_02N. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2A.3.4.3.1-1: SystemInformationBlockType2: Additional spurious emissions test requirement for “NS_02N”

| Derivation Path: TS 36.508 [12] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2(NS-02N) |  |  |

6.2A.3.4.3.2	Message contents exceptions (network signalled value "NS_24")

1.	Information element additionalSpectrumEmission is set to NS_24. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2A.3.4.3.2-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_24"

| Derivation Path: TS 36.508 [12] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 24 (NS_24) |  |  |

6.2A.3.4.3.3	Message contents exceptions (network signalled value "NS_03N")

1.	Information element additionalSpectrumEmission is set to NS_03N. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2A.3.4.3.3-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_03N"

| Derivation Path: TS 36.508 [12] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | TBD (NS_03N) |  |  |

6.2A.3.4.3.4	Message contents exceptions (network signalled value "NS_04N")

1.	Information element additionalSpectrumEmission is set to NS_04N. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2A.3.4.3.4-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_04N"

| Derivation Path: TS 36.508 [12] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | TBD (NS_04N) |  |  |

6.2A.3.4.3.5	Message contents exceptions (network signalled value "NS_05N")

1.	Information element additionalSpectrumEmission is set to NS_05N. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2A.3.4.3.5-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_05N"

| Derivation Path: TS 36.508 [12] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | TBD (NS_05N) |  |  |

6.2A.3.4.3.6	Message contents exceptions (network signalled value "NS_11N")

1.	Information element additionalSpectrumEmission is set to NS_11N. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2A.3.4.3.6-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_11N"

| Derivation Path: TS 36.508 [12] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | TBD (NS_11N) |  |  |

6.2A.3.4.3.7	Message contents exceptions (network signalled value "NS_12N")

1.	Information element additionalSpectrumEmission is set to NS_12N. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2A.3.4.3.7-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_12N"

| Derivation Path: TS 36.508 [12] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | TBD (NS_12N) |  |  |

6.2A.3.4.3.8	Message contents exceptions (network signalled value "NS_06N")

1.	Information element additionalSpectrumEmission is set to NS_06N. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2A.3.4.3.8-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_06N"

| Derivation Path: TS 36.508 [12] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | TBD (NS_06N) |  |  |

6.2A.3.4.3.9	Message contents exceptions (network signalled value "NS_07N")

1.	Information element additionalSpectrumEmission is set to NS_07N. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2A.3.4.3.9-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_07N"

| Derivation Path: TS 36.508 [12] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | TBD (NS_07N) |  |  |

6.2A.3.4.3.10	Message contents exceptions (network signalled value "NS_08N")

1.	Information element additionalSpectrumEmission is set to NS_08N. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2A.3.4.3.10-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_08N"

| Derivation Path: TS 36.508 [12] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | TBD (NS_08N) |  |  |

6.2A.3.5	Test requirements

The maximum output power derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2.4A.5-1 to Table 6.2.4A.5-10a. The allowed A-MPR values specified in Table 6.2.4A-1 are in addition to the allowed MPR requirements specified in clause 6.2.3 A. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.5 A.3-1 and 6.2.5A.3-2 apply.

Table 6.2A.3.5-1: UE Power Class test requirements (network signalled value “NS_02N”)

| Config ID | Power class 3 |  |  |  |  |  | Power class 5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | A_MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | MPR (dB) | A_MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | N/A | 23 | 2 | 25.7 | 20.3 | 0 | N/A | 20 | 2 | 22.7 | 17.3 |
| 2 | 1 | N/A | 22 | 2 | 25.7 | 19.3 | 1 | N/A | 19 | 3.5 | 22.7 | 14.8 |
| 3 | 2 | N/A | 21 | 2 | 25.7 | 18.3 | 2 | N/A | 18 | 4 | 22.7 | 13.3 |
| 4 | 1 | N/A | 22 | 2 | 25.7 | 19.3 | 1 | N/A | 19 | 3.5 | 22.7 | 14.8 |
| 5 | 2 | N/A | 21 | 2 | 25.7 | 18.3 | 2 | N/A | 18 | 4 | 22.7 | 13.3 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [7] clause 6.2.5 |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.5-1a: UE Power Class test requirements, subPRB allocation (network signalled value “NS_02N”)

| Config ID | Power class 3 |  |  |  |  |  | Power class 5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | A_MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | MPR (dB) | A_MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | N/A | 23 | 2 | 25.7 | 20.3 | 0 | N/A | 20 | 2 | 22.7 | 17.3 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [7] clause 6.2.5 |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.5-2: UE Power Class test requirements (network signalled value "NS_24")

| Config ID | Power class 3 |  |  |  |  |  | Power class 5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | A-MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | MPR (dB) | A-MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 0 | 0.5 | 19.5 | 3.5 | 22.7 | 15.3 |
| 2 | 1 | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 1 | 0.5 | 19.0 | 3.5 | 22.7 | 14.8 |
| 3 | 2 | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 0.5 | 18.0 | 4.0 | 22.7 | 13.3 |
| 4 | 1 | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 1 | 0.5 | 19.0 | 3.5 | 22.7 | 14.8 |
| 5 | 2 | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 0.5 | 18.0 | 4.0 | 22.7 | 13.3 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [7] clause 6.2.5 |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.5-2a: UE Power Class test requirements, subPRB allocation (network signalled value "NS_24")

| Config ID | Power class 3 |  |  |  |  |  | Power class 5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | A-MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | MPR (dB) | A-MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 0 | 0.5 | 19.5 | 3.5 | 22.7 | 15.3 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [7] clause 6.2.5 |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.5-3: UE Power Class test requirements (network signalled value "NS_03N")

FSS

Table 6.2A.3.5-3a: UE Power Class test requirements, subPRB allocation (network signalled value "NS_03N")

FSS

Table 6.2A.3.5-4: UE Power Class test requirements (network signalled value "NS_04N")

FSS

Table 6.2A.3.5-4a: UE Power Class test requirements, subPRB allocation (network signalled value "NS_04N")

FSS

Table 6.2A.3.5-5: UE Power Class test requirements (network signalled value "NS_05N")

FSS

Table 6.2A.3.5-5a: UE Power Class test requirements, subPRB allocation (network signalled value "NS_05N")

FSS

Table 6.2A.3.5-6: UE Power Class test requirements (network signalled value "NS_11N")

FSS

Table 6.2A.3.5-6a: UE Power Class test requirements, subPRB allocation (network signalled value "NS_11N")

FSS

Table 6.2A.3.5-7: UE Power Class test requirements (network signalled value "NS_12N")

FSS

Table 6.2A.3.5-7a: UE Power Class test requirements, subPRB allocation (network signalled value "NS_12N")

FSS

Table 6.2A.3.5-8: UE Power Class test requirements (network signalled value "NS_06N")

FSS

Table 6.2A.3.5-8a: UE Power Class test requirements, subPRB allocation (network signalled value "NS_06N")

FSS

Table 6.2A.3.5-9: UE Power Class test requirements (network signalled value "NS_07N")

FSS

Table 6.2A.3.5-9a: UE Power Class test requirements, subPRB allocation (network signalled value "NS_07N")

FSS

Table 6.2A.3.5-10: UE Power Class test requirements (network signalled value "NS_08N")

FSS

Table 6.2A.3.5-10a: UE Power Class test requirements, subPRB allocation (network signalled value "NS_08N")

FSS

==============Next change==============

6.2B.3	UE additional maximum output power reduction for category NB1 and NB2 UE

Editor’s note: The following aspects are either missing or not yet determined:

- Mapping of NS values to AdditionalSpectrumEmission information element for NS_03N, NS_04N, NS_ 05N, NS_06N, NS_07N, NS_08N, NS_11N and NS_12N.

- Band 254 Minimum conformance requirements for NS_11N and NS_12N

- Band 254 Test configurations tables for NS_03N, NS_04N, NS_05N, NS_11N and NS_12N

- Band 254 Test requirements for NS_03N, NS_04N, NS_05N, NS_11N and NS_12N

- Band 252 Test configurations tables for NS_06N, NS_07N and NS_08N

- Band 252 Test requirements for NS_06N, NS_07N and NS_08N

6.2B.3.1	Test purpose

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction A-MPR is allowed for the output power as specified in Table 6.2B.1-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

6.2B.3.2	Test applicability

The requirements of this test apply in test case 6.5B.4.4 Additional Spurious Emissions for network signalled values NS_02N, NS_03N, NS_04N, NS_05N, NS_06N, NS_07N, NS_08N, NS_11N, NS_12N, and NS_24 to all types of NB-IOT NTN UE release 17 and forward of UE category NB1 and NB2 that support satellite access operation.

6.2B.3.3	Minimum conformance requirements

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power as specified in Table 6.2B.1-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For UE Power Class 3 and 5 the specific requirements and identified subclauses are specified in Table 6.2B.3-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2B.3.3-1 are in addition to the allowed MPR requirements specified in subclause 6.2B.2.

Table 6.2B.3.3-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | NB-IOT NTN Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5B.4.2 | Table 5.2-1 | N/A |  |
| NS_02N | 6.5B.4.4.3.1 | 255 | N/A |  |
| NS_03N | 6.5B.4.4.3.3 | 254 | N/A |  |
| NS_04N | 6.5B.4.4.3.5 | 254 |  |  |
| NS_05N | 6.5B.4.4.3.5 | 254 |  |  |
| NS_06N | 6.5B.4.4.7 6.5B.3.3.1 | 252 | N/A |  |
| NS_07N | 6.5B.4.4.8 6.5B.3.3.1 | 252 | N/A |  |
| NS_08N | 6.5B.4.4.9 6.5B.3.3.1 | 252 | N/A |  |
| NS_11N | 6.5B.4.4.3.5 | 254 |  |  |
| NS_12N |  | 254 |  |  |
| NS_24 | 6.5B.4.4.3 | 256 | PC3 | PC5 |
|  |  |  | ≤3.5 | ≤ 0.5 |

The normative reference for this requirement is TS 36.102 [11] subclause 6.2B.3

6.2B.3.4	Test description

6.2B.3.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NB-IOT NTN operating bands specified in sub-clause 5.2B. All these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in Table 6.2B.3.4.1-1 to 6.2B.3.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of NPDSCH and NPDCCH before measurement are specified in Annex C.2.

Table 6.2B.3.4.1-1: Test Configuration Table (network signalled value “NS_02N”)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 |  | NC |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Low range, Mid range, High range |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A for Maximum Power Reduction (MPR) test case | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 (Note 1) |  | QPSK | 3@0 | 15 |
| 2 (Note 1) |  | QPSK | 3@3 | 15 |
| 3 (Note 1) |  | QPSK | 3@9 | 15 |
| 4 (Note 1) |  | QPSK | 6@0 | 15 |
| 5 (Note 1) |  | QPSK | 6@6 | 15 |
| 6 (Note 1) |  | QPSK | 12@0 | 15 |
| Note 1: Applicable to UE supporting UL multi-tone transmissions |  |  |  |  |

Table 6.2B.3.4.1-2: Test Configuration Table (network signalled value "NS_24")

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 |  | NC |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Low range, Mid range, High range |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A for Maximum Power Reduction (MPR) test case | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 (Note 1) |  | QPSK | 3@0 | 15 |
| 2 (Note 1) |  | QPSK | 3@3 | 15 |
| 3 (Note 1) |  | QPSK | 3@9 | 15 |
| 4 (Note 1) |  | QPSK | 6@0 | 15 |
| 5 (Note 1) |  | QPSK | 6@6 | 15 |
| 6 (Note 1) |  | QPSK | 12@0 | 15 |
| Note 1: Applicable to UE supporting UL multi-tone transmissions |  |  |  |  |

Table 6.2B.3.4.1-3

FFS

Table 6.2B.3.4.1-4: Test Configuration Table (network signalled value "NS_04N")

FFS

Table 6.2B.3.4.1-5: Test Configuration Table (network signalled value "NS_05N")

FFS

Table 6.2B.3.4.1-6: Test Configuration Table (network signalled value "NS_11N")

FFS

Table 6.2B.3.4.1-7: Test Configuration Table (network signalled value "NS_12N")

FFS

Table 6.2B.3.4.1-8: Test Configuration Table (network signalled value "NS_06N")

FFS

Table 6.2B.3.4.1-9: Test Configuration Table (network signalled value "NS_07N")

FFS

Table 6.2B.3.4.1-10: Test Configuration Table (network signalled value "NS_08N")

FFS

==============Next change==============

6.2B.3.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 8.1.6 and with the following exceptions for each network signalled value.

6.2B.3.4.3.1	Message contents exceptions (network signalled value “NS_02N”)

Information element additionalSpectrumEmission is set to NS_02N. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2B.3.4.3.1-1: SystemInformationBlockType2-NB: Additional spurious emissions test requirement for “NS_02N”

| Derivation Path: TS 36.508 [12] clause 8.1.4.3, Table 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2(NS_02N) |  |  |

6.2B.3.4.3.2	Message contents exceptions (network signalled value "NS_24")

Information element additionalSpectrumEmission is set to NS_24. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2B.3.4.3.2-1: SystemInformationBlockType2-NB: Additional spurious emissions test requirement for "NS_24"

| Derivation Path: TS 36.508 [12] clause 8.1.4.3, Table 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 24 (NS_24) |  |  |

6.2B.3.4.3.3	Message contents exceptions (network signalled value "NS_03N")

1.	Information element additionalSpectrumEmission is set to NS_03N. This can be set in the SystemInformationblockType2-NB as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2B.3.4.3.3-1: SystemInformationBlockType2-NB: Additional spurious emissions test requirement for "NS_03N"

| Derivation Path: TS 36.508 [12] clause 8.1.4.3, Table 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | TBD (NS_03N) |  |  |

6.2B.3.4.3.4	Message contents exceptions (network signalled value "NS_04N")

1.	Information element additionalSpectrumEmission is set to NS_04N. This can be set in the SystemInformationblockType2-NB as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2B.3.4.3.4-1: SystemInformationBlockType2-NB: Additional spurious emissions test requirement for "NS_04N"

| Derivation Path: TS 36.508 [12] clause 8.1.4.3, Table 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | TBD (NS_04N) |  |  |

6.2B.3.4.3.5	Message contents exceptions (network signalled value "NS_05N")

1.	Information element additionalSpectrumEmission is set to NS_05N. This can be set in the SystemInformationblockType2-NB as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2B.3.4.3.5-1: SystemInformationBlockType2-NB: Additional spurious emissions test requirement for "NS_05N"

| Derivation Path: TS 36.508 [12] clause 8.1.4.3, Table 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | TBD (NS_05N) |  |  |

6.2B.3.4.3.6	Message contents exceptions (network signalled value "NS_11N")

1.	Information element additionalSpectrumEmission is set to NS_11N. This can be set in the SystemInformationblockType2 -NB as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2B.3.4.3.6-1: SystemInformationBlockType2-NB: Additional spurious emissions test requirement for "NS_11N"

| Derivation Path: TS 36.508 [12] clause 8.1.4.3, Table 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | TBD (NS_11N) |  |  |

6.2B.3.4.3.7	Message contents exceptions (network signalled value "NS_12N")

1.	Information element additionalSpectrumEmission is set to NS_12N. This can be set in the SystemInformationblockType2-NB as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2B.3.4.3.7-1: SystemInformationBlockType2-NB: Additional spurious emissions test requirement for "NS_12N"

| Derivation Path: TS 36.508 [12] clause 8.1.4.3, Table 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | TBD (NS_12N) |  |  |

6.2B.3.4.3.8	Message contents exceptions (network signalled value "NS_06N")

1.	Information element additionalSpectrumEmission is set to NS_06N. This can be set in the SystemInformationblockType2-NB as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2B.3.4.3.8-1: SystemInformationBlockType2-NB: Additional spurious emissions test requirement for "NS_06N"

| Derivation Path: TS 36.508 [12] clause 8.1.4.3, Table 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | TBD (NS_06N) |  |  |

6.2B.3.4.3.9	Message contents exceptions (network signalled value "NS_07N")

1.	Information element additionalSpectrumEmission is set to NS_07N. This can be set in the SystemInformationblockType2-NB as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2B.3.4.3.9-1: SystemInformationBlockType2-NB: Additional spurious emissions test requirement for "NS_07N"

| Derivation Path: TS 36.508 [12] clause 8.1.4.3, Table 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | TBD (NS_07N) |  |  |

6.2B.3.4.3.10	Message contents exceptions (network signalled value "NS_08N")

1.	Information element additionalSpectrumEmission is set to NS_08N. This can be set in the SystemInformationblockType2-NB as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2B.3.4.3.10-1: SystemInformationBlockType2-NB: Additional spurious emissions test requirement for "NS_08N"

| Derivation Path: TS 36.508 [12] clause 8.1.4.3, Table 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | TBD (NS_08N) |  |  |

6.2B.3.5	Test requirement

The maximum output power derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2B.3.5-1 to Table 6.2B.3.5-11. The allowed A-MPR values specified in Table 6.2B.3.3-1 are in addition to the allowed MPR requirements specified in clause 6.2B.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in TS.36.521-1[14] Table 6.2.5 F.3-1 and 6.2.5F.3-2 apply.

Table 6.2B.3.5-1: UE Power Class test requirements (network signalled value “NS_02N”)

| Test ID | MPR (dB) | A_MPR (dB) | Power class 3 |  |  |  | Power class 5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0.5 | N/A | 22.5 | 2.0 | 25.7 | 19.8 | 19.5 | 2.0 | 22.7 | 16.8 |
| 2 | 0 | N/A | 23 | 2.0 | 25.7 | 20.3 | 20 | 2.0 | 22.7 | 17.3 |
| 3 | 0.5 | N/A | 22.5 | 2.0 | 25.7 | 19.8 | 19.5 | 2.0 | 22.7 | 16.8 |
| 4 | 1 | N/A | 22 | 2.0 | 25.7 | 19.3 | 19 | 2.0 | 22.7 | 16.3 |
| 5 | 1 | N/A | 22 | 2.0 | 25.7 | 19.3 | 19 | 2.0 | 22.7 | 16.3 |
| 6 | 2 | N/A | 21 | 2.0 | 25.7 | 18.3 | 18 | 2.0 | 22.7 | 15.3 |
| NOTE 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [7] clause 6.2.5F |  |  |  |  |  |  |  |  |  |  |

Table 6.2B.3.5-2: UE Power Class test requirements (network signalled value "NS_24")

| Test ID | MPR (dB) | A_MPR (dB) | Power class 3 |  |  |  | Power class 5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0.5 | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 19.5 | 3.5 | 22.7 | 15.3 |
| 2 | 0 | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 19.5 | 3.5 | 22.7 | 15.3 |
| 3 | 0.5 | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 19.5 | 3.5 | 22.7 | 15.3 |
| 4 | 1 | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 19.0 | 3.5 | 22.7 | 15.3 |
| 5 | 1 | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 19.0 | 3.5 | 22.7 | 14.8 |
| 6 | 2 | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 18.0 | 4.0 | 22.7 | 13.3 |
| NOTE 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [7] clause 6.2.5F |  |  |  |  |  |  |  |  |  |  |

Table 6.2B.3.5-3

FFS

Table 6.2B.3.5-4: UE Power Class test requirements (network signalled value "NS_03N")

FSS

Table 6.2B.3.5-5: UE Power Class test requirements (network signalled value "NS_04N")

FSS

Table 6.2B.3.5-6: UE Power Class test requirements (network signalled value "NS_05N")

FSS

Table 6.2B.3.5-7: UE Power Class test requirements (network signalled value "NS_11N")

FSS

Table 6.2B.3.5-8: UE Power Class test requirements (network signalled value "NS_12N")

FSS

Table 6.2B.3.5-9: UE Power Class test requirements (network signalled value "NS_06N")

FSS

Table 6.2B.3.5-10: UE Power Class test requirements (network signalled value "NS_07N")

FSS

Table 6.2B.3.5-11: UE Power Class test requirements (network signalled value "NS_08N")

FSS

==============End of change==============