3GPP TSG RAN WG5 Meeting #109	R5-255983

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

Agenda Item:	6.6.9.7

Source:	Keysight Technologies UK

Title:	Discussion for handling of 200 OK in response to SIP INVITE for MCX Client Terminated Call

Document for:	Endorsement

# 1	Introduction

For MCX Client Terminated call testing, there is ambiguity in the contents of MIME body of 200 OK message that is sent in response to SIP INVITE message.

# 2	Discussion

For MCX Client Terminated call testing, the current test specifications refer (directly, or indirectly) to TS 37.579-1 Table 5.5.2.17.1.1-1 for 200 OK response to SIP INVITE, that include both SDP and MCS Info in the MIME body by default.

However, in the core specifications (TS 24.379 for MCPTT, TS 24.281 for MCVideo, and TS 24.282 for MCData), there is no requirement for the called client to include MCS Info in the 200 OK responding to SIP INVITE, except for specific scenarios (e.g. TS 24.379 cl. 6.2.3.1.1, case 7a).

Proposal 2-1

It is proposed to update the MCX conformance specification to allow Client behaviour that does not include the MCS Info in the 200 OK sent in response to SIP INVITE as proposed in [1] and update the affected test cases because of subsequent implementation of [1].

# 3	Proposal

It is proposed to implement the proposal 2-1.

# 4	References

[1] R5-255981: Correction to 200 OK in response to SIP INVITE for MCX Client Terminated Call