3GPP TSG-RAN5 Meeting #109	R5-256028

# Dallas, United States, Nov 17-21, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5197 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 7.1.1.14.2.2 NES spatial domain Type 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The existing checking steps in test sequence are hard to verify the CSI sub-configurations. And there are some typos in the message contents. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update checking steps as: - Check 1 CSI report at step 5 with only sub-configuration #1 activated. - Check 2 CSI reports at step 8 with two subconfigurations activated. - The message tables for 2 NZP CSI-RS reqsources are splitted for readability. - Fix typos in message contents. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test purpose could not be verified properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.14.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

7.1.1.14.2.2	NES spatial domain Type 2 adaptation / Aperiodic CSI reporting

7.1.1.14.2.2.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and supporting spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for aperiodic CSI reporting }

ensure that {

when { UE receives RRCReconfiguration message with CSI-ReportConfig that contains a list of sub-configurations each corresponding to a list of one or more CSI-RS resources }

then { UE performs aperiodic CSI reporting on PUSCH as indicated by CSI triggering state for aperiodic CSI reporting }

}

7.1.1.14.2.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.214, clauses 5.2.1.4.2, 5.2.1.5.1 and 5.2.3. Unless otherwise stated these are Rel-18 requirements.

[TS 38.214, clause 5.2.1.4.2]

…

If the UE is configured with a CSI-ReportConfig that contains a list of sub-configurations, provided by csi-ReportSubConfigToAddModList:

…

-	A sub-configuration can be configured with a list of NZP CSI-RS resources, provided by nzp-CSI-RS-ResourceList, which indicates one or more NZP CSI-RS resources, within a NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement which corresponds to the CSI-ReportConfig. If there is no sub-configuration configured with a power offset provided by powerOffset, the list of NZP CSI-RS resources has no intersection with a list of NZP CSI-RS resources configured for any other sub-configuration(s) within the CSI-ReportConfig, otherwise, the list of NZP CSI-RS resources is identical to or has no intersection with a list of NZP CSI-RS resources configured for any other sub-configuration(s) within the CSI-ReportConfig.

-	If a sub-configuration is configured with a list of NZP CSI-RS resources with more than one resource, the UE shall derive the CSI parameters other than CRI conditioned on the reported CRI, where the CRI k (k ≥ 0) for the sub-configuration corresponds to the configured (k+1)-th entry of associated NZP-CSI-RS-Resource in the list of NZP CSI-RS resources.

-	If a sub-configurations is not configured with nzp-CSI-RS-ResourceList then the sub-configuration shall be associated with all the NZP CSI-RS resources within a NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement which corresponds to the CSI-ReportConfig.

-	the UE reports CSI(s) for one or more sub-configurations according to Clauses 5.2.1.5.1, 5.2.1.5.2, 5.2.3 and 5.2.4, and according to the higher layer parameter reportQuantity configured for that CSI-ReportConfig.

-	The UE does not expect the higher layer parameter reportQuantity to be set to 'cri-RSRP', 'cri-SINR', 'cri-SINR- Index', 'cri-RSRP-Index', 'none', 'ssb-Index-RSRP', 'ssb-Index-SINR', 'ssb-Index-RSRP- Index', 'ssb-Index-SINR- Index', or 'tdcp'.

…

[TS 38.214, clause 5.2.1.5.1]

For CSI-RS resource sets associated with Resource Settings configured with the higher layer parameter resourceType set to 'aperiodic', 'periodic', or 'semi-persistent', trigger states for Reporting Setting(s) (configured with the higher layer parameter reportConfigType set to 'aperiodic') and/or Resource Setting for channel and/or interference measurement on one or more component carriers are configured using the higher layer parameter CSI-AperiodicTriggerStateList. For a reporting setting for which the CSI-ReportConfig contains a list of sub-configurations provided by the higher layer parameter csi-ReportSubConfigToAddModList, one or more trigger states can be configured with each indicating one or more of the sub-configurations. For aperiodic CSI report triggering, a single set of CSI triggering states are higher layer configured, wherein the CSI triggering states can be associated with any candidate DL BWP.

[TS 38.214, clause 5.2.3]

A UE shall perform aperiodic CSI reporting using PUSCH on serving cell c upon successful decoding of a DCI format 0_1 or DCI format 0_2 which triggers an aperiodic CSI trigger state. A UE shall perform aperiodic CSI reporting using PUSCH on the serving cell with the smallest serving cell index scheduled by DCI format 0_3 which triggers an aperiodic CSI trigger state.

7.1.1.14.2.2.3	Test description

7.1.1.14.2.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cell 1.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

7.1.1.14.2.2.3.2	Test procedure sequence

Table 7.1.1.14.2.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message to configure CSI-ReportConfig containing a list of sub-configurations and CSI-ReportSubConfigTriggerList. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS transmits MAC control element to indicate aperiodic CSI trigger State. | <-- | MAC PDU (Aperiodic CSI Trigger State Subselection MAC CE (T0=1)) | - | - |
| 4 | The SS transmits a downlink assignment to trigger aperiodic CSI reporting for sub-configuration #1. (Note 1) | <-- | DCI format 0_1 | - | - |
| 5 | Check: Does the UE transmit 1 CSI report within 20ms? | --> | PUSCH (CSI report) | 1 | P |
| 6 | The SS transmits MAC control element to indicate aperiodic CSI trigger State. | <-- | MAC PDU (Aperiodic CSI Trigger State Subselection MAC CE (T1=1)) | - | - |
| 7 | The SS transmits a downlink assignment to trigger aperiodic CSI reporting for both sub-configuration #1 and sub-configuration #2. (Note 1) | <-- | DCI format 0_1 | - | - |
| 8 | Check: Does the UE transmit 2 CSI reports within 20ms? | --> | PUSCH (2 CSI reports) | 1 | P |
| Note 1: The CSI request field is value ‘1' in the DCI format 0_1. |  |  |  |  |  |

7.1.1.14.2.2.3.3	Specific message contents

Table 7.1.1.14.2.2.3.3-1: RRCReconfiguration (Step 1, Table 7.1.1.14.2.2.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | Table 7.1.1.14.2.2.3.3-2 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.14.2.2.3.3-2: CellGroupConfig

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-19 with condition MEAS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |  |  |  |  |
| spCellConfigDedicated |  | ServingCellConfig |  | Table 7.1.1.14.2.2.3.3-3 |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 7.1.1.14.2.2.3.3-3: ServingCellConfig

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig | CSI-MeasConfig | Table 7.1.1.14.2.2.3.3-4 |  |
| servingCellMO | Not present |  |  |
| } |  |  |  |

Table 7.1.1.14.2.2.3.3-4: CSI-MeasConfig

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-38 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | 2 entries |  |  |
| NZP-CSI-RS-Resource[1] | NZP-CSI-RS-Resource-1 | entry 1, CSI-RS resource 1 for sub-configuration #1 |  |
| NZP-CSI-RS-Resource[2] | NZP-CSI-RS-Resource-2 | entry 2, CSI-RS resource 2 for sub-configuration #2 |  |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSet[1] | NZP-CSI-RS-ResourceSet | entry 1, Table 7.1.1.14.2.2.3.3-5 |  |
| } |  |  |  |
| csi-SSB-ResourceSetToAddModList | Not present |  |  |
| csi-ReportConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig { | 1 entry |  |  |
| CSI-ReportConfig[1] | CSI-ReportConfig | entry 1, Table 7.1.1.14.2.2.3.3-8 |  |
| } |  |  |  |
| reportTriggerSize | 1 |  |  |
| aperiodicTriggerStateList CHOICE { |  |  |  |
| setup | CSI-AperiodicTriggerStateList | Table 7.1.1.14.2.2.3.3-9 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.14.2.2.3.3-5: NZP-CSI-RS-ResourceSet (Table 7.1.1.14.2.2.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-87 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 2 entries |  |  |
| NZP-CSI-RS-ResourceId[1] | 0 | entry 1, CSI-RS resource 1 |  |
| NZP-CSI-RS-ResourceId[2] | 1 | entry 2, CSI-RS resource 2 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.14.2.2.3.3-6: NZP-CSI-RS-Resource-1

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-85 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceId | 0 | CSI-RS resource 1 |  |
| resourceMapping | CSI-RS-ResourceMapping | Table 7.1.1.14.2.2.3.3-7 |  |
| } |  |  |  |

Table 7.1.1.14.2.2.3.3-6A: NZP-CSI-RS-Resource-2

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-85 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceId | 1 | CSI-RS resource 2 |  |
| resourceMapping | CSI-RS-ResourceMapping-2 | Table 7.1.1.14.2.2.3.3-7A |  |
| } |  |  |  |

Table 7.1.1.14.2.2.3.3-7: CSI-RS-ResourceMapping-1

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| firstOFDMSymbolInTimeDomain | 3 | l0 = 3 for CSI-RS resource 1 | FR1 |
|  | 11 | l0 = 11 for CSI-RS resource 1 | FR2 |
| } |  |  |  |

Table 7.1.1.14.2.2.3.3-7A: CSI-RS-ResourceMapping-2

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| firstOFDMSymbolInTimeDomain | 7 | l0 = 7 for CSI-RS resource 2 | FR1 |
|  | 13 | l0 = 13 for CSI-RS resource 2 | FR2 |
| } |  |  |  |

Table 7.1.1.14.2.2.3.3-8: CSI-ReportConfig (Table 7.1.1.14.2.2.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| csi-ReportSubConfigToAddModList-r18 SEQUENCE (SIZE (1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfig-r18 { | 2 entries |  |  |
| CSI-ReportSubConfig-r18[1] SEQUENCE { |  | entry 1 |  |
| reportSubConfigId-r18 | 0 | sub-configuration #1 |  |
| reportSubConfigParams-r18 CHOICE { |  |  |  |
| a2-parameters SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceIndex-r18 { | 1 entry |  |  |
| NZP-CSI-RS-ResourceIndex-r18[1] | 0 | entry 1, CSI-RS resource 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| powerOffset-r18 | Not present |  |  |
| } |  |  |  |
| CSI-ReportSubConfig-r18[2] SEQUENCE { |  | entry 2 |  |
| reportSubConfigId-r18 | 1 | sub-configuration #2 |  |
| reportSubConfigParams-r18 CHOICE { |  |  |  |
| a2-parameters SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceIndex-r18 { | 1 entry |  |  |
| NZP-CSI-RS-ResourceIndex-r18[1] | 1 | entry 1, CSI-RS resource 2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| powerOffset-r18 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.14.2.2.3.3-9: CSI-AperiodicTriggerStateList (Table 7.1.1.14.2.2.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-32 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-AperiodicTriggerStateList ::= SEQUENCE (SIZE (1..maxNrOfCSI-AperiodicTriggers)) OF CSI-AperiodicTriggerState { | 2 entries |  |  |
| CSI-AperiodicTriggerState[1] SEQUENCE { |  | entry 1 |  |
| associatedReportConfigInfoList SEQUENCE (SIZE(1..maxNrofReportConfigPerAperiodicTrigger)) OF CSI-AssociatedReportConfigInfo { | 1 entry |  |  |
| CSI-AssociatedReportConfigInfo[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 0 |  |  |
| resourcesForChannel CHOICE { |  |  |  |
| nzp-CSI-RS SEQUENCE { |  |  |  |
| resourceSet | 1 |  |  |
| qcl-info SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF TCI-StateId { | 1 entry |  |  |
| TCI-StateId[1] | 0 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| csi-IM-ResourcesforInteference | Not present |  |  |
| nzp-CSI-RS-ResourcesforInterference | Not present |  |  |
| csi-ReportSubConfigTriggerList-r18 SEQUENCE (SIZE(1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfigId-r18 { | 1 entry |  |  |
| CSI-ReportSubConfigId-r18[1] | 0 | sub-configuration #1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| CSI-AperiodicTriggerState[2] SEQUENCE { |  | entry 2 |  |
| associatedReportConfigInfoList SEQUENCE (SIZE(1..maxNrofReportConfigPerAperiodicTrigger)) OF CSI-AssociatedReportConfigInfo { | 1 entry |  |  |
| CSI-AssociatedReportConfigInfo[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 0 |  |  |
| resourcesForChannel CHOICE { |  |  |  |
| nzp-CSI-RS SEQUENCE { |  |  |  |
| resourceSet | 1 |  |  |
| qcl-info SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF TCI-StateId { | 1 entry |  |  |
| TCI-StateId[1] | 0 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| csi-IM-ResourcesforInteference | Not present |  |  |
| nzp-CSI-RS-ResourcesforInterference | Not present |  |  |
| csi-ReportSubConfigTriggerList-r18 SEQUENCE (SIZE(1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfigId-r18 { | 2 entries |  |  |
| CSI-ReportSubConfigId-r18[1] | 0 | entry 1, sub-configuration #1 |  |
| CSI-ReportSubConfigId-r18[2] | 1 | entry 2, sub-configuration #2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.14.2.2.3.3-10: PUSCH-Config

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-118 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| pusch-TimeDomainAllocationList CHOICE { |  |  |  |
| setup { |  |  |  |
| PUSCH-TimeDomainResourceAllocationList SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation { | 1 entry |  |  |
| PUSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | Entry 1 |  |
| k2 | 4 |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 27 | Start symbol(S)=0, Length(L)=14 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of Changes>