# 3GPP TSG-RAN5 Meeting #109	R5-256087

# Dallas, US, Nov 17 - 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5202 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of test 8.1.5.10.9 for R18 IDC enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IDC_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Update IDC TC 8.1.5.10.9 UE Assistance Information / IDC / FDM assistance configuration to align the configuration description with other IDC TCs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the time instances table and related description of TC 8.1.5.10.9 UE Assistance Information / IDC / FDM assistance configuration to align the configuration description with other IDC TCs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work Plan is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.10.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.508-1 CR 3652 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The configuration used in “Table 8.1.5.10.9.3.2-1” is introduced in TS 38.508-1 CR 3652. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

8.1.5.10.9	UE Assistance Information / IDC / FDM assistance configuration

8.1.5.10.9.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE configured to provide IDC assistance information based on idc-FDM-AssistanceConfig and UE did not transmit a UEAssistanceInformation message with idc-FDM-Assistance and UE is experiencing IDC problems that it cannot solve by itself }

then { The UE transmits a UEAssistanceInformation message to provide IDC assistance information }

}

(2)

with { UE in NR RRC_CONNECTED state and UE already transmits a UEAssistanceInformation message with idc-FDM-Assistance }

ensure that {

when { UE configured to provide IDC assistance information based on idc-FDM-AssistanceConfig and the current idc-FDM-Assistance information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message and UE is experiencing IDC problems that it cannot solve by itself }

then { The UE transmits a UEAssistanceInformation message to provide IDC assistance information }

}

8.1.5.10.9.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in TS 38.331, clause 5.3.5.9, 5.7.4.2 and 5.4.7.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.5.9]

……

1>	if the received otherConfig includes the idc-AssistanceConfig:

2>	if idc-AssistanceConfig is set to setup:

3>	consider itself to be configured to provide IDC assistance information in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide IDC assistance information;

…

[TS 38.331, clause 5.7.4.2]

Upon initiating the procedure, the UE shall:

1>	if configured to provide delay budget report:

…

1>	if configured to provide IDC assistance information based on idc-FDM-AssistanceConfig included in idc-AssistanceConfig of a cell group:

2>	if the UE did not transmit a UEAssistanceInformation message with idc-FDM-Assistance since it was configured to provide IDC assistance information:

3>	if on one or more frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself; or

3>	if on one or more supported UL CA or NR-DC combination comprising of frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

2>	else if the current idc-FDM-Assistance information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

……

[TS 38.331, clause 5.7.4.3]

The UE shall set the contents of the UEAssistanceInformation message as follows:

……

1>	if transmission of the UEAssistanceInformation message is initiated to provide IDC assistance information according to 5.7.4.2 or 5.3.5.3:

……

2>	if there is at least one affected frequency range overlapping with one candidate frequency range included in candidateServingFreqRangeListNR, and the center frequency of the affected frequency range is within the candidate frequency range included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include the field affectedCarrierFreqRangeList with an entry for each affected frequency range;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeList, include centerFreq and affectedBandwidth;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeList, include interferenceDirection and optionally victimSystemType, and set it accordingly;

2>	if there is at least one supported UL CA or NR-DC combinations comprising of candidate frequency ranges included in candidateServingFreqRangeListNR, and each affected frequency range in the UL CA or NR-DC combination overlapping with one candidate frequency range included in candidateServingFreqRangeListNR, and the center frequency of the affected frequency range is within the candidate frequency range included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include the field affectedCarrierFreqRangeCombList with an entry for each supported UL CA or NR-DC combination comprising of frequency ranges that is affected by IDC problems;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeCombList, include centerFreq and affectedBandwidth;

3>	for each UL CA or NR-DC combination included in the field affectedCarrierFreqRangeCombList, include interferenceDirection and optionally victimSystemType, and set it accordingly;

…

8.1.5.10.9.3	Test description

8.1.5.10.9.3.1	Pre-test conditions

System Simulator:

-	NR Cell 14/NRf4 (2390-2400MHz for band n40) and WLAN AP 1 (Cell 27) as specified in TS 38.508-1[4] clause 4.13.

-	NR Cell 1/NRf1 (2496-2556MHz for band n41) and WLAN AP 2 (Cell 27b) as specified in TS 38.508-1[4] clause 4.13.

UE:

-	UE’s WLAN is on.

Preamble:

-	The UE is in state 3N-A on NR Cell 14 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.5.10.9.3.2	Test procedure sequence

Table 8.1.5.10.9.3.2-1 for FR1 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration "T0" indicates the initial conditions. Subsequent configurations marked "T1", "T2" and "T3" are applied at the points indicated in the Main behaviour description in Table 8.1.5.10.9.3.2-2. The configuration "C3" for "T0", "C4" for "T1", "C5" for "T2" and "C6" for "T3" are specified in TS 38.508-1 [4] Table 4.13-2.

Table 8.1.5.10.9.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 14 | WLAN AP 1 (Cell 27) | NR Cell 1 | WLAN AP 2 (Cell 27b) | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | Refer to “C3” in TS 38.508-1 [4] |  |  |  |  |  |  |
| T1 | Refer to “C4” in TS 38.508-1 [4] |  |  |  |  |  |  |
| T2 | Refer to “C5” in TS 38.508-1 [4] |  |  |  |  |  |  |
| T3 | Refer to “C6” in TS 38.508-1 [4] |  |  |  |  |  |  |

Table 8.1.5.10.9.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a RRCReconfiguration message including OtherConfig with idc-FDM-AssistanceConfig-r18 including CandidateServingFreqRangeListNR-r18 including center frequency and bandwidth for NR Cell 14 and NR Cell 1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS re-adjusts the power levels according to row "T1" in Table 8.1.5.10.9.3.2-1. | - | - | - | - |
| 4 | The UE associates with the WLAN AP1 and obtains the local IP address of WLAN AP1. Note: The UL signal to WLAN AP shall generate IDC problem. The UE shall detect IDC problem with “T1” in RRC_CONNECTED in NR Cell 14. | - | - | - | - |
| 5 | Check: Does the UE transmit an UEAssistanceInformation message including idc-FDM-Assistance-r18 with at least one AffectedCarrierFreqRange-r18 entry with centerFreq-r18 and affectedBandwidth-r18 information? | --> | NR RRC: UEAssistanceInformation | 1 | P |
| 6 | The SS re-adjusts the power levels according to row "T2" in Table 8.1.5.10.9.3.2-1. | - | - | - | - |
| 7 | The SS transmits an RRCReconfiguration message including to order the UE to perform inter-frequency handover to NR Cell 1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 8 | The UE transmit RRCReconfigurationComplete message in NR Cell 1. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 8A | The SS re-adjusts the power levels according to row "T3" in Table 8.1.5.10.9.3.2-1. | - | - | - | - |
| 9 | The UE associates with the WLAN AP2 and obtains the local IP address of WLAN AP2. Note: The UL signal to WLAN AP shall generate IDC problem. The UE shall detect IDC problem with “T3” in RRC_CONNECTED in NR Cell 1. | - | - | - | - |
| 10 | Check: Does the UE transmit an UEAssistanceInformation message including idc-FDM-Assistance-r18 with at least one AffectedCarrierFreqRange-r18 entry with centerFreq-r18 and affectedBandwidth-r18 information different from the information in Step 5? | --> | NR RRC: UEAssistanceInformation | 2 | P |

<End of modified section>