3GPP TSG-RAN5 Meeting # 109	R5-256191

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0895 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new pics for parallelMeasurementWithoutRestriction-r17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For the NGSO NTN RRM measurement procedure test case,. Klayer1_measurement is different value in test case 14.5.1.2 based on UE capability parallelMeasurementWithoutRestriction-r17 [TS 38.306 4.2.7.2] parallelMeasurementWithoutRestriction-r17 Indicates whether the UE supports measurements on cells belonging to different satellites as the serving cell in parallel with normal operation (i.e. data/control transmission and/or reception, and L1 measurements) of serving cell without scheduling restrictions. The feature is applicable only when the serving satellite is NGSO. If the serving cell belongs to GSO satellite, the scheduling restriction is not applied on the premise that a mixed type of satellites on the same frequency layer is not supported in this release. If not reported, for measurements in parallel with normal operation of serving cell scheduling restrictions shall apply. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the new pics for “parallelMeasurementWithoutRestriction-r17” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE cannot declare UE capability parallelMeasurementWithoutRestriction-r17 in PICS during operating NTN RRM test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

### A.4.3.2	Physical Layer Baseline Implementation Capabilities

Table A.4.3.2-1: UE Physical Layer Baseline Implementation Capabilities

| Item | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | M | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support PDSCH reception based on semi-persistent scheduling | 38.306, 4.2.7.10 | Rel-15 | pc_downlinkSPS | No |  |  |
| ==============Skip unchanged part============== |  |  |  |  |  |  |  |
| 250 | Support of search space set group switching capability 2 for FR1 according to Table 10.4-1 of TS 38.213 [30] for SSSG switching | 38.306, 4.2.7.2 | Rel-17 | pc_searchSpaceSetGrp_switchCap2_r17 | No |  | FR1 only UE indicating support of this feature shall also indicate support of sssg-Switching-1bitInd-r17. |
| 251 | Support of RSRP reporting for a group of two reference signals | 38.306, 4.2.7.2 | Rel-15 | pc_groupBeamReporting | No |  |  |
| YYY | Support of measurements on cells belonging to different satellites as the serving cell in parallel with normal operation (i.e. data/control transmission and/or reception, and L1 measurements) of serving cell without scheduling restrictions. | 38.306, 4.2.7.2 | Rel-17 | pc_parallelMeasurementWithoutRestriction-r17 | No |  | The pics is applicable only when the serving satellite is NGSO. |

==============End of change==============