3GPP TSG RAN WG5 Meeting #109 	R5-256249

# Dallas, USA 17th Nov 2025 – 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.522 | CR |  | rev | - | Current version: | 19.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update in Table 4.3.1-1 by splitting the Test Case 6.5.2.4 ACLR into Test Clauses 6.5.2.4.1 NR ACLR & 6.5.2.4.2 UTRA ACLR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Marquis Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In 3GPP TS 38.521-5 the Test Case 6.5.2.4 ACLR is defined into two separate clauses in 6.5.2.4.1 NR ACLR & 6.5.2.4.2 UTRA ACLR while also same defined in Table 4.1.1-1 ref. TS 38.521-1 [1] |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update in Table 4.3.1-1 by splitting the Test Case 6.5.2.4 ACLR into Test Clauses 6.5.2.4.1 NR ACLR & 6.5.2.4.2 UTRA ACLR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect information will remain in Table 4.3.1-1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Reference Table>

4.1.1	FR1 standalone conformance test cases

Table 4.1.1-1: Applicability of RF SA FR1 conformance test cases, ref. TS 38.521-1 [1]

| Clause | TC Title | Release | Applicability |  | Tested Bands/CA-Configurations Selection | Branch | Additional Information |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |  |
| 6 | Transmitter Characteristics |  |  |  |  |  |  |  |
| 6.2.1 | UE maximum output power | Rel-15 | C001l | UEs supporting 5GS FR1 and not supporting RedCap and not supporting eRedCap | D001a | PC1 PC2 PC3 | TC 6.2.1 is skipped if TC 6.2G.1 is executed. |  |
| 6.2.2 | UE maximum output power reduction | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a | PC1 PC2 PC3 | Test execution is not necessary if TS 38.521-1 TC 6.5.2.4.1 is executed. Skip TC 6.2.2 if UE supports NSA and TS 38.521-3 TC 6.2B.2.3 or 6.5B.2.3.3.1 has been executed. TC 6.2.2 is skipped if TC 6.2G.2 is executed. |  |
| 6.2.3 | UE additional maximum output power reduction | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a | PC1 PC2 PC3 PC1.5 | Test execution is not necessary if TS 38.521-1 TC 6.5.2.3, 6.5.2.4.2 and 6.5.3.3 are executed. Skip TC 6.2.3 if UE supports NSA and TS 38.521-3 TC 6.2B.3.3 has been executed or TS 38.521-3 TCs 6.5B.2.3.2, 6.5B.2.3.3.2 and 6.5B.4.3 have been executed. TC 6.2.3 is skipped if TC 6.2G.3 is executed. |  |
| 6.2.4 | Configured transmitted power | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a | PC1 PC2 PC3 PC1.5 | TC 6.2.4 is skipped if TC 6.2G.4 is executed. |  |
| 6.2A.1.1 | UE maximum output power for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 E015a | Inter-band CA: PC2, PC3 Intra-band contiguous CA (NOTE 1) Intra-band non-contiguous CA (NOTE 1) |  |  |
| 6.2A.2.1 | UE maximum output power reduction for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 E015a | Inter-band CA: PC2, PC3 Intra-band contiguous CA: PC2, PC3 Intra-band non-contiguous CA: PC2 (NOTE 1), PC3 (NOTE 1) | Test execution is not necessary if TS 38.521-1 TC 6.5A.2.4.1.1 is executed. |  |
| 6.2A.3.1 | UE additional maximum output power reduction for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 E015a | Inter-band CA: PC2, PC3 Intra-band contiguous CA: PC2, PC3 Intra-band non-contiguous CA (NOTE 1) | Test execution is not necessary if TS 38.521-1 TC 6.5A.2.3 and 6.5A.3.3 are executed. |  |
| 6.2A.4.1 | Configured transmitted power for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 E015a | Inter-band CA: PC2, PC3 Intra-band contiguous CA: PC2, PC3 Intra-band non-contiguous CA (NOTE 1) |  |  |
| 6.2B.1.1 | UE maximum output power for NR-DC | Rel-16 | C004a | UEs supporting 5GS FR1 and NR-DC | E015b |  |  |  |
| 6.2B.2.1 | UE maximum output power reduction for NR-DC | Rel-16 | C004a | UEs supporting 5GS FR1 and NR-DC | E015b | PC1 PC2 PC3 | Test execution is not necessary if TS 38.521-1 TC 6.5B.2.4 is executed. |  |
| 6.2B.3.1 | UE additional maximum output power reduction for NR-DC | Rel-16 | N/A | No requirements in TS 38.521-1 TC 6.5B.2.3 Additional Spectrum Emission mask for NR-DC and TS 38.521-1 TC 6.5B.3.3 Additional Spurious Emissions for NR-DC. |  |  |  |  |
| 6.2B.4.1 | Configured transmitted power level for NR-DC | Rel-16 | C004a | UEs supporting 5GS FR1 and NR-DC | E015b |  |  |  |
| 6.2C.1 | Configured transmitted power for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.2C.3 | UE maximum output power for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.2C.4 | UE maximum output power reduction for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 Test execution is not necessary if TS 38.521-1 TC 6.5C.2.4.1 is executed. |  |
| 6.2C.5 | UE additional maximum output power reduction for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.2D.1 | UE maximum output power for UL MIMO | Rel-15 | N/A | No test point applicable to Rel-15 UE | D022 | PC1.5 PC2 PC3 |  |  |
|  |  | Rel-16 | C003b | UEs supporting 5GS FR1 and ULFPTx |  |  |  |  |
| 6.2D.1_1 | UE maximum output power for SUL with UL MIMO | Rel-17 | C179b | UEs supporting 5GS FR1 and SUL and ULFPTx | D024 |  |  |  |
| 6.2D.2 | UE maximum output power reduction for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 | PC1.5 PC2 PC3 | Test execution is not necessary if TS 38.521-1 TC 6.5D.2.4.1 is executed. |  |
|  |  | Rel-16 | C003b | UEs supporting 5GS FR1 and ULFPTx |  |  |  |  |
| 6.2D.2_1 | UE maximum output power reduction for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 | PC2 PC3 | Test execution is not necessary if TS 38.521-1 TC 6.5D.2.4.1_1 is executed. |  |
| 6.2D.2_2 | UE maximum output power reduction for UL MIMO for UE supporting 4Tx | Rel-18 | C402 | UEs supporting 5GS FR1 and (4-layer codebook based UL MIMO or ULFPTx) | D022a |  |  |  |
| 6.2D.3 | UE additional maximum output power reduction for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 | PC1.5 PC2 PC3 | Test execution is not necessary if TS 38.521-1 TC 6.5D.2.3, 6.5D.2.4.2 and 6.5D.3.3 are executed. |  |
| 6.2D.3_1 | UE additional maximum output power reduction for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 | PC2 PC3 |  |  |
| 6.2D.3_2 | UE additional maximum output power reduction for UL MIMO for UE supporting 4Tx | Rel-18 | C402 | UEs supporting 5GS FR1 and (4-layer codebook based UL MIMO or ULFPTx) | D022a |  |  |  |
| 6.2D.4 | Configured transmitted power for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 | PC1.5 PC2 PC3 |  |  |
|  |  | Rel-16 | C003b | UEs supporting 5GS FR1 and ULFPTx |  |  |  |  |
| 6.2D.4_1 | Configured transmitted power for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
|  |  | Rel-17 | C179b | UEs supporting 5GS FR1 and SUL and ULFPTx |  |  |  |  |
| 6.2D.4_2 | Configured transmitted power for UL MIMO for UE supporting 4Tx | Rel-18 | C402 | UEs supporting 5GS FR1 and (4-layer codebook based UL MIMO or ULFPTx) | D022a |  |  |  |
| 6.2E.1.2 | UE maximum output power for V2X / concurrent operation | Rel-16 | C079a | UEs supporting 5GS FR1 and NR sidelink and NR V2X inter-band concurrent operation | E050 |  |  |  |
| 6.2E.2.1 | UE maximum output power reduction for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 | PC3 PC2 | Test execution is not necessary if TS 38.521-1 TC 6.5E.2.4.1 is executed. |  |
| 6.2E.2.1D | UE maximum output power reduction for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 | PC3 PC2 |  |  |
| 6.2E.2.2 | UE maximum output power reduction for V2X / concurrent operation | Rel-16 | C079c | UEs supporting 5GS FR1 and NR sidelink and UEs supporting 5GS FR1 and NR sidelink and NR V2X inter-band concurrent operation or NR V2X intra-band concurrent operation | E051 | Inter-band concurrent operation Intra-band concurrent operation: PC3 , PC2 |  |  |
| 6.2E.3.1 | UE additional maximum output power reduction for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 | PC3 PC2 | Test execution is not necessary if TS 38.521-1 TC 6.5E.2.3.1 is executed. |  |
| 6.2E.3.1D | UE additional maximum output power reduction for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 | PC3 PC2 | Test execution is not necessary if TS 38.521-1 TC 6.5E.2.3.1D is executed. |  |
| 6.2E.3.2 | UE additional maximum output power reduction for V2X / concurrent operation | Rel-16 | N/A | No test case details are specified in TS 38.521-1. |  |  |  |  |
| 6.2E.4.1 | Configured transmitted power for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 | PC3 PC2 |  |  |
| 6.2E.4.1D | Configured transmitted power for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 | PC3 PC2 |  |  |
| 6.2E.4.2 | Configured transmitted power for V2X / concurrent operation | Rel-16 | C079c | UEs supporting 5GS FR1 and NR sidelink and UEs supporting 5GS FR1 and NR sidelink and NR V2X inter-band concurrent operation or NR V2X intra-band concurrent operation | E051 | Inter-band concurrent operation Intra-band concurrent operation: PC3, PC2 |  |  |
| 6.2F.1 | Void |  |  |  |  |  |  |  |
| 6.2F.2 | UE maximum output power reduction for shared spectrum access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |  |
| 6.2F.3 | UE additional maximum output power reduction for shared spectrum access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |  |
| 6.2F.4 | Configured transmitted power for shared spectrum access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |  |
| 6.2G.1 | UE maximum output power for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 |  |  |
| 6.2G.2 | UE maximum output power reduction for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 | Test execution is not necessary if TS 38.521-1 TC 6.5G.2.3.1 is executed. |  |
| 6.2G.2_1 | UE maximum output power reduction for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  |  |  |
| 6.2G.3 | UE additional maximum output power reduction for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 | Test execution is not necessary if TS 38.521-1 TC 6.5G.2.2 and 6.5G.3.3 are executed. |  |
| 6.2G.3_1 | UE additional maximum output power reduction for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  |  |  |
| 6.2G.4 | Configured transmitted power for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 |  |  |
| 6.2G.4_1 | Configured transmitted power for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  |  |  |
| 6.2H.1.2 | UE maximum output power reduction for intra-band UL contiguous CA with UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 | Test execution is not necessary if TS 38.521-1 TC 6.5H.1.2.1 is executed. |  |
| 6.2H.1.3 | UE additional maximum output power reduction for intra-band UL contiguous CA with UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 | Test execution is not necessary if TS 38.521-1 TC 6.5H.1.2.3 is executed. |  |
| 6.2H.1.4 | Configured transmitted power for intra-band UL contiguous CA with UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.2H.3.1 | UE maximum output power for inter-band UL CA with UL MIMO | Rel-17 | C328 | UEs supporting 5GS FR1 and inter-band CA (2UL CA) with UL MIMO. | E045 | PC1.5 PC2 PC3 |  |  |
| 6.2I.1 | UE maximum output power for RedCap | Rel-17 | C177 | RedCap UEs supporting 5GS FR1 | D001b |  |  |  |
| 6.2I.2 | UE maximum output power for eRedCap | Rel-18 | C372 | eRedCap UEs supporting 5GS FR1 | D001b |  |  |  |
| 6.2I.3 | Void |  |  |  |  |  |  |  |
| 6.2I.4 | Void |  |  |  |  |  |  |  |
| 6.2J.1 | UE maximum output power for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 6.2J.2 | Configured transmitted power for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 6.2L.3.1 | UE maximum output power for inter-band UL CA with Tx Diversity | Rel-17 | C329 | UEs supporting 5GS FR1 and inter-band CA (2UL CA) with Tx Diversity. | E046 | PC1.5 PC2 PC3 |  |  |
| 6.3.1 | Minimum output power | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | TC 6.3.1 is skipped if TC 6.3G.1 is executed. |  |
| 6.3.3.2 | General ON/OFF time mask | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | Skip TC 6.3.3.2 if UE supports NSA and TS 38.521-3 TC 6.3B.3.1 or 6.3B.3.2 or 6.3B.3.3 has been executed. TC 6.3.3.2 is skipped if TC 6.3G.3.1 is executed. |  |
| 6.3.3.4 | PRACH time mask | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | Skip TC 6.3.3.4 if UE supports NSA and TS 38.521-3 TC 6.3B.4.1 or 6.3B.4.2 or 6.3B.4.3 has been executed. TC 6.3.3.4 is skipped if TC 6.3G.3.2 is executed. |  |
| 6.3.3.6 | SRS time mask | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | TC 6.3.3.6 is skipped if TC 6.3G.3.3 is executed. |  |
| 6.3.4.2 | Absolute power tolerance | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | TC 6.3.4.2 is skipped if TC 6.3G.4.1 is executed. |  |
| 6.3.4.3 | Relative power tolerance | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | TC 6.3.4.3 is skipped if TC 6.3G.4.2 is executed. |  |
| 6.3.4.4 | Aggregate power tolerance | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | TC 6.3.4.4 is skipped if TC 6.3G.4.3 is executed. |  |
| 6.3A.1.1 | Minimum output power for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 |  |  |  |
| 6.3A.3.1 | Transmit ON/OFF time mask for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 |  |  |  |
| 6.3A.3.2 | Time mask for switching between two uplink carriers | Rel-16 | C051 | UEs supporting 5GS FR1 and Inter-band CA (2UL CA) and dynamic 1Tx-2Tx UL Tx switching | E019 |  |  |  |
| 6.3A.3.3 | Time mask for switching between two uplink carriers with two transmit antenna connectors | Rel-17 | C051b | UEs supporting 5GS FR1 and Inter-band CA (2UL CA) and dynamic 2Tx-2Tx UL Tx switching | E019a |  |  |  |
| 6.3A.3.4 | Time mask for switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors (3UL CA) | Rel-17 | C051d | UEs supporting 5GS FR1 and inter-band/intra-band CA (3UL CA) and dynamic 1Tx-2Tx UL Tx switching. | E019b |  |  |  |
| 6.3A.3.5 | Time mask for switching between two uplink bands with two transmit antenna connectors (3UL CA) | Rel-17 | C051f | UEs supporting 5GS FR1 and inter-band/intra-band CA (3UL CA) and dynamic 2Tx-2Tx UL Tx switching. | E019c |  |  |  |
| 6.3A.3.6 | Time mask for switching across three uplink bands | Rel-18 | C412 | UEs supporting 5GS FR1 and inter-band CA and dynamic UL Tx switching across three bands. | E054 |  |  |  |
| 6.3A.3.6_1 | Time mask for switching across three uplink bands (more than 3CC) | Rel-18 | C413 | UEs supporting 5GS FR1 and inter-band + intra-band CA and dynamic UL Tx switching across three bands. | E054 |  |  |  |
| 6.3A.3.7 | Time mask for switching across four uplink bands | Rel-18 | C414 | UEs supporting 5GS FR1 and inter-band CA and dynamic UL Tx switching across four bands. | E055 |  |  |  |
| 6.3A.4.1.1 | Absolute power tolerance for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 |  |  |  |
| 6.3A.4.2.1 | Relative power tolerance for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 |  |  |  |
| 6.3A.4.3.1 | Aggregate power tolerance for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 |  |  |  |
| 6.3C.1 | Minimum output power for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.3C.3.1 | General transmit ON/OFF time mask for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.3C.3.2 | General transmit ON/OFF time mask for switching between two uplink carriers | Rel-16 | C178 | UEs supporting 5GS FR1 and SUL and dynamic UL 1Tx-2Tx switching | E044 |  | NOTE 2 |  |
| 6.3C.3.3 | Time mask for switching between two uplink carriers with two transmit antenna connectors | Rel-17 | C178a | UEs supporting 5GS FR1 and SUL and dynamic UL 2Tx-2Tx switching | E044a |  |  |  |
| 6.3C.3.4 | Time mask for switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors | Rel-17 | C178b | UEs supporting 5GS FR1 and SUL with intra-band contiguous CA and dynamic UL 1Tx-2Tx switching | E044b |  |  |  |
| 6.3C.3.5 | Time mask for switching between two uplink bands with two transmit antenna connectors | Rel-17 | C178c | UEs supporting 5GS FR1 and SUL with intra-band contiguous CA and dynamic UL 2Tx-2Tx switching | E044c |  |  |  |
| 6.3C.3.6 | Time mask for switching across three uplink bands (3CC) | Rel-18 | C346 | UEs supporting 5GS FR1 and SUL with inter-band CA and dynamic UL switching across three bands | E049 |  |  |  |
| 6.3C.3.6_1 | Time mask for switching across three uplink bands (more than 3CC) | Rel-18 | C347 | UEs supporting 5GS FR1 and SUL with intra-band + inter-band CA and dynamic UL switching across three bands | E049 |  |  |  |
| 6.3C.4.1 | Absolute power tolerance for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.3C.4.2 | Relative power tolerance for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.3C.4.3 | Aggregate power tolerance for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.3D.1 | Minimum output power for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.3D.1_1 | Minimum output power for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.3D.1_2 | Minimum output power UL MIMO for UE supporting 4Tx | Rel-18 | C400 | UEs supporting 5GS FR1 and 4-layer codebook based UL MIMO | D022a |  | NOTE 1 |  |
| 6.3D.3 | Transmit ON/OFF time mask for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.3D.3_1 | Transmit ON/OFF time mask for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.3D.3_2 | Transmit ON/OFF time mask for UL MIMO for UE supporting 4Tx | Rel-18 | C400 | UEs supporting 5GS FR1 and 4-layer codebook based UL MIMO | D022a |  |  |  |
| 6.3D.4.1 | Absolute power tolerance for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.3D.4.1_1 | Absolute power tolerance for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.3D.4.1_2 | Absolute power tolerance for UL MIMO for UE supporting 4Tx | Rel-18 | C400 | UEs supporting 5GS FR1 and 4-layer codebook based UL MIMO | D022a |  |  |  |
| 6.3D.4.2 | Relative power tolerance for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.3D.4.2_1 | Relative power tolerance for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.3D.4.2_2 | Relative power tolerance for UL MIMO for UE supporting 4Tx | Rel-18 | C400 | UEs supporting 5GS FR1 and 4-layer codebook based UL MIMO | D022a |  | NOTE 1 |  |
| 6.3D.4.3 | Aggregate power tolerance for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 | PC1.5 PC2 PC3 |  |  |
| 6.3D.4.3_1 | Aggregate power tolerance for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.3D.4.3_2 | Aggregate power tolerance for UL MIMO for UE supporting 4Tx | Rel-18 | C400 | UEs supporting 5GS FR1 and 4-layer codebook based UL MIMO | D022a |  |  |  |
| 6.3E.1.1 | Minimum output power for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 6.3E.1.1D | Minimum output power for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  |  |  |
| 6.3E.3.1 | Transmit ON/OFF time mask for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 6.3E.3.1D | Transmit ON/OFF time mask for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  |  |  |
| 6.3E.4.1.1 | Absolute power tolerance for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 6.3E.4.1.1D | Absolute power tolerance for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  |  |  |
| 6.3F.1 | Minimum output power | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  | NOTE 1 |  |
| 6.3F.3.2 | General ON/OFF time mask | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |  |
| 6.3F.4.2 | Absolute power tolerance for shared spectrum access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |  |
| 6.3F.4.3 | Relative power tolerance for shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |  |
| 6.3F.4.4 | Aggregate power tolerance for shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |  |
| 6.3G.1 | Minimum output power for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 |  |  |
| 6.3G.1_1 | Minimum output power for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  | NOTE 1 |  |
| 6.3G.3.1 | General ON/OFF time mask for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 |  |  |
| 6.3G.3.1_1 | General ON/OFF time mask for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  |  |  |
| 6.3G.3.2 | PRACH time mask for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 |  |  |
| 6.3G.3.3 | SRS time mask for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 |  |  |
| 6.3G.4.1 | Absolute power tolerance for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 |  |  |
| 6.3G.4.1_1 | Absolute power tolerance for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  |  |  |
| 6.3G.4.2 | Relative power tolerance for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 |  |  |
| 6.3G.4.2_1 | Relative power tolerance for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  | NOTE 1 |  |
| 6.3G.4.3 | Aggregate power tolerance for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 |  |  |
| 6.3G.4.3_1 | Aggregate power tolerance for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  |  |  |
| 6.3H.1.1 | Minimum output power for intra-band UL contiguous CA with UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.3H.1.3 | Transmit ON/OFF time mask for intra-band UL contiguous CA with UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.3J.1 | Minimum output power for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 6.3J.3.1 | General ON/OFF time mask for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 6.3J.3.3 | PRACH time mask for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 6.3J.3.4 | SRS time mask for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 6.3J.4.1 | Absolute power tolerance for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 6.3J.4.2 | Relative power tolerance for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 6.3J.4.3 | Aggregate power tolerance for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 6.4.1 | Frequency error | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | Skip TC 6.4.1 if UE supports NSA and TS 38.521-3 TC 6.4B.1.1 or 6.4B.1.2 or 6.4B.1.3 has been executed. TC 6.4.1 is skipped if TC 6.4G.1 is executed. |  |
| 6.4.2.1 | Error vector magnitude | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | Skip TC 6.4.2.1 if UE supports NSA and TS 38.521-3 TC 6.4B.2.1.1 or 6.4B.2.2.1 or 6.4B.2.3.1 has been executed. TC 6.4.2.1 is skipped if TC 6.4G.2.1 is executed. |  |
| 6.4.2.1a | Error Vector Magnitude including symbols with transient period | Rel-16 | C156 | UEs supporting 5GS FR1 AND Band supporting enhancedUL-TransientPeriod | D001a |  |  |  |
| 6.4.2.2 | Carrier leakage | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | Skip TC 6.4.2.2 if UE supports NSA and TS 38.521-3 TC 6.4B.2.1.2 or 6.4B.2.2.2 or 6.4B.2.3.2 has been executed. TC 6.4.2.2 is skipped if TC 6.4G.2.2 is executed. |  |
| 6.4.2.3 | In-band emissions | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | Skip TC 6.4.2.3 if UE supports NSA and TS 38.521-3 TC 6.4B.2.2.3 or 6.4B.2.3.3 has been executed. TC 6.4.2.3 is skipped if TC 6.4G.2.3 is executed. |  |
| 6.4.2.4 | EVM equalizer spectrum flatness | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | Skip TC 6.4.2.4 if UE supports NSA and TS 38.521-3 TC 6.4B.2.1.4 or 6.4B.2.2.4 or 6.4B.2.3.4 has been executed. TC 6.4.2.4 is skipped if TC 6.4G.2.4 is executed. |  |
| 6.4.2.5 | EVM equalizer spectrum flatness for Pi/2 BPSK | Rel-15 | C050 | UEs supporting 5GS FR1 Power Class 3 and pi/2-BPSK modulation scheme for power boosting in FR1 | D017a |  |  |  |
|  |  | Rel-16 | C111 | UEs supporting 5GS FR1 and pi/2-BPSK modulation scheme and low PAPR DMRS | D001a |  |  |  |
| 6.4.2.6 | Phase continuity requirements for DMRS bundling | Rel-17 | C337 | UEs supporting 5GS FR1, dmrs-BundlingPUCCH-Rep-r17 and either dmrs-BundlingPUSCH-multiSlot-r17 or dmrs-BundlingPUSCH-RepTypeA-r17 or dmrs-BundlingPUSCH-RepTypeB-r17 | D001a |  | NOTE 1 |  |
| 6.4A.1.1 | Frequency error for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA and not supporting Tx Diversity) | E015 | Inter-band CA Intra-band contiguous CA Intra-band non-contiguous CA |  |  |
| 6.4A.2.1.1 | Error vector magnitude for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA and not supporting Tx Diversity) | E015 | Inter-band CA Intra-band contiguous CA Intra-band non-contiguous CA |  |  |
| 6.4A.2.2.1 | Carrier leakage for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA and not supporting Tx Diversity) | E015 | Inter-band CA Intra-band contiguous CA Intra-band non-contiguous CA(NOTE 1) |  |  |
| 6.4A.2.3.1 | In-band emissions for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA and not supporting Tx Diversity) | E015 | Inter-band CA Intra-band contiguous CA Intra-band non-contiguous CA |  |  |
| 6.4C.1 | Frequency error for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.4C.2.1 | Error vector magnitude for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.4C.2.2 | Carrier leakage for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.4C.2.3 | In-band emissions for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.4C.2.4 | EVM equalizer spectrum flatness for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.4C.2.5 | EVM equalizer spectrum flatness for Pi/2 BPSK for SUL | Rel-16 | C112 | UEs supporting 5GS FR1 and SUL and pi/2-BPSK modulation scheme and low PAPR DMRS | D003 |  | NOTE 2 |  |
| 6.4D.1 | Frequency error for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.4D.1_1 | Frequency error for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.4D.1_2 | Frequency error for UL MIMO for UE supporting 4Tx | Rel-18 | C400 | UEs supporting 5GS FR1 and 4-layer codebook based UL MIMO | D022a |  |  |  |
| 6.4D.2.1 | Error vector magnitude for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.4D.2.1_1 | Error Vector Magnitude for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.4D.2.1_2 | Error Vector Magnitude for UL MIMO for UE supporting 4Tx | Rel-18 | C400 | UEs supporting 5GS FR1 and 4-layer codebook based UL MIMO | D022a |  |  |  |
| 6.4D.2.2 | Carrier leakage for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.4D.2.2_1 | Carrier leakage for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.4D.2.2_2 | Carrier leakage for UL MIMO for UE supporting 4Tx | Rel-18 | C400 | UEs supporting 5GS FR1 and 4-layer codebook based UL MIMO | D022a |  |  |  |
| 6.4D.2.3 | In-band emissions for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.4D.2.3_1 | In-band emissions for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.4D.2.3_2 | In-band emissions for UL MIMO for UE supporting 4Tx | Rel-18 | C400 | UEs supporting 5GS FR1 and 4-layer codebook based UL MIMO | D022a |  |  |  |
| 6.4D.2.4 | EVM equalizer spectrum flatness for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.4D.2.4_1 | EVM equalizer spectrum flatness for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.4D.2.4_2 | EVM equalizer spectrum flatness for UL MIMO for UE supporting 4Tx | Rel-18 | C400 | UEs supporting 5GS FR1 and 4-layer codebook based UL MIMO | D022a |  |  |  |
| 6.4D.3 | Time alignment error for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.4D.3_1 | Time alignment error for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.4D.3_2 | Time alignment error for UL MIMO for UE supporting 4Tx | Rel-18 | C400 | UEs supporting 5GS FR1 and 4-layer codebook based UL MIMO | D022a |  |  |  |
| 6.4D.4 | Requirements for coherent UL MIMO | FFS | FFS | FFS | FFS |  | NOTE 1 |  |
| 6.4E.1.1 | Frequency error for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 6.4E.1.1D | Frequency error for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  |  |  |
| 6.4E.2.2.1 | Error Vector Magnitude for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 6.4E.2.2.1D | Error Vector Magnitude for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  |  |  |
| 6.4E.2.3.1 | Carrier leakage for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 6.4E.2.3.1D | Carrier leakage for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  |  |  |
| 6.4E.2.4.1 | In-band emissions for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 6.4E.2.4.1D | In-band emissions for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  |  |  |
| 6.4E.2.5.1 | EVM equalizer spectrum flatness for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 6.4E.2.5.1D | EVM equalizer spectrum flatness for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  |  |  |
| 6.4F.1 | Frequency error | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  | NOTE 1 |  |
| 6.4F.2.1 | Error Vector Magnitude | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  | NOTE 1 |  |
| 6.4F.2.2 | Carrier leakage | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  | NOTE 1 |  |
| 6.4G.1 | Frequency error for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 |  |  |
| 6.4G.1_1 | Frequency error for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  |  |  |
| 6.4G.2.1 | Error vector magnitude for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 |  |  |
| 6.4G.2.1_1 | Error Vector Magnitude for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  |  |  |
| 6.4G.2.2 | Carrier leakage for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 |  |  |
| 6.4G.2.2_1 | Carrier leakage for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  |  |  |
| 6.4G.2.3 | In-band emissions for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting | D030 | PC1.5 PC2 PC3 |  |  |
| 6.4G.2.3_1 | In-band emissions for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  |  |  |
| 6.4G.2.4 | EVM equalizer spectrum flatness for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting Tx Diversity | D030 | PC1.5 PC2 PC3 |  |  |
| 6.4G.2.4_1 | EVM equalizer spectrum flatness for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  |  |  |
| 6.4H.1.1 | Frequency error for intra-band UL contiguous CA with UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.4H.1.2.1 | Error Vector Magnitude for intra-band UL contiguous CA with UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.4H.1.2.2 | Carrier leakage for intra-band UL contiguous CA with UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.4H.1.2.3 | In-band emissions for intra-band UL contiguous CA with UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.4H.1.3 | Time alignment error for intra-band UL contiguous CA with UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.4H.1.4 | Coherent UL MIMO for intra-band UL contiguous CA with UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 | NOTE 1 |  |
| 6.4J.1 | Frequency error for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  |  |  |
| 6.4J.2.1 | Error Vector Magnitude for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  |  |  |
| 6.4J.2.1a | Error Vector Magnitude including symbols with transient period for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  |  |  |
| 6.4J.2.4 | EVM equalizer spectrum flatness for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  |  |  |
| 6.4J.2.5 | EVM equalizer spectrum flatness for Pi/2 BPSK for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  |  |  |
| 6.5.1 | Occupied bandwidth | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | Skip TC 6.5.1 if UE supports NSA and TS 38.521-3 TC 6.5B.1.2 or 6.5B.1.3 has been executed. TC 6.5.1 is skipped if TC 6.5G.1 is executed. |  |
| 6.5.2.2 | Spectrum emission mask | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a | PC1 PC2 PC3 | Skip TC 6.5.2.2 if UE supports NSA and TS 38.521-3 TC 6.5B.2.2.1 or 6.5B.2.3.1 has been executed. TC 6.5.2.2 is skipped if TC 6.5G.2.1 is executed. |  |
| 6.5.2.3 | Additional spectrum emission mask | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a | PC1 PC2 PC3 | Skip TC 6.5.2.3 if UE supports NSA and TS 38.521-3 TC 6.5B.2.3.2 has been executed. TC 6.5.2.3 is skipped if TC 6.5G.2.2 is executed. |  |
| 6.5.2.4.1 | NR ACLR | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a | PC1 PC2 PC3 | Skip TC 6.5.2.4.1 if UE supports NSA and TS 38.521-3 TC 6.5B.2.3.3.1 has been executed. TC 6.5.2.4.1 is skipped if TC 6.5G.2.3.1 is executed. |  |
| 6.5.2.4.2 | UTRA ACLR | Rel-15 | C001a | UEs supporting 5GS FR1 PC3 | D001a |  | Skip TC 6.5.2.4.2 if UE supports NSA and TS 38.521-3 TC 6.5B.2.3.3.2 has been executed. TC 6.5.2.4.2 is skipped if TC 6.5G.2.3.2 is executed. |  |
| 6.5.3.1 | General spurious emissions | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | Skip TC 6.5.3.1 if UE supports NSA and TS 38.521-3 TC 6.5B.3.1.1 or 6.5B.3.2.1 or 6.5B.3.3.1(non-exception requirements) has been executed. TC 6.5.3.1 is skipped if TC 6.5G.3.1 is executed. |  |
| 6.5.3.2 | Spurious emissions for UE co-existence | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | Skip TC 6.5.3.2 if UE supports NSA and TS 38.521-3 TC 6.5B.3.3.2 (non-exception requirements) has been executed. |  |
| 6.5.3.3 | Additional spurious emissions | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | Skip TC 6.5.3.3 if UE supports NSA and TS 38.521-3 TC 6.5B.4.3 has been executed. TC 6.5.3.3 is skipped if TC 6.5G.3.3 is executed. |  |
| 6.5.4 | Transmit intermodulation | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a |  | Skip TC 6.5.4 if UE supports NSA and TS 38.521-3 TC 6.5B.5.3 has been executed. TC 6.5.4 is skipped if TC 6.5G.4 is executed. |  |
| 6.5A.1.1 | Occupied bandwidth for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 | Inter-band CA Intra-band contiguous CA Intra-band non-contiguous CA: PC2 (NOTE 1), PC3 (NOTE 1) |  |  |
| 6.5A.2.2.1 | Spectrum emission mask for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 E015a | Inter-band CA: PC2, PC3 Intra-band contiguous CA: PC2, PC3 Intra-band non-contiguous CA |  |  |
| 6.5A.2.3.1 | Additional spectrum emission mask for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 | Inter-band CA: PC2, PC3Intra-band contiguous CA: PC2, PC3 Intra-band non-contiguous CA (NOTE 1) |  |  |
| 6.5A.2.4.1.1 | NR ACLR for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 E015a | Inter-band CA: PC2, PC3 Intra-band contiguous CA: PC2, PC3 Intra-band non-contiguous CA: PC2 (NOTE 1), PC3 (NOTE 1) |  |  |
| 6.5A.2.4.2.1 | UTRA ACLR for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 |  |  |  |
| 6.5A.3.1.1 | General spurious emissions for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 | Inter-band CA Intra-band contiguous CA: PC2, PC3 |  |  |
| 6.5A.3.2.1 | Spurious emissions for UE co-existence for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 | Inter-band CA Intra-band contiguous CA: PC2, PC3 Intra-band non-contiguous CA | NOTE 3 Skip the testing if UE supports NSA and the same UL DC configuration and TS 38.521-3 TC 6.5B.3.3.2 has been executed |  |
| 6.5A.3.3.1 | Additional spurious emissions for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 | Inter-band CA: PC2, PC3Intra-band contiguous CA: PC2, PC3 Intra-band non-contiguous CA (NOTE 1) |  |  |
| 6.5A.4.1 | Transmit intermodulation for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 | Inter-band CA Intra-band contiguous CA Intra-band non-contiguous CA |  |  |
| 6.5C.1 | Occupied bandwidth for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.5C.2.2 | Spectrum emission mask for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.5C.2.3 | Additional spectrum emission mask for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.5C.2.4.1 | NR ACLR for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.5C.2.4.2 | UTRA ACLR for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.5C.3.1 | General spurious emissions for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.5C.3.2 | Spurious emissions for UE co-existence for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 NOTE 3 Skip the testing if UE supports NSA and the same UL DC configuration and TS 38.521-3 TC 6.5B.3.3.2 has been executed |  |
| 6.5C.3.3 | Additional spurious emissions for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.5C.4 | Transmit intermodulation for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 2 |  |
| 6.5D.1 | Occupied bandwidth for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.5D.1_2 | Occupied bandwidth for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.5D.1_3 | Occupied bandwidth for UL MIMO for UE supporting 4Tx | Rel-18 | C400 | UEs supporting 5GS FR1 and 4-layer codebook based UL MIMO | D022a |  |  |  |
| 6.5D.2.2 | Spectrum emission mask for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
|  |  | Rel-16 | C003b | UEs supporting 5GS FR1 and ULFPTx |  |  |  |  |
| 6.5D.2.2_1 | Spectrum emission mask for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
|  |  | Rel-17 | C179b | UEs supporting 5GS FR1 and SUL and ULFPTx |  |  |  |  |
| 6.5D.2.2_2 | Spectrum emission mask for UL MIMO for UE supporting 4Tx | Rel-18 | C402 | UEs supporting 5GS FR1 and (4-layer codebook based UL MIMO or ULFPTx) | D022a |  | NOTE 1 |  |
| 6.5D.2.3 | Additional spectrum emission mask for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.5D.2.3_1 | Additional spectrum emission mask for UL MIMO for UE supporting 4Tx | Rel-18 | C402 | UEs supporting 5GS FR1 and (4-layer codebook based UL MIMO or ULFPTx) | D022a |  | NOTE 1 |  |
| 6.5D.2.4.1 | NR ACLR for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
|  |  | Rel-16 | C003b | UEs supporting 5GS FR1 and ULFPTx |  |  |  |  |
| 6.5D.2.4.1_1 | NR ACLR for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.5D.2.4.1_2 | NR ACLR for UL MIMO for UE supporting 4Tx | Rel-18 | C402 | UEs supporting 5GS FR1 and (4-layer codebook based UL MIMO or ULFPTx) | D022a |  | NOTE 1 |  |
| 6.5D.2.4.2 | UTRA ACLR for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.5D.2.4.2_1 | UTRA ACLR for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.5D.2.4.2_2 | UTRA ACLR for UL MIMO for UE supporting 4Tx | Rel-18 | C402 | UEs supporting 5GS FR1 and (4-layer codebook based UL MIMO or ULFPTx) | D022a |  | NOTE 1 |  |
| 6.5D.3.1 | General spurious emissions for UL MIMO | Rel-15 only | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.5D.3.2 | Spurious emissions for UE co-existence for UL MIMO | Rel-15 only | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.5D.3.3 | Additional spurious emissions for UL MIMO | Rel-15 only | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.5D.3_1.1 | General spurious emissions for UL MIMO (Rel-16 onward) | Rel-16 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
|  |  | Rel-16 | C003b | UEs supporting 5GS FR1 and ULFPTx |  |  |  |  |
| 6.5D.3_1.2 | Spurious emission for UE co-existence for UL MIMO (Rel-16 onward) | Rel-16 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
|  |  | Rel-16 | C003b | UEs supporting 5GS FR1 and ULFPTx |  |  |  |  |
| 6.5D.3_1.3 | Additional spurious emissions for UL MIMO (Rel-16 onward) | Rel-16 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
|  |  | Rel-16 | C003b | UEs supporting 5GS FR1 and ULFPTx |  |  |  |  |
| 6.5D.3_2.1 | General spurious emissions for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.5D.3_2.2 | Spurious emissions for UE co-existence for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.5D.3_2.3 | Additional spurious emissions for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.5D.3_3.1 | General spurious emissions for UL MIMO for UE supporting 4Tx | Rel-18 | C402 | UEs supporting 5GS FR1 and (4-layer codebook based UL MIMO or ULFPTx) | D022a |  | NOTE 1 |  |
| 6.5D.3_3.2 | Spurious emissions for UE co-existence for UL MIMO for UE supporting 4Tx | Rel-18 | C402 | UEs supporting 5GS FR1 and (4-layer codebook based UL MIMO or ULFPTx) | D022a |  | NOTE 1 |  |
| 6.5D.3_3.3 | Additional spurious emissions for UL MIMO for UE supporting 4Tx | Rel-18 | C402 | UEs supporting 5GS FR1 and (4-layer codebook based UL MIMO or ULFPTx) | D022a |  | NOTE 1 |  |
| 6.5D.4 | Transmit intermodulation for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and 2-layer codebook based UL MIMO | D022 |  |  |  |
| 6.5D.4_1 | Transmit intermodulation for SUL with UL MIMO | Rel-17 | C179 | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 6.5D.4_2 | Transmit intermodulation for UL MIMO for UE supporting 4Tx | Rel-18 | C400 | UEs supporting 5GS FR1 and 4-layer codebook based UL MIMO | D022a |  |  |  |
| 6.5E.1.1 | Occupied bandwidth for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 6.5E.1.1D | Occupied bandwidth for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  |  |  |
| 6.5E.2.2.1 | Spectrum emission mask for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 6.5E.2.2.1D | Spectrum emission mask for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  |  |  |
| 6.5E.2.2.2 | Spectrum emission mask for V2X / concurrent operation | Rel-16 | C079c | UEs supporting 5GS FR1 and NR sidelink and UEs supporting 5GS FR1 and NR V2X inter-band concurrent operation or NR V2X intra-band concurrent operation | E051 | Inter-band concurrent operation: PC3 Intra-band concurrent operation: PC3, PC2 |  |  |
| 6.5E.2.3.1 | Additional Spectrum emission mask for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 6.5E.2.3.1D | Additional Spectrum emission mask for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  |  |  |
| 6.5E.2.4.1 | Adjacent channel leakage ratio for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 6.5E.2.4.1D | Adjacent channel leakage ratio for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  | NOTE 1 |  |
| 6.5E.2.4.2 | Adjacent channel leakage for V2X / concurrent operation | Rel-16 | C079c | UEs supporting 5GS FR1 and NR sidelink and UEs supporting 5GS FR1 and NR V2X inter-band concurrent operation or NR V2X intra-band concurrent operation | E051 | Inter-band concurrent operation: PC3 Intra-band concurrent operation: PC3, PC2 |  |  |
| 6.5E.3.1.1 | General spurious emissions for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 6.5E.3.1.1D | General spurious emissions for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  |  |  |
| 6.5E.3.2.1 | Spurious emissions for UE co-existence for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  | NOTE 1 |  |
| 6.5E.3.2.1D | Spurious emissions for UE co-existence for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  |  |  |
| 6.5E.3.2.2 | Spurious emissions for UE co-existence for V2X / concurrent operation | Rel-16 | C079c | UEs supporting 5GS FR1 and NR sidelink and UEs supporting 5GS FR1 and NR V2X inter-band concurrent operation or NR V2X intra-band concurrent operation | E051 | Inter-band concurrent operation: PC3 Intra-band concurrent operation: PC3, PC2 |  |  |
| 6.5E.3.3.1 | Additional spurious emissions requirements for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 6.5E.4.1 | Transmit intermodulation for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 6.5E.4.1D | Transmit intermodulation for V2X / non-concurrent operation / SL-MIMO | Rel-16 | C079b | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO | D016 |  |  |  |
| 6.5E.4.2 | Transmit intermodulation for V2X con-current operation | Rel-16 | C079d | UEs supporting 5GS FR1 and NR sidelink and UEs supporting 5GS FR1 and NR V2X intra-band concurrent operation | E052 | Intra-band concurrent operation: PC3, PC2 |  |  |
| 6.5F.1 | Occupied bandwidth for shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |  |
| 6.5F.2.2 | Spectrum emission mask for operation with shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |  |
| 6.5F.2.4 | Adjacent channel leakage ratio for operation with shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |  |
| 6.5F.2.4.2 | Shared spectrum channel access ACLR with additional requirement for NS_29 | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |  |
| 6.5F.3.1 | General spurious emissions | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  | NOTE 1 |  |
| 6.5F.3.3 | Additional spurious emissions for shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |  |
| 6.5F.4 | Transmit intermodulation for shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  | NOTE 1 |  |
| 6.5G.1 | Occupied bandwidth for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting txDiversity-r16 | D030 | PC1.5 PC2 PC3 |  |  |
| 6.5G.1_1 | Occupied bandwidth for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  |  |  |
| 6.5G.2.1 | Spectrum emission mask for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting txDiversity-r16 | D030 | PC1.5 PC2 PC3 |  |  |
| 6.5G.2.1_1 | Spectrum emission mask for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  | NOTE 1 |  |
| 6.5G.2.2 | Additional spectrum emission mask for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting txDiversity-r16 | D030 | PC1.5 PC2 PC3 |  |  |
| 6.5G.2.2_1 | Additional spectrum emission mask for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  | NOTE 1 |  |
| 6.5G.2.3.1 | NR ACLR for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting txDiversity-r16 | D030 | PC1.5 PC2 PC3 |  |  |
| 6.5G.2.3.2 | UTRA ACLR for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting txDiversity-r16 | D030 | PC3 |  |  |
| 6.5G.2.3.2_1 | UTRA ACLR for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  | NOTE 1 |  |
| 6.5G.2.3_1.1 | NR ACLR for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  | NOTE 1 |  |
| 6.5G.3.1 | General spurious emissions for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting txDiversity-r16 | D030 | PC1.5 PC2 PC3 |  |  |
| 6.5G.3.2 | Spurious emissions for UE co-existence for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting txDiversity-r16 | D030 | PC1.5 PC2 PC3 |  |  |
| 6.5G.3.3 | Additional spurious emissions for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting txDiversity-r16 | D030 | PC1.5 PC2 PC3 |  |  |
| 6.5G.3.3_1 | Additional Spurious emissions for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  | NOTE 1 |  |
| 6.5G.3_1.1 | General spurious emissions for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  | NOTE 1 |  |
| 6.5G.3_1.2 | Spurious emissions for UE co-existence for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  | NOTE 1 |  |
| 6.5G.4 | Transmit intermodulation for Tx Diversity | Rel-15 | C001g | UEs supporting 5GS FR1 and supporting txDiversity-r16 | D030 | PC1.5 PC2 PC3 |  |  |
| 6.5G.4_1 | Transmit intermodulation for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  |  |  |
| 6.5H.1.1 | Occupied bandwidth for intra-band UL contiguous CA for UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.5H.1.2.1 | Spectrum emission mask for intra-band UL contiguous CA for UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.5H.1.2.2 | Additional spectrum emission mask for intra-band UL contiguous CA for UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.5H.1.2.3 | NR ACLR for intra-band UL contiguous CA for UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.5H.1.3.1 | General spurious emissions for intra-band UL contiguous CA for UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.5H.1.3.2 | Spurious emissions for UE co-existence for intra-band UL contiguous CA for UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.5H.1.3.3 | Additional spurious emissions for intra-band UL contiguous CA for UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.5H.1.4 | Transmit intermodulation for intra-band UL contiguous CA for UL MIMO | Rel-15 | C285 | UEs supporting 5GS FR1 and intra-band contiguous CA (2UL CA) and MIMO. | E042 | PC2 PC3 |  |  |
| 6.5H.3.3.1 | General spurious emissions for inter-band UL CA with UL MIMO | Rel-17 | C328 | UEs supporting 5GS FR1 and inter-band CA (2UL CA) with UL MIMO. | E045 | PC1.5 PC2 PC3 |  |  |
| 6.5H.3.3.2 | Spurious emissions for UE co-existence for inter-band UL CA with UL MIMO | Rel-17 | C328 | UEs supporting 5GS FR1 and inter-band CA (2UL CA) with UL MIMO. | E045 | PC1.5 PC2 PC3 |  |  |
| 6.5J.1 | Occupied bandwidth for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 6.5J.2.2 | Spectrum emissions mask for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 6.5J.3.1 | General spurious emissions for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 6.5L.3.3.1 | General spurious emissions for inter-band UL CA with Tx Diversity | Rel-17 | C329 | UEs supporting 5GS FR1 and inter-band CA (2UL CA) with Tx Diversity. | E046 | PC1.5 PC2 PC3 |  |  |
| 6.5L.3.3.2 | Spurious emissions for UE co-existence for inter-band UL CA with Tx Diversity | Rel-17 | C329 | UEs supporting 5GS FR1 and inter-band CA (2UL CA) with Tx Diversity. | E046 | PC1.5 PC2 PC3 |  |  |
| 7 | Receiver Characteristics |  |  |  |  |  |  |  |
| 7.3.2 | Reference sensitivity power level | Rel-15 | C001l | UEs supporting 5GS FR1 and not supporting RedCap | D001 | 2Rx 4Rx 8Rx PC1.5 PC2 PC3 |  |  |
| 7.3.2_1 | Reference sensitivity power level for XR | Rel-18 | C416 | XR UEs supporting 5GS FR1 and supportOf2RxXR-r18 if 2Rx XR UE | D001 | 2Rx 4Rx | NOTE 1 |  |
| 7.3A.1 | Reference sensitivity power level for 2DL CA without exception | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 | 2Rx 4Rx PC2 PC3 |  |  |
| 7.3A.1_1 | Reference sensitivity power level for 2DL CA exceptions | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 E015a E015 | 2Rx 4Rx PC2 PC3 |  |  |
| 7.3A.2 | Reference sensitivity power level for 3DL CA without exceptions | Rel-15 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 | 2Rx 4Rx PC2 PC3 |  |  |
| 7.3A.2_1 | Reference sensitivity power level for 3DL CA exceptions | Rel-15 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 E015 | 2Rx 4Rx PC2 PC3 |  |  |
| 7.3A.3 | Reference sensitivity power level for 4DL CA | Rel-15 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 | 2Rx 4Rx PC2 PC3 |  |  |
| 7.3C.2 | Reference sensitivity power level for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |  |
| 7.3D.2 | Reference sensitivity power level for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |  |
| 7.3D.2_1 | Reference sensitivity power level for SUL with UL MIMO | Rel-17 | C179a | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 7.3D.2_3 | Reference sensitivity power level for UL MIMO for UE supporting 4Tx | Rel-18 | C401 | UEs supporting 5GS FDD FR1 and 4-layer codebook based UL MIMO | D012a |  |  |  |
| 7.3E.2 | Reference sensitivity for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  | NOTE 1 |  |
| 7.3E.3 | Reference sensitivity for V2X / Concurrent operation | Rel-16 | C079a | UEs supporting 5GS FR1 and NR sidelink and UEs supporting 5GS FR1 and NR V2X inter-band concurrent operation | E050 | Inter-band concurrent operation: PC3 |  |  |
| 7.3F.2 | Reference sensitivity for shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |  |
| 7.3G.1 | Reference sensitivity for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  |  |  |
| 7.3I.2 | Reference sensitivity power level for RedCap | Rel-17 | C177 | RedCap UEs supporting 5GS FR1 | D001b |  |  |  |
| 7.3I.3 | Reference sensitivity power level for eRedCap | Rel-18 | C372 | eRedCap UEs supporting 5GS FR1 | D001b |  |  |  |
| 7.3J.2 | Reference sensitivity power level for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 7.4 | Maximum input level | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 7.4 if UE supports NSA and TS 38.521-3 TC 7.4B.2 or 7.4B.3 has been executed. |  |
| 7.4A.1 | Maximum input level for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |  |
| 7.4A.2 | Maximum input level for CA (3DL CA) | Rel-15 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |  |
| 7.4A.3 | Maximum input level for CA (4DL CA) | Rel-15 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 |  |  |  |
| 7.4A.4 | Maximum input level for CA (5DL CA) | Rel-16 | C313 | UEs supporting 5GS FR1 and CA (5DL CA) | E018a |  | Skip TC 7.4A.4 if UE supports NSA and TS 38.521-3 TC 7.4B.3_1.4 has been executed. |  |
| 7.4D | Maximum input level for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |  |
| 7.4D_1 | Maximum input level for SUL with UL MIMO | Rel-17 | C179a | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 7.4D_2 | Maximum input level for UL MIMO for UE supporting 4Tx | Rel-18 | C401 | UEs supporting 5GS FDD FR1 and 4-layer codebook based UL MIMO | D012a |  |  |  |
| 7.4E.1 | Maximum input level for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 7.4F | Maximum input level for shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |  |
| 7.4G.1 | Maximum input level for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  |  |  |
| 7.4J | Maximum input level for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 7.5 | Adjacent channel selectivity | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | NOTE 1 Skip TC 7.5 if UE supports NSA and TS 38.521-3 TC 7.5B.2 or 7.5B.3 has been executed. |  |
| 7.5A.1 | Adjacent channel selectivity for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |  |
| 7.5A.2 | Adjacent channel selectivity for CA (3DL CA) | Rel-15 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |  |
| 7.5A.3 | Adjacent channel selectivity for CA (4DL CA) | Rel-15 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 |  |  |  |
| 7.5A.4 | Adjacent channel selectivity for CA (5DL CA) | Rel-16 | C313 | UEs supporting 5GS FR1 and CA (5DL CA) | E018a |  | Skip TC 7.5A.4 if UE supports NSA and TS 38.521-3 TC [FFS] has been executed. |
| 7.5D | Adjacent channel selectivity for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |  |
| 7.5D_1 | Adjacent channel selectivity for SUL with UL MIMO | Rel-17 | C179a | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 7.5D_2 | Adjacent channel selectivity for UL MIMO for UE supporting 4Tx | Rel-18 | C401 | UEs supporting 5GS FDD FR1 and 4-layer codebook based UL MIMO | D012a |  |  |  |
| 7.5E.1 | Adjacent channel selectivity for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 7.5E.2 | Adjacent channel selectivity for V2X / concurrent operation | Rel-16 | C079a | UEs supporting 5GS FR1 and NR sidelink and UEs supporting 5GS FR1 and NR V2X inter-band concurrent operation | E050 | Inter-band concurrent operation: PC3 |  |  |
| 7.5F.1 | Adjacent channel selectivity for shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |  |
| 7.5G.1 | Adjacent channel selectivity for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  |  |  |
| 7.5J | Adjacent channel selectivity for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 7.6.2 | In-band blocking | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 7.6.2 if UE supports NSA and TS 38.521-3 TC 7.6B.2.2 or 7.6B.2.3 has been executed. |  |
| 7.6.3 | Out-of-band blocking | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  |  |  |
| 7.6.4 | Narrow band blocking | Rel-15 | C001 | UEs supporting 5GS FR1 | D004 |  | Skip TC 7.6.4 if UE supports NSA and TS 38.521-3 TC 7.6B.4.2 or 7.6B.4.3 has been executed. |  |
| 7.6A.2.1 | In-band blocking for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |  |
| 7.6A.2.2 | In-band blocking for CA (3DL CA) | Rel-15 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |  |
| 7.6A.2.3 | In-band blocking for CA (4DL CA) | Rel-15 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 |  | Skip TC 7.6A.2.3 if UE supports NSA and TS 38.521-3 TC 7.6B.2.3_1.3 has been executed. |  |
| 7.6A.2.4 | In-band blocking for CA (5DL CA) | Rel-16 | C313 | UEs supporting 5GS FR1 and CA (5DL CA) | E018a |  | NOTE 1 |  |
| 7.6A.3.1 | Out-of-band blocking for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |  |
| 7.6A.3.2 | Out-of-band blocking for CA (3DL CA) | Rel-15 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |  |
| 7.6A.3.3 | Out-of-band blocking for CA (4DL CA) | Rel-15 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 |  |  |  |
| 7.6A.3.4 | Out-of-band blocking for CA (5DL CA) | Rel-15 | C313 | UEs supporting 5GS FR1 and CA (5DL CA) | E018a |  | NOTE 1 |  |
| 7.6A.4.1 | Narrow band blocking for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |  |
| 7.6A.4.2 | Narrow band blocking for CA (3DL CA) | Rel-15 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |  |
| 7.6A.4.3 | Narrow band blocking for CA (4DL CA) | Rel-15 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 |  | Skip TC 7.6A.4.3 if UE supports NSA and TS 38.521-3 TC 7.6B.4.3_1.3 has been executed. |  |
| 7.6A.4.4 | Narrow band blocking for CA (5DL CA) | Rel-15 | C313 | UEs supporting 5GS FR1 and CA (5DL CA) | E018a |  | NOTE 1 |  |
| 7.6C.2 | In-band blocking for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |  |
| 7.6C.2_1 | Inband Blocking for SUL with DL CA | Rel-17 | C078 | UEs supporting 5GS FR1 and SUL and intra-band contiguous CA (2DL CA) | E031 |  |  |  |
| 7.6C.3 | Out-of-band blocking for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |  |
| 7.6C.3_1 | Out-of-band blocking for SUL with DL CA | Rel-17 | C078 | UEs supporting 5GS FR1 and SUL and intra-band contiguous CA (2DL CA) | E031 |  |  |  |
| 7.6D.2 | In-band blocking for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |  |
| 7.6D.2_1 | In-band blocking for SUL with UL MIMO | Rel-17 | C179a | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 7.6D.2_2 | In-band blocking for UL MIMO for UE supporting 4Tx | Rel-18 | C401 | UEs supporting 5GS FDD FR1 and 4-layer codebook based UL MIMO | D012a |  |  |  |
| 7.6D.3 | Out-of-band blocking for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |  |
| 7.6D.3_1 | Out-of-band blocking for SUL with UL MIMO | Rel-17 | C179a | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 7.6D.3_2 | Out-of-band blocking for UL MIMO for UE supporting 4Tx | Rel-18 | C401 | UEs supporting 5GS FDD FR1 and 4-layer codebook based UL MIMO | D012a |  |  |  |
| 7.6D.4 | Narrow band blocking for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |  |
| 7.6D.4_1 | Void |  |  |  |  |  |  |  |
| 7.6D.4_3 | Narrow band blocking for UL MIMO for UE supporting 4Tx | Rel-18 | C401 | UEs supporting 5GS FDD FR1 and 4-layer codebook based UL MIMO | D012a |  |  |  |
| 7.6E.2.1 | In-band blocking for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 7.6E.3.1 | Out-of-band blocking for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 7.6F.2.1 | In-band blocking for shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |  |
| 7.6F.3.1 | Out-of-band blocking for shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |  |
| 7.6G.1 | In-band blocking for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  |  |  |
| 7.6G.2 | Out-of-band blocking for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  |  |  |
| 7.6G.3 | Narrow band blocking for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  |  |  |
| 7.6J.2 | In-band blocking for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 7.6J.3 | Out-of-band blocking for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 7.7 | Spurious response | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  |  |  |
| 7.7A.1 | Spurious response for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |  |
| 7.7A.2 | Spurious response for CA (3DL CA) | Rel-15 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |  |
| 7.7A.3 | Spurious response for CA (4DL CA) | Rel-15 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 |  |  |  |
| 7.7A.4 | Spurious response for CA (5DL CA) | Rel-15 | C313 | UEs supporting 5GS FR1 and CA (5DL CA) | E018a |  | NOTE 1 |  |
| 7.7C | Spurious response for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |  |
| 7.7D | Spurious response for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |  |
| 7.7D_1 | Spurious response for SUL with UL MIMO | Rel-17 | C179a | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 7.7D_2 | Spurious response for UL MIMO for UE supporting 4Tx | Rel-18 | C401 | UEs supporting 5GS FDD FR1 and 4-layer codebook based UL MIMO | D012a |  |  |  |
| 7.7E.1 | Spurious response for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 7.7F.1 | Spurious response for shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |  |
| 7.7G.1 | Spurious response for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  |  |  |
| 7.7J | Spurious response for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 7.8.2 | Wide band Intermodulation | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 7.8.2 if UE supports NSA and TS 38.521-3 TC 7.8B.2.2 or 7.8B.2.3 has been executed. |  |
| 7.8A.2.1 | Wide band Intermodulation for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |  |
| 7.8A.2.2 | Wide band Intermodulation for CA (3DL CA) | Rel-15 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |  |
| 7.8A.2.3 | Wide band Intermodulation for CA (4DL CA) | Rel-15 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 |  |  |  |
| 7.8A.2.4 | Wide band Intermodulation for CA (5DL CA) | Rel-16 | C313 | UEs supporting 5GS FR1 and CA (5DL CA) | E018a |  | NOTE 1 |  |
| 7.8D.2 | Wide band Intermodulation for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |  |
| 7.8D.2_1 | Wide band Intermodulation for SUL with UL MIMO | Rel-17 | C179a | UEs supporting 5GS FR1 and SUL and UL MIMO | D024 |  |  |  |
| 7.8D.2_2 | Wide band Intermodulation for UL MIMO for UE supporting 4Tx | Rel-18 | C401 | UEs supporting 5GS FDD FR1 and 4-layer codebook based UL MIMO | D012a |  |  |  |
| 7.8E.2.1 | Wide band Intermodulation for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |  |
| 7.8F.2 | Wide band Intermodulation for shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |  |
| 7.8G.1 | Intermodulation characteristics for Tx Diversity for UE supporting 4Tx | Rel-18 | C403 | UEs supporting 5GS FR1 and supporting 4Tx Diversity | D030a |  |  |  |
| 7.8J.2 | Wide band intermodulation for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  | NOTE 1 |  |
| 7.9 | Spurious emissions | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 7.9 if UE supports NSA and TS 38.521-3 TC 7.9B.1 or 7.9B.2 or 7.9B.3 has been executed. |  |
| 7.9A.1 | Spurious emissions for CA (2DL CA) | Rel-15 | C005 | UEs supporting 5GS FR1 and inter-band 2DL CA with a DL-only band | E002 |  |  |  |
| 7.9J | Spurious emissions for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  |  |  |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.521-1. NOTE 2: The test case is optional for Rel-17 RedCap UE implementing SUL. NOTE 3: Uplink configurations CA_nA-nB, DC_A_nB, DC_B_nA, SUL_A-nD and SUL_B-nC can be noted as same UL CA configuration, where band nC and nD are the corresponding SUL bands with same UL frequency range of band (n)A and (n)B |  |  |  |  |  |  |  |  |

<Start of Changes>

4.3.1 FR1 standalone conformance test cases for Satellite Access

Table 4.3.1-1: Applicability of RF SA FR1 conformance test cases, ref. TS 38.521-5 [12]

| Clause | TC Title | Release | Applicability |  | Tested Bands/CA-Configurations Selection | Branch | Additional Information |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |
| 6 | Transmitter Characteristics |  |  |  |  |  |  |
| 6.2.1 | UE maximum output power | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 6.2.2 | Maximum Power Reduction (MPR) | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 6.2.3 | UE additional maximum output power reduction | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 6.2.4 | Configured transmitted power | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 6.3.1 | Minimum output power | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 6.3.3 | Tx ON/OFF Time Mask | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 6.3.4.2 | Absolute power tolerance | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 6.3.4.3 | Relative power tolerance | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 6.3.4.4 | Aggregate power tolerance | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 6.4.1_1 | Frequency error with GSO ephemeris | Rel-17 | C001p | UEs supporting 5GS FDD FR1 satellite access and only GSO or both GSO and NGSO | D027 |  |  |
| 6.4.1_2 | Frequency error with NGSO ephemeris | Rel-17 | C001q | UEs supporting 5GS FDD FR1 satellite access and only NGSO or both GSO and NGSO | D027 |  |  |
| 6.4.2.1 | Error Vector Magnitude | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 6.4.2.1a | Error Vector Magnitude including symbols with transient period | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 6.4.2.2 | Carrier Leakage | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 6.4.2.3 | In-band emissions | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 6.4.2.4 | EVM equalizer spectrum flatness | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 6.4.2.5 | EVM equalizer spectrum flatness for Pi/2 BPSK | Rel-17 | C111a | UEs supporting 5GS FDD FR1 satellite access and pi/2-BPSK modulation scheme and low PAPR DMRS | D027 |  |  |
| 6.5.1 | Occupied bandwidth | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 6.5.2.2 | Spectrum Emissions Mask | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 6.5.2.4 | Adjacent Channel Leakage Ratio | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 6.5.2.4.1 | NR ACLR | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 6.5.2.4.2 | UTRA ACLR | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access PC3 | D027 | GSO NGSO |  |
| 6.5.3.1 | General Spurious | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 6.5.3.2 | Spurious emissions for UE co-existence | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO | NOTE 1 |
| 6.5.3.3 | Additional Spurious emissions | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 6.5.4 | Transmit intermodulation | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 7 | Receiver Characteristics |  |  |  |  |  |  |
| 7.3.2 | Reference sensitivity power level | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 7.3.2_1 | Reference sensitivity power level for flexible Tx-Rx frequency separation | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 7.4 | Maximum input level | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 7.5 | Adjacent channel selectivity | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 7.6.2 | In-band blocking | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 7.6.3 | Out of Band blocking | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 7.6.4 | Narrow band blocking | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 7.7 | Spurious response | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 7.8.2 | Wide band Intermodulation | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| 7.9 | Spurious emission | Rel-17 | C001o | UEs supporting 5GS FDD FR1 satellite access | D027 | GSO NGSO |  |
| Note 1: The test case is incomplete for any band but has basic test configurations already. NOTE 1 can be removed only when the test case is complete for at least one band for at least one feature included in the test case. Detailed completion status can be found in the corresponding test case section in 38.521-5. |  |  |  |  |  |  |  |

<End of change>