3GPP TSG-RAN5 Meeting #109	R5-256503

# Dallas, US, 17th – 21st November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | 0122 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Transmit EIRP density and other updates for band 254 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, IoT_NTN_FDD_LS_band-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Minimum conformance requirements have been clarified for Transmit EIRP density. - Test procedure for Transmit EIRP density test cases are incomplete. - Test requirements for Transmit EIRP density test cases require alignment with updated minimum conformance requirements. - Minimum conformance requirements for band 254 have been clarified for Additional Spectrum Emission Mask for category M1 and NB-IoT NTN - Minimum conformance requirements for band 254 have been clarified for Additional spurious emissions for category M1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Minimum conformance requirements for Transmit EIRP density updated and associated editor’s note removed. - Test procedure for Transmit EIRP density test cases have been completed. - Transmit EIRP density test requriements updated - Minimum conformance requirements for band 254 for Additional Spectrum Emission Mask for category M1 and NB-IoT NTN updated. - Minimum conformance requirements for band 254 for Additional spurious emissions for category M1 updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For band 254, Transmit EIRP density test case will remain incomplete and Additional Spectrum Emission Mask and Additional spurious emissions will remain ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.1, 6.2A.1_1, 62B.1, 62B.1_1, 6.5A.3.3.3, 6.5A.4.4.3.4, 6.5B.3.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR depends on RAN4 CR R4-2520566. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

### 6.2A.1	UE maximum output power for category M1

6.2A.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2A.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 17 and forward of UE category M1 that support satellite access operation.

6.2A.1.3	Minimum conformance requirements

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2A.1-1: UE Power Class

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 |  |  | 23 | +/-2 | 20 | +/-2 |
| 255 |  |  | 23 | +/-2 | 20 | +/-2 |
| 254 |  |  | 23 | +/-2 | 20 | +/-2 |
| 253 |  |  | 23 | +/-2 | 20 | +/-2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance |  |  |  |  |  |  |

The default power class PPowerClass_Default for an operating band is Power Class 3 unless otherwise stated.

The normative reference for this requirement is TS 36.102 [11] clause 6.2.A.

The UE shall meet the following additional requirements for maximum transmission power density specified in Table 6.2A.1-2 when NS is signalled and when the configured channel overlaps with any portion of the specified frequency range.

Table 6.2A.1-2: Additional requirements for transmit power density

| EUTRABand | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum power density |  | Measurement method |
| --- | --- | --- | --- | --- | --- | --- |
| 254 | NS_04N | 1.4 | 1610 - 1618.25 | 27dBm/4kHz |  | Mean spectral limit (note 2) |
|  | NS_05N | 1.4 | 1618.25 - 1626.5 |  |  |  |
|  | NS_11N | 1.4 | 1610 - 1618.25 | 15dBm/4kHz |  | Peak spectral limit (note 2) |
|  | NS_12N | 1.4 | 1618.25 - 1626.5 |  |  |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. NOTE 2: Indicated limit is applied to any frequency of the 1610-1626.5MHz band where a UE can operate. |  |  |  |  |  |  |

6.2A.1.4	Test description

6.2A.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on operating bands for satellite access that specified in sub-clause 5.2A. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2A.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.2A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 4.3.1 |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] subclause 4.3.1 |  | 1.4MHz |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |
| Ch BW | N/A for Max UE output power testing |  | Mod'n | RB allocation |
|  |  |  |  | HD-FDD |
| 1.4MHz |  |  | QPSK | 1 |
| 1.4MHz |  |  | QPSK | 2 |
| NOTE 1: Test Channel Bandwidth is checked separately for each E-UTRA satellite access band, the applicable channel bandwidths are specified in Table 5.3A-1. NOTE 2: The 1 RB allocation shall be tested at RB#0 with narrowband index 0 for low and mid range, RB #5 with max narrowband index for high range test frequency. NOTE 3: The RBstart of non-1RB allocation shall be RB #0 with narrowband index 0 for low and mid range, RB# (6 - RB allocation) with max narrowband index for high range test frequency. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.2A.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through any preconfigured means.

7.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] table 5.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and table 5.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB31 information during the duration of the test as define in TS 36.508[12] clause 5.6.3.1.

8.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

9.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.2A.1.4.3.

6.2A.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 6.2A.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one sub-frame (1ms). For HD-FDD slots with transient periods and Half-duplex guard subframe are not under test.

6.2A.1.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 4.6.

6.2A.1.5	Test requirements

The maximum output power derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2A.1.5-1.

Table 6.2A.1.5-1: UE Power Class test requirements

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) | Class 6 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 256 |  |  | 23 | ±2.7 | 20 | ±2.7 |  |  |
| 255 |  |  | 23 | ±2.7 | 20 | ±2.7 |  |  |
| 254 |  |  | 23 | ±2.7 | 20 | ±2.7 |  |  |
| 253 |  |  | 23 | ±2.7 | 20 | ±2.7 |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. |  |  |  |  |  |  |  |  |

### 6.2A.1_1	UE maximum output power for category M1: Transmit EIRP density

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

- Band 254 Test tolerances for Transmit EIRP density.

- Band 254 Measurement uncertainties for Transmit EIRP density.

6.2A.1_1.1	Test purpose

Same as 6.2A.1.1.

6.2A.1_1.2	Test applicability

Same as 6.2A.1.2.

6.2A.1_1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2A.1.3.

6.2A.1_1.4	Test description

6.2A.1_1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on operating bands for satellite access that specified in sub-clause 5.2A. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2A.1_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.2A.1_1.4.1-1: Test Configuration Table for transmit EIRP power density measurements

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 4.3.1 |  | Low range, Mid range for NS_04N and NS_11N Mid range, High range for NS_05N and NS_12N |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] subclause 4.3.1 |  | 1.4MHz |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |
| Ch BW | N/A for Max UE output power testing |  | Mod'n | RB allocation |
|  |  |  |  | HD-FDD |
| 1.4MHz |  |  | QPSK | 1 |
| NOTE 1: Test Channel Bandwidth is checked separately for each E-UTRA satellite access band, the applicable channel bandwidths are specified in Table 5.3A-1. NOTE 2: The 1 RB allocation shall be tested at RB#0 with narrowband index 0 for low and mid range, RB #5 with max narrowband index for high range test frequency. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.2A.1_1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through any preconfigured means.

7.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] table 5.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and table 5.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB31 information during the duration of the test as define in TS 36.508[12] clause 5.6.3.1.

8.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

9.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.2A.1_1.4.3.

6.2A.1_1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 6.2A.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in each 4 kHz interval within the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one sub-frame (1ms). For HD-FDD slots with transient periods and Half-duplex guard subframe are not under test.

4.	Derive mean or peak values of all 4 kHz intervals within the channel bandwidth (depending on the network signaled value) of the radio access mode. These values will correspond to transmit power mean or peak density values respectively assuming 0 dBi UE antenna gain.

5.	Repeat steps 1, 2, 3 and 4 for all applicable network signalled values.

6.2A.1_1.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 4.6 with additional exceptions defined in 6.2A.3.4.3 associated to each applicable network signalled value used in transmit EIRP density measurements.

6.2A.1_1.5	Test requirements

Mean and peak ttransmit EIRP density values derived in step 4 shall be within the range prescribed by the additional requirements for transmit EIRP density and tolerance in Table 6.2A.1_1.5-1.

Table 6.2A.1_1.5-1: Additional requirements for transmit power density

| EUTRABand | NS value |  | Channel bandwidth (MHz) |  | Frequency range (MHz) |  | Maximum power density |  | Measurement method | Tolerance |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 254 | NS_04N |  | 1.4 |  | 1610 - 1618.25 |  | 27dBm/4kHz |  | Mean spectral limit (note 2) | FFS |  |
|  | NS_05N |  | 1.4 |  | 1618.25 - 1626.5 |  |  |  |  |  |  |
|  | NS_11N |  | 1.4 |  | 1610 - 1618.25 |  | 15dBm/4kHz |  | Peak spectral limit (note 2) | FFS |  |
|  | NS_12N |  | 1.4 |  | 1618.25 - 1626.5 |  |  |  |  |  |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. NOTE 2: Indicated limit is applied to any frequency of the 1610-1626.5MHz band where a UE can operate. |  |  |  |  |  |  |  |  |  |  |  |

<<< Skip Unchanged Sections >>>

### 6.2B.1	UE maximum output power for category NB1 and NB2

6.2B.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2B.1.2	Test applicability

This test case applies to all types of NB-IoT UE release 17 and forward of category NB1 and NB2 that support satellite access operation.

6.2B.1.3	Minimum conformance requirements

Category NB1 and NB2 UE Power Classes are specified in Table 6.2B.1.3-1 and define the maximum output power for any transmission bandwidth within the category NB1 and NB2 channel bandwidth. For 3.75kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one slot (2ms) excluding the 2304Ts gap when UE is not transmitting. For 15kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one sub-frame (1ms).

Table 6.2B.1.3-1: UE Power Class

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 23 | +/-2 | 20 | +/-2 |
| 255 | 23 | +/-2 | 20 | +/-2 |
| 254 | 23 | +/-2 | 20 | +/-2 |
| 253 | 23 | +/-2 | 20 | +/-2 |

The normative reference for this requirement is TS 36.102 [11] clause 6.2B.1.

The UE shall meet the following additional requirements for maximum transmission power density specified in Table 6.2B.1-2 when NS is signalled and when the configured channel overlaps with any portion of the specified frequency range.

Table 6.2B.1-2: Additional requirements for transmit power density

| EUTRABand | NS value | Channel bandwidth (MHz) |  | Frequency range (MHz) |  | Maximum power density |  | Measurement method |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 254 | NS_04N | 0.2 |  | 1610 - 1618.25 |  | 27dBm/4kHz |  | Mean spectral limit (note 2) |
|  | NS_05N | 0.2 |  | 1618.25 - 1626.5 |  |  |  |  |
|  | NS_11N | 1.4 |  | 1610 - 1618.25 |  | 15dBm/4kHz |  | Peak spectral limit (note 2) |
|  | NS_12N | 1.4 |  | 1618.25 - 1626.5 |  |  |  |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. NOTE 2: Indicated limit is applied to any frequency of the 1610-1626.5MHz band where a UE can operate. |  |  |  |  |  |  |  |  |

6.2B.1.4	Test description

6.2B.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA operating bands defined for NB-IoT NTN in clause 5.2B. All of these configurations shall be tested with applicable test parameters and are shown in table 6.2B.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in TS 36.521 Annex A.2. Configurations of NPDSCH and NPDCCH before measurement are specified in TS 36.521 Annex C.2.

Table 6.2B.1.4.1-1: Test Configuration Initial Conditions for FDD

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Low range, Mid range, High range |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 (Note 2) |  | BPSK | 1@0 | 3.75 |
| 2 (Note 3) |  | BPSK | 1@47 | 3.75 |
| 3 (Note 2) |  | QPSK | 1@0 | 15 |
| 4 (Note 3) |  | QPSK | 1@11 | 15 |
| 5 (Note 1) |  | QPSK | 3@3 | 15 |
| NOTE 1: Applicable to UE supporting UL multi-tone transmissions. NOTE 2: only applicable for low range. NOTE 3: only applicable for high range. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only the main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS 36.521 Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1.1 and H.4.0.

4.	The UL Reference Measurement channel is set according to Table 6.2B.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	UE location according to TS 36.508 [12] clause 8.2.6.1 is provided to the UE through any preconfigured means.

7.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] table 8.2.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and table 8.2.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB31-NB information during the duration of the test as define in TS 36.508[12] clause 8.2.6.3.1.

8.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

9.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.2B.1.4.3.

6.2B.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via NPDCCH with DCI format N0 for C_RNTI to schedule the UL RMC according to Table 6.2B.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC (UE should be already transmitting PUMAX after Initial Conditions setting).

2.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one sub-frame (1ms) for sub-carrier spacing of 15 kHz or one slot (2ms) excluding the 2304Ts gap when UE is not transmitting for sub-carrier spacing of 3.75 kHz. For TDD slots with transient periods are not under test. For Half-Duplex guard subframes are not under test.

NOTE 1:	For configuration IDs applicable to UE depending on UE capability in Test Configuration Table with different UL sub-carrier spacing, the SS shall release the connection through State 3A-NB and finally ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5 using the appropriate UL subcarrier spacing in Random Access Response message.

6.2B.1.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 8.1.6.

6.2B.1.5	Test requirements

The maximum output power derived in step 2 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2B.1.5-1.

Table 6.2B.1.5-1: UE Power Class test requirement

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 23 | +/-2.7 | 20 | +/-2.7 |
| 255 | 23 | +/-2.7 | 20 | +/-2.7 |
| 254 | 23 | +/-2.7 | 20 | +/-2.7 |
| 253 | 23 | +/-2.7 | 20 | +/-2.7 |

### 6.2B.1_1	UE maximum output power for category NB1 and NB2: Transmit EIRP density

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

- Band 254 Test tolerances for Transmit EIRP density.

- Band 254 Measurement uncertainties for Transmit EIRP density.

6.2B.1_1.1	Test purpose

Same as 6.2B.1.1.

6.2B.1_!.2	Test applicability

Same as 6.2B.1.2.

6.2B.1_1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2B.1.3.

6.2B.1_1.4	Test description

6.2B.1_1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA operating bands defined for NB-IoT NTN in clause 5.2B. All of these configurations shall be tested with applicable test parameters and are shown in table 6.2B.1_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in TS 36.521 Annex A.2. Configurations of NPDSCH and NPDCCH before measurement are specified in TS 36.521 Annex C.2.

Table 6.2B.1_1.4.1-1 Test Configuration Initial Conditions for FDD for transmit EIRP power density measurements

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Low range, Mid range for NS_04N and NS_11N Mid range, High range for NS_05N and NS_12N |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 (Note 1) |  | BPSK | 1@0 | 3.75 |
| 2 (Note 2) |  | BPSK | 1@47 | 3.75 |
| 3 (Note 1) |  | QPSK | 1@0 | 15 |
| 4 (Note 2) |  | QPSK | 1@11 | 15 |
| NOTE 1: only applicable for low range. NOTE 2: only applicable for high range. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only the main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS 36.521 Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1.1 and H.4.0.

4.	The UL Reference Measurement channel is set according to Table 6.2B.1_1.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	UE location according to TS 36.508 [12] clause 8.2.6.1 is provided to the UE through any preconfigured means.

7.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] table 8.2.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and table 8.2.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB31-NB information during the duration of the test as define in TS 36.508[12] clause 8.2.6.3.1.

8.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

9.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.2B.1_1.4.3.

6.2B.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via NPDCCH with DCI format N0 for C_RNTI to schedule the UL RMC according to Table 6.2B.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC (UE should be already transmitting PUMAX after Initial Conditions setting).

2.	Measure the mean power of the UE in each 4 kHz interval within the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one sub-frame (1ms) for sub-carrier spacing of 15 kHz or one slot (2ms) excluding the 2304Ts gap when UE is not transmitting for sub-carrier spacing of 3.75 kHz. For TDD slots with transient periods are not under test. For Half-Duplex guard subframes are not under test.

3.	Derive mean or peak values of all 4 kHz intervals within the channel bandwidth (depending on the network signaled value) of the radio access mode. These values will correspond to transmit power mean or peak density values respectively assuming 0 dBi UE antenna gain.

4.	Repeat steps 1, 2 and 3 for all applicable network signaled values.

NOTE 1:	For configuration IDs applicable to UE depending on UE capability in Test Configuration Table with different UL sub-carrier spacing, the SS shall release the connection through State 3A-NB and finally ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5 using the appropriate UL subcarrier spacing in Random Access Response message.

6.2B.1.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 8.1.6 with additional exceptions defined in 6.2B.3.4.3 associated to each applicable network signalled value used in transmit EIRP density measurements.

6.2B.1_1.5	Test requirements

Mean and peak ttransmit EIRP density values derived in step 3 shall be within the range prescribed by the additional requirements for transmit EIRP density and tolerance in Table 6.2B.1_1.5-1.

Table 6.2B.1_1.5-1: Additional requirements for transmit EIRP density

| EUTRABand | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum power density | Measurement method | Tolerance |
| --- | --- | --- | --- | --- | --- | --- |
| 254 | NS_04N | 0.2 | 1610 - 1618.25 | 27dBm/4kHz | Mean spectral limit (note 2) | FFS |
|  | NS_05N | 0.2 | 1618.25 - 1626.5 |  |  |  |
|  | NS_11N | 0.2 | 1610 - 1618.25 | 15dBm/4kHz | Peak spectral limit (note 2) | FFS |
|  | NS_12N | 0.2 | 1618.25 - 1626.5 |  |  |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. NOTE 2: Indicated limit is applied to any frequency of the 1610-1626.5MHz band where a UE can operate. |  |  |  |  |  |  |

<<< Skip Unchanged Sections >>>

6.5A.3.3.3	Minimum conformance requirements

When "NS_02N" or "NS_03N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.3-1.

Table 6.5A.3.3.3-1: Additional requirements for "NS_02N" and "NS_03N"

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ±0 – 0.7 | -2 for PC3 -5 for PC5 | 4 kHz |
| ±0.7 – 2.8 | -12 for PC3 -15 for PC5 | 4 kHz |
| ±>2.8 | -13 for PC3 and PC5 | 4 kHz |

When "NS_04N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.3-2 for any channel bandwidth configured within 1610-1618.25MHz.

Table 6.5A.3.3.3-2: Additional requirements for "NS_04N"

| ΔfOOB (kHz) | Spectrum emission limit (dBm) |  | Measurement bandwidth |  | Measurement method |
| --- | --- | --- | --- | --- | --- |
| ± 0-160 | -2 |  | 30kHz |  |  |
| ± 160-2300 | -2 to -26 |  |  |  | Average |
| ± 2300-18500 | -26 |  |  |  |  |
| NOTE 1: Spectrum emissions are linearly interpolated in dBm versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |  |  |

When "NS_05N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.3-3 for any channel bandwidth configured within 1618.25-1626.5MHz.

Table 6.5A.3.3.3-3: Additional requirements for "NS_05N"

| ΔfOOB (kHz) | Spectrum emission limit (dBm) |  | Measurement bandwidth |  | Measurement method |
| --- | --- | --- | --- | --- | --- |
| ± 0-160 | -5 |  | 30kHz |  |  |
| ± 160-225 | -5 to -8.5 |  |  |  |  |
| ± 225-650 | -8.5 to -15 |  |  |  | Average |
| ± 650-1365 | -15 |  |  |  |  |
| ± 1365-1800 | -23 to -26 |  |  |  |  |
| ± 1800-16500 | -26 |  |  |  |  |
| NOTE 1: Spectrum emissions are linearly interpolated in dBm versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |  |  |

The normative reference for this requirement is TS 36.102 [11] clause 6.5A.3.3.

<<< Skip Unchanged Sections >>>

6.5A.4.4.3.4	Minimum requirement (network signalled value "NS_04N, NS_05N")

When "NS_04N or NS_05N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.3.4-1 where BWchannel equals to 1.4MHz. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5A.4.2.3-1 from the edge of the channel bandwidth.

Table 6.5A.4.4.3.4-1: Additional out-of-band requirements for "NS_04N, NS_05N"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | Measurement method |
| --- | --- | --- | --- |
|  | BWchannel |  |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Average (note 2) l |
| 1605 ≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
| 1628.5 ≤ f ≤ 1631.5 | -30 | 30kHz |  |
| 1631.5 ≤ f ≤ 1636.5 | -30 | 100kHz |  |
| 1636.5 ≤ f ≤ 1646.5 | -30 | 300kHz | Average |
| 1646.5 ≤ f ≤ 1666.5 | -30 | 1MHz |  |
| 1666.5 ≤ f ≤ 2200 | -30 | 3MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. NOTE 2: In the sub-band 1573.42MHz to 1580.42 MHz the average measurement time is 20ms. |  |  |  |

Table 6.5A.4.4.3.4-2: Void

The normative reference for this requirement is TS 36.102[11] subclause 6.5A.4.4.5.

<<< Skip Unchanged Sections >>>

6.5B.3.3.3	Minimum conformance requirements

When "NS_02N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.3.3.3-1.

Table 6.5B.3.3.3-1: Additional requirements for "NS_02N"

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0.09–0.28 | -2 for PC3 -5 for PC5 | 4 kHz |
| ± 0.28–0.85 | -12 for PC3 -15 for PC5 | 4 kHz |
| ± >0.85 | -13 for PC3 and PC5 | 4 kHz |

NOTE:	ΔfOOB = 0.09 MHz corresponds to an authorized bandwidth, as defined in C63.26-2015 [22], of 0.38 MHz.

When "NS_03N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.3.3.3-2.

Table 6.5B.3.3.3-2: Additional requirements for "NS_03N"

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0.09–0.28 | -2 for PC3 -5 for PC5 | 4 kHz |
| ± 0.28–0.85 | -12 for PC3 -15 for PC5 | 4 kHz |
| ± 0.85–1.7 | -13 for PC3 and PC5 | 4 kHz |

NOTE:	ΔfOOB = 0.09 MHz corresponds to an authorized bandwidth, as defined in C63.26-2015 [22], of 0.38 MHz.

When "NS_04N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.3.3.3-3 for any channel bandwidth configured within 1610-1618.25MHz.

Table 6.5B.3.3.3-3: Additional requirements for "NS_04N"

| ΔfOOB (kHz) | Spectrum emission limit (dBm) |  | Measurement bandwidth |  | Measurement method |
| --- | --- | --- | --- | --- | --- |
| ± 0-160 | -2 |  | 30kHz |  |  |
| ± 160-2300 | -2 to -26 |  |  |  | Average |
| ± 2300-18500 | -26 |  |  |  |  |
| NOTE 1: Spectrum emissions are linearly interpolated in dBm versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |  |  |

When "NS_05N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.3.3.3-4 for any channel bandwidth configured within 1618.25-1626.5MHz.

Table 6.5B.3.3.3-4: Additional requirements for "NS_05N"

| ΔfOOB (kHz) | Spectrum emission limit (dBm) |  | Measurement bandwidth |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
| ± 0-160 | -5 |  | 30kHz |  |  |
| ± 160-225 | -5 to -8.5 |  |  |  |  |
| ± 225-650 | -8.5 to -15 |  |  |  | Average |
| ± 650-1365 | -15 |  |  |  |  |
| ± 1365-1800 | -23 to -26 |  |  |  |  |
| ± 1800-16500 | -26 |  |  |  |  |
| NOTE 1: Spectrum emissions are linearly interpolated in dBm versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |  |  |

<<< END OF CHANGES >>>