3GPP TSG-RAN5 Meeting #109 	R5-256515

# Dallas, Texas, USA, 17th November 2025 – 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4268 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of common message contents for FR2 multi-Rx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Innovation Center Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_multiRX_DL-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Message contents specific for FR2 multi-Rx test cases are currently missing in the specification. Much of this content can be reused by multiple test cases, so it is more practical to define it in Annex H and refer to the corresponding content from the test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added new section H.3.14 with message contents specific for multi-Rx test cases - Implemented many of the required message contents. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case configuration would remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | H.3.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

## H.3.14	RRC messages and IE content exceptions for FR2 Multi-Rx

Table H.3.14-1: CSI-MeasConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-38 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment |  | Condition |  |  |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |  |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList { |  | 2 entries |  |  | CSI-RS |  |  |
| NZP-CSI-RS-ResourceSet[1] |  | NZP-CSI-RS-ResourceSet (Table H.3.14-2A) | set 1 |  |  |  |  |
| NZP-CSI-RS-ResourceSet[2] |  | NZP-CSI-RS-ResourceSet (Table H.3.14-2A) | set 2 |  |  |  |  |
| } |  |  |  |  |  |  |  |
| csi-SSB-ResourceSetToAddModList { |  | 2 entries |  |  | SSB |  |  |
| CSI-SSB-ResourceSet[1] |  | CSI-SSB-ResourceSet (Table H.3.14-2) | set 1 |  |  |  |  |
| CSI-SSB-ResourceSet[2] |  | CSI-SSB-ResourceSet (Table H.3.14-2) | set 2 |  |  |  |  |
| } |  |  |  |  |  |  |  |
| csi-ResourceConfigToAddModList { |  | 1 entry |  |  |  |  |  |
| CSI-ResourceConfig[1] |  | CSI-ResourceConfig | entry 1 |  |  |  |  |
| } |  |  |  |  |  |  |  |
| csi-ReportConfigToAddModList { |  | 1 entry |  |  |  |  |  |
| CSI-ReportConfig[1] |  | CSI-ReportConfig | entry 1 |  |  |  |  |
| } |  |  |  |  |  |  |  |
| reportTriggerSize |  | 1 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB | Configuration for SSB based CSI |
| CSI-RS | Configuration for CSI-RS based CSI |

Table H.3.14-2: CSI-SSB-ResourceSet

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-47 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-SSB-ResourceSet ::= SEQUENCE { |  |  |  |
| csi-SSB-ResourceSetId | 0 | set 1 |  |
|  | 1 | set 2 |  |
| csi-SSB-ResourceList { |  |  |  |
| SSB-Index[1] | 0 | set 1 |  |
|  | 1 | set 2 |  |
| } |  |  |  |
| } |  |  |  |

Table H.3.14-2A: NZP-CSI-RS-ResourceSet

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-87 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment |  | Condition |  |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |  |  |  |
| nzp-CSI-ResourceSetId |  | 0 | set 1 |  |  |  |
|  |  | 1 | set 2 |  |  |  |
| nzp-CSI-RS-Resources { |  |  |  |  |  |  |
| { |  |  |  |  |  |  |
| nzp-CSI-RS-ResourceId |  | 0 | set 1 |  |  |  |
| resourceMapping |  | CSI-RS-ResourceMapping |  |  |  |  |
| periodicityAndOffset |  | CSI-ResourcePeriodicityAndOffset |  |  |  |  |
| qcl-InfoPeriodicCSI-RS |  | TCI-StateId |  |  |  |  |
| } |  |  |  |  |  |  |
| { |  |  |  |  |  |  |
| nzp-CSI-RS-ResourceId |  | 1 | set 1 |  |  |  |
| resourceMapping |  | CSI-RS-ResourceMapping |  |  |  |  |
| periodicityAndOffset |  | CSI-ResourcePeriodicityAndOffset |  |  |  |  |
| qcl-InfoPeriodicCSI-RS |  | TCI-StateId |  |  |  |  |
| } |  |  |  |  |  |  |
| { |  |  |  |  |  |  |
| nzp-CSI-RS-ResourceId |  | 2 | set 2 |  |  |  |
| resourceMapping |  | CSI-RS-ResourceMapping |  |  |  |  |
| periodicityAndOffset |  | CSI-ResourcePeriodicityAndOffset |  |  |  |  |
| qcl-InfoPeriodicCSI-RS |  | TCI-StateId |  |  |  |  |
| } |  |  |  |  |  |  |
| { |  |  |  |  |  |  |
| nzp-CSI-RS-ResourceId |  | 3 | set 2 |  |  |  |
| resourceMapping |  | CSI-RS-ResourceMapping |  |  |  |  |
| periodicityAndOffset |  | CSI-ResourcePeriodicityAndOffset |  |  |  |  |
| qcl-InfoPeriodicCSI-RS |  | TCI-StateId |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| resourceType-r18 |  | periodic |  |  |  |  |
| } |  |  |  |  |  |  |

Table H.3.14-3: CSI-ResourceConfig

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-12B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark |  | Comment |  | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |  |  |
| csi-ResourceConfigId | 0 |  |  |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |  |  |
| nzp-CSI-RS-ResourceSetList { | 2 entries |  |  |  | CSI-RS |
| NZP-CSI-RS-ResourceSetId[0] | 0 |  |  |  |  |
| NZP-CSI-RS-ResourceSetId[0] | 1 |  |  |
| } |  |  |  |  |  |
| csi-SSB-ResourceSetList { | 2 entries |  |  |  | SSB |
| CSI-SSB-ResourceSetId[0] | 0 |  | set 1 |  |  |
| CSI-SSB-ResourceSetId[1] | 1 |  | set 2 |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| bwp-Id | BWP-Id |  |  |
| resourceType | Periodic |  |  |
| } |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB | Configuration for SSB based CSI |
| CSI-RS | Configuration for CSI-RS based CSI |

Table H.3.14-4: CSI-ReportConfig

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-12G |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  | PERIODIC |
| reportSlotConfig CHOICE { |  |  |  |
| slots16 | 4 | Periodicity 16 slots and offset 4 | 2_MS |
| slots40 | 4 | Periodicity 40 slots and offset 4 | 5_MS |
| pucch-CSI-ResourceList SEQUENCE { | 1 entry |  |  |
| (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| uplinkBandwidthPartId | BWP-Id of active BWP |  |  |
| pucch-Resource | 8 | PUCCH-format2 as configured in TS 38.508-1 [14], Table 4.6.3-112 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity CHOICE { |  |  |  |
| ssb-Index-RSRP | NULL |  | SS-RSRP |
| cri-RSRP | NULL |  | CSI-RSRP |
| } |  |  |  |
| groupBasedBeamReporting | enabled |  |  |
| groupBasedBeamReporting-v1710 SEQUENCE { |  |  |  |
| nrofReportedGroups-r17 | n1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| PERIODIC | Configuration for periodic reporting |
| SS-RSRP | L1-RSRP measurement based on SSB |
| CSI-RSRP | L1-RSRP measurement based on CSI-RS |
| 2_MS | CSI Report with 2ms periodicity (16 slots @ SCS=120kHz) |
| 5_MS | CSI Report with 2ms periodicity (40 slots @ SCS=120kHz) |

Table H.3.14-5: CSI-ResourceConfig

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-12B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF { | 1 entry |  | CSI-RS |
| NZP-CSI-RS-ResourceSetId[0] | 0 |  |  |
| } |  |  |  |
| csi-SSB-ResourceSetList SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF { | 2 entries |  | SSB |
| CSI-SSB-ResourceSetId[0] | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB | Configuration for SSB based CSI |
| CSI-RS | Configuration for CSI-RS based CSI |

<< End of Changes >>