3GPP TSG-RAN5 Meeting #109	R5-256526

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5227 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to test case 9.2.7.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_Ph1-CT_n3GPPA-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The wrong generic procedure is referenced. - The UL grant procedure is not relevant to WLAN. - The NR RRC_CONNECTED procedure is not relevant for WLAN. - Editorial issues exist. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrected references to generic procedure. - Removed references to UL grants and introduced a timer to handle the optional Service Request. - Removed reference to RRC procedure. - Added the prefix ‘5GMM’ to NAS messages and other editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case cannot be implemented |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 9.2.7	Service request

#### 9.2.7.1	Service request / IDLE mode uplink user data transport / Rejected / Restricted service area, Abnormal / T3517

9.2.7.1.1	Test Purpose (TP)

Same test purposes as clause 9.1.7.1.1

9.2.7.1.2	Conformance requirements

Same conformance requirements as in clause 9.1.7.1.2

9.2.7.1.3	Test description

9.2.7.1.3.1	Pre-test conditions

System Simulator:

-	WLAN Cell 27

UE:

-	None.

Preamble:

-	The UE is in state 3W-A with PDU session Active state according to TS 38.508-1 [4]

9.2.7.1.3.2	Test procedure sequence

Table 9.2.7.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The generic procedure for SS-requested IPsec Secure tunnel disconnection, specified in subclause 4.5A.5 of TS 38.508-1 [4], takes place performing disconnection of security association. | - | - | - | - |
| 2 | Trigger UE to initiate IPSec SA (Note 1) | - | - | - | - |
| - | Exception: In parallel to steps 3, the UE initiates an IPsec security association and one child security association as defined in TS 24.502 [35] clause 7.3.2 | - | - | - | - |
| 3 | Check: Does UE transmit a SERVICE REQUEST message with Service type IE set to ‘data’? | --> | 5GMM: SERVICE REQUEST | 1 | P |
| 4 | The SS transmits a SERVICE REJECT message with 5GMM cause = "Restricted service area". | <-- | 5GMM: SERVICE REJECT | - | - |
| 5-6 | Void. | - | - | - | - |
| 6A | The SS starts timer 5 sec. (Note 2) | - | - | - | - |
| - | EXCEPTION: Steps 6Ba1 to 6Bb3 describe behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE is implemented in a certain way. | - | - | - | - |
| - | EXCEPTION: In parallel to step 6Ba1, the UE initiates an IPsec security association as defined in TS 24.502 [35] clause 7.3.2 | - | - | - | - |
| 6Ba1 | Check: Does the UE transmit REGISTRATION REQUEST for mobility registration? NOTE: Allowed for Rel-15 UEs. | --> | 5GMM: REGISTRATION REQUEST | 2 | P |
| 6Ba2 | Stop timer 5 sec. | - | - | - | - |
| 6Bb1 | Timer 5 sec expires. | - | - | - | - |
| 6Bb2 | The generic procedure for SS-requested IPsec Secure tunnel disconnection, specified in subclause 4.5A.5 of TS 38.508-1 [4], takes place performing disconnection of security association. | - | - | - | - |
| - | EXCEPTION: In parallel to step 6Bb3, the UE initiates an IPsec security association as defined in TS 24.502 [35] clause 7.3.2 | - | - | - | - |
| 6Bb3 | Check: Does the UE transmit REGISTRATION REQUEST for mobility registration over a new signalling connection? | --> | 5GMM: REGISTRATION REQUEST | 2 | P |
| 7-8A | Void. | - | - | - | - |
| 8B | SS sends a REGISTRATION ACCEPT message containing a 5G-GUTI. | <-- | 5GMM: REGISTRATION ACCEPT | - | - |
| 8C | The SS starts timer 5 sec. (Note 2) | - | - | - | - |
| 8D | The UE send a REGISTRATION COMPLETE. | --> | 5GMM: REGISTRATION COMPLETE | - | - |
| - | EXCEPTION: Steps 8Ea1 to 8Eb3 describe behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE is implemented in a certain way. | - | - | - | - |
| 8Ea1 | The UE transmits a SERVICE REQUEST message. | --> | 5GMM: SERVICE REQUEST | - | - |
| 8Ea2 | Stop timer 5 sec. | - | - | - | - |
| 8Eb1 | Timer 5 sec expires. | - | - | - | - |
| 8Eb2 | The generic procedure for SS-requested IPsec Secure tunnel disconnection, specified in subclause 4.5A.5 of TS 38.508-1 [4], takes place performing disconnection of security association. | - | - | - | - |
| - | EXCEPTION: In parallel to step 8Eb3, the UE initiates an IPsec security association as defined in TS 24.502 [35] clause 7.3.2 | - | - | - | - |
| 8Eb3 | The UE transmits a SERVICE REQUEST message. | --> | 5GMM: SERVICE REQUEST | - | - |
| 8F-8G | Void. | - | - | - | - |
| 8H | The SS does not respond to SERVICE REQUEST message. (Note 3) | - | - | - | - |
| - | EXCEPTION: Steps 9 to 10 are repeated for 4 times. | - | - | - | - |
| - | EXCEPTION: In parallel to step 9, the UE initiates an IPsec security association as defined in TS 24.502 [35] clause 7.3.2 | - | - | - | - |
| 9 | Check: does the UE transmit a SERVICE REQUEST message after T3517 expiry but before 60s over a new signalling connection? | --> | 5GMM: SERVICE REQUEST | 3 | P |
| 10 | The SS does not respond to SERVICE REQUEST message. (Note 3) | - | - | - | - |
| - | EXCEPTION: In parallel to step 11, the UE initiates an IPsec security association as defined in TS 24.502 [35] clause 7.3.2 | - | - | - | - |
| 11 | Check: Does the UE initiate IPSec secure tunnel as per 3GPP TS 24.502 [33] clause 7.3.2within 60 seconds (minimum value of T3525)? | - | - | 3,4 | F |
| 12 | The SS starts timer 10 sec after Step 11 to see if UE performs the optional Steps listed below. | - | - | - | - |
| - | EXCEPTION: Steps 12a1 to 12a7a2 describe behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE is implemented in a certain way. | - | - | - | - |
| 12a1 | Trigger UE to initiate IPSec SA (Note 1) | - | - | - | - |
|  | Exception: In parallel to step 12a2 the UE initiates an IPsec security association and one child security association as defined in TS 24.502 [35] clause 7.3.2 | - | - | - | - |
| 12a2 | The UE transmits a SERVICE REQUEST message after T3525 expiry over a new signalling connection. | --> | 5GMM: SERVICE REQUEST | - | - |
| 12a3 | The SS transmits a SERVICE ACCEPT message. | <-- | 5GMM: SERVICE ACCEPT | - | - |
| 12a4 – 12a7a2 | Void |  |  |  |  |
| Note 1: This could be done by e.g. MMI or by AT command +CGACT. Note 2: An arbitrary chosen timer to avoid message crossing. Note 3: The UE locally releases the signalling connection after T3517 expiry. |  |  |  |  |  |

9.2.7.1.3.3	Specific message contents

Table 9.2.7.1.3.3-3: SERVICE REQUEST (steps 3, 8Ea1, 8Eb3, 9 and 12a2, Table 9.2.7.1.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.7.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Service type |  |  |  |
| Service type value | ‘0001’B | data |  |

Table 9.2.7.1.3.3-4: SERVICE REJECT (step 4, Table 9.2.7.1.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.7.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GMM cause | '0001 1100'B | Restricted service area |  |