3GPP TSG-RAN5 Meeting #108	R5-255054

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5475 | rev | 1 | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE DRX testcase 7.1.6.2 for CAT-M1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu EMEA Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test, LTE_MTCe2_L1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test fails as RLC ACK for RRCReconfigComplete falls under CDRX off duration for CAT-M1 device. Hence, longer duration for DRX should be configured for CAT-M1 device. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to configure drx-Config-r13 IE if the UE supports CE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated with TTCN CR R5s250290 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Updated the reason for change in coversheet |  |  |  |  |  |  |  |  |

<Start Of change>

#### 7.1.6.2	DRX operation / Short cycle not configured / DRX command MAC control element reception

7.1.6.2.1	Test Purpose (TP)

(1)

with { UE in CONNECTED mode }

ensure that {

when { long DRX cycle is configured and a DRX Command MAC control element is received }

then { UE successfully decodes the MAC control PDU }

}

(2)

with { UE in CONNECTED mode }

ensure that {

when { long DRX cycle is configured and the HARQ RTT Timer is running and a DRX Command MAC control element is received }

then { UE continues running the HARQ RTT timer }

}

(3)

with { UE in CONNECTED mode }

ensure that {

when { long DRX cycle is configured and the drx-RetransmissionTimer is running and a DRX Command MAC control element is received }

then { UE continues running the drx-RetransmissionTimer and monitors the PDCCH }

}

7.1.6.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.321, clauses 3.1 and 5.7.

[TS 36.321, clause 3.1]

Active Time: Time related to DRX operation, as defined in subclause 5.7, during which the UE monitors the PDCCH in PDCCH-subframes.

...

DRX Cycle: Specifies the periodic repetition of the On Duration followed by a possible period of inactivity (see figure 3.1-1 below).

Figure 3.1-1: DRX Cycle

drx-InactivityTimer: Specifies the number of consecutive PDCCH-subframe(s) after successfully decoding a PDCCH indicating an initial UL or DL user data transmission for this UE.

drx-RetransmissionTimer: Specifies the maximum number of consecutive PDCCH-subframe(s) for as soon as a DL retransmission is expected by the UE.

drxShortCycleTimer: Specifies the number of consecutive subframe(s)the UE shall follow the short DRX cycle.

drxStartOffset: Specifies the subframe where the DRX Cycle starts.

...

HARQ RTT Timer: This parameter specifies the minimum amount of subframe(s) before a DL HARQ retransmission is expected by the UE.

...

onDurationTimer: Specifies the number of consecutive PDCCH-subframe(s) at the beginning of a DRX Cycle.

PDCCH-subframe: For FDD UE operation, this represents any subframe; for TDD, only downlink subframes and subframes including DwPTS.

[TS 36.321, clause 5.7]

-	if a HARQ RTT Timer expires in this subframe and the data in the soft buffer of the corresponding HARQ process was not successfully decoded:

-	start the drx-RetransmissionTimer for the corresponding HARQ process.

-	if a DRX Command MAC control element is received:

-	stop onDurationTimer;

-	stop drx-InactivityTimer.

-	if drx-InactivityTimer expires or a DRX Command MAC control element is received in this subframe:

-	if the short DRX cycle is configured:

-	start or restart drxShortCycleTimer;

-	use the Short DRX Cycle.

-	else:

-	use the Long DRX cycle.

-	if drxShortCycleTimer  expires in this subframe:

-	use the long DRX cycle.

-	during the Active Time, for a PDCCH-subframe except if the subframe is required for uplink transmission for half-duplex FDD UE operation and except if the subframe is part of a configured measurement gap:

-	monitor the PDCCH;

-	if the PDCCH indicates a DL transmission or if a DL assignment has been configured for this subframe:

-	start the HARQ RTT Timer for the corresponding HARQ process;

-	stop the drx-RetransmissionTimer for the corresponding HARQ process.

-	if the PDCCH indicates a new transmission (DL or UL):

-	start or restart drx-InactivityTimer.

-	when not in Active Time, CQI/PMI/RI on PUCCH and SRS shall not be reported.

Regardless of whether the UE is monitoring PDCCH or not the UE receives and transmits HARQ feedback when such is expected.

7.1.6.2.3	Test description

7.1.6.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Loopback Activated (state 4) according to [18] configured to return no data in UL.

7.1.6.2.3.2	Test procedure sequence

The definition of NormalSF(current SFN,current subframe number,y) can be found in clause 7.1.6.1.3.2.

Table 7.1.6.2.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0A | SS Transmits RRCConnectionReconfiguration to configure specific DRX parameters | <-- | - | - | - |
| 0B | The UE transmits RRCConnectionReconfigurationComplete | --> | - | - | - |
| 1 | In a PDCCH sub frame which is X PDCCH sub frames before the PDCCH sub-frame in which the onDurationTimer expires, with drx-InactivityTimer< X < the number of PDCCH subframes encapsulated by HARQ RTT timer, the SS indicates the transmission of a DL MAC PDU on the PDCCH. The SS transmits an invalid MAC PDU.(Note 1) i.e., on the subframe with the subframe number csfn2 = [csfn1+NormalSF(SFN1, csfn1, onDurationTimer-1-X)] modulo 10, and the system frame number SFN2 = SFN1+floor([csfn1+NormalSF(SFN1, csfn1, onDurationTimer-1-X)]/10); and [(SFN1 * 10) + csfn1] modulo (Long DRX Cycle) = drxStartOffset. | <-- | MAC PDU | - | - |
| 2 | Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 1? | --> | HARQ NACK | 1 | P |
| 3 | In a PDCCH sub frames before the onDurationTimer expires, the SS indicates the transmission of a DL MAC PDU on the PDCCH. The SS transmits a DL MAC PDU with DRX MAC Control element. UE successfully decodes the MAC PDU. i.e., on the subframe with the subframe number csfn3 = [csfn1+NormalSF(SFN1, csfn1, onDurationTimer-1-X+Y)] modulo 10, and the system frame number SFN3 = SFN1+floor([csfn1+NormalSF(SFN1, csfn1, onDurationTimer-1-X+Y)]/10); and 0<Y<X) | <-- | MAC PDU(DRX MAC Control element) | - | - |
| 4 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 3? | --> | HARQ ACK | 1 | P |
| 5 | In the PDCCH sub frame when the drx-RetransmissionTimer for the MAC PDU indicated in Step 1 on the PDCCH is started the SS indicates the transmission of a DL MAC PDU. The SS transmits an invalid MAC PDU.(Note 1) i.e., on the subframe with the subframe number csfn5 = [csfn4+NormalSF(SFN4, csfn4, 0)] modulo 10, and the system frame number SFN5 = SFN4+floor([csfn4+NormalSF(SFN4, csfn4, 0)]/10); where csfn4 = [csfn2+HARQ RTT Timer] modulo 10, and the SFN4 = SFN2+floor([csfn2+HARQ RTT Timer]/10); For TDD the HARQ RTT timer is the HARQ RTT timer for the DL transmission in Step 1 | <-- | MAC PDU | - | - |
| 6 | Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 5? | --> | HARQ NACK | 2,3 | P |
| 7 | Z PDCCH sub frames, where Z>drx-InactivityTimer, before the PDCCH sub-frame in which the drx-RetransmissionTimer for the DL MAC PDU in Step 5 expires, the SS indicates the transmission of a DL MAC PDU. The SS transmits a DL MAC PDU with DRX MAC Control element. i.e., on the subframe with the subframe number = [csfn6+NormalSF(SFN6, csfn6, drx-RetransmissionTimer –Z)] modulo 10, and the system frame number = SFN6+floor([csfn6+NormalSF(SFN6, csfn6, drx-RetransmissionTimer –Z)]/10); where csfn6 = [csfn5+HARQ RTT Timer] modulo 10, and the SFN6 = SFN5+floor([csfn5+HARQ RTT Timer]/10); For TDD the HARQ RTT timer is the HARQ RTT timer for the DL transmission in Step 5. | <-- | MAC PDU(DRX MAC Control element) | - | - |
| 8 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 7? | --> | HARQ ACK | 2,3,1 | P |
| 9 | In the last sub frame when the Drx-RetransmissionTimer for the DL MAC PDU indicated on the PDCCH in Step 5 is still running, the SS indicates the transmission of a DL MAC PDU. i.e., on the subframe with the subframe number = [csfn6+NormalSF(SFN6, csfn6, drx-RetransmissionTimer –1)] modulo 10, and the system frame number = SFN6+floor([csfn6+NormalSF(SFN6, csfn6, drx-RetransmissionTimer –1)]/10); | <-- | MAC PDU | - | - |
| 10 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 9? | --> | HARQ ACK | 2,3 | P |
| Note 1: Invalid MAC PDU is a MAC PDU that fails the CRC check. Note 2: All DL MAC PDUs are transmitted with the NDI set on the PDCCH. Note 3: Timer tolerances for the MAC DRX related timers measured in subframes or PDCCH subframes is 0. These timers are: drx-InactivityTimer, drx-RetransmissionTimer, HARQ RTT Timer. |  |  |  |  |  |

7.1.6.2.3.3	Specific message contents

Table 7.1.6.2.3.3-1: specific Parameters in RRCConnectionReconfiguration (step 0A of table 7.1.6.2.3.2-1)

| Derivation Path: 36.508 Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| measConfig |  | Not present |  |  |
| mobilityControlInfo |  | Not present |  |  |
| dedicatedInfoNASList |  | Not present |  |  |
| radioResourceConfigDedicated SEQUENCE { |  |  |  |  |
| srb-ToAddModList |  | Not present |  |  |
| drb-ToAddModList |  | Not present |  |  |
| drb-ToReleaseList |  | Not present |  |  |
| mac-MainConfig CHOICE { |  |  |  |  |
| explicitValue SEQUENCE { |  |  |  |  |
| ul-SCH-Config SEQUENCE { |  |  |  |  |
| maxHARQ-Tx |  | n5 |  |  |
| retxBSR-Timer |  | sf10240 |  |  |
| ttiBundling |  | FALSE |  |  |
| } |  |  |  |  |
| drx-Config CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| onDurationTimer |  | psf20 |  |  |
| drx-InactivityTimer |  | psf6 |  |  |
| drx-RetransmissionTimer |  | psf6 |  |  |
| longDRX-CycleStartOffset CHOICE { |  |  |  |  |
| sf640 |  | 4 |  |  |
| } |  |  |  |  |
| shortDRX |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| timeAlignmentTimerDedicated |  | infinity |  |  |
| phr-Config CHOICE { |  |  |  |  |
| release |  | NULL |  |  |
| } |  |  |  |  |
| drx-Config-r13 CHOICE{ |  |  |  | CEmodeA CEmodeB |
| setup SEQUENCE { |  |  |  |  |
| onDurationTimer-v1310 |  | psf300 |  |  |
| drx-RetransmissionTimer-v1310 |  | psf40 |  |  |
| drx-ULRetransmissionTimer-r13 |  | Psf320 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

<End of change>