3GPP TSG-RAN WG5 Meeting #109	R5-256685

# Dallas, USA, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5201 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of XR test case 7.1.3.5.10 DSR triggering |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - PDCP Discard / PSI based discard / DSR triggering TC is missing and needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - PDCP Discard / PSI based discard / DSR triggering TC has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WP can not be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.5.10 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.509 CR 0090 TS 38.523-2 CR 0641 TS 38.508-1 CR 3651 TS 38.523-1 CR 5198 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is related to Discussion paper R5-256071. The related applicability CR is R5-256078 (CR number 0641). The related test environment CR is R5-256076 (CR number 3651). The related new test loop mode CR is R5-256079 (CR number 0090). The related reference spec number CR is R5-256072 (CR number 5198). The interim number xx for spec 26.522 needs to be assigned with hard number by MCC. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

7.1.3.5.10	PDCP Discard / PSI based discard / DSR triggering

7.1.3.5.10.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { discardTimerForLowImportance is configured and PSI based SDU discard is activated and the smallest remaining value of discardTimerForLowImportance among all the data buffered for a LCG becomes below remainingTimeThreshold }

then { UE does not trigger a DSR }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { discardTimerForLowImportance is configured and PSI based SDU discard is activated and the smallest remaining value of discardTimer among all the data buffered for a LCG becomes below remainingTimeThreshold }

then { UE triggers a DSR and Remaining Time and Buffer Size fields are only determined based on high importance data }

}

7.1.3.5.10.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.323, clause 5.2.1 and 5.3, TS 26.522, clause 4.2.2 and clause 4.2.4. Unless otherwise stated these are Rel-18 requirements.

[TS 26.522, clause 4.2.2]

The one-byte RTP HE for the marking of PDU Sets and End of Bursts is defined as follows:

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | len   |E| R |D|  PSI  |      PSSN         |     PSN   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                     PSSize                    |     NPDS

+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+

|

+.+.+.+.+.+.+.+.+

[TS 26.522, clause 4.2.4]

The semantics of the fields of the RTP HE for PDU Set marking are defined as follows:

-	End PDU of the PDU Set [E] (1 bit): This field is a flag that shall be set to 1 for the last PDU of the PDU Set and set to 0 for all other PDUs of the PDU Set.

-	End of Data Burst [D] (1 bit): This field is a flag that shall be set to 1 for the last PDU of a Data Burst. It shall be set to 0 for all other PDUs. A Data Burst may consist of one or more PDU Sets.

NOTE 1:	The bit encodes the End of Data Burst indication as per the guidelines provided in clause 4.2.6.1.

-	Reserved [R] (2 bits): This field is reserved for future usage (e.g., dynamic burst indication). It shall be set to 0 by the RTP sender and shall be ignored by the RTP receiver.

-	PDU Set Importance [PSI] (4 bits): The PDU Set Importance field indicates the importance of this PDU Set compared to other PDU Sets within the same Multimedia Session. This information may help RAN to discard PDUs, when needed. Lower values shall indicate a higher importance PDU Set, with the highest importance PDU Set indicated by 1 and the lowest importance PDU Set indicated by 15. When the RTP sender cannot define an importance, it shall set the value to 0.

NOTE 2:	A complete set of guidelines for setting the PSI field for the 3GPP audio/video codecs are provided in clause 4.2.6.2

-	PDU Set Sequence Number [PSSN] (10 bits): The sequence number of the PDU Set to which the current PDU belongs, acting as a 10-bit numerical identifier for the PDU Set. The PSSN shall be incremented monotonically by 1 for each subsequent PDU Set.

NOTE 3:	This value wraps around at 1023, however, using the 16-bit RTP packet sequence number and PSSN pair, a receiver may uniquely distinguish between any PDU Sets.

-	PDU Sequence Number within a PDU Set [PSN] (6 bits): The sequence number of the current PDU within the PDU Set. The PSN shall be set to 0 for the first PDU in the PDU Set and incremented monotonically for every PDU in the PDU Set in the order of transmission from the sender.

NOTE 4:	A receiver may use the RTP packet sequence number together with the PSN to distinguish between PDUs within a PDU Set that contains more than 64 PDUs.

-	PDU Set Size [PSSize] (24 bits): The PDU Set Size indicates the total size of all PDUs of the PDU Set to which this PDU belongs. This field is optional and subject to an SDP signaling offer/answer negotiation, where the RTP sender shall indicate whether it will provide the size of the PDU Set for that RTP stream. If not enabled, the field shall not be present within the RTP HE. If enabled, but the RTP sender is not able to determine the PDU Set Size for a particular PDU Set, it shall set the value to 0 in all PDUs of that PDU Set. The PSSize shall indicate the size of a PDU Set including RTP/UDP/IP header encapsulation overhead of its corresponding PDUs. The PSSize shall be expressed in bytes. It is recommended to add the PDU Set Size field when the Number of PDUs in the PDU Set field is present.

NOTE 5:	This field may be optionally present given the signaling of the "pdu-set-size" extension attribute in the SDP offer/answer negotiation as per clause 4.2.5.

-	Number of PDUs in the PDU Set [NPDS] (16 bits): The number of PDUs within the PDU Set indicates the total number of PDUs belonging to the same PDU Set. This field is optional and subject to an SDP signaling offer/answer negotiation, where the RTP sender may indicate whether it will provide the number of PDUs within the PDU Set for that RTP stream. If enabled, but the RTP sender is not able to determine the Number of PDUs in the PDU Set, it shall set the value to 0 in all PDUs of that PDU Set.  It is recommended to add the Number of PDUs in the PDU Set field when the PDU Set Size field is present.

NOTE 6:	This field may be optionally present given the signaling of the "num-pdus-in-pdu-set" extension attribute in the SDP offer/answer negotiation as per clause 4.2.5.

NOTE 7:	Guidelines to set the PDU Set Size in bytes by an RTP sender are provided in clause 4.2.6.3.

NOTE 8:	The use of NPDS for the NAT46/NAT64 correction is FFS.

[TS 38.321, clause 5.18.34]

The network activates and deactivates PSI-based SDU discard by sending the PSI-Based SDU Discard Activation/Deactivation MAC CE described in clause 6.1.3.73. The PSI-based SDU discard is initially deactivated upon (re-)configuration by upper layers and after reconfiguration with sync.

The MAC entity shall for each DRB configured with discardTimerForLowImportance:

1>	if a PSI-Based SDU Discard Activation/Deactivation MAC CE is received activating the PSI-based SDU discard for the DRB:

2>	indicate the activation of the PSI-based SDU discard for the DRB to upper-layers.

1>	if a PSI-Based SDU Discard Activation/Deactivation MAC CE is received deactivating the PSI-based SDU discard for the DRB:

2>	indicate the deactivation of the PSI-based SDU discard for the DRB to upper-layers.

[TS 38.321, clause 6.1.3.73]

The PSI-Based SDU Discard Activation/Deactivation MAC CE is identified by MAC subheader with an one-octet eLCID as specified in Table 6.2.1-1b.

It has a fixed size and consists of one octet defined as follows (Figure 6.1.3.73-1):

-	Di: This field indicates the activation/deactivation status of the PSI-based SDU discard of DRB i, where i is the ascending order of the DRB ID among the DRBs configured with discardTimerForLowImportance and with RLC entity(ies) associated with this MAC entity. The Di field set to 1 indicates that the PSI-based SDU discard shall be activated for DRB i. The Di field set to 0 indicates that the PSI-based SDU discard shall be deactivated for DRB i.

Figure 6.1.3.73-1: PSI-Based SDU Discard Activation/Deactivation MAC CE

[TS 38.323, clause 5.2.1]

At reception of a PDCP SDU from upper layers, the transmitting PDCP entity shall:

-	if discardTimerForLowImportance is configured and PSI based SDU discard is activated, and the PDCP SDU belongs to a low importance PDU Set:

-	start the discardTimerForLowImportance associated with this PDCP SDU;

-	else:

-	start the discardTimer associated with this PDCP SDU (if configured).

NOTE 0:	Identification of PSI of a PDU Set and determination of low importance PDU Set are left up to UE implementation.

For a PDCP SDU received from upper layers, the transmitting PDCP entity shall:

-	associate the COUNT value corresponding to TX_NEXT to this PDCP SDU;

NOTE 1:	Associating more than half of the PDCP SN space of contiguous PDCP SDUs with PDCP SNs, when e.g., the PDCP SDUs are discarded or transmitted without acknowledgement, may cause HFN desynchronization problem. How to prevent HFN desynchronization problem is left up to UE implementation.

-	perform header compression of the PDCP SDU using ROHC as specified in the clause 5.7.4 and/or using EHC as specified in the clause 5.12.4;

-	perform uplink data compression of the PDCP SDU as specified in clause 5.14.4;

-	perform integrity protection, and ciphering using the TX_NEXT as specified in the clause 5.9 and 5.8, respectively;

-	set the PDCP SN of the PDCP Data PDU to TX_NEXT modulo 2[pdcp-SN-SizeUL];

-	increment TX_NEXT by one;

-	submit the resulting PDCP Data PDU to lower layer as specified below.

[TS 38.323, clause 5.3]

When the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU.

When the discardTimer or discardTimerForLowImportance expires for a PDCP SDU, the transmitting PDCP entity shall:

-	if pdu-SetDiscard is configured:

-	discard all PDCP SDUs belonging to the PDU Set to which the PDCP SDU belongs along with the corresponding PDCP Data PDUs;

NOTE 1:	PDCP SDUs subsequently received from upper layers are also discarded if they belong to the PDU Set.

-	else:

-	discard the PDCP SDU along with the corresponding PDCP Data PDU.

If the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers.

For SRBs, when upper layers request a PDCP SDU discard, the PDCP entity shall discard all stored PDCP SDUs and PDCP PDUs.

NOTE 2:	Discarding a PDCP SDU already associated with a PDCP SN causes a SN gap in the transmitted PDCP Data PDUs, which increases PDCP reordering delay in the receiving PDCP entity. It is up to UE implementation how to minimize SN gap after SDU discard.

[TS 38.300, clause 16.15.2]

During the data forwarding for handover, the source gNB provides the Indication of End of Data Burst, Indication of Data Burst Size, and Time to Next Burst received from the UPF to target gNB in case one or more PDUs of the data burst is forwarded to target gNB.

Finally, 5GC may provide the Multi-modal Service ID (MMSID) to NG-RAN, as part of the QoS parameters of the QoS flow, when establishing and/or updating the corresponding QoS Flows. It is up to the gNB’s implementation to use this information for multi-modality service.

In the uplink, the UE needs to be able to identify PDU Sets and Data Bursts dynamically, including PSI. How this is done is left up to UE implementation but when possible for a QoS flow, this is indicated to the gNB via UE Assistance Information.

7.1.3.5.10.3	Test description

7.1.3.5.10.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.0 with exceptions as below:

The PDCP setting for discard timer is listed in Table 7.1.3.5.10.3.1-1 applicable for the configured UM DRB and Table 7.1.3.5.10.3.1-2 for 1 AM DRB on NR cell configured and Table 7.1.3.5.10.3.3-1 for SR configuration except that PDCP is configured for 18 bit SN.

Table 7.1.3.5.10.3.1-1: PDCP Settings

| Parameter | Value |
| --- | --- |
| Discard_Timer | 500 ms |

Table 7.1.3.5.10.3.1-2: Logical Channel Configuration Settings

| Parameter | DRB1 | DRB2 |
| --- | --- | --- |
| LogicalChannelIdentity | LCH4(DRB-Identity +3) | LCH5(DRB-Identity +3) |
| Priority | 7 | 6 |
| prioritizedBitRate | 0kbs | 0kbs |
| logicalChannelGroup | 2 (LCG ID#2) | 1 (LCG ID#1) |
| remainingTimeThreshold | 50ms | 50ms |

The test loop mode J for application layer packet transmission in TS 38.509 [6] is used in this test case.

The mapping relationship from an RTP packet (with header extension) to a PDCP PDU can be specified as below:

- One RTP header extension is corresponding to one RTP packet;

- One RTP packet is encapsulated into one UDP packet with UDP header;

- One UDP packet is encapsulated into one IP packet with UDP header;

- One IP packet is mapped to one QoS flow and sent to gNB from 5GC through GTP-U header;

- One QoS flow is mapped to one SDAP SDUs in one DRB.

- One SDAP PDU is encapsulated into one PDCP PDU with PDCP header.

For this test case, UDP/IP/SDAP headers can be configured by SS default configuration. For RTP header, the field X which indicates if the header extension exists needs to be set to 1. The other fields in RTP header can be configured by SS default configuration.

For this test case, only RTP HE for PDU Set Marking is used, thus the RTP header extension should be one-byte format defined in TS 26.522 [xx].

The RTP header is defined as follows:

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|V=2|P|X|   C   |M|     PT      |       sequence number         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                       timestamp                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|         synchronization source (SSRC) identifier              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|              contributing source (CSRC) identifiers           |

The one-byte RTP HE for the marking of PDU Sets and End of Bursts is defined as follows:

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | len   |E| R |D|  PSI  |      PSSN         |     PSN   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                     PSSize                    |     NPDS

+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+

|

+.+.+.+.+.+.+.+.+

The specific RTP Header Extension used for this test case is defined in Table 7.1.3.5.10.3.1-1. The specific RTP/UDP/IP headers are specified in Table 7.1.3.5.10.3.1-2, Table 7.1.3.5.10.3.1-3 and Table 7.1.3.5.10.3.1-4 accordingly.

- Two PDU Sets are used for this TC, the PDU Set Sequence Number [PSSN] are defined as 0 and 1.

- Three PDUs are included in the PDU Set with PSSN 0, PDU Sequence Numbers within a PDU Set [PSN] are defined as  0,1 and 2; three PDUs are included in the PDU Set with PSSN 1, PDU Sequence Numbers within a PDU Set [PSN] are defined as  0,1 and 2.

- The value of PDU Set Importance (PSI) of the PDU Set with PSSN 0 is marked as 15, which means the PDU Set has lowest importance and should be configured with discard timer for low importance; the value of PDU Set Importance (PSI) of the PDU Set with PSSN 1 is marked as 1, which means the PDU Set has highest importance and should be configured with normal discard timer.

- The the PDU Set with PSSN 1 is transmitted after PDU Set with PSSN 0.

Table 7.1.3.5.10.3.1-1: RTP Header Extension for PDU Set Marking

| Parameter | Size | Comment | For PDU Set 0 | For PDU Set 1 |
| --- | --- | --- | --- | --- |
| Magic word | 2 Bytes | Used to identify the 1-byte format RTP header extension and serves as the fixed start identifier for this format | 0xBEDE | 0xBEDE |
| length | 2 Bytes | The length of total extended data | 0x0002 | 0x0002 |
| ID | 4 bits | The extended ID to mark “PDU Set Marking” | 0001 | 0001 |
| len | 4 bits | The length of single extended data. Fixed 2 bit in PDU Set marking | 0010 | 0010 |
| E | 1 bit | End of PDU Set | 0 (For PDU 1) 0 (For PDU 2) 1 (For PDU 3) | 0 (For PDU 4) 0 (For PDU 5) 1 (For PDU 6) |
| R | 1 bit | Reserved. Set to 0 for transmitter | 00 | 00 |
| D | 1 bit | End of Data Burst | 0 (For PDU 1) 0 (For PDU 2) 0 (For PDU 3) | 0 (For PDU 4) 0 (For PDU 5) 1 (For PDU 6) |
| PSI | 4 bits | PDU Set Importance | 1111 | 0001 |
| PSSN | 10 bits | PDU Set Sequence Number | 0000000000 | 0000000001 |
| PSN | 6 bits | PDU Sequence Number within a PDU Set | 000000 (For PDU 1) 000001 (For PDU 2) 000010 (For PDU 3) | 000000 (For PDU 4) 000001 (For PDU 5) 000010 (For PDU 6) |
| PSSize (Optional) | 24 bits | Total size of PDU Set (including RTP/UDP/IP header). Set to 0 if uncertain | N/A | N/A |
| NPDS (Optional) | 16 bits | Total number of PDUs in the PDU Set. Set to 0 if uncertain | N/A | N/A |

Table 7.1.3.5.10.3.1-2: RTP Header definition

| Parameter | Size | Comment | For PDU Set 0 | For PDU Set 1 |
| --- | --- | --- | --- | --- |
| V | 2 bit | Version number, typically 2 | 10 | 10 |
| P | 1 bit | Padding indicator; if set to 1, the packet contains padding bytes at the end. | 0 | 0 |
| X | 1 bit | Extension flag; if set to 1, a header extension is present. | 1 | 1 |
| CC(CSRC count) | 4 bit | Indicates the number of CSRC identifiers that follow the fixed header. | 0000 | 0000 |
| M | 1 bit | Marker bit, usually markign the end of video frame/audio | 0 (For PDU 1) 0 (For PDU 2) 1 (For PDU 3) | 0 (For PDU 4) 0 (For PDU 5) 1 (For PDU 6) |
| PT | 7 bit | Payload type, identifying the format of the payload (e.g., H.264 = 96) | 1100000 | 1100000 |
| sequence number | 2 Byte | RTP packet sequence number | 0000000000000000 (For PDU 1) 0000000000000001 (For PDU 2) 0000000000000010 (For PDU 3) | 0000000000000000 (For PDU 4) 0000000000000001 (For PDU 5) 0000000000000010 (For PDU 6) |
| timestamp | 4 Byte | The timestamp, reflecting the sampling instant of the first byte in the payload | 00000000000000000000001011101100 | 00000000000000000001011101110000 |
| SSRC | 4 Byte | Synchronization source identifier, uniquely identifying the source of the stream | 00010010001101000101011001111000 | 00010010001101000101011001111000 |
| CSRC | 4 Byte | Contributing Source lists the original sources of a media stream, present only if the CC (CSRC count) field is greater than zero | N/A | N/A |

Table 7.1.3.5.10.3.1-3: UDP Header definition

| Parameter | Size | Comment | For PDU Set 0 | For PDU Set 1 |
| --- | --- | --- | --- | --- |
| Source Port | 16 bits | Identifies the port number of the sending application, ranging from 0 to 65535, with dynamic ports (49152-65535) typically used by clients (e.g., 54321 used in this case) | 1101010000110001 | 1101010000110001 |
| Destination Port | 16 bits | Specifies the port number of the target application on the receiving end. (e.g., RTP = 5004) | 0001001110001100 | 0001001110001100 |
| Length | 16 bits | Indicates the total size of the UDP packet in bytes.(e.g., RTP header 12 Bytes+RTP header extension 8 Bytes +RTP payload 10Bytes+UDP header 8Bytes= 38 Bytes) | 0000000000100110 | 0000000000100110 |
| Checksum | 16 bits | Indicates the number of CSRC identifiers that follow the fixed header. | 0000000000000000 | 0000000000000000 |

Table 7.1.3.5.10.3.1-4: IP Header definition

| Parameter | Size | Comment | For PDU Set 0 | For PDU Set 1 |
| --- | --- | --- | --- | --- |
| Version | 4 bits | Specifies the IP protocol version, which is 4 for IPv4 | 0100 | 0100 |
| IHL | 4 bits | Internet Header Length, Indicates the length of the IP header in 32-bit words (e.g. IPv4 header is 20 Bytes) | 0101 | 0101 |
| DSCP | 6 bits | Differentiated Services Code Point, Used for classifying and managing network traffic quality of service | 000000 | 000000 |
| ECN | 2 bits | Explicit Congestion Notification, Allows end-to-end notification of network congestion without dropping packets | 00 | 00 |
| Total Length | 16 bits | Defines the entire packet size including header and data in bytes (e.g. UDP packet 29 Bytes +IPv4 header 20 Bytes=49 Bytes) | 0000000000110001 | 0000000000110001 |
| Identification | 16 bits | Uniquely identifies a group of fragments of a single IP datagram (e.g. from 0x1234 to 0x1239) | 0001001000110100 (For PDU 1) 0001001000110101 (For PDU 2) 0001001000110110 (For PDU 3) | 0001001000110111 (For PDU 4) 0001001000111000 (For PDU 5) 0001001000111001 (For PDU 6) |
| Flags | 3 bits | Controls fragmentation with bits for reserved, don't fragment, and more fragments (e.g. 2 means no fragmentation) | 010 | 010 |
| Fragment Offset | 13 bits | Specifies the position of a fragment relative to the original datagram | 0000000000000 | 0000000000000 |
| Time to Live | 8 bits | Prevents infinite loops by limiting packet lifetime in the network (e.g. 64 seconds) | 0000000001000000 | 0000000001000000 |
| Protocol | 8 bits | Identifies the transport layer protocol carried in the data portion (e.g. 17 for UDP) | 0000000000010001 | 0000000000010001 |
| Header Checksum | 16 bits | Provides error detection for the IP header contents | 1010010100110010 (For PDU 1) 1010010100110001 (For PDU 2) 1010010100110000 (For PDU 3) | 1010010100101111 (For PDU 4) 1010010100101110 (For PDU 5) 1010010100101101 (For PDU 6) |
| Source Address | 32 bits | Contains the IPv4 address of the packet sender (e.g. 192.168.1.2) | 1100000010101000 0000000100000010 | 1100000010101000 0000000100000010 |
| Destination Address | 32 bits | Contains the IPv4 address of the packet recipient (e.g. 192.168.1.3) | 1100000010101000 0000000100000011 | 1100000010101000 0000000100000011 |

7.1.3.5.10.3.2	Test procedure sequence

Table 7.1.3.5.10.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The SS does not allocate UL grants unless when explicitly stated so in the procedure. | - | - | - | - |
| 1 | The SS sends the PSI-Based SDU Discard Activation MAC CE to the UE | <-- | MAC PDU (PSI-Based SDU Discard Activation/Deactivation MAC CE) | - | - |
| 2 | The SS transfers 6 constructed PDUs into 6 PDCP Data PDUs and the PDCP SN = "0" within TX_NEXT. The first three PDCP Data PDUs belong to the PDU Set 0; The last three PDCP Data PDUs belong to the PDU Set 1. |  | - | - | - |
| - | EXCEPTION: Step 3 shall be repeated for k=0 to 2 (increment=1) with the below specified PDU size sent to the UE: Data PDU#1 = 55 bytes for k=0 Data PDU#2 = 62 bytes for k=1 Data PDU#3 = 78 bytes for k=2 | - | - | - | - |
| 3 | The SS sends a PDCP Data PDU via RLC-UM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = k After having sent a PDU, the SS sets PDCP SN is set to k+1 within TX_NEXT. | <-- | PDCP DATA PDU (SN=k) | - | - |
| 4 | Wait for the smallest remaining value of discardTimerForLowImportance below the remainingTimeThreshold. Note: According to TS38.508-1, timer tolerance should be 10% of remainingTimeThreshold and discardTimerForLowImportance. | - | - | - | - |
| 5 | The SS resumes normal UL grant allocation. | - | - | - | - |
| 6 | Check: Does UE transmit a DSR report in the next 1 second? | --> | DSR | 1 | F |
| - | EXCEPTION: The SS does not allocate UL grants unless when explicitly stated so in the procedure. | - | - | - | - |
| - | EXCEPTION: Step 7 shall be repeated for k=3 to 5 (increment=1) with the below specified PDU size sent to the UE: Data PDU#4 = 94 bytes for k=3 Data PDU#5 = 110 bytes for k=4 Data PDU#6 = 126 bytes for k=5 | - | - | - | - |
| 7 | The SS sends a PDCP Data PDU via RLC-UM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = k After having sent a PDU, the SS sets PDCP SN to k+1 within TX_NEXT. | <-- | PDCP DATA PDU (SN=k) | - | - |
| 8 | Wait for the smallest remaining value of discardTimer below the remainingTimeThreshold. Note: According to TS38.508-1, timer tolerance should be 10% of remainingTimeThreshold and discardTimer. | - | - | - | - |
| 9 | The SS resumes normal UL grant allocation. | - | - | - | - |
| 10 | Check: Does UE transmit a DSR to the SS? | --> | DSR | 2 | P |

7.1.3.5.10.3.3	Specific message contents

Table 7.1.3.5.10.3.3-1: SchedulingRequest-Config (Preamble)

| Derivation Path: 38.508-1 [4], Table 4.6.3-155 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| sr-TransMax | n64 |  |  |

Table 7.1.3.5.10.3.3-2: PDCP-Config

| Derivation Path: 38.508-1 [4], Table 4.6.3-99 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DiscardTimerForLowImportance-r18 | ms100 |  |  |

<< END OF CHANGES >>