3GPP TSG-RAN5 Meeting #109	R5-256837

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 1193 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update A-MPR test case for NS 202 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_mmWave_protect-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Update A-MPR test case for NS 202 |  |  |  |  |  |  |  |  |
| Summary of change: |  | 6.2.3.3.3.3 updated with 38.101-2 latest values |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work item NR_mmWave_protect-UEConTest, cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.3.3, 6.2.3.3.3.3, 6.2.3.3.3.4. Table 6.2.3.5-7, Table 6.2.3.5-7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the final outcome of R5-255577 and the result of 1 revision. |  |  |  |  |  |  |  |  |

==============First change==============

### 6.2.3	UE maximum output power with additional requirements

Editor's note:	The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS for power class other than 1, 3, 5 and 7..

6.2.3.1	Test purpose

Additional spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power.

6.2.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

6.2.3.3	Minimum conformance requirements

##### 6.2.3.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band (the IE field freqBandIndicatorNR) and an associated value of additionalSpectrumEmission in the relevant RRC information elements.

To meet these additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in clause 6.2.1.1.3. Unless stated otherwise, an A-MPR of 0 dB shall be used.

Table 6.2.3.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.3.1-2. Unless otherwise stated, the allowed total back off is maximum of A-MPR and MPR specified in clause 6.2.2.

Table 6.2.3.3.1-1: Additional maximum power reduction (A-MPR)

| Network Signalling label | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_200 |  |  |  |  | N/A |
| NS_201 (NOTE 1) | 6.5.3.3.3 | n258 |  |  | 6.2.3.3.2 |
| NS_202 | 6.5.3.3.3 | n257, n258 | 50, 100, 200, 400 | Table 5.3.2-1 | 6.2.3.3.3 |
| NS_203 | 6.5.3.3.3 | n258 | 50, 100, 200, 400 | Table 5.3.2-1 | 6.2.3.3.4 |
| NS_205 | 6.5.3.3.3 | n257, n258 | 50,100, 200, 400 | Table 5.3.2-1 | 6.2.3.3.6 |
| NOTE 1: NS_201 is obsolete, the associated additional spurious emission requirements are not applicable. |  |  |  |  |  |

Table 6.2.3.3.1-2: Mapping of Network Signalling label

| NR Band | Value of additionalSpectrumEmission (NOTE 1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n257 | NS_200 | NS_202 | NS_205 |  |  |  |  |  |
| n258 | NS_200 | NS_2012 | NS_202 | NS_203 | NS_205 |  |  |  |
| n260 | NS_200 |  |  |  |  |  |  |  |
| n261 | NS_200 |  |  |  |  |  |  |  |
| NOTE 1: additionalSpectrumEmission corresponds to an information element of the same name defined in sub-clause 6.3.2 of TS 38.331 [19]. NOTE 2: NS_201 is obsolete, the associated additional spurious emission requirements are not applicable. |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2.3.1.

##### 6.2.3.3.2	Void

6.2.3.3.2.1	Void

6.2.3.3.2.2	Void

6.2.3.3.2.3	Void

##### 6.2.3.3.3	A-MPR for NS_202

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2.3.3.

6.2.3.3.3.1	A-MPR for NS_202 for power class 1

For power class 1, A-MPR for NS_202 shall be 11.0 dB.

6.2.3.3.3.2	A-MPR for NS_202 for power class 2

For power class 2, A-MPR for NS_202 specified in clause 6.2.3.3.3.3 applies.

6.2.3.3.3.3	A-MPR for NS_202 for power class 3

For power class 3, A-MPR for NS_202 shall be 2.0 dB.

6.2.3.3.3.4	A-MPR for NS_202 for power class 4

For power class 4, A-MPR for NS_202 specified in clause 6.2.3.3.3.3 applies.

6.2.3.3.3.5	A-MPR for NS_202 for power class 5

For power class 5, A-MPR for NS_202 specified in clause 6.2.3.3.3.3 applies.

6.2.3.3.3.6	A-MPR for NS_202 for power class 6

For power class 6, A-MPR for NS_202 specified in clause 6.2.3.3.3.3 applies.

6.2.3.3.3.7	A-MPR for NS_202 for power class 7

For power class 7, A-MPR for NS_202 specified in clause 6.2.3.3.3.3 applies.

##### 6.2.3.3.4	A-MPR for NS_203

6.2.3.3.4.1	A-MPR for NS_203 for power class 1

For power class 1, A-MPR for NS_203 shall be 3.0 dB if Offset frequency < BWchannel, 0.0 dB otherwise. 
The Offset frequency is defined as the frequency from 24.25 GHz to the lower edge of the channel bandwidth.

6.2.3.3.4.2	A-MPR for NS_203 for power class 2

For power class 2, A-MPR for NS_203 specified in clause 6.2.3.3.4.3 applies.

6.2.3.3.4.3	A-MPR for NS_203 for power class 3

For power class 3, A-MPR for NS_203 shall be 0 dB.

6.2.3.3.4.4	A-MPR for NS_203 for power class 4

For power class 4, A-MPR for NS_203 specified in clause 6.2.3.3.4.3 applies.

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2.3.4.

6.2.3.3.4.5	A-MPR for NS_203 for power class 5

For power class 6, AMPR for NS_203 specified in clause 6.2.3.3.4.3 applies.

6.2.3.3.4.6	A-MPR for NS_203 for power class 6

For power class 6, AMPR for NS_203 specified in clause 6.2.3.3.4.3 applies.

6.2.3.3.4.7	A-MPR for NS_203 for power class 7

For power class 7, A-MPR for NS_203 specified in clause 6.2.3.3.4.3 applies.

##### 6.2.3.3.5	[Reserved]

##### 6.2.3.3.6	A-MPR for NS_205

6.2.3.3.6.1	A-MPR for NS_205 for power class 1

For power class 1, A-MPR for NS_205 shall be 7.0 dB if offset frequency < BWchannel, 6.0 dB otherwise.

Offset frequency is defined as the frequency from the upper edge of the protected frequency range i.e. 24 GHz to the lower edge of the channel bandwidth.

6.2.3.3.6.2	A-MPR for NS_205 for power class 2

For power class 2, A-MPR for NS_205 specified in clause 6.2.3.6.3 applies.

6.2.3.3.6.3	A-MPR for NS_205 for power class 3

For power class 3, A-MPR for NS_205 shall be 2.0 dB if offset frequency < BWchannel, 0.0 dB otherwise.

Offset frequency is defined as the frequency from the upper edge of the protected frequency range i.e. 24 GHz to the lower edge of the channel bandwidth.

6.2.3.3.6.4	A-MPR for NS_205 for power class 4

For power class 4, A-MPR for NS_205 specified in clause 6.2.3.6.3 applies.

6.2.3.3.6.5	A-MPR for NS_205 for power class 5

For power class 5, A-MPR for NS_205 specified in clause 6.2.3.6.3 applies.

6.2.3.3.6.6	A-MPR for NS_205 for power class 6

For power class 6, A-MPR for NS_205 specified in clause 6.2.3.6.3 applies.

6.2.3.3.6.7	A-MPR for NS_205 for power class 7

For power class 7, A-MPR for NS_205 specified in clause 6.2.3.6.3 applies.

6.2.3.4	Test description

6.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.2.3.4.1-2 to Table 6.2.3.4.1-6. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.3.4.1-1: Void

Table 6.2.3.4.1-2: Test configuration table for NS_202

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] clause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] clause 4.3.1 |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] clause 4.3.1 |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | 120kHz |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | - | Modulation | RB allocation (NOTE 1) |
| 1 (NOTE 4) |  | DFT-s-OFDM QPSK | Inner_Full |
| 2 |  | DFT-s-OFDM QPSK | Inner_1RB_Left for PC2, PC3, PC4, PC6 and PC7 Inner_Partial for PC1 (NOTE 2) |
| 3 (NOTE 3) |  | DFT-s-OFDM 64QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3，PC4, PC6 and PC7 or Table 6.1-2 for PC1. NOTE 2: When testing Low range configure uplink RB to Inner_1RB_Left for PC2, PC3, PC4, PC6 and PC7 or Inner_Partial_Left_Region1 for PC1 and when testing High range configure uplink RB to Inner_1RB_Right for PC2, PC3, PC4, PC6 and PC7 or Inner_Partial_Right_Region1 for PC1. NOTE 3: Test ID only applicable to PC1. NOTE 4: Test ID only applicable to PC2, PC3, PC4, PC6 and PC7. |  |  |  |

Table 6.2.3.4.1-3: Test configuration table for NS_203

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] clause 4.1 |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] clause 4.3.1 |  |  |  | Low range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] clause 4.3.1 |  |  |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |  |
| Test ID | Frequency | Channel Bandwidth | Downlink Configuration | Uplink Configuration |  |
|  |  |  | - | Modulation | RB allocation (NOTE 1) |
| 1 | Default | Default |  | DFT-s-OFDM QPSK | Inner_Full |
| 2 | Default | Default |  | DFT-s-OFDM QPSK | Inner_1RB_Left for PC2, PC3, PC4, PC6 and PC7 Inner_Partial_Left_Region1 for PC1 |
| 3 (NOTE 2) | Low range + Channel Bandwidth (NOTE 3) | Default |  | DFT-s-OFDM QPSK | Inner_Partial_Left_Region1 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3, PC4. PC6 and PC7 or Table 6.1-2 for PC1. NOTE 2: Test ID only applicable to PC1 NOTE 3: Test frequencies for test ID 3 is specified in Table 6.2.3.4.1-4. |  |  |  |  |  |

Table 6.2.3.4.1-4: NS_203 test ID3 test frequencies for NR operating band n258, SCS 120kHz and ΔFRaster 120 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] | kSSB | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50 | 32 | Downlink & Uplink | Low + CHBW | 24325.08 | 2017917 | 24302.04 | 2017533 | 0 | 120 | 22260 | 2017819 | 11 | 1 | 0 (0) | 2 |
| 100 | 66 | Downlink & Uplink | Low + CHBW | 24400.08 | 2019167 | 24352.56 | 2018375 | 0 | 120 | 22263 | 2018683 | 10 | 2 | 0 (0) | 4 |
| 200 | 132 | Downlink & Uplink | Low + CHBW | 24550.08 | 2021667 | 24455.04 | 2020083 | 0 | 120 | 22269 | 2020411 | 8 | 3 | 0 (0) | 6 |
| 400 | 264 | Downlink & Uplink | Low + CHBW | 24850.08 | 2026667 | 24660.00 | 2023499 | 0 | 120 | 22281 | 2023867 | 4 | 1 | 1 (4) | 10 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-5: [Reserved]

Table 6.2.3.4.1-6: Test configuration table for NS_205

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  | Low range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |  |
| Test ID | Frequency | Channel Bandwidth | Downlink Configuration | Uplink Configuration |  |
| 12 | Default | Default |  | Modulation | RB allocation (NOTE 1) |
| 22 | Default | Default |  | DFT-s-OFDM QPSK | Outer_Full |
| 32,4 | Mid | Default |  | DFT-s-OFDM QPSK | Inner_Partial_Left_Region1 |
| 42,4 | Mid | Default |  | DFT-s-OFDM QPSK | Outer_Full |
| 53 | Default | Default |  | DFT-s-OFDM QPSK | Inner_Partial_Left_Region1 |
| 63 | Default | Default |  | DFT-s-OFDM QPSK | Inner_Full |
| 73,4 | Mid | Default |  | DFT-s-OFDM QPSK | Inner_1RB_Left |
| 83,4 | Mid | Default |  | DFT-s-OFDM QPSK | Inner_Full |
|  |  |  |  | DFT-s-OFDM QPSK | Inner_1RB_Left |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3, PC4. PC6 and PC7 or Table 6.1-2 for PC1. NOTE 2: The test ID applies to PC1. NOTE 3: The test ID applies to PC2, PC3, PC4, PC6 and PC7. NOTE 4: The test ID applies to band n258. |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] clause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The DL and UL Reference Measurement channels are set according to Table 6.2.3.4.1-2 to Table 6.2.3.4.1-6.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.2.3.4.3

6.2.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.3.4.1-2 to Table 6.2.3.4.1-3. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

3.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 msec starting from the first TPC command in this step to ensure that the UE transmits at its maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

4.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

5.	Measure UE EIRP in the Tx beam peak direction in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3.5-5 to Table 6.2.3.5-24. EIRP test procedure is defined in Annex K. The measuring duration is one active uplink subframe. EIRP is calculated considering both polarizations, theta and phi.

6.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.

6.2.3.4.3	Message contents

Message contents are according to TS 38.508-1 [10] clause 4.6, with the following exceptions for each network signalling value.

1.	Information element AdditionalSpectrumEmission for NR can be set in SIB1 according to TS 38.331[19]. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement

| Derivation Path: TS 38.508-1 [10] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (NS_202) | for band n257 |  |
| AdditionalSpectrumEmission | 2 (NS_202) | for band n258 |  |
| AdditionalSpectrumEmission | 3 (NS_203) | for band n258 |  |
| AdditionalSpectrumEmission | 4 (NS_205) | for band n257 |  |
| AdditionalSpectrumEmission | 5 (NS_205) | for band n258 |  |

6.2.3.5	Test requirement

The UE EIRP derived in step 5 shall not exceed the values specified in Table 6.2.3.5-5 to Table 6.2.3.5-24.

Table 6.2.3.5-1: Void

Table 6.2.3.5-2: Void

Table 6.2.3.5-3: Void

Table 6.2.3.5-4: Void

Table 6.2.3.5-5: UE Power Class 1 test requirements (network signalling value "NS_202")

| Band | Test ID | PPowerclass | MPRf,c | A- MPRf,c | T(MAX(MPRf,c, A- MPRf,c,)) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n257, n258 | 2 | 40 | 0 | 11 | 7 | 22-TT | 55 |
|  | 3 |  | 6.5 | 11 | 7 | 22-TT | 55 |

Table 6.2.3.5-6: UE Power Class 2 test requirements (network signalling value "NS_202")

| Band | Test ID | PPowerclass | MPRf,c | A- MPRf,c | T(MAX(MPRf,c, A- MPRf,c,)) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n257, n258 | 1 | 29 | 0 | 1 | 1.5 | 26.5-TT | 43 |
|  | 2 |  | 0 | 1 | 1.5 | 26.5-TT | 43 |

Table 6.2.3.5-7: UE Power Class 3 test requirements (network signalling value "NS_202")

| Band | Test ID | PPowerclass | MPRf,c | A- MPRf,c | T(MAX(MPRf,c, A- MPRf,c,)) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n257, n258 | 1 | 22.4 | 0 | 2 | 1.5 | 18.9-TT-MBP,n | 43 |
|  | 2 |  | 0 | 2 | 1.5 | 18.9-TT-MBP,n | 43 |
| Note 1: MBP,n is the Multiband Relaxation factor for the tested band. This shall fulfil the requirements in Table 6.2.1.1.3.3-5. |  |  |  |  |  |  |  |

Table 6.2.3.5-8: UE Power Class 4 test requirements (network signalling value "NS_202")

| Band | Test ID | PPowerclass | MPRf,c | A- MPRf,c | T(MAX(MPRf,c, A- MPRf,c,)) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n257, n258 | 1 | 34 | 0 | 1 | 1.5 | 31.5-TT | 43 |
|  | 2 |  | 0 | 1 | 1.5 | 31.5-TT | 43 |

Table 6.2.3.5-9: UE Power Class 5 and 6 test requirements (network signalling value "NS_202")

| Band | Test ID | PPowerclass | MPRf,c | A- MPRf,c | T(MAX(MPRf,c, A- MPRf,c,)) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n257 | 1 | 30 | 0 | 1 | 1.5 | 27.5-TT-MBP,n | 43 |
| n258 | 2 | 30.4 | 0 | 1 | 1.5 | 27.9-TT-MBP,n | 43 |
| Note 1: MBP,n is the Multiband Relaxation factor for the tested band. This shall fulfil the requirements in Table 6.2.1.1.3.5-4 for PC5 and FSS for PC6. |  |  |  |  |  |  |  |

Table 6.2.3.5-9a: UE Power Class 7 test requirements (network signalling value "NS_202")

| Band | Test ID | PPowerclass | MPRf,c | A- MPRf,c | T(MAX(MPRf,c, A- MPRf,c,)) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n257, n258 | 1 | 16.4 | 0 | 1 | 1.5 | 13.9-TT-MBP,n | 43 |
|  | 2 |  | 0 | 1 | 1.5 | 13.9-TT-MBP,n | 43 |
| Note 1: MBP,n is the Multiband Relaxation factor for the tested band. This shall fulfil the requirements in Table 6.2.1.1.3.3-5. |  |  |  |  |  |  |  |

Table 6.2.3.5-10: UE Power Class 1 test requirements (network signalling value "NS_203")

| Band | Test ID | PPowerclass | MPRf,c | A- MPRf,c | T(MAX(MPRf,c, A- MPRf,c,)) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n258 | 1 | 40 | 0 | 3 | 2 | 35-TT | 55 |
|  | 2 |  | 0 | 3 | 2 | 35-TT | 55 |
|  | 3 |  | 0 | 0 | 0 | 40-TT | 55 |

Table 6.2.3.5-11: UE Power Class 2 test requirements (network signalling value "NS_203")

| Band | Test ID | PPowerclass | MPRf,c | A- MPRf,c | T(MAX(MPRf,c, A- MPRf,c,)) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n258 | 1 | 29 | 0 | 0 | 0 | 29-TT | 43 |
|  | 2 |  | 0 | 0 | 0 | 29-TT | 43 |

Table 6.2.3.5-12: UE Power Class 3 test requirements (network signalling value "NS_203")

| Band | Test ID | PPowerclass | MPRf,c | A- MPRf,c | T(MAX(MPRf,c, A- MPRf,c,)) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n258 | 1 | 22.4 | 0 | 0 | 0 | 22.4-TT-ΔMBP,n | 43 |
|  | 2 |  | 0 | 0 | 0 | 22.4-TT-ΔMBP,n | 43 |
| Note 1: MBP,n is the Multiband Relaxation factor for the tested band. This shall fulfil the requirements in Table 6.2.1.1.3.3-5. |  |  |  |  |  |  |  |

Table 6.2.3.5-13: UE Power Class 4 test requirements (network signalling value "NS_203")

| Band | Test ID | PPowerclass | MPRf,c | A- MPRf,c | T(MAX(MPRf,c, A- MPRf,c,)) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n258 | 1 | 34 | 0 | 0 | 0 | 34-TT | 43 |
|  | 2 |  | 0 | 0 | 0 | 34-TT | 43 |

Table 6.2.3.5-14: UE Power Class 5 and 6 test requirements (network signalling value "NS_203")

| Band | Test ID | PPowerclass | MPRf,c | A- MPRf,c | T(MAX(MPRf,c, A- MPRf,c,)) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n258 | 1 | 30.4 | 0 | 0 | 0 | 30.4-TT-ΔMBP,n | 43 |
|  | 2 | 30.4 | 0 | 0 | 0 | 30.4-TT-ΔMBP,n | 43 |
| Note 1: MBP,n is the Multiband Relaxation factor for the tested band. This shall fulfil the requirements in Table 6.2.1.1.3.5-4 for PC5 and FFS for PC6. |  |  |  |  |  |  |  |

Table 6.2.3.5-14a: UE Power Class 7 test requirements (network signalling value "NS_203")

| Band | Test ID | PPowerclass | MPRf,c | A- MPRf,c | T(MAX(MPRf,c, A- MPRf,c,)) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n258 | 1 | 16.4 | 0 | 0 | 0 | 16.4-TT-ΔMBP,n | 43 |
|  | 2 |  | 0 | 0 | 0 | 16.4-TT-ΔMBP,n | 43 |
| Note 1: MBP,n is the Multiband Relaxation factor for the tested band. This shall fulfil the requirements in Table 6.2.1.1.3.3-5. |  |  |  |  |  |  |  |

Table 6.2.3.5-15: Test Tolerance (Power class 3)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | 3.24 dB | 3.11 dB |

Table 6.2.3.5-16: Test Tolerance (Power class 1)

| Test Metric | FR2a |
| --- | --- |
| Max device size ≤ 30 cm | 3.38 dB |

Table 6.2.3.5-17: Test Tolerance (Power class 5)

| Test Metric | FR2a |
| --- | --- |
| Max device size ≤ 30 cm | 3.38 dB |

Table 6.2.3.5-18: Test Tolerance (Power class 7)

| Test Metric | FR2a |
| --- | --- |
| Max device size ≤ 30 cm | 3.24 dB |

Table 6.2.3.5-19: UE Power Class 1 test requirements (network signalling value "NS_205")

| Band | Test ID | PPowerclass | MPRf,c | A- MPRf,c | T(MAX(MPRf,c, A- MPRf,c,)) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n257 | 1 | 40 | 6.5 | 6 | 5 | 28.5-TT | 55 |
|  | 2 |  | 0 | 6 | 5 | 29-TT | 55 |
| n258 | 1 |  | 6.5 | 7 | 5 | 28-TT | 55 |
|  | 2 |  | 0 | 7 | 5 | 28-TT | 55 |
|  | 3 |  | 6.5 | 6 | 5 | 28.5-TT | 55 |
|  | 4 |  | 0 | 6 | 5 | 29-TT | 55 |

Table 6.2.3.5-20: UE Power Class 2 test requirements (network signalling value "NS_205")

| Band | Test ID | PPowerclass | MPRf,c | A- MPRf,c | T(MAX(MPRf,c, A- MPRf,c,)) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n257 | 5 | 29 | 0 | 0 | 0 | 29-TT | 43 |
|  | 6 |  | 2.5 | 0 | 2 | 24.5-TT | 43 |
| n258 | 5 |  | 0 | 2 | 1.5 | 25.5-TT | 43 |
|  | 6 |  | 2.5 | 2 | 2 | 24.5-TT | 43 |
|  | 7 |  | 0 | 0 | 0 | 29-TT | 43 |
|  | 8 |  | 2.5 | 0 | 2 | 24.5-TT | 43 |

Table 6.2.3.5-21: UE Power Class 3 test requirements (network signalling value "NS_205")

| Band | Test ID | PPowerclass | MPRf,c | A- MPRf,c | T(MAX(MPRf,c, A- MPRf,c,)) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n257 | 5 | 22.4 | 0 | 0 | 0 | 22.4-TT-MBP,n | 43 |
|  | 6 |  | 2.5 | 0 | 2 | 17.9-TT-MBP,n | 43 |
| n258 | 5 |  | 0 | 2 | 1.5 | 18.9-TT-MBP,n | 43 |
|  | 6 |  | 2.5 | 2 | 2 | 17.9-TT-MBP,n | 43 |
|  | 7 |  | 0 | 0 | 0 | 22.4-TT-MBP,n | 43 |
|  | 8 |  | 2.5 | 0 | 2 | 17.9-TT-MBP,n | 43 |
| Note 1: MBP,n is the Multiband Relaxation factor for the tested band. This shall fulfil the requirements in Table 6.2.1.1.3.3-5. |  |  |  |  |  |  |  |

Table 6.2.3.5-22: UE Power Class 4 test requirements (network signalling value "NS_205")

| Band | Test ID | PPowerclass | MPRf,c | A- MPRf,c | T(MAX(MPRf,c, A- MPRf,c,)) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n257 | 5 | 34 | 0 | 0 | 0 | 34-TT | 43 |
|  | 6 |  | 2.5 | 0 | 2 | 29.5-TT | 43 |
| n258 | 5 |  | 0 | 2 | 1.5 | 30.5-TT | 43 |
|  | 6 |  | 2.5 | 2 | 2 | 29.5-TT | 43 |
|  | 7 |  | 0 | 0 | 0 | 34-TT | 43 |
|  | 8 |  | 2.5 | 0 | 2 | 29.5-TT | 43 |

Table 6.2.3.5-23: UE Power Class 5 and 6 test requirements (network signalling value "NS_205")

| Band | Test ID | PPowerclass | MPRf,c | A- MPRf,c | T(MAX(MPRf,c, A- MPRf,c,)) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n257 | 5 | 30 | 0 | 0 | 0 | 30-TT-MBP,n | 43 |
|  | 6 |  | 2.5 | 0 | 2 | 25.5-TT-MBP,n | 43 |
| n258 | 5 | 30.4 | 0 | 2 | 1.5 | 26.9-TT-MBP,n | 43 |
|  | 6 |  | 2.5 | 2 | 2 | 25.9-TT-MBP,n | 43 |
|  | 7 |  | 0 | 0 | 0 | 30.4-TT-MBP,n | 43 |
|  | 8 |  | 2.5 | 0 | 2 | 25.9-TT-MBP,n | 43 |
| Note 1: MBP,n is the Multiband Relaxation factor for the tested band. This shall fulfil the requirements in Table 6.2.1.1.3.5-4 for PC5 and 6.2.1.1.6-4 for PC6. |  |  |  |  |  |  |  |

Table 6.2.3.5-24: UE Power Class 7 test requirements (network signalling value "NS_205")

| Band | Test ID | PPowerclass | MPRf,c | A- MPRf,c | T(MAX(MPRf,c, A- MPRf,c,)) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n257 | 5 | 16.4 | 0 | 0 | 0 | 16.4-TT-MBP,n | 43 |
|  | 6 |  | 2.5 | 0 | 2 | 11.9-TT-MBP,n | 43 |
| n258 | 5 |  | 0 | 2 | 1.5 | 12.9-TT-MBP,n | 43 |
|  | 6 |  | 2.5 | 2 | 2 | 11.9-TT-MBP,n | 43 |
|  | 7 |  | 0 | 0 | 0 | 16.4-TT-MBP,n | 43 |
|  | 8 |  | 2.5 | 0 | 2 | 11.9-TT-MBP,n | 43 |
| Note 1: MBP,n is the Multiband Relaxation factor for the tested band. This shall fulfil the requirements in Table 6.2.1.1.3.3-5. |  |  |  |  |  |  |  |

==============Next change==============

<Contents of the next change, including the text of the whole of the affected clause. Insert sepearator if there are more than one additional changes>

==============End of change==============