3GPP TSG-RAN5 Meeting #108	R5-253797r1

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 1034 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of reference sensitivity TP analysis for new EN-DC combos within FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Test points of reference sensitivity for the EN-DC combinations need to be introduced. DC_1A-11A_n77A PC2&PC3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding the TP analysis of reference sensitivity for these NRCA combinations into Table 4.3.3.1-2. - Test points for these EN-DC combinations are clarified in RAN4, no further test point analysis is needed in RAN5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Test point analysis of reference sensitivity for these EN-DC combinations will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 CR 1961 TS 38.521-3 CR 1966 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Related WIC: NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: revert changes for DC_1A_n77(2A) PC2&PC3, DC_3A_n77(2A) PC2&PC3. |  |  |  |  |  |  |  |  |

-------- Start of changes --------

4.3.3.1	Reference sensitivity test cases for EN-DC

This clause contains information on test point analysis and status for FR1 EN-DC test cases in TS 38.521-3 [4] clause 7. The analyses are performed per EN-DC configuration in Table 4.3.3.1-1, Table 4.3.3.1-2 and Table 4.3.3.1-3.

Table 4.3.3.1-1: Reference Sensitivity test cases per EN-DC configuration (2CC)

| Band or band configuration | Single UL | Refsens exception, victim band (NOTE 2) | Requirement coverage (NOTE 2) | Comments |
| --- | --- | --- | --- | --- |
| DC_(n)XAA |  |  |  |  |
| 5 | Only single UL requirements defined | 5 | Exception | RAN5#92-e |
| 12 | Only single UL requirements defined | 12 | Exception | RAN5#92-e |
| 38 | Only single UL requirements defined | N/A | NE | RAN5#88-e |
| 41 | Yes | N/A | NE | RAN5#88-e |
| 48 | Only single UL requirements defined | N/A | NE | RAN5#88-e |
| 71 | Yes | 71 | Exception | RAN5#88-e |
| DC_XA_nXA |  |  |  |  |
| DC_2A_n2A | Only single UL requirements defined | N/A | NE | RAN5#88-e |
| DC_3A_n3A | Yes | 3 | Exception | RAN5#88-e |
| DC_5A_n5A | Only single UL requirements defined | N/A | NE | RAN5#88-e |
| DC_7A_n7A | Only single UL requirements defined | N/A | NE | RAN5#88-e |
| DC_41A_n41A | Only single UL requirements defined | N/A | NE | RAN5#88-e |
| DC_48A_n48A | Only single UL requirements defined | N/A | NE | RAN5#88-e |
| DC_66A_n66A | Only single UL requirements defined | N/A | NE | RAN5#88-e |
| DC_71A_n71A | Only single UL requirements defined | N/A | NE | RAN5#92-e |
| DC_XA_nYA (NOTE 1) |  |  |  |  |
| DC_1A_n3A | Yes | 1 (IMD3), CBI | NE, IMD, CBI, CBI avoid | RAN5#89-e |
| DC_1A_n7A | No | N/A | NE | RAN5#94-e |
| DC_1A_n8A | No | 1 (IMD4) | NE, IMD | RAN5#95-e |
| DC_1A_n28A | No | 1 (HD3) | NE, HD, HD avoid | RAN5#98 |
| DC_1A_n41A | No | 1 (CBI, PC2&PC3), n41 (CBI) | NE, CBI | RAN5#98 |
| DC_1A_n77A | No | n77 (HD2), 1 (IMD2, PC2&PC3), 1 (IMD4, PC2&PC3) | NE, HD, IMD | RAN5#104 |
| DC_1A_n78A | No | 1 (IMD4) | NE, IMD | RAN5#88-e |
| DC_1A_n79A | No | N/A | NE | RAN5#103 |
| DC_2A_n41A | No | 2 (CBI) | NE, CBI (PC2&PC3) | RAN5#106 |
| DC_2A_n66A | No | 2 (IMD3), n66 (IMD5) | NE, IMD | RAN5#94-e |
| DC_2A_n71A | No | 2 (HD3), n71 (HM) | NE, HD, HM | RAN5#93-e |
| DC_2A_n77A | No | n77 (HD), 2 (HM), 2 (IMD2, PC2&PC3), 2 (IMD4, PC2&PC3), 2 (IMD5), 2 (CBI, PC2) | NE, HD, HD avoid, HM, HM avoid, IMD, CBI | RAN5#103 |
| DC_2A_n78A | No | n78 (HD2), 2 (IMD2), 2 (IMD4) | NE, HD, IMD | RAN5#94-e |
| DC_3A_n1A | Yes | n1 (IMD3), 3 (CBI) | NE, IMD, CBI, CBI avoid | RAN5#107 |
| DC_3A_n5A | Yes | 3 (IMD4), n5 (IMD2) | NE, IMD | RAN5#94-e |
| DC_3A_n8A | No | 3 (IMD5), n8 (IMD4) | NE, IMD | RAN5#96-e |
| DC_3A_n28A | No | N/A | NE | RAN5#95-e |
| DC_3A_n77A | No | n77 (HD2), 3 (HM, PC2&PC3), 3 (IMD2), 3 (IMD4) | NE, HD, HD avoid HM, HM avoid, IMD | RAN5#106 |
| DC_3A_n78A | Yes | 3 (IMD2, PC2&PC3), 3 (IMD4, PC2&PC3), 3 (HM), n78 (HD2) | NE, IMD, HD, HD avoid, HM, HM avoid | RAN5#106 |
| DC_3A_n79A | No | N/A | NE | RAN5#103 |
| DC_5A_n66A | Yes | 5 (IMD2) | NE (anchor agnostic), IMD | RAN5#93-e |
| DC_5A_n77A | No | n77 (HD), 5 (IMD4, PC2&PC3), 5 (IMD5) | NE, HD, HD avoid, IMD | RAN5#94-e |
| DC_5A_n78A | No | n78 (HD4), 5 (IMD4) 5 (HM) | NE, HD, HD avoid, IMD HM, HM avoid | RAN5#106 |
| DC_7A_n1A | No | N/A | NE | RAN5#90-e |
| DC_7A_n3A | No | 7 (IMD4¦3*fB3-1*fB7¦) | NE, IMD | RAN5#90-e |
| DC_7A_n5A | Yes | n5 (IMD3) | NE, IMD | RAN5#94-e |
| DC_7A_n8A | No | N/A | NE | RAN5#95-e |
| DC_7A_n28A | No | N/A | NE | RAN5#94-e |
| DC_7A_n66A | No | 7 (IMD4) | NE, IMD | RAN5#99 |
| DC_7A_n71A | No | 7 (HD) | NE, HD, HD avoid | RAN5#99 |
| DC_7A_n77A | No | 7 (IMD4) | NE, IMD | RAN5#99 |
| DC_7A_n78A | No | CBI | NE, CBI | RAN5#94-e |
| DC_8A_n1A | No | n1 (IMD4 ¦2*fB1-2*fB8¦) | NE, IMD | RAN5#90-e |
| DC_8A_n3A | No | 8 (IMD4 ¦3*fB8-1*fB3¦), n3 (IMD5 ¦4*fB8-1*fB3¦) | NE, IMD | RAN5#90-e |
| DC_8A_n20A | Yes | 8 (IMD3), n20 (IMD3) | NE, IMD | RAN5#94-e |
| DC_8A_n28A | No | N/A | NE | RAN5#95-e |
| DC_8A_n41A | No | n41 (HD3), 8 (IMD3) | NE HD, ,HD avoid IMD | RAN5#98 |
| DC_8A_n77A | No | 8 (HD4), 8 (IMD4) | NE, HD, HD avoid, IMD | RAN5#92-e |
| DC_8A_n78A | No | n78 (HD4), 8 (IMD4) | NE, HD, HD avoid, IMD | RAN5#96-e |
| DC_8A_n79A | No | n79 (HD5), 8 (IMD5) | NE, HD, HD avoid, IMD | RAN5#96-e |
| DC_11A_n77A | No | N/A | NE | RAN5#91-e |
| DC_11A_n78A | No | N/A | NE | RAN5#91-e |
| DC_11A_n79A | No | 11 (HM) | NE, HM, HM avoid | RAN5#92-e |
| DC_12A_n2A | No | N/A | NE | RAN5#98 |
| DC_12A_n66A | No | n66 (HD3) | NE, HD, HD avoid | RAN5#97 |
| DC_12A_n78A | Yes (DC_12_n78) | n78 (HD5), 12 (IMD5) | NE, HD, HD avoid, IMD | RAN5#98 |
| DC_13A_n77A | No | n77 (HD), 13 (HM), 13 (IMD5, PC2&PC3) | NE, HD, HD avoid, HM, HM avoid, IMD | RAN5#94-e |
| DC_14A_n2A | No | N/A | NE | RAN5#98 |
| DC_18A_n77A | No | n77 (HD5), 18 (IMD4), 18 (IMD5) | NE, HD, IMD | RAN5#104 |
| DC_18A_n78A | No | 18 (IMD4) | NE, IMD | RAN5#104 |
| DC_19A_n77A | No | n77 (HD5), n77 (HD4) | NE, HD | RAN5#99 |
| DC_19A_n78A | No | n78(HD4), 19(HM, PC2&PC3), 19(IMD4, PC2&PC3) | NE, HD, HM, IMD | RAN5#102 |
| DC_19A_n79A | No | 19 (HM ¦5*fB19DL-1*fB79¦, PC2&PC3) | NE, HM, HM avoid | RAN5#101 |
| DC_20A_n1A | No | N/A | NE | RAN5#90-e |
| DC_20A_n3A | No | n3 (IMD4), 20 (IMD4) | NE, IMD | RAN5#89-e |
| DC_20A_n7A | DC_20_n7 | 20 (IMD3) | NE, IMD | RAN5#94-e |
| DC_20A_n8A | DC_20_n8 | 20 (IMD3), n8 (IMD3) | NE, IMD | RAN5#95-e |
| DC_20A_n78A | No | 20 (IMD4), n78 (HD) | NE, IMD, HD, HD avoid | RAN5#88-e |
| DC_21A_n28A | Yes | 21 (IMD5) | NE, IMD | RAN5#99 |
| DC_21A_n78A | No | N/A | NE | RAN5#103 |
| DC_21A_n79A | No | 21 (HM, PC2&PC3), 21 (IMD3, PC2&PC3) | NE, HM, HM avoid, IMD | RAN5#101 |
| DC_25A_n41A | No | 25 | CBI | RAN5#98 |
| DC_26A_n41A | No | n41 (HD3), 26 (IMD3), 26 (HM ¦3*fB26DL-1*fB41¦) | NE, HD, HD avoid, IMD, HM | RAN5#106 |
| DC_26A_n77A | No | n77 (HD4), 26 (IMD4) | NE, HD, HD avoid, IMD | RAN5#92-e |
| DC_26A_n78A | No | n78 (HD4), 26 (IMD4) | NE, HD, HD avoid, IMD | RAN5#92-e |
| DC_26A_n79A | No | 26 (HM ¦5*fB26DL-1*fB79¦) | NE, HM, HM avoid | RAN5#106 |
| DC_28A_n5A | No | 28 (CBI) | NE, CBI, CBI avoid | RAN5#107 |
| DC_28A_n7A | No | N/A | NE | RAN5#94-e |
| DC_28A_n77A | No | n77 (HD), n77 (HM), 28 (IMD5, PC2&PC3) | NE, HD, HD avoid, HM, IMD | RAN5#106 |
| DC_28A_n78A | No | n78 (HD), 28 (IMD5) | NE, HD, HD avoid, IMD | RAN5#106 |
| DC_30A_n66A | No | n66 (CBI) | NE, CBI | RAN5#97 |
| DC_38A_n78A | No | 38 (CBI) | NE, CBI | RAN5#99 |
| DC_39A_n41A4 | No | n41 (CBI) | NE, CBI | RAN5#104 |
| DC_40A_n1A | No | N/A | NE | RAN5#89-e |
| DC_40A_n41A4 | No | 40 (CBI) | NE, CBI | RAN5#107 |
| DC_40A_n78A | No | 40 (HM) | NE, HM, HM avoid | RAN5#89-e |
| DC_40A_n79A | No | N/A | NE | RAN5#94-e |
| DC_41A_n28A | No | N/A | NE | RAN5#98 |
| DC_41A_n77A (NOTE 3) | No | 41 (HM ¦3*fB41DL-2*fB77¦, PC2&PC3), 41 (CBI, PC2&PC3), n77 (CBI) | NE, HM, CBI | RAN5#106 |
| DC_41A_n78A (NOTE 3) | No | 41 (HM ¦3*fB41DL-2*fB78¦), 41 (CBI), n78 (CBI) | NE, HM, CBI | RAN5#106 |
| DC_42A_n77A | Only single UL requirements defined | N/A | NE | RAN5#91-e |
| DC_48A_n66A | No | 48 (HD2), 66 (IMD5) | NE (anchor agnostic), HD, IMD | RAN5#92-e |
| DC_66A_n2A | DC_66_n2 | n2 (IMD3), 66 (IMD5) | NE (anchor agnostic), IMD | RAN5#93-e |
| DC_66A_n5A | No | n5 (IMD2) | NE, IMD | RAN5#97 |
| DC_66A_n25A | No | 66, n25 (IMD3), 66 (IMD5) | NE, IMD | RAN5#99 |
| DC_66A_n41A | No | 66 (CBI) | NE, CBI | RAN5#98 |
| DC_66A_n71A | No | 66 (IMD4) | NE (anchor agnostic), IMD | RAN5#93-e |
| DC_66A_n77A | No | n77 (HD), 66 (IMD2, PC2&PC3), 66 (IMD5, PC2&PC3), 66 (CBI, PC2) | NE, HD, HD avoid, IMD, CBI | RAN5#103 |
| DC_66A_n78A | No | n78 (HD2), 66 (IMD5) | NE (anchor agnostic), HD, IMD | RAN5#93-e |
| DC_71A_n2A | No | n2 (HD3), 71 (HM¦3*fB71DL-1*fB2¦) | NE (anchor agnostic), HD, HM, HM avoid | RAN5#107 |
| DC_71A_n66A | No | n66 (IMD4) | NE (anchor agnostic), IMD | RAN5#98 |
| NOTE 1: If single UL is allowed in a DC_XA_nYA configuration the IMD requirements does not apply for UEs supporting UE capability singleUL-Transmission. NOTE 2: Notations used NE: Non-exception as defined in TS 38.101-3 [7], meaning standalone LTE in TS 36.101 [8] and NR in 38.101-1 [5] requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.1 HM: RX Harmonic Mixing, as defined in TS 38.101-3 [7], 7.3B.2.3.2 IMD: Intermodulation Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.5 CBI: Cross Band Isolation, as defined in TS 38.101-3 [7], 7.3B.2.3.4 HD avoid: single carrier requirements apply with aggressor of UL Harmonic Distortion active. HM avoid: single carrier requirements apply with aggressor of RX Harmonic Mixing active. CBI avoid: single carrier requirements apply with aggressor of Cross Band Isolation active. NOTE 3: HM cannot be avoided with aggressor still active since CBI exception always exists. Therefore, no HM avoid test point is added. NOTE 4: Exception for this configuration is tested only when the UE reports supporting Simultaneous Rx/Tx capability. |  |  |  |  |

Table 4.3.3.1-2: Reference Sensitivity test cases per EN-DC configuration (3CC)

| Band or band configuration | Single UL | UL config | Refsens exception, victim band | Requirement coverage (NOTE 2) | Fallback DC configurations | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| DC_(n)XCA |  |  |  |  |  |  |
| 41 | Yes | DC_(n)41AA | N/A | NE | DC_(n)41AA DC_41A_n41A | RAN5#89-e |
| 41 | Yes | DC_41A_n41A | N/A | NE | DC_(n)41AA DC_41A_n41A | RAN5#89-e |
| 48 | Only single UL requirements defined | DC_(n)48AA | N/A | NE | DC_(n)48AA DC_48A_n48A | RAN5#89-e |
| 48 | Only single UL requirements defined | DC_48A_n48A | N/A | NE | DC_(n)48AA DC_48A_n48A | RAN5#89-e |
| DC_XA_nYC |  |  |  |  |  |  |
| DC_3A_n78C | Yes | DC_3A_n78A | N/A | NE |  | RAN5#93-e |
| DC_XA_nY(2A) |  |  |  |  |  |  |
| FFS |  |  |  |  |  |  |
| DC_XA_nYA-nZA |  |  |  |  |  |  |
| DC_1A_n3A-n78A | Yes | DC_1A_n3A | n78 (3 band IMD2) | NE, IMD |  | RAN5#107 |
|  | No | DC_1A_n78A | n3 (3 band IMD2) | NE, IMD |  | RAN5#107 |
| DC_1A_n28A-n78A | No | DC_1A_n28A | n78 (3 band IMD3) | NE, IMD |  | RAN5#92-e |
|  | No | DC_1A_n78A | n28 (3 band IMD5) | NE, IMD |  | RAN5#92-e |
| DC_1A_n78A-n79A | No | DC_1A_n78A | n79 (3 band IMD3, PC2&PC3) | IMD |  | RAN5#103 |
|  | No | DC_1A_n79A | n78 (3 band IMD5, PC2&PC3) | IMD |  | RAN5#103 |
| DC_3A_n28A-n78A | No | DC_3A_n28A | n78 (3 band IMD5) | NE, IMD |  | RAN5#92-e |
|  | Yes | DC_3A_n78A | No 3CC exception | No test required |  |  |
| DC_3A_n78A-n79A | Yes | DC_3A_n78A | n79 (3 band IMD3, PC2&PC3) | IMD |  | RAN5#106 |
|  | No | DC_3A_n79A | n78 (3 band IMD5, PC2&PC3) | IMD |  | RAN5#106 |
| DC_7A_n5A-n78A | Yes | DC_7A_n5A | No 3CC exception | No test required |  | RAN5#94-e |
|  | No | DC_7A_n78A | No 3CC exception | No test required |  | RAN5#94-e |
| DC_7A_n28A-n78A | No | DC_7A_n28A | n78 (3 band IMD2) | NE, IMD |  | RAN5#92-e |
|  | No | DC_7A_n78A | n28 (3 band IMD2) | NE, IMD |  |  |
| DC_19A_n78A-n79A | No | DC_19A_n78A | n79 (3 band IMD2, PC2&PC3) | IMD |  | RAN5#106 |
|  | No | DC_19A_n79A | n78 (3 band IMD2, PC2&PC3) | IMD |  | RAN5#106 |
| DC_20A_n28A-n78A | No | DC_20A_n28A | n78 (3 band IMD4) | NE, IMD |  | RAN5#93-e |
|  | No | DC_20A_n78A | n28 (3 band IMD4) | NE, IMD |  | RAN5#93-e |
| DC_21A_n78A-n79A | No | DC_21A_n78A | n79 (3 band IMD2, PC2&PC3) | IMD |  | RAN5#103 |
|  | No | DC_21A_n79A | n78 (3 band IMD2, PC2&PC3) | IMD |  | RAN5#103 |
| DC_28A_n7A-n78A | No | DC_28A_n7A | n78 (3 band IMD2) | NE, IMD |  | RAN5#94-e |
|  | No | DC_28A_n78A | n7 (3 band IMD2) | NE, IMD |  | RAN5#94-e |
| DC_XA-YA_nZA |  |  |  |  |  |  |
| DC_1A-3A_n28A | No | DC_1A_n28A | 3 (3 band IMD5) | IMD |  | RAN5#92-e |
|  | No | DC_3A_n28A | 1 (3 band IMD4) | IMD |  | RAN5#92-e |
| DC_1A-3A_n41A | No | DC_1A_n41A | No 3CC exception | No test required |  | RAN5#98 |
|  | No | DC_3A_n41A | No 3CC exception | No test required |  | RAN5#98 |
| DC_1A-3A_n77A | Yes | DC_1A_n77A | 3 (3 band IMD2, PC2&PC3) 3 (3 band IMD4, PC2&PC3) | IMD |  | RAN5#104 |
|  | Yes | DC_3A_n77A | 1 (3 band IMD2, PC2&PC3) | IMD |  | RAN5#104 |
| DC_1A-3A_n78A | No | DC_1A_n78A | 3 (3 band IMD2, PC2&PC3) | IMD |  | RAN5#102 |
|  | Yes | DC_3A_n78A | 1 (3 band IMD5, PC2&PC3) | IMD |  | RAN5#102 |
| DC_1A-3A_n79A | No | DC_1A_n79A | No 3CC exception | No test required |  | RAN5#101 |
|  | No | DC_3A_n79A | 1(3 band IMD5, PC2&PC3) | IMD |  | RAN5#102 |
| DC_1A-7A_n3A | Yes | DC_1A_n3A | No 3CC exception | No test required |  |  |
|  | No | DC_7A_n3A |  |  |  |  |
| DC_1A-7A_n28A | No | DC_1A_n28A | 7 (3 band IMD2) | IMD |  | RAN5#92-e |
|  | No | DC_7A_n28A | No 3 band exception | No test required |  | RAN5#92-e |
| DC_1A-7A_n78A | No | DC_1A_n78A | 7 (IMD4 ¦3*fB1-1*fB78¦) | IMD |  |  |
|  | No | DC_7A_n78A | 1 (IMD4 ¦2*fB7-2*fB78¦) | IMD |  |  |
| DC_1A-8A_n3A | Yes | DC_1A_n3A | No 3CC exception | No test required |  |  |
|  | No | DC_8A_n3A |  |  |  |  |
| DC_1A-11A_n77A | Yes (DC_1_n77) | DC_1A_n77A | 11 (3 band IMD2, PC2&PC3) | NE, IMD |  | RAN5#108 |
| DC_1A-18A_n77A | No | DC_1A_n77A | 18 (3 band IMD5, PC2&PC3) | IMD |  | RAN5#105 |
|  | No | DC_18A_n77A | 1 (3 band IMD3, PC2&PC3) | IMD |  | RAN5#105 |
| DC_1A-19A_n78A | No | DC_1A_n78A | 19 (3 band IMD5, PC2&PC3) | IMD |  | RAN5#102 |
|  | No | DC_19A_n78A | 1 (3 band IMD3, PC2&PC3) | IMD |  | RAN5#102 |
| DC_1A-19A_n79A | No | DC_1A_n79A | 19 (3 band IMD3, PC2&PC3) | IMD |  | RAN5#102 |
|  | No | DC_19A_n79A | 1 (3 band IMD4, PC2&PC3) | IMD |  | RAN5#102 |
| DC_1A-20A_n3A | Yes | DC_1A_n3A | No 3CC exception | No test required |  |  |
|  | No | DC_20A_n3A |  |  |  |  |
| DC_1A-20A_n8A | No | DC_1A_n8A | 20 (3 band IMD4) | IMD |  | RAN5#95-e |
|  | Yes | DC_20A_n8A | No 3CC exception | - |  | RAN5#95-e |
| DC_1A-20A_n28A | No | DC_1A_n28A | No 3CC exception | No test required |  | RAN5#93-e |
|  | No | DC_20A_n28A | No 3CC exception | No test required |  | RAN5#93-e |
| DC_1A-20A_n78A | No | DC_1A_n78A | 20 (IMD5) | IMD |  |  |
|  | No | DC_20A_n78A | 1 (IMD3) | IMD |  |  |
| DC_1A-21A_n78A | No | DC_1A_n78A | 21 (3 band IMD2, PC2&PC3) 21 (3 band IMD5, PC2&PC3) | IMD |  | RAN5#102 |
|  | No | DC_21A_n78A | 1 (3 band IMD2, PC2&PC3) 1 (3 band IMD5, PC2&PC3) | IMD |  | RAN5#102 |
| DC_1A-21A_n79A | No | DC_1A_n79A | No 3CC exception | NE |  | RAN5#103 |
|  | No | DC_21A_n79A | No 3CC exception | NE |  | RAN5#103 |
| DC_1A-28A_n3A | Yes | DC_1A_n3A | 1 (3 band IMD4) | IMD | Yes | RAN5#90-e |
|  | No | DC_28A_n3A | No 3CC exception | - | No |  |
| DC_1A-28A_n5A | No | DC_1A_n5A | No 3CC exception | No test required |  | RAN5#95-e |
|  | No | DC_28A_n5A |  |  |  |  |
| DC_1A-28A_n78A | No | DC_1A_n78A | 28 (3 band IMD3) | IMD |  | RAN5#92-e |
|  | No | DC_28A_n78A | 1 (3 band IMD5) | IMD | DC_28A_n78A |  |
| DC_1A-41A_n28A | No | DC_1A_n28A | 41 (IMD2 ¦1*fB1+1*fn28¦) | IMD |  | RAN5#98 |
|  | No | DC_41A_n28A | No 3CC exception | No test required |  | RAN5#98 |
| DC_1A-41A_n41A | No | DC_1A_n41A | No 3CC exception | No test required |  | RAN5#98 |
| DC_1A-41A_n77A | No | DC_1A_n77A | 41 (IMD4 ¦1*fn77-3*fB1¦, PC2&PC3) 41 (IMD5 ¦3*fB1-2*fn77¦, PC2&PC3) | IMD |  | RAN5#104 |
|  | No | DC_41A_n77A | No 3CC exception | No test required |  | RAN5#98 |
| DC_1A-42A_n78A | No | DC_1A_n78A | No 3CC exception | NE |  | RAN5#103 |
| DC_1A-42A_n79A | No | DC_1A_n79A | 42 (3 band IMD5, PC2&PC3) | IMD |  | RAN5#102 |
| DC_2A-66A_n2A | No | DC_2A_n2A | No 3CC exception | No test required |  | RAN5#104 |
|  | No | DC_66A_n2A | 2 (3 band IMD3) | NE, IMD |  |  |
| DC_2A-66A_n5A | No | DC_2A_n5A | 66 (3 band IMD4) | IMD |  | RAN5#98 |
|  | Yes (DC_66_n5) | DC_66A_n5A | No 3CC exception | No test required |  |  |
| DC_2A_n5A-n77A | No | DC_2A_n5A | n77 (3 band IMD3) | NE, IMD |  | RAN5#101 |
|  | No | DC_2A_n77A | No 3CC exception | NE |  |  |
| DC_2A-13A_n77A | No | DC_2A_n77A | No 3CC exception | NE |  | RAN5#103 |
|  | No | DC_13A_n77A | 2 (3 band IMD3, PC2&PC3) | NE, IMD |  |  |
| DC_2A-66A_n41A | No | DC_2A_n41A | 2 (3 band IMD4) | IMD |  | RAN5#96-e |
|  | No | DC_66A_n41A | No 3CC exception |  |  |  |
| DC_2A-66A_n77A | No | DC_2A_n77A | 66 (3 band IMD2, PC2&PC3), 66 (3 band IMD4, PC2&PC3), 66 (3 band IMD5) | NE, IMD |  | RAN5#103 |
|  | No | DC_66A_n77A | 2(3 band IMD2, PC2&PC3) | NE, IMD |  |  |
| DC_3A-7A_n1A | Yes | DC_3A_n1A | No 3CC exception | No test required |  | RAN5#90-e |
|  | No | DC_7A_n1A | No 3CC exception |  |  |  |
| DC_3A-7A_n5A | Yes | DC_3A_n5A | 7 (3 band IMD2) | IMD |  | RAN5#95-e |
|  | Yes | DC_7A_n5A | No 3CC exception | - |  | RAN5#95-e |
| DC_3A-7A_n8A | No | DC_3A_n8A | 7 (3 band IMD2, IMD3) | IMD |  | RAN5#96-e |
|  | No | DC_7A_n8A | No 3CC exception | No test required |  | RAN5#96-e |
| DC_3A-7A_n28A | No | DC_3A_n28A | 7 (3 band IMD3) | IMD |  | RAN5#92-e |
|  | No | DC_7A_n28A | 3 (3 band IMD2) | IMD |  | RAN5#92-e |
| DC_3A-8A_n28A | No | DC_3A_n28A | No 3CC exception | No test required |  | RAN5#95-e |
|  | No | DC_8A_n28A |  |  |  |  |
| DC_3A-18A_n77A | Yes | DC_3A_n77A | No 3CC exception | No test required |  | RAN5#98 |
|  | No | DC_18A_n77A | 3 (3 band IMD3, PC2&PC3) | IMD |  | RAN5#105 |
| DC_3A-19A_n78A | Yes | DC_3A_n78A | No 3CC exception | No test required |  | RAN5#102 |
|  | No | DC_19A_n78A | 3 (3 band IMD3, PC2&PC3) | IMD |  | RAN5#102 |
| DC_3A-19A_n79A | No | DC_3A_n79A | 19 (3 band IMD3, PC2&PC3) | IMD |  | RAN5#101 |
|  | No | DC_19A_n79A | 3 (3 band IMD4, PC2&PC3) | IMD |  | RAN5#101 |
| DC_3A-20A_n1 | Yes | DC_3A_n1A | No 3CC exception | No test required |  |  |
|  | No | DC_20A_n1A |  |  |  |  |
| DC_3A-20A_n8A | No | DC_3A_n8A | No 3CC exception | No test required |  | RAN5#96-e |
|  | Yes (DC_20_n8) | DC_20A_n8A | 3 (3 band IMD4) | IMD |  | RAN5#96-e |
| DC_3A-20A_n28A | No | DC_3A_n28A | No 3CC exception | No test required |  | RAN5#92-e |
|  | No | DC_20A_n28A | 3 (3 band IMD4) | IMD |  | RAN5#92-e |
| DC_3A-20A_n78 | Yes | DC_3A_n78A | No 3CC exception |  |  |  |
|  | No | DC_20A_n78A | 3 (3 band IMD3) | IMD |  |  |
| DC_3A-21A_n78 | Yes | DC_3A_n78A | 21 (3 band IMD4, PC2&PC3) 21 (3 band IMD5, PC2&PC3) | IMD |  | RAN5#102 |
|  | No | DC_21A_n78A | 3 (3 band IMD2) | IMD |  | RAN5#101 |
| DC_3A-21A_n79 | No | DC_3A_n79A | No 3CC exception | No test required |  | RAN5#101 |
|  | No | DC_21A_n79A | 3 (3 band IMD3, PC2&PC3) | IMD |  | RAN5#102 |
| DC_3A-28A_n78A | Yes | DC_3A_n78A | No 3CC exception |  | DC_3A_n78A | RAN5#92-e |
|  | No | DC_28A_n78A | 3 (3 band IMD3) | IMD | DC_28A_n78A |  |
| DC_3A-40A_n1A | Yes | DC_3A_n1A | 40 (3 band IMD5) | IMD5 if UE supporting dual UL |  |  |
|  | No | DC_40A_n1A | No 3CC exception |  |  |  |
| DC_3A-41A_n28A | No | DC_3A_n28A | 41 (IMD2 ¦1*fB3+1*fn28¦), 41 (IMD3 ¦2*fB3-fn28¦) | IMD |  | RAN5#98 |
|  | No | DC_41A_n28A | 3 (IMD2 ¦1*fB41-1*fn28¦) | IMD |  | RAN5#98 |
| DC_3A-41A_n41A | No | DC_3A_n41A | No 3CC exception | No test required |  | RAN5#98 |
| DC_3A-41A_n77A | Yes | DC_3A_n77A | 41 (IMD5 ¦3*fB3-2*fB77¦, PC2&PC3) | IMD |  | RAN5#105 |
|  | No | DC_41A_n77A | 3 (IMD3 ¦2*fB41-1*fB77¦) | IMD |  | RAN5#105 |
| DC_3A-42A_n78A | Yes | DC_3A_n78A | No 3CC exception | NE |  | RAN5#103 |
| DC_3A-42A_n79A | No | DC_3A_n79A | No 3CC exception | NE |  | RAN5#103 |
| DC_7A-8A_n3A | No | DC_7A_n3A | 8 (3 band IMD3) | IMD |  | RAN5#95-e |
|  | No | DC_8A_n3A | 7 (3 band IMD2) 7 (3 band IMD3) | IMD |  | RAN5#95-e |
| DC_7A-20A_n1A | No | DC_20A_n1A | 20 (3 band IMD5) | IMD |  | RAN5#90-e |
|  | No | DC_7A_n1A | No 3CC exception | _ |  |  |
| DC_7A-20A_n3A | No | DC_7A_n3A | 20 (IMD2 ¦1*fB7-1*fB3) | NE, IMD |  |  |
|  | No | DC_20A_n3A | 7 (IMD2 ¦1*fB3+1*fB20) | NE, IMD |  |  |
| DC_7A-20A_n8A | No | DC_7A_n8A | 20 (3 band IMD3) | IMD |  | RAN5#95-e |
|  | Yes | DC_20A_n8A | 7 (3 band IMD3) | IMD |  | RAN5#95-e |
| DC_7A-20A_n28A | No | DC_7A_n28A | No 3CC exception | No test required |  | RAN5#92-e |
|  | No | DC_20A_n28A | 7 (3 band IMD5) | IMD |  |  |
| DC_7A-20A_n78A | No | DC_7A_n78A | 20 (3 band IMD2) 20 (3 band IMD5) | IMD |  |  |
|  | No | DC_20A_n78A | 7 (3 band IMD2) | IMD |  |  |
| DC_7A-28A_n3A | No | DC_7A_n3A | 28 (3 band IMD2) | IMD |  | RAN5#90-e |
|  | No | DC_28A_n3A | 7 (3 band IMD3) | IMD |  |  |
| DC_7A-28A_n5A | Yes | DC_7A_n5A | 28 (3 band IMD5) | IMD |  | RAN5#95-e |
|  | No | DC_28A_n5A | 7 (3 band IMD5) | IMD |  | RAN5#95-e |
| DC_7A-28A_n78A | No | DC_7A_n78A | 28 (3 band IMD2) 28 (3 band IMD5) | IMD |  | RAN5#92-e |
|  | No | DC_28A_n78A | 7 (3 band IMD2) | IMD | DC_28A_n78A |  |
| DC_13A_n2A-n77A | No | DC_13A_n2A | n77 (3 band IMD3) | NE, IMD |  | RAN5#103 |
|  | No | DC_13A_n77A | n2 (3 band IMD3, PC2&PC3) | NE, IMD |  |  |
| DC_13A-66A_n2A | No | DC_13A_n2A | 66 (3 band IMD4) | NE, IMD |  | RAN5#101 |
|  | No | DC_66A_n2A | No 3CC exception | NE |  |  |
| DC_13A-66A_n77A | No | DC_13A_n77A | 66 (3 band IMD3, PC2&PC3) | NE, IMD |  | RAN5#103 |
|  | No | DC_66A_n77A | No 3CC exception | NE |  |  |
| DC_18A-41A_n77A | No | DC_18A_n77A | No 3CC exception | No test required |  | RAN5#95-e |
|  | No | DC_41A_n77A | 18 (IMD5 ¦2*fB77-3*fB41¦) | IMD |  |  |
| DC_18A-41A_n78A | No | DC_18A_n78A | No 3CC exception | No test required |  | RAN5#95-e |
|  | No | DC_41A_n78A | 18 (IMD5 ¦2*fB78-3*fB41¦) | IMD |  |  |
| DC_19A-21A_n78A | No | DC_19A_n78A | No 3CC exception | No test required |  | RAN5#102 |
|  | No | DC_21A_n78A | 19 (3 band IMD3, PC2&PC3) 19 (3 band IMD4, PC2&PC3) | IMD |  | RAN5#102 |
| DC_19A-21A_n79A | No | DC_19A_n79A | 21 (3 band IMD5, PC2&PC3) | IMD |  | RAN5#102 |
|  | No | DC_21A_n79A | No 3CC exception | No test required |  | RAN5#101 |
| DC_19A-42A_n78A | No | DC_19A_n78A | No 3CC exception | NE |  | RAN5#103 |
| DC_19A-42A_n79A | No | DC_19A_n79A | No 3CC exception | NE |  | RAN5#103 |
| DC_21A-42A_n78A | No | DC_21A_n78A | No 3CC exception | NE |  | RAN5#103 |
| DC_21A-42A_n79A | No | DC_21A_n79A | No 3CC exception | NE |  | RAN5#103 |
| DC_66A_n2A-n77A | No | DC_66A_n2A | No 3CC exception | NE |  | RAN5#103 |
|  | No | DC_66A_n77A | n2 (3 band IMD2, PC2&PC3) | NE, IMD |  |  |
| DC_66A_n5A-n77A | No | DC_66A_n5A | n77 (3 band IMD3) | NE, IMD |  | RAN5#101 |
|  | No | DC_66A_n77A | No 3CC exception | NE |  |  |
| DC_(n)XAA-nYA |  |  |  |  |  |  |
| FFS |  |  |  |  |  |  |
| DC_XA_nXA_nYA |  |  |  |  |  |  |
| FFS |  |  |  |  |  |  |
| NOTE 1: If single UL is allowed in a DC_XA_nYA configuration the IMD requirements does not apply for UEs supporting UE capability singleUL-Transmission. NOTE 2: Notations used NE: Non-exception as defined in TS 38.101-3 [7], meaning standalone LTE in TS 36.101 [8] and NR in 38.101-1 [5] requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.1 HM: RX Harmonic Mixing, as defined in TS 38.101-3 [7], 7.3B.2.3.2 IMD: Intermodulation Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.5 CBI: Cross Band Isolation, as defined in TS 38.101-3 [7], 7.3B.2.3.4 |  |  |  |  |  |  |

-------- End of changes --------