3GPP TSG-RAN WG5 Meeting #108	R5-253844r1

Bengaluru, India, 25th – 29th Aug, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-2 | CR | 0196 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of SLPP default message contents and test condition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Support of sidelink positioning in NR systems is one of the objectibve of the work item Expanded and improved NR positioning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added SL-RTT, SL-AOA, SL-TDOA and SL-TOA simulated test environment - Added default RRC message contents for SL-PRS Configuration. - Added default SLPP PROVIDE ASSISTANCE DATA message contents - Added default SLPP REQUEST CAPABILITIES message contents |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The release 18 positioning work item will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1, 8.1A, 8.3, 8.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: - Corrected the city name on the cover sheet. - Corrected typos and format errors. - Added a new table for CommonIEsRequestLocationInformation. |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

8	Default Conditions for NR

8.1	LCS Sub-Test Cases

Some test cases defined in clause 9 may include several sub-test cases dependent on the positioning method(s) supported by the UE. Each sub-test case is identified by a sub-test case number as defined in Table 8.1-1. The applicable sub-tests for each test case are specified in the test procedure sequence clause of each test case. If no sub-tests are defined for a specific test case it means that this particular test case is not dependent on a specific positioning method and is applicable in all cases, independently of the positioning method(s) supported by the UE.

Table 8.1-1: Sub-Test Case Numbers for NR

| Sub-Test Case Number | Supported Positioning Methods |
| --- | --- |
| 1 | Void |
| 2 | Void |
| 3 | Void |
| 4 | Void |
| 5 | UE supporting OTDOA (LTE) |
| 6 | UE supporting ECID (LTE) |
| 7 | UE supporting GNSS(1) and OTDOA (LTE) |
| 8 | Void |
| 9 | Void |
| 10 | Void |
| 11 | UE supporting WLAN (Rel-13 only) |
| 12 | UE supporting MBS(2) (Rel-13 only) |
| 13 | UE supporting Bluetooth |
| 14 | UE supporting Sensor (Rel-13 only) |
| 15 | UE supporting GNSS(1) |
| 16 | UE supporting MBS(2) (Rel-14 onwards) |
| 17 | UE supporting WLAN (Rel-14 onwards) |
| 18 | UE supporting Sensor (Rel-14 onwards) |
| 19 | UE supporting Multi-RTT (Rel-16 onwards) |
| 20 | UE supporting DL-AoD (Rel-16 onwards) |
| 21 | UE supporting DL-TDOA (Rel-16 onwards) |
| 22 | UE supporting NR E-CID (Rel-16 onwards) |
| 23 | UE supporting MBS(2) (Rel-16 onwards) |
| 24 | UE supporting Sensor (Rel-16 onwards) |
| 25 | UE supporting GNSS(1) (Rel-15 onwards) |
| 26 | UE supporting Multi-RTT (Rel-17 onwards) |
| 27 | UE supporting DL-AoD (Rel-17 onwards) |
| 28 | UE supporting DL-TDOA (Rel-17 onwards) |
| 29 | UE supporting UL-TDOA (Rel-17 onwards) |
| 30 | UE supporting Multi-RTT (Rel-18 onwards) |
| 31 | UE supporting DL-AoD (Rel-18 onwards) |
| 32 | UE supporting DL-TDOA (Rel-18 onwards) |
| 33 | UE supporting SL-RTT (Rel-18 onwards) |
| 34 | UE supporting SL-AoA (Rel-18 onwards) |
| 35 | UE supporting SL-TDOA (Rel-18 onwards) |
| 36 | UE supporting SL-TOA (Rel-18 onwards) |
| NOTE 1: The GNSS combination of BDS, Galileo, GLONASS, GPS supported by the UE NOTE 2: Metropolitan Beacon System (MBS) is a specific type of Terrestrial Beacon System (TBS) [29] |  |

8.1A	Test Configurations

For ease of use of this document a number of Test Configurations corresponding to Network Deployment Types are defined in Table 8.1A-1.

Table 8.1A-1: Test Configuration

| Test Configuration | Network Deployment Type |
| --- | --- |
| A | EN-DC |
| B | NG-RAN NR |
| C | NE-DC |
| D | NG-RAN E-UTRA |
| E | NGEN-DC |

For ease of use of this document a number of sidelink positioning coverage scenarios are defined in Table 8.1A-2.

Table 8.1A-2: Sidelink positioning coverage scenarios

| Test Configuration | Sidelink Positioning Coverage Scenarios |
| --- | --- |
| A | In-coverage |
| B | Partial-coverage |
| C | out-of-coverage |

< Unchanged sections omitted >

8.2.8	Simulated general NR environment

For NR FR2, the connection between the SS and the DUT shall be OTA. The SS shall ensure that the NR cell is suitable throughout the test. The OTA link shall be sufficient to provide stable LPP/SLPP message transmissions between the SS and the DUT.

For NR FR1 and the other technologies used for the test cases in Chapter 9 (e.g. LTE, GNSS, WLAN…) the connection shall be conducted, following the settings described in this section.

< Unchanged sections omitted >

8.2.13	Simulated UL-TDOA environment

For UL-TDOA signalling test cases NR Cell 1 is used, as defined in 3GPP TS 38.508-1 [30].

The simulated general NR environment as specified in 8.2.8 also applies.

UL-TDOA test is only applicable for Test Configuration B (Table 8.1A-1). The NR Cell 1 frequency to be used for testing and other default conditions is as specified for signalling test cases in 3GPP TS 38.508-1 [30].

The simulated general NR environment as specified in 8.2.8 also applies.

NR E-CID tests are only applicable for Test Configuration B (Table 8.1A-1). The NR Cell 1 frequency to be used for testing and other default conditions is as specified for signalling test cases in 3GPP TS 38.508-1 [30].

8.2.14	Simulated SL-RTT environment

For SL-RTT signalling test cases NR Cell 1 is used for Test Configuration A (Table 8.1A-2) and Test Configuration B (Table 8.1A-2), as defined in 3GPP TS 38.508-1 [30]. The NR Cell 1 frequency to be used for testing and other default conditions are as specified for signalling test cases in 3GPP TS 38.508-1 [30].

In the test, there is one target UE receiving SL-PRS and performing SL Rx-TX time difference measurements and two anchor UEs NR-SS-UE1 and anchor NR-SS-UE2 (if the UE supports SL-Target UE-based SL-RTT). The synchronization source type is GNSS for all anchor UEs.

The NR-SS-UEs transmit SL-PRS according to the sl-PRS-TxInfo provided in the SLPP PROVIDE ASSISTANCE DATA as defined in subclause 8.5.

The simulated general NR environment as specified in 8.2.8 also applies.

8.2.15	Simulated SL-AoA environment

For SL-AoA signalling test cases NR Cell 1 is used for Test Configuration A (Table 8.1A-2) and Test Configuration B (Table 8.1A-2), as defined in 3GPP TS 38.508-1 [30]. The NR Cell 1 frequency to be used for testing and other default conditions are as specified for signalling test cases in 3GPP TS 38.508-1 [30].

In the test, there is one target UE receiving SL-PRS and performing SL-PRS-RSRP measurements and two anchor UEs NR-SS-UE1 and anchor NR-SS-UE2 (if the UE supports SL-Target UE-based SL-AoA). The synchronization source type is GNSS for all anchor UEs.

The NR-SS-UEs transmit SL-PRS according to the sl-PRS-TxInfo provided in the SLPP PROVIDE ASSISTANCE DATA as defined in subclause 8.5.

The simulated general NR environment as specified in 8.2.8 also applies.

8.2.16	Simulated SL-TDOA environment

For SL-TDOA signalling test cases NR Cell 1 is used for Test Configuration A (Table 8.1A-2) and Test Configuration B (Table 8.1A-2), as defined in 3GPP TS 38.508-1 [30]. The NR Cell 1 frequency to be used for testing and other default conditions are as specified for signalling test cases in 3GPP TS 38.508-1 [30].

In the test, there is one target UE receiving SL-PRS and performing SL Rx-Tx time difference measurements and three anchor UEs NR-SS-UE1, NR-SS-UE2 and anchor NR-SS-UE3 (if the UE supports SL-Target UE-based SL-TDOA). The synchronization source type is GNSS for all anchor UEs.

The NR-SS-UEs transmit SL-PRS according to the sl-PRS-TxInfo provided in the SLPP PROVIDE ASSISTANCE DATA as defined in subclause 8.5.

The simulated general NR environment as specified in 8.2.8 also applies.

8.2.17	Simulated SL-TOA environment

For SL-TOA signalling test cases NR Cell 1 is used for Test Configuration A (Table 8.1A-2) and Test Configuration B (Table 8.1A-2), as defined in 3GPP TS 38.508-1 [30]. The NR Cell 1 frequency to be used for testing and other default conditions are as specified for signalling test cases in 3GPP TS 38.508-1 [30].

In the test, there is one target UE receiving SL-PRS and performing SL-RTOA measurements and two anchor UEs NR-SS-UE1 and anchor NR-SS-UE2 (if the UE supports SL-Target UE-based SL-TOA). The synchronization source type is GNSS for all anchor UEs.

The NR-SS-UEs transmit SL-PRS according to the sl-PRS-TxInfo provided in the SLPP PROVIDE ASSISTANCE DATA as defined in subclause 8.5.

The simulated general NR environment as specified in 8.2.8 also applies.

< Unchanged sections omitted >

8.3	Default RRC and NAS message and information elements contents

The default values of common RRC and NAS messages and information elements are used as defined in 3GPP
TS 38.508-1 [30] with the following exceptions:

< Unchanged sections omitted >

Table 8.3.2-4: SRS-PosResource-r16

| Derivation Path: TS 38.331, clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-PosResource-r16::=SEQUENCE { |  |  |  |
| srs-PosResourceId-r16 | 0 |  |  |
| transmissionComb-r16 CHOICE { |  |  |  |
| n4-r16 SEQUENCE { |  |  |  |
| combOffset-n4-r16 | 0 |  |  |
| cyclicShift-n4-r16 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceMapping-r16 SEQUENCE { |  |  |  |
| startPosition-r16 | 1 |  |  |
| nrofSymbols-r16 | n2 |  |  |
| } |  |  |  |
| freqDomainShift-r16 | 0 |  |  |
| freqHopping-r16 SEQUENCE { |  |  |  |
| c-SRS-r16 | 0 |  |  |
| } |  |  |  |
| groupOrSequenceHopping-r16 | neither |  |  |
| resourceType-r16 CHOICE { |  |  |  |
| periodic-r16 SEQUENCE { |  |  |  |
| periodicityAndOffset-p-r16 CHOICE { |  |  |  |
| sl160 | 20 |  | FR1 |
|  | 23 |  | FR1 AND (HD_FDD OR TDD) AND SCS15Khz |
|  | 27 |  | FR1 AND TDD AND SCS30Khz |
| sl1280 | 163 |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sequenceId-r16 | 0 |  |  |
| spatialRelationInfoPos-r16 CHOICE { |  |  |  |
| servingRS-r16 SEQUENCE { |  |  |  |
| servingCellId | 0 |  |  |
| referenceSignal-r16 CHOICE { |  |  |  |
| ssb-IndexServing-r16 | 1 | Set according to Table 4.4.2-2 in TS 38.508-1. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| HD_FDD | pc_halfDuplexFDD_TypeA_RedCap_r17 (i.e HD_FDD UE are performing test on FDD band) |

8.3.3	RRC message contents for SL-PRS Configuration

Table 8.3.3-1: RRCReconfiguration

| Derivation Path: TS 38.508-1 [30], Table 4.6.1-13 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| sl-ConfigDedicatedNR-r16 CHOICE { |  |  |  |  |
| setup |  | SL-ConfigDedicatedNR | Table 8.3.3-2 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.3.3-2: SL-ConfigDedicatedNR

| Derivation Path: TS 38.508-1 [30], Table 4.6.6-7 with condition SCHEDULING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-ConfigDedicatedNR-r16 ::= SEQUENCE { |  |  |  |  |
| sl-PHY-MAC-RLC-Config-r16 SEQUENCE { |  |  |  |  |
| sl-FreqInfoToAddModList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfig-r16 { |  | 1 entry |  |  |
| SL-FreqConfig-r16[1] |  | SL-FreqConfig | Table 8.3.3-3 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.3.3-3: SL-FreqConfig

| Derivation Path: TS 38.508-1 [30], Table 4.6.6-10 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-FreqConfig-r16 ::= SEQUENCE { |  |  |  |  |
| sl-BWP-ToAddModList-r16 SEQUENCE (SIZE (1..maxNrofSL-BWPs-r16)) OF SL-BWP-Config-r16 { |  | 1 entry |  |  |
| SL-BWP-Config-r16[1] |  | SL-BWP-Config | Table 8.3.3-4 |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.3.3-4: SL-BWP-Config

| Derivation Path: TS 38.508-1 [30], Table 4.6.6-1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SL-BWP-Config-r16 ::= SEQUENCE { |  |  |  |  |
| sl-BWP-Id | BWP-Id | As defined in TS 38.508-1 [30] Table 4.6.3-12 |  |
| sl-BWP-Generic-r16 SEQUENCE { |  |  |  |
| sl-BWP-r16 | BWP | As defined in TS 38.508-1 [30] Table 4.6.3-8 |  |
| sl-LengthSymbols-r16 | sym14 |  |  |
| sl-StartSymbol-r16 | sym0 |  |  |
| sl-PSBCH-Config-r16 | Not present |  |  |
| sl-TxDirectCurrentLocation-r16 | Not present |  |  |
| } |  |  |  |
| sl-BWP-PoolConfig-r16 | SL-BWP-PoolConfig | Table 8.3.3-5 | pc_sl_PRS_RxInSharedResourcePool |
|  | Not present |  |  |
| sl-BWP-PoolConfigA2X-r18 | Not present |  |  |
| sl-BWP-PRS-PoolConfig-r18 | Not present |  |  |
| sl-BWP-PRS-PoolConfig-r18 CHOICE { |  |  |  |
| setup | SL-BWP-PRS-PoolConfig-r18 | As defined in TS 38.508-1 [30] Table 4.6.6-4A | pc_sl_PRS_RxInDedicatedResourcePool |
| } |  |  |  |
| } |  |  |  |  |

Table 8.3.3-5: SL-BWP-PoolConfig

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment |  | Condition |
| SL-BWP-PoolConfig-r16 ::= SEQUENCE { |  |  |  |  |
| sl-RxPool-r16 SEQUENCE (SIZE (1..maxNrofRXPool-r16)) OF SL-ResourcePool-r16 { | 1 entry |  |  |  |
| SL-ResourcePool-r16[1] | SL-ResourcePool | As defined in TS 38.508-1 [30] Table 4.6.6-25 with condition SLPP |  |  |
| } |  |  |  |  |
| sl-TxPoolSelectedNormal-r16 | Not present |  |  |  |
| sl-TxPoolScheduling-r16 SEQUENCE { |  |  |  |  |
| sl-PoolToReleaseList-r16 | Not present |  |  |  |
| sl-PoolToAddModList-r16 SEQUENCE (SIZE (1..maxNrofTXPool-r16)) OF SL-ResourcePoolConfig-r16 { | 1 entry |  |  |  |
| SL-ResourcePoolConfig-r16[1] SEQUENCE { |  | entry 1 |  |  |
| sl-ResourcePoolID-r16 | 1 |  |  |  |
| sl-ResourcePool-r16 | SL-ResourcePool | As defined in TS 38.508-1 [30] Table 4.6.6-25 with condition SLPP |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-TxPoolExceptional-r16 | Not present |  |  |  |
| } |  |  |  |  |

< Unchanged sections omitted >

8.5	Default SLPP message and information elements contents

This clause contains the default values of SLPP messages and information elements used, unless indicated otherwise in specific clauses of this specification.

-	SLPP REQUEST CAPABILITIES

Table 8.5-1: RequestCapabilities

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| applicationLayerID | Not present |  |  |
|  | '00 00 01 00'H | Application Layer ID in initiating UE side | UE-only mode |
| transactionID | (0..255) |  |  |
| endTransaction | FALSE |  |  |
| sequenceNumber | Not present |  |  |
| sessionID | Not present |  |  |
|  | Present. Any value acceptable. | OCTET STRING (SIZE (6)) | UE-only mode |
| acknowledgement | Not present |  |  |
| slpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestCapabilities SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| requestCapabilities SEQUENCE { |  |  |  |
| commonIEsRequestCapabilities | Present | OCTET STRING containing CommonIEsRequestCapabilities |  |
| commonSL-PRS-MethodsIEsRequestCapabilities | Present | OCTET STRING containing CommonSL-PRS-MethodsIEsRequestCapabilities |  |
| sl-AoA-RequestCapabilities | Present | OCTET STRING containing SL-AoA-RequestCapabilities |  |
| sl-RTT-RequestCapabilities | Present | OCTET STRING containing SL-RTT-RequestCapabilities |  |
| sl-TDOA-RequestCapabilities | Present | OCTET STRING containing SL-TDOA-RequestCapabilities |  |
| sl-TOA-RequestCapabilities | Present | OCTET STRING containing SL-TOA-RequestCapabilities |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UE-only mode | The SLPP signalling procedure is between sidelink UEs |

-	SLPP PROVIDE ASSISTANCE DATA

Table 8.5-2: ProvideAssistanceData

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| applicationLayerID | Not present |  |  |
|  | '00 00 01 00'H | Application Layer ID in initiating UE side | UE-only mode |
| transactionID | (0..255) |  |  |
| endTransaction | FALSE |  |  |
| sequenceNumber | Not present |  |  |
| sessionID | Not present |  |  |
|  | Present. Any value acceptable. | OCTET STRING (SIZE (6)) | UE-only mode |
| acknowledgement | Not present |  |  |
| slpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| provideAssistanceData SEQUENCE { |  |  |  |
| commonIEsProvideAssistanceData | Not present |  |  |
| commonSL-PRS-MethodsIEsProvide AssistanceData | Present | OCTET STRING containing CommonSL-PRS-MethodsIEsProvideAssistanceData as defined in Table 8.5.1.1-1 |  |
| sl-AoA-ProvideAssistanceData | Present | OCTET STRING containing SL-AoA-ProvideAssistanceData as defined in Table 8.5.1.2-1 | Sub-test 34 |
| sl-RTT-ProvideAssistanceData | Present | OCTET STRING containing SL-RTT-ProvideAssistanceData as defined in Table 8.5.1.3-1 | Sub-test 33 |
| sl-TDOA-ProvideAssistanceData | Present | OCTET STRING containing SL-TDOA-ProvideAssistanceData as defined in Table 8.5.1.4-1 | Sub-test 35 |
| sl-TOA-ProvideAssistanceData | Present | OCTET STRING containing SL-TOA-ProvideAssistanceData as defined in Table 8.5.1.5-1 | Sub-test 36 |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UE-only mode | The SLPP signalling procedure is between sidelink UEs |

-	SLPP REQUEST LOCATION INFORMATION

Table 8.5-3: RequestLocationInformation

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| applicationLayerID | Not present |  |  |
|  | '00 00 01 00'H | Application Layer ID in initiating UE side | UE-only mode |
| transactionID | (0..255) |  |  |
| endTransaction | FALSE |  |  |
| sequenceNumber | Not present |  |  |
| sessionID | Not present |  |  |
|  | Present. Any value acceptable. | OCTET STRING (SIZE (6)) | UE-only mode |
| acknowledgement | Not present |  |  |
| slpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| requestLocationInformation SEQUENCE { |  |  |  |
| commonIEsRequestLocationInformation | Present | OCTET STRING containing CommonIEsRequestLocationInformation as defined in Table 8.5-4 |  |
| commonSL-PRS-MethodsIEsRequestLocationInformation | Not present |  |  |
| sl-AoA-RequestLocationInformation | Present | OCTET STRING containing SL-AoA-RequestLocationInformation as defined in Table 8.5-5 | Sub-test 34 |
| sl-RTT-RequestLocationInformation | Present | OCTET STRING containing SL-RTT-RequestLocationInformation as defined in Table 8.5-6 | Sub-test 33 |
| sl-TDOA-RequestLocationInformation | Present | OCTET STRING containing SL-TDOA-RequestLocationInformation as defined in Table 8.5-7 | Sub-test 35 |
| sl-TOA-RequestLocationInformation | Present | OCTET STRING containing SL-TOA-RequestLocationInformation as defined in Table 8.5-8 | Sub-test 36 |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UE-only mode | The SLPP signalling procedure is between sidelink UEs |

-	CommonIEsRequestLocationInformation

Table 8.5-4: CommonIEsRequestLocationInformation

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CommonIEsRequestLocationInformation ::= SEQUENCE { |  |  |  |
| locationInformationType | locationMeasurementsPreferred |  |  |
| periodicalReporting | Not present |  |  |
| additionalInformation | onlyReturnInformationRequested |  |  |
| qos SEQUENCE { |  |  |  |
| horizontalAccuracy | Not present |  |  |
| verticalCoordinateRequest | FALSE |  |  |
| verticalAccuracy | Not present |  |  |
| rangeAccuracy | Not present |  |  |
| azimuthAccuracy | Not present |  |  |
| elevationAccuracy | Not present |  |  |
| responseTime SEQUENCE { |  |  |  |
| time | 128 |  |  |
| tenMilliSeconds | Not present |  |  |
| } |  |  |  |
| velocityRequest | Not present |  |  |
| } |  |  |  |
| environment | Not present |  |  |
| scheduledLocationTime | Not present |  |  |
| locationCoordinateTypes | Not present |  |  |
| velocityTypes | Not present |  |  |
| } |  |  |  |

-	SL-AoA REQUEST LOCATION INFORMATION

Table 8.5-5: SL-AoA-RequestLocationInformation

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-AoA-RequestLocationInformation ::= SEQUENCE { |  |  |  |
| sl-AoA-Request | aoa |  |  |
| measurementReportingType | Not present |  |  |
| measurementsForMultipleARP-IDs-Rx | Not present |  |  |
| sl-ARP-InfoRequest | Not present |  |  |
| sl-LOS-NLOS-IndicatorRequest | Not present |  |  |
| sl-PRS-RSRP-Request | Not present |  |  |
| sl-RSRPP-Request | Not present |  |  |
| sl-AdditionalPathsRequest | Not present |  |  |
| } |  |  |  |

-	SL-RTT REQUEST LOCATION INFORMATION

Table 8.5-6: SL-RTT-RequestLocationInformation

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-RTT-RequestLocationInformation ::= SEQUENCE { |  |  |  |
| sl-ARP-InfoRequest | Not present |  |  |
| sl-LOS-NLOS-IndicatorRequest | Not present |  |  |
| sl-PRS-RSRP-Request | Not present |  |  |
| sl-RSRPP-Request | Not present |  |  |
| sl-AdditionalPathsRequest | Not present |  |  |
| dummy | Not present |  |  |
| multipleSL-PRS-RxTxTimeDiffRequest | Not present |  |  |
| measurementsForMultipleARP-IDs-Rx | Not present |  |  |
| associatedSL-PRS-TxTimeStampRequest | Not present |  |  |
| } |  |  |  |

-	SL-TDOA REQUEST LOCATION INFORMATION

Table 8.5-7: SL-TDOA-RequestLocationInformation

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-RTT-RequestLocationInformation ::= SEQUENCE { |  |  |  |
| sl-ARP-InfoRequest | Not present |  |  |
| sl-LOS-NLOS-IndicatorRequest | Not present |  |  |
| sl-PRS-RSRP-Request | Not present |  |  |
| sl-RSRPP-Request | Not present |  |  |
| sl-AdditionalPathsRequest | Not present |  |  |
| measurementsForMultipleARP-IDs-Rx | Not present |  |  |
| maxNumberOfMeasurementsPerARP | Not present |  |  |
| } |  |  |  |

-	SL-TOA REQUEST LOCATION INFORMATION

Table 8.5-8: SL-TOA-RequestLocationInformation

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-RTT-RequestLocationInformation ::= SEQUENCE { |  |  |  |
| sl-ARP-InfoRequest | Not present |  |  |
| sl-LOS-NLOS-IndicatorRequest | Not present |  |  |
| sl-PRS-RSRP-Request | Not present |  |  |
| sl-RSRPP-Request | Not present |  |  |
| sl-AdditionalPathsRequest | Not present |  |  |
| measurementsForMultipleARP-IDs-Rx | Not present |  |  |
| maxNumberOfMeasurementsPerARP | Not present |  |  |
| } |  |  |  |

8.5.1	Default assistance data information elements

8.5.1.1	Common SL-PRS Assistance Data Elements

This clause defines the common SL-PRS assistance data elements which shall be provided to the UE in the tests in SLPP Provide Assistance Data messages.

Table 8.5.1.1-1: CommonSL-PRS-MethodsIEsProvideAssistanceData

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CommonSL-PRS-MethodsIEsProvideAssistance Data ::= SEQUENCE { |  |  |  |
| sl-PRS-AssistanceDataInfo SEQUENCE (SIZE (1..maxNrOfUEs)) OF SL-PRS-AssistanceData { | 3 entries |  |  |
| SL-PRS-AssistanceData[1] SEQUENCE { |  | entry 1 |  |
| applicationLayerID | '00 00 01 00'H | NR-SS-UE1 |  |
| sl-PRS-SequenceID | 0 |  |  |
| anchorUE-LocationInformation | AnchorLocationCoordinates for NR-SS-UE1 | As defined in TS 37.571-5 [12] clause 13.1. |  |
| arp-LocationInfo SEQUENCE { |  |  |  |
| referencePoint | Not present |  |  |
| arp-LocationInfoList SEQUENCE (SIZE (1..4)) OF ARP-LocationInfoElement { | 1 entry |  |  |
| ARP-LocationInfoElement[1] SEQUENCE { |  | entry 1 |  |
| sl-PRS-ARP-ID | 1 |  |  |
| arp-LocationInfo | RelativeLocation | As defined in TS 37.571-5 [12] clause 13.2. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-PRS-TxInfo SEQUENCE { |  |  |  |
| sl-PRS-Priority | 1 |  |  |
| sl-PRS-DelayBudget | 1000 |  |  |
| sl-PRS-Bandwidth | mhz5 |  |  |
| sl-PRS-Periodicity | Not present |  |  |
| sl-PRS-Transmission | true |  |  |
| } |  |  |  |
| sl-POS-ARP-ID-Tx | Not present |  |  |
| } |  |  |  |
| SL-PRS-AssistanceData [2] SEQUENCE { |  | entry 2 | In case of sub-test 33 pc_Target_UEB_SL_RTT, sub-test 34 pc_Target_UEB_SL_AoA, sub-test 35 pc_Target_UEB_SL_TDOA or sub-test 36 pc_Target_UEB_SL_TOA supported by the UE |
| applicationLayerID | '00 00 03 00'H | NR-SS-UE2 |  |
| sl-PRS-SequenceID | 1 |  |  |
| anchorUE-LocationInformation | AnchorLocationCoordinates for NR-SS-UE2 | As defined in TS 37.571-5 [12] clause 13.1. |  |
| arp-LocationInfo SEQUENCE { |  |  |  |
| referencePoint | Not present |  |  |
| arp-LocationInfoList arp-LocationInfoList SEQUENCE (SIZE (1..4)) OF ARP-LocationInfoElement { | 1 entry |  |  |
| ARP-LocationInfoElement[1] SEQUENCE { |  | entry 1 |  |
| sl-PRS-ARP-ID | 1 |  |  |
| arp-LocationInfo | RelativeLocation | As defined in TS 37.571-5 [12] clause 13.2. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-PRS-TxInfo SEQUENCE { |  |  |  |
| sl-PRS-Priority | 1 |  |  |
| sl-PRS-DelayBudget | 1000 |  |  |
| sl-PRS-Bandwidth | mhz5 |  |  |
| sl-PRS-Periodicity | Not present |  |  |
| sl-PRS-Transmission | true |  |  |
| } |  |  |  |
| sl-POS-ARP-ID-Tx | Not present |  |  |
| } |  |  |  |
| SL-PRS-AssistanceData [3] SEQUENCE { |  | entry 3 | In case of sub-test 35 pc_Target_UEB_SL_TDOA supported by the UE |
| applicationLayerID | '00 00 04 00'H | NR-SS-UE3 |  |
| sl-PRS-SequenceID | 2 |  |  |
| anchorUE-LocationInformation | AnchorLocationCoordinates for NR-SS-UE3 | As defined in TS 37.571-5 [12] clause 13.1. |  |
| arp-LocationInfo SEQUENCE { |  |  |  |
| referencePoint | Not present |  |  |
| arp-LocationInfoList arp-LocationInfoList SEQUENCE (SIZE (1..4)) OF ARP-LocationInfoElement { | 1 entry |  |  |
| ARP-LocationInfoElement[1] SEQUENCE { |  | entry 1 |  |
| sl-PRS-ARP-ID | 1 |  |  |
| arp-LocationInfo | RelativeLocation | As defined in TS 37.571-5 [12] clause 13.2. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-PRS-TxInfo SEQUENCE { |  |  |  |
| sl-PRS-Priority | 1 |  |  |
| sl-PRS-DelayBudget | 1000 |  |  |
| sl-PRS-Bandwidth | mhz5 |  |  |
| sl-PRS-Periodicity | Not present |  |  |
| sl-PRS-Transmission | true |  |  |
| } |  |  |  |
| sl-POS-ARP-ID-Tx | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-PRS-Error | Not present |  |  |
| } |  |  |  |

8.5.1.2	SL-AoA Assistance Data Elements

This clause defines the SL-AoA assistance data elements which shall be provided to the UE in the tests in SLPP Provide Assistance Data messages.

Table 8.5.1.2-1: sl-AoA-ProvideAssistanceData

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-AoA-ProvideAssistanceData ::= SEQUENCE { |  |  |  |
| sl-AoA-AssistanceDataInfo | Not present |  |  |
| sl-AoA-Error | Not present |  |  |
| } |  |  |  |

8.5.1.3	SL-RTT Assistance Data Elements

This clause defines the SL-RTT assistance data elements which shall be provided to the UE in the tests in SLPP Provide Assistance Data messages.

Table 8.5.1.3-1: SL-RTT-ProvideAssistanceData

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-RTT-ProvideAssistanceData ::= SEQUENCE { |  |  |  |
| } |  |  |  |

8.5.1.4	SL-TDOA Assistance Data Elements

This clause defines the SL-TDOA assistance data elements which shall be provided to the UE in the tests in SLPP Provide Assistance Data messages.

Table 8.5.1.4-1: SL-TDOA-ProvideAssistanceData

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-TDOA-ProvideAssistanceData ::= SEQUENCE { |  |  |  |
| sl-RTD-Info SEQUENCE ( SIZE (1.. maxNrOfUEs)) OF RTD-InfoListPerAnchorUE { | 3 entries |  |  |
| RTD-InfoListPerAnchorUE[1] SEQUENCE { |  | entry 1 |  |
| applicationLayerID | '00 00 01 00'H | NR-SS-UE1 |  |
| referenceRTD-Info SEQUENCE { |  |  |  |
| syncSourceType | gnss |  |  |
| applicationLayerID | Not present |  |  |
| nrCell-Identify | Not present |  |  |
| } |  |  |  |
| rtd-BetweenAnchorUEs CHOICE { |  |  |  |
| subframeOffset | 0 |  |  |
| } |  |  |  |
| rtd-Quality | Not present |  |  |
| syncSourceType | gnss |  |  |
| } |  |  |  |
| RTD-InfoListPerAnchorUE[2] SEQUENCE { |  | entry 2 | In case of sub-test 35 pc_Target_UEB_SL_TDOA supported by the UE |
| applicationLayerID | '00 00 03 00'H | NR-SS-UE2 |  |
| referenceRTD-Info SEQUENCE { |  |  |  |
| syncSourceType | gnss |  |  |
| applicationLayerID | Not present |  |  |
| nrCell-Identify | Not present |  |  |
| } |  |  |  |
| rtd-BetweenAnchorUEs CHOICE { |  |  |  |
| subframeOffset | 0 |  |  |
| } |  |  |  |
| rtd-Quality | Not present |  |  |
| syncSourceType | gnss |  |  |
| } |  |  |  |
| RTD-InfoListPerAnchorUE[3] SEQUENCE { |  | entry 3 | In case of sub-test 35 pc_Target_UEB_SL_TDOA supported by the UE |
| applicationLayerID | '00 00 04 00'H | NR-SS-UE3 |  |
| referenceRTD-Info |  |  |  |
| referenceRTD-Info SEQUENCE { |  |  |  |
| syncSourceType | gnss |  |  |
| applicationLayerID | Not present |  |  |
| nrCell-Identify | Not present |  |  |
| } |  |  |  |
| rtd-BetweenAnchorUEs CHOICE { |  |  |  |
| subframeOffset | 0 |  |  |
| } |  |  |  |
| rtd-Quality | Not present |  |  |
| syncSourceType | gnss |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-TDOA-Error | Not present |  |  |
| } |  |  |  |

8.5.1.5	SL-TOA Assistance Data Elements

This clause defines the SL-TOA assistance data elements which shall be provided to the UE in the tests in SLPP Provide Assistance Data messages.

Table 8.5.1.5-1: SL-TOA-ProvideAssistanceData

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-TOA-ProvideAssistanceData ::= SEQUENCE { |  |  |  |
| sl-RTD-Info SEQUENCE ( SIZE (1.. maxNrOfUEs)) OF RTD-InfoListPerAnchorUE { | 2 entries |  |  |
| RTD-InfoListPerAnchorUE[1] SEQUENCE { |  | entry 1 |  |
| applicationLayerID | '00 00 01 00'H | NR-SS-UE1 |  |
| referenceRTD-Info SEQUENCE { |  |  |  |
| syncSourceType | gnss |  |  |
| applicationLayerID | Not present |  |  |
| nrCell-Identify | Not present |  |  |
| } |  |  |  |
| rtd-BetweenAnchorUEs CHOICE { |  |  |  |
| subframeOffset | 0 |  |  |
| } |  |  |  |
| rtd-Quality | Not present |  |  |
| syncSourceType | gnss |  |  |
| } |  |  |  |
| RTD-InfoListPerAnchorUE[2] SEQUENCE { |  | entry 2 | In case of sub-test 36 pc_Target_UEB_SL_TOA supported by the UE |
| applicationLayerID | '00 00 03 00'H | NR-SS-UE2 |  |
| referenceRTD-Info SEQUENCE { |  |  |  |
| syncSourceType | gnss |  |  |
| applicationLayerID | Not present |  |  |
| nrCell-Identify | Not present |  |  |
| } |  |  |  |
| rtd-BetweenAnchorUEs CHOICE { |  |  |  |
| subframeOffset | 0 |  |  |
| } |  |  |  |
| rtd-Quality | Not present |  |  |
| syncSourceType | gnss |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-TOA-Error | Not present |  |  |
| } |  |  |  |