3GPP TSG-RAN5 Meeting #108	 R5-253937r1

# Bengaluru, India, 25th – 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3418 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates of Reference Sensitivity requirements for PC2 n5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | HPUE_NR_FR1_bands_R19-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Updates of Reference Sensitivity requirements for PC2 n5 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the following tables for Reference Sensitivity requirements for PC2 n2 and PC2 n5: - Table 7.3.2.3-1c: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity - Table 7.3.2.3-1d Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity - Table 7.3.2.5-2c: Reference Sensitivity for PC2 UE on FDD bands for UE not supporting Tx Diversity - Table 7.3.2.5-2d: Reference Sensitivity for PC2 UE on FDD bands for UE supporting Tx Diversity |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The correspomding Reference Sensitivity information for PC2 part of the bands are not specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st Revision: Fixed 3GU issue on cover page |  |  |  |  |  |  |  |  |

<Start of changes>

### 7.3.2	Reference sensitivity power level

<skipped unchanged sections>

Table 7.3.2.3-1c: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  | 0 | 0 | 0 | 0 | 0 | 0 | - | 0 | 0 | 0 |
| n3 |  | 0.5 | 0.5 | 0.5 | 0.5 | 0.6 | 0.8 | 1.1 | 1.5 | 2.3 | 2.8 |
| n5 | 0.5 | 0.5 | 0.7 | 0.8 | 1.5 | 2.6 | 2.8 |  |  |  |  |
| n7 |  | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | - | 0.5 | - | 2.0 |
| n8 |  | 0.5 | 0.7 | 0.8 | 2.3 | 2.8 | 3.2 | 3.1 |  |  |  |
| n14 |  | 0.6 | 0.8 |  |  |  |  |  |  |  |  |
| n28 | 0.6 | 0.6 | 0.7 | 0.8 | 1.3 | 2.4 | 2.9 |  | 3 |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2.3-1d Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  | 0 | 0 | 0 | 0 | 0 | 0 | - | 0 | 0 | 0 |
| n3 |  | 1.4 | 1.5 | 1.5 | 1.5 | 1.6 | 1.7 | 2.8 | 5 | 5.5 | 6.0 |
| n5 | 1.1 | 1.1 | 1.2 | 1.4 | 4.5 | 6.1 | 6.5 |  |  |  |  |
| n7 |  | 0.9 | 1.0 | 1.0 | 1.0 | 1.1 | 1.1 | - | 1.1 | - | 5.3 |
| n8 |  | 1.3 | 1.4 | 2.1 | 5.8 | 6.1 | 6.5 | 7.0 |  |  |  |
| n14 |  | 1.1 | 1.3 |  |  |  |  |  |  |  |  |
| n28 | 1.1 | 1.1 | 1.1 | 1.3 | 3.0 | 6.6 | 7.9 |  | 8.2 |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2G.4 |  |  |  |  |  |  |  |  |  |  |  |

<skipped unchanged sections>

Table 7.3.2.5-2c: Reference Sensitivity for PC2 UE on FDD bands for UE not supporting Tx Diversity

| Operating Band | 3 MHz (dB) | 5 MHz (dB) |  | 10 MHz (dB) |  | 15 MHz (dB) |  | 20 MHz (dB) |  | 25 MHz (dB) |  | 30 MHz (dB) |  | 35 MHz (dB) |  | 40 MHz (dB) |  | 45 MHz (dB) |  | 50 MHz (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  | REFSENS |  | REFSENS |  | REFSENS |  | REFSENS |  | REFSENS |  | REFSENS |  | - |  | REFSENS |  | REFSENS |  | REFSENS |  |
| n3 |  | REFSENS+0.5 |  | REFSENS +0.5 |  | REFSENS +0.5 |  | REFSENS +0.5 |  | REFSENS +0.6 |  | REFSENS +0.8 |  | REFSENS +1.1 |  | REFSENS +1.5 |  | REFSENS +2.3 |  | REFSENS +2.8 |  |
| n5 | REFSENS+0.5 |  | REFSENS+0.5 |  | REFSENS+0.7 |  | REFSENS+0.8 |  | REFSENS+1.5 |  | REFSENS+2.6 |  | REFSENS+2.8 |  | REFSENS |  | REFSENS |  | REFSENS |  | REFSENS |  |
| n7 |  | REFSENS+0.5 |  | REFSENS+0.5 |  | REFSENS+0.5 |  | REFSENS+0.5 |  | REFSENS+0.5 |  | REFSENS+0.5 |  | - |  | REFSENS +0.5 |  | - |  | REFSENS +2.0 |  |
| n8 |  | REFSENS+0.5 |  | REFSENS+0.7 |  | REFSENS+0.8 |  | REFSENS+2.3 |  | REFSENS+2.8 |  | REFSENS+3.2 |  | REFSENS+3.1 |  | - |  | - |  | - |  |
| n14 |  | REFSENS +0.6 |  | REFSENS +0.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n28 | REFSENS +0.6 | REFSENS +0.6 |  | REFSENS +0.7 |  | REFSENS +0.8 |  | REFSENS +1.3 |  | REFSENS +2.4 |  | REFSENS +2.9 |  |  |  | REFSENS +3.0 |  |  |  |  |  |
|  | NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 2: void. NOTE 3: void. NOTE 4: void. NOTE 5: void. NOTE 6: The symbol “REFSENS” refers to the Reference Sensitivity values in Table 7.3.2.5-1a for two antenna ports and in Table 7.3.2.5-2a for four antenna ports. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2.5-2d: Reference Sensitivity for PC2 UE on FDD bands for UE supporting Tx Diversity

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  | REFSENS | REFSENS | REFSENS | REFSENS | REFSENS | REFSENS | - | REFSENS | REFSENS | REFSENS |
| n3 |  | REFSENS +1.4 | REFSENS +1.5 | REFSENS +1.5 | REFSENS +1.5 | REFSENS +1.6 | REFSENS +1.7 | REFSENS +2.8 | REFSENS +5 | REFSENS +5.5 | REFSENS +6.0 |
| n5 | REFSENS+1.1 | REFSENS+1.1 | REFSENS+1.2 | REFSENS+1.4 | REFSENS+4.5 | REFSENS+6.1 | REFSENS+6.5 | REFSENS | REFSENS | REFSENS | REFSENS |
| n7 |  | REFSENS +0.9 | REFSENS +1.0 | REFSENS+1.0 | REFSENS +1.0 | REFSENS+1.1 | REFSENS +1.1 | - | REFSENS +1.1 | - | REFSENS +5.3 |
| n8 |  | REFSENS +1.3 | REFSENS +1.4 | REFSENS+2.1 | REFSENS +5.8 | REFSENS+6.1 | REFSENS +6.5 | REFSENS +7.0 |  |  |  |
| n14 |  | REFSENS +1.1 | REFSENS +1.3 |  |  |  |  |  |  |  |  |
| n28 | REFSENS +1.1 | REFSENS +1.1 | REFSENS +1.1 | REFSENS +1.3 | REFSENS +3.0 | REFSENS +6.6 | REFSENS +7.9 |  | REFSENS +8.2 |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2G.4 NOTE 2: void. NOTE 3: void. NOTE 4: void. NOTE 5: The symbol “REFSENS” refers to the Reference Sensitivity values in Table 7.3.2.5-1a for two antenna ports and in Table 7.3.2.5-2a for four antenna ports. |  |  |  |  |  |  |  |  |  |  |  |

<End of changes>