3GPP TSG-RAN5 Meeting #108	R5-254580r2

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5124 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test case for Search Space Set Group Switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As discussed in R5-254544, new test case is needed to test SSSG switching – Rel-17 power saving enhancement feature. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new SSSG test case 7.1.1.12.5 – "PDCCH monitoring adaptation / SSSG Switching" |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Coverage doesnot exist for SSSG switching. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.12.5 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0624 TS/TR 38.508-2 CR 0862 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-254580r1: 1. Updated TP5, main behavior and specific message content table. 2. Search space IDs updated from 2,3,4 to 2,4,5. R5-254580r2: 1. Updated TP5, step numbering, main behavior and specific message content tables. |  |  |  |  |  |  |  |  |

<Start of modified section>

##### 7.1.1.12.5	PDCCH monitoring adaptation / Search Space Set Group Switching

7.1.1.12.5.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives searchSpaceGroupIdList-r17 in pdcch-Config in a RRC message }

then { UE starts monitoring PDCCH according to search space sets with group index 0 }

}

(2)

with { UE in RRC_CONNECTED state and not configured with searchSpaceSwitchDelay-r17 }

ensure that {

when { UE detects DCI format 1_1 with PDCCH monitoring adaptation field set to search space group ID 1 }

then { UE starts monitoring PDCCH as per search space sets with group index 1 after Pswitch symbols delay as configured per scs }

}

(3)

with { UE in RRC_CONNECTED state and configured with searchSpaceSwitchDelay-r17 }

ensure that {

when { UE detects DCI format 1_1 with PDCCH monitoring adaptation field set to search space set group ID 1 }

then { UE starts monitoring PDCCH as per search space sets with group index 1 after searchSpaceSwitchDelay-r17 symbols delay from the last symbol of PDCCH carrying DCI 1_1 }

}

(4)

with { UE supporting sssg-Switching-2BitInd-r17 and configured with searchSpaceSwitchDelay-r17 in RRC_CONNECTED state }

ensure that {

when { UE detects DCI format 1_1 with 2-bit PDCCH monitoring adaptation field set to search space set group index 2 }

then { UE starts monitoring PDCCH as per search space sets with group index 2 after searchSpaceSwitchDelay-r17 symbols delay from the last symbol of PDCCH carrying DCI 1_1 }

}

(5)

with { UE in RRC_CONNECTED and searchSpaceSwitchTimer-r17 is running and monitoring group index 1 }

ensure that {

when { searchSpaceSwitchTimer-r17 expires }

then { the UE continues monitoring PDCCH as per search space sets with group index 1 for Pswitch symbols }

}

(6)

with { UE in RRC_CONNECTED }

ensure that {

when { searchSpaceGroupIdList-r17 is removed in RRC message }

then { the UE starts monitoring PDCCH in all configured search space sets }

}

7.1.1.12.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.212, clause 7.3.1.2.2 and TS 38.213, clause 10.4. Unless otherwise stated these are Rel-17 requirements.

[TS 38.212, clause 7.3.1.2.2]

PDCCH monitoring adaptation indication – 0, 1 or 2 bits

…

-	1 or 2 bits, if pdcch-SkippingDurationList is not configured and if searchSpaceGroupIdList-r17 is configured

-	1 bit if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0 and search space set(s) with group index 1, and if the UE is not configured by searchSpaceGroupIdList-r17 with any search space set with group index 2;

-	2 bits if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0, search space set(s) with group index 1 and search space set(s) with group index 2;

-	2 bits, if pdcch-SkippingDurationList is configured and if searchSpaceGroupIdList-r17 is configured

-	0 bit, otherwise

[TS 38.213, clause 10.4]

A UE can be provided

-	a group index for a respective Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList for PDCCH monitoring on a serving cell,

-	a group index for a respective Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList-r17 for PDCCH monitoring on an active DL BWP of a serving cell.

If the UE is not provided searchSpaceGroupIdList or searchSpaceGroupIdList-r17 for a search space set, the following procedures that are based on search space set group switching are not applicable for PDCCH monitoring according to the search space set.

…

When a UE is provided searchSpaceGroupIdList or searchSpaceGroupIdList-r17, the UE resets PDCCH monitoring according to search space sets with group index 0, if provided by searchSpaceGroupIdList or searchSpaceGroupIdList-r17.

A UE can be provided by searchSpaceSwitchDelay or searchSpaceSwitchDelay-r17 a number of symbols Pswitch where a minimum value of Pswitch is provided in Table 10.4-1 for UE processing capability 1 and UE processing capability 2 and SCS configuration μ. UE processing capability 1 for SCS configuration μ applies unless the UE indicates support for UE processing capability 2.

Table 10.4-1: Minimum value of Pswitch [symbols]

| μ | Minimum Pswitch value for UE processing capability 1 [symbols] | Minimum Pswitch value for UE processing capability 2 [symbols] |
| --- | --- | --- |
| 0 | 25 | 10 |
| 1 | 25 | 12 |
| 2 | 25 | 22 |
| 3 | 40 | - |
| 5 | 160 | - |
| 6 | 320 | - |

A UE can be provided, by searchSpaceSwitchTimer, a timer value for a serving cell that the UE is provided searchSpaceGroupIdList or, if provided, for a set of serving cells provided by cellGroupsForSwitchList. The UE decrements the timer value by one after each slot based on a reference SCS configuration that is the smallest SCS configuration μ among all configured DL BWPs in the serving cell, or in the set of serving cells. The UE maintains the reference SCS configuration during the timer decrement procedure.

…

A UE can be provided group indexes for a Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList-r17 for PDCCH monitoring on an active DL BWP of a serving cell and, if the UE is not provided pdcch-SkippingDurationList for the active DL BWP of the serving cell, a DCI format 0_1 and a DCI format 0_2 that schedule PUSCH transmissions and a DCI format 1_1 and a DCI format 1_2 that schedule PDSCH receptions can include a PDCCH monitoring adaptation field of 1 bit or of 2 bits for the serving cell.

If the field has 1 bit and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on the active DL BWP of the serving cell

-	a '0' value for the bit indicates start of PDCCH monitoring according to search space sets with group index 0 and stop of PDCCH monitoring according to search space sets with other group indexes, if any

-	a '1' value for the bit indicates start of PDCCH monitoring according to search space sets with group index 1 and stop of PDCCH monitoring according to search space sets with other group indexes, if any, and the UE sets the timer value to the one provided by searchSpaceSwitchTimer-r17, if provided

If the field has 2 bits and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on the active DL BWP of the serving cell

-	a '00' value for the bit indicates start of PDCCH monitoring according to search space sets with group index 0 and stop of PDCCH monitoring according to search space sets with other group indexes, if any

-	a '01' value for the bit indicates start of PDCCH monitoring according to search space sets with group index 1 and stop of PDCCH monitoring according to search space sets with other group indexes, if any, and the UE sets the timer value to the one provided by searchSpaceSwitchTimer-r17, if provided

-	a '10' value for the bit indicates start of PDCCH monitoring according to search space sets with group index 2 and stop of PDCCH monitoring according to search space sets with other group indexes, if any, and the UE sets the timer value to the one provided by searchSpaceSwitchTimer-r17, if provided

-	a '11' value is reserved

A UE can be provided a set of durations by pdcch-SkippingDurationList and group indexes for a Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList-r17 for PDCCH monitoring on an active DL BWP of a serving cell and, a DCI format 0_1 and a DCI format 0_2 that schedule PUSCH transmissions, and a DCI format 1_1 and a DCI format 1_2 that schedule PDSCH receptions can include a PDCCH monitoring adaptation field of 2 bits.

If the set of durations includes one value and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on the active DL BWP of the serving cell

-	a '00' value for the bits indicates start of PDCCH monitoring according to search space sets with group index 0 and stop of PDCCH monitoring according to search space sets with group index 1, if any

-	a '01' value for the bits indicates start of PDCCH monitoring according to search space sets with group index 1 and stop of PDCCH monitoring according to search space sets with group index 0, if any, and the UE sets the timer value to the one provided by searchSpaceSwitchTimer-r17, if provided

-	a '10' value for the bits indicates skipping PDCCH monitoring for a duration provided by the value in the set of durations

-	a '11' value is reserved

If the set of durations includes two values and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on active DL BWP of the serving cell

-	a '00' value for the bits indicates start of PDCCH monitoring according to search space sets with group index 0 and stop of PDCCH monitoring according to search space sets with group index 1, if any

-	a '01' value for the bits indicates start of PDCCH monitoring according to search space sets with group index 1 and stop of PDCCH monitoring according to search space sets with group index 0, if any, and the UE sets the timer value to the one provided by searchSpaceSwitchTimer-r17, if provided

-	a '10' value for the bits indicates skipping PDCCH monitoring for a duration provided by the first value in the set of durations

-	a '11' value for the bits indicates skipping PDCCH monitoring for a duration provided by the second value in the set of durations

When the PDCCH monitoring adaptation field indicates to a UE to start PDCCH monitoring according to search space sets with a first group index and stop PDCCH monitoring according to search space sets with a second group index, the UE applies the indication

-	at the beginning of a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field when μ∈0, 1, 2, 3,

-	at the beginning of a first slot, of a slot group of Xs slots, that is at least Pswitch symbols after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field when μ∈5, 6

…

If the UE changes to a new active DL BWP of the serving cell by the expiration of bwp-InactivityTimer, or by RA procedure requested by higher layers [11, TS 38.321], or by RRC configuration, the UE

-	resumes PDCCH monitoring according to the search space sets on the new active BWP of the serving cell when UE is in a PDCCH skipping duration, if the UE is not provided searchSpaceGroupIdList-r17 on the new active DL BWP

-	monitors PDCCH according to search space sets with group index 0 on the new active BWP of the serving cell, if the UE is provided searchSpaceGroupIdList-r17.

If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring an active DL BWP of on a serving cell and the timer is running, the UE

-	resets the timer after a slot of the active DL BWP of the serving cell if the UE detects a DCI format in a PDCCH reception in the slot for with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI/G-RNTI for multicast/G-CS-RNTI

-	otherwise, decrements the timer value by one after a slot of the active DL BWP of the serving cell

When the timer expires in a first slot, the UE monitors PDCCH on the serving cell according to search space sets with group index 0 starting in a second slot that

-	is not earlier than Pswitch symbols after the first slot when μ∈0, 1, 2, 3,

-	is a first slot in a slot group of Xs slots that is not earlier than Pswitch symbols after the first slot when μ∈5, 6,

-	is not earlier than a slot where a PDCCH skipping duration expires, if applicable

When a UE receives

-	a first PDCCH in a first slot that provides a DCI format with a PDCCH monitoring adaptation field having a first value indicating skipping PDCCH monitoring, or indicating start of PDCCH monitoring according to a search space sets with a first group index and stop of PDCCH monitoring according to search space sets with a second group index, for an active DL BWP and

-	a second PDCCH that provides a DCI format with a PDCCH monitoring adaptation field having a second value indicating skipping PDCCH monitoring, or indicating start of PDCCH monitoring according to search space sets with a first group index and stop of PDCCH monitoring according to search space sets with a second group index different than the first group index, for the active DL BWP where the second PDCCH is received

-	in the first slot if the first value indicates skipping PDCCH monitoring

-	before a slot that is at least Pswitch symbols after the first slot if the first value indicates start of PDCCH monitoring according to search space sets with a first group index

the UE does not expect the second value to be different than the first value.

A UE does not expect to receive in a second slot a PDCCH on an active DL BWP that provides a DCI format indicating skipping PDCCH monitoring, or start of PDCCH monitoring according to search space sets with group index 1 or 2 for the active DL BWP, if the second slot is not at least Pswitch symbols after a first slot where the timer expires.

7.1.1.12.5.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink.

7.1.1.12.5.3.2	Test procedure sequence

Table 7.1.1.12.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits NR RRCReconfiguration message to configure search space set group as per Table 7.1.1.12.5.3.3-1. (Note 1) | <-- | (NR RRC: RRCReconfiguration) | - | - |
| 2 | The UE transmits NR RRCReconfigurationComplete message. (Note 2) | --> | (NR RRC: RRCReconfigurationComplete) | - | - |
| 3 | SS transmits a downlink assignment in search space ID 2 corresponding to group ID 0 addressed to C-RNTI. | <-- | DCI format 1_1 | - | - |
| 4 | SS transmits in the indicated downlink assignment a MAC PDU. | <-- | MAC PDU | - | - |
| 5 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 4? | --> | HARQ ACK | 1 | P |
| 6 | SS transmits a downlink assignment in search space ID 2 addressed to C-RNTI with PDCCH monitoring adaptation field indicating group ID 1. | <-- | DCI format 1_1 | - | - |
| 7 | SS transmits in the indicated downlink assignment a MAC PDU. | <-- | MAC PDU | - | - |
| 8 | The UE transmits a HARQ ACK for the DL MAC PDU in Step 7. | --> | HARQ ACK | - | - |
| 9 | In a PDCCH occasion of next available slot after Pswitch symbols from step 6, SS transmits a downlink assignment in search space ID 5 corresponding to group ID 1 addressed to C-RNTI. (Note 3) | <-- | DCI format 1_1 | - | - |
| 10 | SS transmits in the indicated downlink assignment a MAC PDU. | <-- | MAC PDU | - | - |
| 11 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 10? | --> | HARQ ACK | 2 | P |
| 12 | SS waits for searchSpaceSwitchTimer-r17 expiry. | - | - | - | - |
| 13 | In a PDCCH occasion with in Pswitch symbols after searchSpaceSwitchTimer-r17 expiry, SS transmits a downlink assignment in search space ID 2 corresponding to group ID 1 addressed to C-RNTI. | <-- | DCI format 1_1 | - | - |
| 14 | SS transmits in the indicated downlink assignment a MAC PDU. | <-- | MAC PDU | - | - |
| 15 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 14? | --> | HARQ ACK | 5 | P |
| 16 | In a PDCCH occasion after Pswitch symbols from step 13, SS transmits NR RRCReconfiguration message to configure search space set group as per Table 7.1.1.12.5.3.3-1. (Note 1) | <-- | (NR RRC: RRCReconfiguration) | - | - |
| 17 | The UE transmits NR RRCReconfigurationComplete message. (Note 2) | --> | (NR RRC: RRCReconfigurationComplete) | 5 | P |
| - | EXCEPTION: Steps 18a1 to 18b7 describe behavior that depends on UE capability. | - | - | - | - |
| 18a1 | IF pc_sssg_Switching_2BitInd_r17 THEN SS transmits a downlink assignment in search space ID 2 addressed to C-RNTI with PDCCH monitoring adaptation field indicating group ID 2. | <-- | DCI format 1_1 | - | - |
| 18a2 | SS transmits in the indicated downlink assignment a MAC PDU. | <-- | MAC PDU | - | - |
| 18a3 | The UE transmits a HARQ ACK for the DL MAC PDU in Step 18a2. | --> | HARQ ACK | - | - |
| 18a4 | In a PDCCH occasion of next available slot after searchSpaceSwitchDelay-r17 symbols from step 18a1, SS transmits a downlink assignment in search space ID 5 corresponding to group ID 2 addressed to C-RNTI. | <-- | DCI format 1_1 | - | - |
| 18a5 | SS transmits in the indicated downlink assignment a MAC PDU. | <-- | MAC PDU | - | - |
| 18a6 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 18a5? | --> | HARQ ACK | 4 | P |
| 18a7 | SS waits for searchSpaceSwitchTimer-r17 expiry started at step 18a4. | - | - | - | - |
| 18b1 | IF NOT pc_sssg_Switching_2BitInd_r17 THEN SS transmits a downlink assignment in search space ID 2 addressed to C-RNTI with PDCCH monitoring adaptation field indicating group ID 1. | <-- | DCI format 1_1 | - | - |
| 18b2 | SS transmits in the indicated downlink assignment a MAC PDU. | <-- | MAC PDU | - | - |
| 18b3 | The UE transmits a HARQ ACK for the DL MAC PDU in Step 18b2. | --> | HARQ ACK | - | - |
| 18b4 | In a PDCCH occasion of next available slot after searchSpaceSwitchDelay-r17 symbols from step 18b1, SS transmits a downlink assignment in search space ID 5 corresponding to group ID 1 addressed to C-RNTI. | <-- | DCI format 1_1 | - | - |
| 18b5 | SS transmits in the indicated downlink assignment a MAC PDU. | <-- | MAC PDU | - | - |
| 18b6 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 18b5? | --> | HARQ ACK | 3 | P |
| 18b7 | SS waits for searchSpaceSwitchTimer-r17 expiry started at step 18b4. | - | - | - | - |
| 19 | SS transmits NR RRCReconfiguration message excluding search space set group configuration. (Note 1) | <-- | NR RRC: RRCReconfiguration | - | - |
| 20 | The UE transmits NR RRCReconfigurationComplete message. (Note 2) | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 21 | SS transmits a downlink assignment on search space ID 5 addressed to C-RNTI. | <-- | DCI format 1_1 | - | - |
| 22 | SS transmits in the indicated downlink assignment a MAC PDU. | <-- | MAC PDU | - | - |
| 23 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 22? | --> | HARQ ACK | 6 | P |
| Note 1: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration. Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 3: Pswitch is derived from TS 38.213 Table 10.4-1 for processing capability 1 as per configured scs μ ∈ {0,1,2,3}. |  |  |  |  |  |

7.1.1.12.5.3.3	Specific message contents

Table 7.1.1.12.5.3.3-1: RRCReconfiguration (step 1, step 16, step 19, Table 7.1.1.12.5.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |
|  | Not present |  | NR |
| nonCriticalExtension | Not present |  | EN-DC |
| nonCriticalExtension SEQUENCE{ |  |  | NR |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.12.5.3.3-2: CellGroupConfig (Table 7.1.1.12.5.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.12.5.3.3-3: ServingCellConfig (Table 7.1.1.12.5.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| pdcch-ServingCellConfig CHOICE { |  |  |  |
| setup | PDCCH-Config |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.12.5.3.3-4: PDCCH-Config (Table 7.1.1.12.5.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config ::= SEQUENCE { |  |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace { | 3 entries |  |  |
| SearchSpace[1] | SearchSpace with condition USS1 |  |  |
| SearchSpace[2] | SearchSpace with condition USS2 |  |  |
| SearchSpace[3] | SearchSpace with condition USS3 |  |  |
| } |  |  |  |
| searchSpacesToAddModListExt-v1700 | Not present |  | Step 19 |
| searchSpacesToAddModListExt-v1700 SEQUENCE(SIZE (1..10)) OF SearchSpaceExt-v1700 { | 3 entries if pc_sssg_Switching_2BitInd_r17, 2 entries otherwise |  | Step 1, Step 16 |
| SearchSpaceExt-v1700[1] SEQUENCE { |  |  |  |
| searchSpaceGroupIdList-r17[0] | 0 |  |  |
| } |  |  |  |
| SearchSpaceExt-v1700[2] SEQUENCE { |  |  |  |
| searchSpaceGroupIdList-r17[0] | 1 |  |  |
| } |  |  |  |
| SearchSpaceExt-v1700[3] SEQUENCE { |  |  |  |
| searchSpaceGroupIdList-r17[0] | 2 |  |  |
| } |  |  |  |
| searchSpaceSwitchConfig-r17 | Not present |  | Step 19 |
| searchSpaceSwitchConfig-r17 ::= SEQUENCE { |  |  | Step 1, Step 16 |
| searchSpaceSwitchTimer-r17 | 20 |  |  |
| searchSpaceSwitchDelay-r17 | 50 |  | Step 16 |
|  | Not present |  | Step 1 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.12.5.3.3-5: SearchSpace (Table 7.1.1.12.5.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-162 with condition USS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 2 |  | USS1 |
|  | 4 |  | USS2 |
|  | 5 |  | USS3 |
| controlResourceSetId | 1 |  |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl5 | 1 |  | USS1 |
| sl5 | 2 |  | USS2 |
| sl10 | 5 |  | USS3 |
| } |  |  |  |
| duration | Not present |  |  |
| monitoringSymbolsWithinSlot | 10000000000000 |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n1 |  |  |
| aggregationLevel4 | n0 |  |  |
| aggregationLevel8 | n0 |  |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| USS1 | Configuration of USS1 |
| USS2 | Configuration of USS2 |
| USS3 | Configuration of USS3 |

Table 7.1.1.12.5.3.3-5: Physical layer parameters for DCI format 1_1 (All steps, Table 7.1.1.12.5.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.3.6.1.2.2-1 with condition SSSG |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| PDSCH-to-HARQ_feedback timing indicator | Corresponding to K1=5 slots as per dl-DataToUL-ACK in Table 4.6.3-112 TS 38.508-1 [4]. | “011”B |  |
|  | Corresponding to K1=8 slots as per dl-DataToUL-ACK in Table 4.6.3-112 TS 38.508-1 [4]. | “110”B | TDD AND SCS15 AND USS1 |
|  | Corresponding to K1=9 slots as per dl-DataToUL-ACK in Table 4.6.3-112 TS 38.508-1 [4]. | “111”B | TDD AND SCS15 AND USS3 |
|  | Corresponding to K1=7 or 3 slots depending on the transmission occasion and as per dl-DataToUL-ACK in Table 4.6.3-112 TS 38.508-1 [4]. | “101”B/”001”B | TDD AND SCS30 AND USS1 |
|  | Corresponding to K1=6 or 2 slots depending on the transmission occasion and as per dl-DataToUL-ACK in Table 4.6.3-112 TS 38.508-1 [4]. | “100”B/”000”B | TDD AND SCS30 AND USS2 |
|  | Corresponding to K1=4 slots as per dl-DataToUL-ACK in Table 4.6.3-112 TS 38.508-1 [4]. | “001”B | TDD AND SCS30 AND USS3 |
|  | Corresponding to K1=8 slots as per dl-DataToUL-ACK in Table 4.6.3-112 TS 38.508-1 [4]. | “110”B | TDD AND SCS120 AND SS1 |
|  | Corresponding to K1=7 slots as per dl-DataToUL-ACK in Table 4.6.3-112 TS 38.508-1 [4]. | “101”B | TDD AND SCS120 AND SS2 |
|  | Corresponding to K1=4 slots as per dl-DataToUL-ACK in Table 4.6.3-112 TS 38.508-1 [4]. | “010”B | TDD AND SCS120 AND SS3 |

| Condition | Explanation |
| --- | --- |
| USS1 | Configuration of USS1 |
| USS2 | Configuration of USS2 |
| USS3 | Configuration of USS3 |

<End of modified section>