3GPP TSG-RAN5 Meeting #108	R5-254855r2

# Bengaluru, India, 25th - 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-5 | CR | 0109 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Transmit power density and A-MPR updates for band n254 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, NR_lic_bands_BW_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Transmit power requirements include now additional requirements for transmit power density for band n254. Current test procedures are missing those requirements. - RAN4 has updated minimum conformance requirements for Transmit power density requirements. Such changes are missing in corresponding test case. - Missing A-MPR message contents for additional Network signalling values to be tested for band n254 (NS_11N and NS_12N). - A-MPR minimum conformance requirements for NS_11N and NS_12N are missing. - A-MPR test requirements for NS_11N and NS_12N are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add new test case for the new requirement. - Align Transmit power minimum conformance requirements with core requirements. - New A-MPR message contents added for NS_11N and NS_12N configuration. - Editor’s note added to indicate A-MPR test case is missing some minimum conformance requirements (NS_11N and NS_12N) for band n254. - A-MPR test requirements for NS_11N and NS_12N added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Transmit power, A-MPR, additional spurious emissions test cases would not test properly associated core requirements for band n254. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.2.1_1 (New), 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: Mapping of NS_11N and NS_12N values added Revision 2: - New test case defined for the new power density requirement - Remove test points different from minimum allocation for power density measurement - New editor’s note added to clarify whether mean EIRP limit from regulatory requirements refers to spatial mean and/or mean EIRP within the channel bandwidth - Key steps in the test procedure for transmit power density measurement set to FFS |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

### 6.2.1	UE maximum output power

#### 6.2.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

#### 6.2.1.2	Test applicability

This test case applies to all types of NR Power Class 3 UE release 17 and forward that support satellite access operation.

#### 6.2.1.3	Minimum conformance requirements

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1 ms).

Table 6.2.1.3-1: UE Power Class

| NR satellite band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n256 | 23 | ±2 |
| n255 | 23 | ±2 |
| n254 | 23 | ±2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated |  |  |

The UE shall meet the following additional requirements for maximum mean transmission power density specified in Table 6.2.1-2 when NS is signaled and when the configured channel overlaps with any portion of the specified frequency range.

Table 6.2.1-2: Additional requirements for transmit power density

| NR Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum power density |
| --- | --- | --- | --- | --- |
| n254 | NS_04N | 5 | 1610 - 1618.25 | -27dBm/4kHz (mean) |
|  | NS_05N | 5 | 1618.25 - 1626.5 |  |
|  |  | 10, 15 | 1610 – 1626.5 |  |
|  | NS_11N | 5 | 1610 - 1618.25 | -15dBm/4kHz (peak limit) |
|  | NS_12N | 5 | 1618.25 - 1626.5 |  |
|  |  | 10, 15 | 1610 – 1626.5 |  |

The normative reference for this requirement is TS 38.101-5 [11] clause 6.2.1.

#### 6.2.1.4	Test description

##### 6.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table table 6.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [12] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [12] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [12] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for maximum output | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | power test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM Pi/2 BPSK | Inner 1RB Left |
| 3 |  | DFT-s-OFDM Pi/2 BPSK | Inner 1RB Right |
| 4 |  | DFT-s-OFDM QPSK | Inner Full |
| 5 |  | DFT-s-OFDM QPSK | Inner 1RB Left |
| 6 |  | DFT-s-OFDM QPSK | Inner 1RB Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [12] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to TS 38.521-1 [2] Annex G.0, G.1, G.2, and G.3.1.

4.	The UL Reference Measurement Channel is set according to Table 6.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 38.508-1 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 38.508-1 [12] table 5.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and table 5.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB19 information during the duration of the test as defined in TS 38.508-1 [12] clause 5.6.3.1.

8.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

9.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [12] clause 4.5. Message contents are defined in clause 6.2.1.4.3.

6.2.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1 ms) and in the uplink symbols. For TDD symbols with transient periods are not under test.

##### 6.2.1.4.3	Message contents

6.2.1.4.3.0	General

Message contents are according to TS 38.508-1 [12] subclause 4.6 and 5.4 with the following exceptions

Table 6.2.1.4.3.0-1: PUSCH-Config

Derivation Path: TS 38.508-1 [12], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

6.2.1.4.3.1	Void

6.2.1.4.3.2	Void

#### 6.2.1.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-1 for Power Class 3.

Table 6.2.1.5-1: Maximum Output Power test requirement for Power Class 3

| NR satellite band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n256 | 23 | ±(2+TT) |
| n255 | 23 | ±(2+TT) |
| n254 | 23 | ±(2+TT) |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-2 |  |  |

Table 6.2.1.5-2: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz |
| --- | --- |
| BW ≤ 30MHz | 0.7 dB |

### 6.2.1_1	UE maximum output power: maximum power density

Editor's Note: This test case is incomplete. The following aspects are either missing or not yet determined:

- To complete test procedure definition, it is required to clarify in RAN4 whether mean power density limit within transmit power density requirements from regulatory requirements refers to spatial mean and/or mean within the channel bandwidth.

- MU Analysis, test tolerances and any associated Annex F updates for new power density metric is pending.

#### 6.2.1_1.1	Test purpose

Same as in 6.2.1.1.

#### 6.2.1_1.2	Test applicability

Same as in 6.2.1.2.

#### 6.2.1_1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2.1.3.

#### 6.2.1_1.4	Test description

##### 6.2.1_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table table 6.2.1_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.1_1.4.1-1: Test Configuration Table for transmit power density measurements

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [12] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [12] subclause 4.3.1 |  | Low range, Mid range for NS_04N and NS_11N Mid range, High range for NS_05N and NS_12N |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [12] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for maximum output | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM Pi/2 BPSK | Inner 1RB Left |
| 2 |  | DFT-s-OFDM Pi/2 BPSK | Inner 1RB Right |
| 3 |  | DFT-s-OFDM QPSK | Inner 1RB Left |
| 4 |  | DFT-s-OFDM QPSK | Inner 1RB Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [12] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to TS 38.521-1 [2] Annex G.0, G.1, G.2, and G.3.1.

4.	The UL Reference Measurement Channel is set according to Table 6.2.1_1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 38.508-1 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 38.508-1 [12] table 5.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and table 5.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB19 information during the duration of the test as defined in TS 38.508-1 [12] clause 5.6.3.1.

8.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

9.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [12] clause 4.5. Message contents are defined in clause 6.2.1_1.4.3.

##### 6.2.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.1_1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	FFS

4.  FFS

5.  Repeat steps 1, 2, 3 and 4 for all applicable network signaled values.

##### 6.2.1_1.4.3	Message contents

Message contents are according to TS 38.508-1 [12] subclause 4.6 and 5.4 with additional exceptions described in 6.2.1.4.3 and the ones described in 6.2.3.4.3 associated to each applicable network signalled value used in maximum power density measurements.

#### 6.2.1_1.5	Test requirement

Mean and peak ttransmit power density values derived in step 4 shall be within the range prescribed by the additional requirements for transmit power density in Table 6.2.1_1.5-1 and tolerances in Table 6.2.1_1.5-2.

Table 6.2.1_1.5-1: Transmit power density test requirement

| NR Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum power density |
| --- | --- | --- | --- | --- |
| n254 | NS_04N | 5 | 1610 - 1618.25 | -27dBm/4kHz ± TT(mean) |
|  | NS_05N | 5 | 1618.25 - 1626.5 |  |
|  |  | 10, 15 | 1610 – 1626.5 |  |
|  | NS_11N | 5 | 1610 - 1618.25 | -15dBm/4kHz ± TT (peak limit) |
|  | NS_12N | 5 | 1618.25 - 1626.5 |  |
|  |  | 10, 15 | 1610 – 1626.5 |  |

Table 6.2.1_1.5-2: Test tolerance (tansmit power density test requirement)

| NR Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Test Tolerance |
| --- | --- | --- | --- | --- |
| n254 | NS_04N | 5 | 1610 - 1618.25 | TBD |
|  | NS_05N | 5 | 1618.25 - 1626.5 |  |
|  |  | 10, 15 | 1610 – 1626.5 |  |
|  | NS_11N | 5 | 1610 - 1618.25 | TBD |
|  | NS_12N | 10, 15 | 1610 – 1626.5 |  |

### 6.2.2	UE maximum output power reduction

#### 6.2.2.1	Test purpose

The number of RB identified in Table 6.2.2.3-1 is based on meeting the requirements for adjacent channel leakage ratio and the maximum power reduction (MPR) due to Cubic Metric (CM).

#### 6.2.2.2	Test applicability

The requirements of this test apply to all types of NR Power Class 3 UE release 17 and forward that support satellite access operation.

NOTE:	Test execution is not necessary if TS 38.521-5 clause 6.5.2.4.1 is executed.

#### 6.2.2.3	Minimum conformance requirements

UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations. For UE power class 3, the allowed maximum power reduction (MPR) is defined in Table 6.2.2.3-1 for channel bandwidths ≤ 100 MHz. The ∆MPR is set to zero.

The allowed MPR for SRS, PUCCH formats 0, 1, 3 and 4, and PRACH shall be as specified for QPSK modulated DFT-s-OFDM of equivalent RB allocation. The allowed MPR for PUCCH format 2 shall be as specified for QPSK modulated CP-OFDM of equivalent RB allocation.

Table 6.2.2.3-1: Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 0.5 |  | 0 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.5 | 0 |  |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |

RB allocation ranges for Outer and Inner RB allocations are specified in TS 38.521-1 [2] 6.2.2.3.

The normative reference for this requirement is TS 38.101-5 [11] clause 6.2.2.

#### 6.2.2.4	Test description

##### 6.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in TS 38.521-1 [2] Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 38.521-1 [2] Annex C.2.

Table 6.2.2.4.1-1: Test Configuration Table for power class 3 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [12] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [12] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [12] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  |  | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default | N/A for Maximum Power | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Default |  | CP-OFDM QPSK | Inner Full |
| 17 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 18 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 19 | Default |  | CP-OFDM QPSK | Outer Full |
| 20 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 21 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 22 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 23 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 24 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 273 | Low |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 283 | High |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 293 | Default |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [12] Annex A, Figure A.3.1.2.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to TS 38.521-1 [2] Annex G.0, G.1, G.2, and G.3.1.

4.	The UL Reference Measurement Channel is set according to Table 6.2.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 38.508-1 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 38.508-1 [12] table 5.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and table 5.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB19 information during the duration of the test as defined in TS 38.508-1 [12] clause 5.6.3.1.

8.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

9.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [12] clause 4.5. Message contents are defined in clause 6.2.2.4.3.

##### 6.2.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 ms for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1 ms over consecutive active uplink slots.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2.2.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [12] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

##### 6.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [12] subclause 4.6 and 5.4 with the following exceptions:

Table 6.2.2.4.3-1: PUSCH-Config

| Derivation Path: TS 38.508-1 [12] subclause 4.6.3 Table 4.6.3-118 PUSCH-Config |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType0 |  | Almost contiguous allocation |
|  | resourceAllocationType1 |  | Contiguous allocation |
| } |  |  |  |

Table 6.2.2.4.3-2: DMRS-UplinkConfig Test ID 33 - 35 in Table 6.2.2.4.1-1

| Derivation Path: TS 38.508-1 [12], Table 4.6.3-51 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-UplinkConfig ::= SEQUENCE { |  |  |  |
| transformPrecodingEnabled SEQUENCE { |  |  |  |
| dmrs-UplinkTransformPrecoding-r16 SEQUENCE { |  |  |  |
| pi2BPSK-ScramblingID0 | Not present |  |  |
| pi2BPSK-ScramblingID1 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.2.4.3-3: ServingCellConfig

| Derivation Path: TS 38.508-1 [12] Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| powerBoostPi2BPSK | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.2.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2.5-1.

Table 6.2.2.5-1: UE MPR test requirements for power class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 2 | 25.0 + TT | 21.0 - TT |
| 2 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2 | 25.0 + TT | 20.5 - TT |
| 3 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2 | 25.0 + TT | 20.5 - TT |
| 4 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2 | 25.0 + TT | 20.5 - TT |
| 5 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 2 | 25.0 + TT | 21.0 - TT |
| 6 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2 | 25.0 + TT | 20.0 - TT |
| 7 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2 | 25.0 + TT | 20.0 - TT |
| 8 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2 | 25.0 + TT | 20.0 - TT |
| 9 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2 | 25.0 + TT | 20.0 - TT |
| 10 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2 | 25.0 + TT | 19.0 - TT |
| 11 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2 | 25.0 + TT | 19.0 - TT |
| 12 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2 | 25.0 + TT | 19.0 - TT |
| 13 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18.0 - TT |
| 14 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18.0 - TT |
| 15 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18.0 - TT |
| 16 | 23 | 0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25.0 + TT | 19.5 - TT |
| 17 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 18 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 19 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 20 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2 | 25.0 + TT | 19.0 - TT |
| 21 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 22 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 23 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 24 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25.0 + TT | 16.0 - TT |
| 25 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25.0 + TT | 16.0 - TT |
| 26 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25.0 + TT | 16.0 - TT |
| 27 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2 | 25.0 + TT | 20.5 - TT |
| 28 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2 | 25.0 + TT | 20.5 - TT |
| 29 | 23 | 0 | 0 | 0 | 23 | 2.0 | 2 | 25.0 + TT | 21.0 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-2. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-2: Test Tolerance (Maximum Power Reduction (MPR))

|  | f ≤ 3.0GHz |
| --- | --- |
| BW ≤ 30MHz | 0.7 dB |

### 6.2.3	UE additional maximum output power reduction

#### 6.2.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR satellite band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [8].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1.3-1 except for 256QAM. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2 except for 256QAM. Outer and inner allocation notation used in clause 6.2.3 is defined in clause 6.2.2. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

#### 6.2.3.2	Test applicability

This test case applies to all types of NR Power Class 3 UE release 17 and forward that support satellite access operation.

#### 6.2.3.3	Minimum conformance requirements

##### 6.2.3.3.1	General

Table 6.2.3.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR satellite band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.3.1-2.

Table 6.2.3.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR satellite Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2.2-1 (NOTE 3) | 5, 10, 15, 20 | Table 5.3.2-1 | N/A |
| NS_24 | 6.5.3.3.13 in 3GPP TS 38.101-1 [5] | n256 | 5, 10, 15, 20 | Table 6.2.3.15-1 in 3GPP TS 38.101-1 [5] | Clause 6.2.3.15 in 3GPP TS 38.101-1 [5]2 |
| NS_02N | 6.5.3.3.3.1 | n255 | 5, 10, 15, 20 |  | N/A |
| NS_100 | 6.5.2.4.2 in 3GPP TS 38.101-1 [5] | n2561 |  |  | Table 6.2.3.3.1-3, and Table 6.2.3.1-2 in 3GPP TS 38.101-1 [5] |
| NS_03N | 6.5.3.3.3.2 | n254 | 5, 10, 15 |  | Clause 6.2.3.3.2 |
| NS_04N | 6.5.3.3.3.3 | n254 | 5 |  | Clause 6.2.3.3.3 |
| NS_05N | 6.5.3.3.3.3 | n254 | 5, 10, 15 |  | Clause 6.2.3.3.4 |
| NS_11N | TBD | n254 | 5 |  | Clause 6.2.3.3 in 3GPP TS 38.101-1 [5] |
| NS_12N | TBD | n254 | 5, 10, 15 |  | Clause 6.2.3.4 in 3GPP TS 38.101-1 [5] |
| NOTE 1: This NS can be signalled for NR satellite bands that have UTRA services deployed. NOTE 2: A-MPR for the upper 5 MHz of the band is not specified, and therefore shall be used as a guard band. NOTE 3: The NS_01 label with the field additionalPmax [8] absent is default for all NR satellite bands. |  |  |  |  |  |

Table 6.2.3.3.1-2: Mapping of network signalling label

| NR satellite band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n256 | NS_01 | NS_24 | NS_100 |  |  |  |  |  |
| n255 | NS_01 | NS_02N |  |  |  |  |  |  |
| n254 | NS_01 | NS_03N | NS_04N | NS_05N |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of 3GPP TS 38.331 [8]. |  |  |  |  |  |  |  |  |

Table 6.2.3.3.1-3: A-MPR for NS_100 (UTRA protection)

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 |
|  | QPSK | ≤ 2 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3 |
| CP-OFDM | QPSK | ≤ 4 |
|  | 16 QAM | ≤ 4 |
|  | 64 QAM | ≤ 4 |

#### 6.2.3.3.2	A-MPR for NS_03N

Table 6.2.3.3.2-1: A-MPR regions for NS_03N

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 1612.5 <= fc < 1613.9 | <= 0.36 | <= 0.36 | A1 |
|  |  |  | >= 2.88 | A2 |
|  | 1613.9 <= fc < 1615.7 |  | >= 3.24 | A3 |
| 10MHz | 1615 <= fc < 1620.1 | <= 1.8 | <= 5.04 | A4 |
|  |  | <= 1.8 | > 5.04 | A5 |
|  |  | > 7.2 | > 0 | A6 |
|  |  | > 1.8 | >= 2.88 | A2 |
|  | 1620.1 <= fc < 1621.5 |  | <= 6.48 | A6 |
|  |  | <= 0.36 | <= 0.36 | A1 |
|  | fc = 1621.5 |  | >= 7.2 | A1 |
| 15MHz | all | <= 3.6 | <= 5.04 | A4 |
|  |  | <= 3.6 | > 5.04 | A5 |
|  |  | > 10.44 |  | A6 |
|  |  | > 3.6 | >= 4.32 | A2 |

Table 6.2.3.3.2-2: A-MPR for NS_03N

|  | Modulation | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 2.5 | 3.0 | 1.0 | 4.0 | 6.5 | 1.5 |
|  | QPSK | 2.5 | 4.0 | 2.5 | 6.0 | 7.0 | 2.0 |
|  | 16QAM | 3.0 | 4.5 | 3.0 | 6.5 | 7.5 | 2.5 |
|  | 64QAM | 3.5 | 5 | 3.5 | 7 | 8 | 3 |
| CP-OFDM | QPSK | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |
|  | 16QAM | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |
|  | 64QAM | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |

#### 6.2.3.3.3	A-MPR for NS_04N and NS_11N

Table 6.2.3.3.3-1: A-MPR regions for NS_04N and NS_11N

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 1612.5 <= fc < 1613.9 | <= 0.36 | <= 0.36 | A1 |
|  |  |  | >= 2.88 | A2 |
|  | 1613.9 <= fc < 1615.7 |  | >= 3.24 | A3 |

Table 6.2.3.3.3-2: A-MPR for NS_04N and NS_11N

|  | Modulation | A1 | A2 | A3 |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 2.5 | 3.0 | 1.0 |
|  | QPSK | 2.5 | 4.0 | 2.5 |
|  | 16QAM | 3.0 | 4.5 | 3.0 |
|  | 64QAM | 3.5 | 5 | 3.5 |
| CP-OFDM | QPSK | 3.5 | 6.0 | 4.0 |
|  | 16QAM | 3.5 | 6.0 | 4.0 |
|  | 64QAM | 3.5 | 6.0 | 4.0 |

#### 6.2.3.3.4	A-MPR for NS_05N and NS_12N

Table 6.2.3.3.4-1: A-MPR regions for NS_05N and NS_12N

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 1622.4 < fc <= 1624 | <= 3.6 | > 0.36 | A3 |
|  |  |  | >= 2.88 | A1 |
| 10MHz | 1615 <= fc < 1620.1 | <= 1.8 | <= 5.04 | A4 |
|  |  | <= 1.8 | > 5.04 | A5 |
|  |  | > 7.2 | > 0 | A6 |
|  |  | > 1.8 | >= 2.88 | A2 |
|  | 1620.1 <= fc <= 1621.5 |  | <= 7.2 | A6 |
|  |  | <= 0.36 | <= 0.36 | A1 |
|  |  | > 7.2 | > 0 | A6 |
| 15MHz | all | <= 3.6 | <= 5.04 | A4 |
|  |  | <= 3.6 | > 5.04 | A5 |
|  |  | > 10.44 |  | A6 |
|  |  | > 3.6 | >= 4.32 | A2 |

Table 6.2.3.3.4-2: A-MPR for NS_05N and NS_12N

|  | Modulation | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 1.5 | 5.0 | 1.5 | 6.5 | 6.5 | 2.0 |
|  | QPSK | 1.5 | 5.0 | 1.5 | 6.5 | 7.0 | 2.5 |
|  | 16QAM | 2.0 | 5.5 | 2.0 | 7.0 | 7.5 | 3.0 |
|  | 64QAM | 2.5 | 6 | 2.5 | 7.5 | 8 | 3.5 |
| CP-OFDM | QPSK | 3.0 | 6.5 |  | 8.0 | 10.0 | 4.5 |
|  | 16QAM | 3.0 | 6.5 |  | 8.0 | 10.0 | 4.5 |
|  | 64QAM | 3.0 | 6.5 |  | 8.0 | 10.0 | 4.5 |

##### 6.2.3.3.5	A-MPR for NS_24

Table 6.2.3.3.5-1: A-MPR regions for NS_24

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  | Region B |  | Region C |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | RBend*12*SCS MHz | LCRB*12*SCS MHz | RBend*12*SCS MHz | LCRB*12*SCS MHz |
| 5MHz | Fc=1,992.5 |  | >3.24 |  |  |  |  |
| 5MHz | Fc=1,997.5 |  | >3.24 |  |  |  |  |
| 5MHz | Fc=2,002.5 |  | >2.16 | >3.78 | ≤1.98 | ≤3.6 | ≤1.98 |
| 10MHz | Fc=1,985 | >5.4 |  |  |  |  |  |
| 10MHz | Fc=1,995 |  | >4.5 | >7.56 | ≤4.32 | ≤7.38 | ≤4.32 |
| 10MHz | Fc=2,000 | >6.84 |  | <2.88 |  | ≥3.06 ≤6.66 | >1.44 |
| 15MHz | Fc=1,987.5 |  | >7.02 | >11.52 | ≤6.84 | ≤11.34 | ≤6.84 |
| 15MHz | Fc=1,997.5 | >9.36 |  | <3.6 |  | ≥3.78 ≤9.18 | >1.44 |
| 20MHz | Fc=1,990 | >13.5 |  | <4.5 |  | ≥4.68 ≤13.32 | >2.16 |
| 20MHz | Fc=1,995 | >12.6 |  | <5.4 |  | ≥5.58 ≤12.42 | >1.44 |
| NOTE 1: The A-MPR values are listed in Table 6.2.3.3.2-2. NOTE 2: For any undefined region, MPR applies |  |  |  |  |  |  |  |

Table 6.2.3.3.5-2: A-MPR for NS_24

| Modulation/Waveform |  | Outer (dB) | Inner (dB) |
| --- | --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | ≤ 1.5 | N/A |
|  | QPSK | ≤ 2 |  |
|  | 16 QAM | ≤ 3 |  |
|  | 64 QAM | ≤ 3.5 |  |
| CP-OFDM | QPSK | ≤ 4 |  |
|  | 16 QAM | ≤ 4 |  |
|  | 64 QAM | ≤ 4.5 |  |

The normative reference for this requirement is TS 38.101-5 [11] clause 6.2.3.1.

#### 6.2.3.4	Test description

##### 6.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Tables 6.2.3.4.1-1 to 6.2.3.4.1-30. The details of the uplink reference measurement channels (RMCs) are specified in clause A.2. Configurations of PDSCH and PDCCH before measurement are specified in clause C.2.

Table 6.2.3.4.1-1: Test Configuration table for NS_100

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [12] subclause 4.1 |  |  |  |  |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [12] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |
| Test Channel Bandwidths as specified in TS 38.508-1 [12] subclause 4.3.1 |  |  |  |  |  |  | Lowest, Highest |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |
| A-MPR test parameters for NS_100 |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | Low | Default | Default | N/A for A-MPR test cases | DFT-s OFDM | Pi/2 BPSK | Edge_1RB_Left |
| 2 | High | Default | Default |  |  | Pi/2 BPSK | Edge_1RB_Right |
| 3 | Default | Default | Default |  |  | Pi/2 BPSK | Outer_Full |
| 4 | Low | Default | Default |  |  | QPSK | Edge_1RB_Left |
| 5 | High | Default | Default |  |  | QPSK | Edge_1RB_Right |
| 6 | Default | Default | Default |  |  | QPSK | Outer_Full |
| 7 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |
| 8 | High | Default | Default |  |  | 16 QAM | Edge_1RB_Right |
| 9 | Default | Default | Default |  |  | 16 QAM | Outer_Full |
| 10 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |
| 11 | High | Default | Default |  |  | 64 QAM | Edge_1RB_Right |
| 12 | Default | Default | Default |  |  | 64 QAM | Outer_Full |
| 13 | Low | Default | Default |  | CP-s OFDM | QPSK | Edge_1RB_Left |
| 14 | High | Default | Default |  |  | QPSK | Edge_1RB_Right |
| 15 | Default | Default | Default |  |  | QPSK | Outer_Full |
| 16 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |
| 17 | High | Default | Default |  |  | 16 QAM | Edge_1RB_Right |
| 18 | Default | Default | Default |  |  | 16 QAM | Outer_Full |
| 19 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |
| 20 | High | Default | Default |  |  | 64 QAM | Edge_1RB_Right |
| 21 | Default | Default | Default |  |  | 64 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |  |  |  |

Table 6.2.3.4.1-2: Test Configuration table for NS_24

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [12] subclause 4.1 |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [12] subclause 4.3.1 |  |  |  |  | Refer to uplink carrier centre frequency (Fc) in test parameters |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [12] subclause 4.3.1 |  |  |  |  | Refer to test parameters (5, 10, 15, 20 MHz) |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest |  |  |  |  |
| A-MPR test parameters for NS_24 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (NOTE 2, 3) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | Region A | Region B | Region C |
| 1-4 | 1,992.5 | 5 | Default | N/A for A-MPR testing | DFT-s OFDM | Pi/2 BPSK QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 5-8 | 1,997.5 | 5 | Default |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 9-20 | 2,002.5 | 5 | Default |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| e | 1,985 | 10 | Default |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM | Edge_1RB_Right | N/A | N/A |
| 25-28 | 1,985 | 10 | Default |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 29-40 | 1,995 | 10 | Default |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 41-52 | 2,000 | 10 | Default |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 36@0 |
| 53-56 | 2,000 | 10 | Default |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 57-68 | 1,987.5 | 15 | Default |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 69-80 | 1,997.5 | 15 | Default |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 50@0 |
| 81-84 | 1,997.5 | 15 | Default |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 85-96 | 1,990 | 20 | Default |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 74@0 |
| 97-100 | 1,990 | 20 | Default |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 101-112 | 1,995 | 20 | Default |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 69@0 |
| 113-116 | 1,995 | 20 | Default |  |  | Pi/2 BPSK QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 117-119 | 1,992.5 | 5 | Default |  | CP-OFDM | QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 120-122 | 1,997.5 | 5 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 123-131 | 2,002.5 | 5 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 132-134 | 1,985 | 10 | Default |  |  | QPSK 16 QAM 64 QAM | Edge_1RB_Right | N/A | N/A |
| 135-137 | 1,985 | 10 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 138-146 | 1,995 | 10 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 147-155 | 2,000 | 10 | Default |  |  | QPSK 16 QAM 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 37@0 |
| 156-158 | 2,000 | 10 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 159-167 | 1,987.5 | 15 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 168-176 | 1,997.5 | 15 | Default |  |  | QPSK 16 QAM 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 51@0 |
| 177-179 | 1,997.5 | 15 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 180-188 | 1,990 | 20 | Default |  |  | QPSK 16 QAM 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 74@0 |
| 189-191 | 1,990 | 20 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 192-200 | 1,995 | 20 | Default |  |  | QPSK 16 QAM 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 69@0 |
| 201-203 | 1,995 | 20 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 204 | 1,992.5 | 5 | Default |  |  | QPSK | Edge_1RB_Right | N/A | N/A |
| 205 | 1,997.5 | 5 | Default |  |  | 16 QAM | Edge_1RB_Right | N/A | N/A |
| 206 | 1,985 | 10 | Default |  |  | QPSK | Edge_1RB_Left | N/A | N/A |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: In test IDs with multiple modulations, each UL Modulation shall be tested separately against Range A, B, and C. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-3: FFS

Table 6.2.3.4.1-4: Test Configuration table for NS_03N

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | (See Freq column) |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | (See Ch BW column) |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_50 (Power Class 3) |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | Low | 5 | Default | N/A | DFT-s-OFDM | QPSK | Edge_1RB_left (A1) | N/A | N/A |
| 2 | Low | 5 | Default |  |  | QPSK | Outer_Full (A2) | N/A | N/A |
| 3 | Low | 5 | Default |  |  | QPSK | Outer_Full (A3) | N/A | N/A |
| 4 | Low | 10 | Default |  |  | QPSK | Edge_1RB_left (A4) |  |  |
| 5 | Low | 10 | Default |  |  | QPSK | Outer_Full (A5) |  |  |
| 6 | Low | 10 | Default |  |  | QPSK | 10@41 (A6) | 3@21 (A6) | 1@10 (A6) |
| 7 | Low | 10 | Default |  |  | QPSK | 40@11 (A2) | 18@6 (A2) | 8@3 (A2) |
| 8 | Low | 10 | Default |  |  | QPSK | 36@0 (A6) | 18@0 (A6) | 9@0 (A6) |
| 9 | Low | 10 | Default |  |  | QPSK | Edge_1RB_left (A1) |  |  |
| 10 | 1621.5 | 10 | Default |  |  | QPSK | Outer_Full(A1) |  |  |
| 11 | Low | 15 | Default |  |  | QPSK | Edge_1RB_left (A4) |  |  |
| 12 | Low | 15 | Default |  |  | QPSK | Outer_Full (A5) |  |  |
| 13 | Low | 15 | Default |  |  | QPSK | 20@59 (A6) | 8@30 (A6) | 3@15 (A6) |
| 14 | Low | 15 | Default |  |  | QPSK | 24@55 (A2) | 12@26 (A2) | 6@12 (A2) |
| 15 | Low | 5 | Default | N/A | CP-OFDM | QPSK | Edge_1RB_left (A1) | N/A | N/A |
| 16 | Low | 5 | Default |  |  | QPSK | Outer_Full (A2) | N/A | N/A |
| 17 | Low | 5 | Default |  |  | QPSK | Outer_Full (A3) | N/A | N/A |
| 18 | Low | 10 | Default |  |  | QPSK | Edge_1RB_left (A4) |  |  |
| 19 | Low | 10 | Default |  |  | QPSK | Outer_Full (A5) |  |  |
| 20 | Low | 10 | Default |  |  | QPSK | 11@41 (A6) | 3@21 (A6) | 1@10 (A6) |
| 21 | Low | 10 | Default |  |  | QPSK | 41@11 (A2) | 18@6 (A2) | 8@3 (A2) |
| 22 | Low | 10 | Default |  |  | QPSK | 36@0 (A6) | 18@0 (A6) | 9@0 (A6) |
| 23 | Low | 10 | Default |  |  | QPSK | Edge_1RB_left (A1) |  |  |
| 24 | 1621.5 | 10 | Default |  |  | QPSK | Outer_Full(A1) |  |  |
| 25 | Low | 15 | Default |  |  | QPSK | Edge_1RB_left (A4) |  |  |
| 26 | Low | 15 | Default |  |  | QPSK | Outer_Full (A5) |  |  |
| 27 | Low | 15 | Default |  |  | QPSK | 20@59 (A6) | 8@30 (A6) | 3@15 (A6) |
| 28 | Low | 15 | Default |  |  | QPSK | 24@55 (A2) | 12@26 (A2) | 6@12 (A2) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-5: Test Configuration table for NS_04N and NS_11N

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 5 MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_50 (Power Class 3) |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 5 | Default | N/A | DFT-s-OFDM | QPSK | Edge_1RB_left (A1) | N/A | N/A |
| 2 | Default | 5 | Default |  |  | QPSK | Outer_Full (A2) | N/A | N/A |
| 3 | Default | 5 | Default |  |  | QPSK | Outer_Full (A3) |  |  |
| 4 | Default | 5 | Default | N/A | CP-OFDM | QPSK | Edge_1RB_left (A1) | N/A | N/A |
| 5 | Default | 5 | Default |  |  | QPSK | Outer_Full (A2) | N/A | N/A |
| 6 | Default | 5 | Default |  |  | QPSK | Outer_Full (A3) |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-6: Test Configuration table for NS_05N and NS_12N

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 5 MHz, 10MHz, 15MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_50 (Power Class 3) |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 5 | Default | N/A | DFT-s-OFDM | QPSK | 5@20 (A3) | N/A | N/A |
| 2 | Default | 5 | Default |  |  | QPSK | Outer_Full (A1) | N/A | N/A |
| 3 | Default | 10 | Default |  |  | QPSK | Edge_1RB_left (A4) |  |  |
| 4 | Default | 10 | Default |  |  | QPSK | Outer_Full (A5) |  |  |
| 5 | Default | 10 | Default |  |  | QPSK | 10@41 (A6) | 3@21 (A6) | 1@10 (A6) |
| 6 | Default | 10 | Default |  |  | QPSK | 40@11 (A2) | 18@6 (A2) | 8@3 (A2) |
| 7 | Default | 10 | Default |  |  | QPSK | 40@0 (A6) | 20@0 (A6) | 10@0 (A6) |
| 8 | Default | 10 | Default |  |  | QPSK | Edge_1RB_left (A1) |  |  |
| 9 | Default | 10 | Default |  |  | QPSK | Edge_1RB_right(A6) |  |  |
| 10 | Default | 15 | Default |  |  | QPSK | Edge_1RB_left (A4) |  |  |
| 11 | Default | 15 | Default |  |  | QPSK | Outer_Full (A5) |  |  |
| 12 | Default | 15 | Default |  |  | QPSK | 20@59 (A6) | 8@30 (A6) | 3@15 (A6) |
| 13 | Default | 15 | Default |  |  | QPSK | 24@55 (A2) | 12@26 (A2) | 6@12 (A2) |
| 14 | Default | 5 | Default | N/A | CP-OFDM | QPSK | 5@20 (A3) | N/A | N/A |
| 15 | Default | 5 | Default |  |  | QPSK | Outer_Full (A1) | N/A | N/A |
| 16 | Default | 10 | Default |  |  | QPSK | Edge_1RB_left (A4) |  |  |
| 17 | Default | 10 | Default |  |  | QPSK | Outer_Full (A5) |  |  |
| 18 | Default | 10 | Default |  |  | QPSK | 11@41 (A6) | 3@21 (A6) | 1@10 (A6) |
| 19 | Default | 10 | Default |  |  | QPSK | 41@11 (A2) | 18@6 (A2) | 8@3 (A2) |
| 20 | Default | 10 | Default |  |  | QPSK | 40@0 (A6) | 20@0 (A6) | 10@0 (A6) |
| 21 | Default | 10 | Default |  |  | QPSK | Edge_1RB_left (A1) |  |  |
| 22 | Default | 10 | Default |  |  | QPSK | Edge_1RB_right(A6) |  |  |
| 23 | Default | 15 | Default |  |  | QPSK | Edge_1RB_left (A4) |  |  |
| 24 | Default | 15 | Default |  |  | QPSK | Outer_Full (A5) |  |  |
| 25 | Default | 15 | Default |  |  | QPSK | 20@59 (A6) | 8@30 (A6) | 3@15 (A6) |
| 26 | Default | 15 | Default |  |  | QPSK | 24@55 (A2) | 12@26 (A2) | 6@12 (A2) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [12] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 and uplink signals according Annex G.0, G.1, G.2 and G.3.1.

4.	The UL Reference Measurement channels are set according to the applicable table from Table 6.2.3.4.1-1 to Table 6.2.3.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 38.508-1 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 38.508-1 [12] table 5.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and table 5.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB19 information during the duration of the test as defined in TS 38.508-1 [12] clause 5.6.3.1.

8.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

9.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [12] clause 4.5. Message contents are defined in clause 6.2.3.4.3.

##### 6.2.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to the applicable table from Table 6.2.3.4.1-1 to Table 6.2.3.4.1-2. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE Allow at least 200 ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration one sub-frame (1 ms).

NOTE:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2.3.4.1-1 to Table 6.2.3.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [12] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

##### 6.2.3.4.3	Message contents

6.2.3.4.3.1	Message contents exceptions for network signalling value "NS_100"

1.	Information element additionalSpectrumEmission is set to NS_100. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_100"

| Derivation Path: TS 38.508-1 [12], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_100) |  |  |

6.2.3.4.3.2	Message contents exceptions for network signalling value "NS_24"

1.	Information element additionalSpectrumEmission is set to NS_24. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.2-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_24"

| Derivation Path: TS 38.508-1 [12], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_24) |  |  |

6.2.3.4.3.3	Message contents exceptions for network signalling value "NS_03N"

1.	Information element additionalSpectrumEmission is set to NS_03N. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.3-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_03N "

| Derivation Path: TS 38.508-1 [12], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_03N) |  |  |

6.2.3.4.3.4	Message contents exceptions for network signalling value "NS_04N"

1.	Information element additionalSpectrumEmission is set to NS_04N. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.341: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_04N "

| Derivation Path: TS 38.508-1 [12], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_04N) |  |  |

6.2.3.4.3.5	Message contents exceptions for network signalling value "NS_05N"

1.	Information element additionalSpectrumEmission is set to NS_05N. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.5-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_05N "

| Derivation Path: TS 38.508-1 [12], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_05N) |  |  |

6.2.3.4.3.6	Message contents exceptions for network signalling value "NS_11N"

1.	Information element additionalSpectrumEmission is set to NS_11N. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.6-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_11N "

| Derivation Path: TS 38.508-1 [12], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 4 (NS_11N) |  |  |

6.2.3.4.3.7	Message contents exceptions for network signalling value "NS_12N"

1.	Information element additionalSpectrumEmission is set to NS_12N. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.7-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_12N "

| Derivation Path: TS 38.508-1 [12], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 5 (NS_12N) |  |  |

#### 6.2.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable Table from table 6.2.3.5-1 to Table 6.2.3.5-3. The allowed A-MPR values specified in Table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2.2. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.1.3-1 apply.

Table 6.2.3.5-1: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz |
| --- | --- |
| BW ≤ 30MHz | 0.7 dB |

Table 6.2.3.5-2: UE Power Class 3 test requirements (NS_100) for n256

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 3 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 4, 5 | 23 | 1 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 6 | 23 | 1 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 7, 8 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 9 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 10, 11 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 12 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 13, 14 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 15 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 16, 17 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 18 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 19, 20 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 21 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-1. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-3: UE Power Class 3 test requirements (NS_24) for n256

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 5, 9-11, 21, 25, 29-31, 41-43, 53, 57-59, 69-71, 81, 85-87, 97, 101-103, 113 | 23 | N/A | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 2, 6, 12-13, 22, 26, 32-33, 44-45, 54, 60-61, 72-73, 82, 88-89, 98, 104-105, 114 | 23 | N/A | 2.0 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 3, 7, 14-16, 23, 27, 34-36, 46-48, 55, 62-64, 74-76, 83, 90-92, 99, 106-108, 115 | 23 | N/A | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 4, 8, 17-18, 24, 28, 37-38, 49-50, 56, 65-66, 77-78, 84, 93-94, 100, 109-110, 116 | 23 | N/A | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 19-20, 39-40, 51-52, 67-68, 79-80, 95-96, 111-112 | 23 | N/A | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 117-118, 120-121, 123-127, 132-133, 135-136, 138-142, 147-151, 156-157, 159-163, 168-172, 177-178, 180-184, 189-190, 192-196, 201-202, 204-206 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 119, 122, 128-129, 134, 137, 143-144, 152-153, 158, 164-165, 173-174, 179, 185-186, 191, 197-198, 203 | 23 | N/A | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 130-131, 145-146, 154-155, 166-167, 175-176, 187-188, 199-200 | 23 | N/A | 7.5 | 0 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-1. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-4: UE Power Class 3 test requirements (NS_03N) for n254

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 1 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18.0 - TT |
| 2 | 23 | 1 | 4.0 | 0 | 19 | 3.5 | 2 | 25.0 + TT | 15.5 - TT |
| 3 | 23 | 1 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18.0 - TT |
| 4 | 23 | 1 | 6.0 | 0 | 17 | 5.0 | 2 | 25.0 + TT | 12.0 - TT |
| 5 | 23 | 1 | 7.0 | 0 | 16 | 5.0 | 2 | 25.0 + TT | 11.0 - TT |
| 6 | 23 | 1 | 2.0 | 0 | 21 | 2.0 | 2 | 25.0 + TT | 19.0 - TT |
| 7 | 23 | 1 | 4.0 | 0 | 19 | 3.5 | 2 | 25.0 + TT | 15.5 - TT |
| 8 | 23 | 1 | 2.0 | 0 | 21 | 2.0 | 2 | 25.0 + TT | 19.0 - TT |
| 9 | 23 | 1 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18.0 - TT |
| 10 | 23 | 1 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18.0 - TT |
| 11 | 23 | 1 | 6.0 | 0 | 17 | 5.0 | 2 | 25.0 + TT | 12.0 - TT |
| 12 | 23 | 1 | 7.0 | 0 | 16 | 5.0 | 2 | 25.0 + TT | 11.0 - TT |
| 13 | 23 | 1 | 2.0 | 0 | 21 | 2.0 | 2 | 25.0 + TT | 19.0 - TT |
| 14 | 23 | 1 | 4.0 | 0 | 19 | 3.5 | 2 | 25.0 + TT | 15.5 - TT |
| 15 | 23 | 3 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25.0 + TT | 16.0 - TT |
| 16 | 23 | 3 | 6.0 | 0 | 17 | 5.0 | 2 | 25.0 + TT | 12.0 - TT |
| 17 | 23 | 3 | 4.0 | 0 | 19 | 3.5 | 2 | 25.0 + TT | 15.5 - TT |
| 18 | 23 | 3 | 8.0 | 0 | 15 | 5.0 | 2 | 25.0 + TT | 10.0 - TT |
| 19 | 23 | 3 | 10.0 | 0 | 13 | 5.0 | 2 | 25.0 + TT | 8.0 - TT |
| 20 | 23 | 3 | 4.0 | 0 | 19 | 3.5 | 2 | 25.0 + TT | 15.5 - TT |
| 21 | 23 | 3 | 6.0 | 0 | 17 | 5.0 | 2 | 25.0 + TT | 12.0 - TT |
| 22 | 23 | 3 | 4.0 | 0 | 19 | 3.5 | 2 | 25.0 + TT | 15.5 - TT |
| 23 | 23 | 3 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25.0 + TT | 16.0 - TT |
| 24 | 23 | 3 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25.0 + TT | 16.0 - TT |
| 25 | 23 | 3 | 8.0 | 0 | 15 | 5.0 | 2 | 25.0 + TT | 10.0 - TT |
| 26 | 23 | 3 | 10.0 | 0 | 13 | 5.0 | 2 | 25.0 + TT | 8.0 - TT |
| 27 | 23 | 3 | 4.0 | 0 | 19 | 3.5 | 2 | 25.0 + TT | 15.5 - TT |
| 28 | 23 | 3 | 6.0 | 0 | 17 | 5.0 | 2 | 25.0 + TT | 12.0 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-2. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-5: UE Power Class 3 test requirements (NS_04N and NS_11N) for n254

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 1 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25 +TT | 18.0 - TT |
| 2 | 23 | 1 | 4.0 | 0 | 19 | 3.5 | 2 | 25 +TT | 15.5 - TT |
| 3 | 23 | 1 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25 +TT | 18.0 - TT |
| 4 | 23 | 3 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25 +TT | 16.0 - TT |
| 5 | 23 | 3 | 6.0 | 0 | 17 | 5.0 | 2 | 25 +TT | 12.0 - TT |
| 6 | 23 | 3 | 4.0 | 0 | 19 | 3.5 | 2 | 25 +TT | 15.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-1. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-6: UE Power Class 3 test requirements (NS_05N and NS_12N) for n254

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 1 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25.0 + TT | 19.5 - TT |
| 2 | 23 | 1 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25.0 + TT | 19.5 - TT |
| 3 | 23 | 1 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25.0 + TT | 11.5 - TT |
| 4 | 23 | 1 | 7.0 | 0 | 16 | 5.0 | 2 | 25.0 + TT | 11 - TT |
| 5 | 23 | 1 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18 - TT |
| 6 | 23 | 1 | 5.0 | 0 | 18 | 4.0 | 2 | 25.0 + TT | 14 - TT |
| 7 | 23 | 1 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18 - TT |
| 8 | 23 | 1 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25.0 + TT | 19.5 - TT |
| 9 | 23 | 1 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18 - TT |
| 10 | 23 | 1 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25.0 + TT | 11.5 - TT |
| 11 | 23 | 1 | 7.0 | 0 | 16 | 5.0 | 2 | 25.0 + TT | 11 - TT |
| 12 | 23 | 1 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18 - TT |
| 13 | 23 | 1 | 5.0 | 0 | 18 | 4.0 | 2 | 25.0 + TT | 14 - TT |
| 14 | 23 | 3 | 0.0 | 0 | 20 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 15 | 23 | 3 | 3.0 | 0 | 20 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 16 | 23 | 3 | 8.0 | 0 | 15 | 5.0 | 2 | 25.0 + TT | 10 - TT |
| 17 | 23 | 3 | 10.0 | 0 | 13 | 5.0 | 2 | 25.0 + TT | 8 - TT |
| 18 | 23 | 3 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25.0 + TT | 14.5 - TT |
| 19 | 23 | 3 | 6.5 | 0 | 16.5 | 4.0 | 2 | 25.0 + TT | 12.5 - TT |
| 20 | 23 | 3 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25.0 + TT | 14.5 - TT |
| 21 | 23 | 3 | 3.0 | 0 | 20 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 22 | 23 | 3 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25.0 + TT | 14.5 - TT |
| 23 | 23 | 3 | 8.0 | 0 | 15 | 5.0 | 2 | 25.0 + TT | 10 - TT |
| 24 | 23 | 3 | 10.0 | 0 | 13 | 5.0 | 2 | 25.0 + TT | 8 - TT |
| 25 | 23 | 3 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25.0 + TT | 14.5 - TT |
| 26 | 23 | 3 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25.0 + TT | 11.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-2. |  |  |  |  |  |  |  |  |  |

<<< END OF CHANGES >>>