3GPP TSG-RAN5 Meeting #108	R5-254884r2

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3596 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | update to NR-NTN specific common clause |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM Europe Inc. - Italy |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For RF, DEMOD, SIG and RRM NR-NTN test cases, the expectation is that the UE as a pre-test condition disables its satellite trajectory prediction so that it does not pre-compensate the doppler and delay under the assumption satellite is moving before the expiry of ntn-UlSyncValidityDuration-r17. This pre-condition is not updated in every single test case procedure and moreover is better suited to be clarified in a common test environment clause. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added pre condition for all RF, DEMOD, SIG and RRM NR-NTN test case in the corresponding common test environment clause for UE to deactivate its satellite trajectory prediction via pre-configured means |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE might fail |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.4, 7.6.4 (new), 6.3.4.0 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: added similar note to disable satellite trajectory prediction for all SIG test cases in clause 6.3.4.0 R2: clarified that for SIG, satellite trajectory prediction disablement is only needed for NGSO. For GSO, it does not make any difference since satellite velocity is configured as 0 Km/s. |  |  |  |  |  |  |  |  |

## 5.6	Test environment for NTN testing

This section defines the test environment which applies to all RF and DEMOD test cases executed for NR NTN UEs, unless otherwise specified.

### 5.6.1	UE location

UE shall determine its location during the test using any of the following means.

1.	UE location for the test shall be provided to the UE via pre-configured means. During the test the UE location is not expected to change unless explicitly stated as a requirement for the test.

2.	Other options such as providing UE location via AT command are not precluded.

UE location provided to the UE before the start of the test is as follows for RF Tx/Rx and DEMOD test cases for both GSO and NGSO config is as follows:

Longitude: 121.56076999

Latitude: 25.08439333 (NGSO satellites), 55.0 (GSO satellites)

Altitude: 0

### 5.6.2	Ephemeris Information

Ephemeris information in this section has been generated as described in clause 5 in TR38.905 [69].

#### 5.6.2.0	Assumptions for Ephemeris generation

This section provides the assumptions used to generate Ephemeris information for GSO and NGSO orbits:

1.	Ephemeris files are generated for 3 types of satellites: GSO (inclination equal to 7º), LEO-600 and LEO-1200.

2.	UE location is as defined in section 5.6.1.

3.	PositionVelocity state vectors are used for EphemerisInfo signaling.

4.	For GSO, an altitude of 35,786 km is considered. For NGSO, 2 different altitudes are considered: 600 km and 1200 km.

5.	One-way delay between UE and satellite to be in the range of 119.375 ms to 128.79 ms for GSO and between 2 ms (lowest value for LEO orbit 600 km) and 6.67 ms (highest value for LEO orbit 1200 km)

6.  Doppler (frequency) shift up to [0.93] ppm for GSO and up to 24ppm for NGSO (LEO-600).

#### 5.6.2.1	Ephemeris for zero Doppler conditions

All NR NTN test cases defined in sections 6, 7 and 8 in TS 38.521-5 [57], unless otherwise stated, shall use the ephemeris configuration in tables 5.6.2.1-1 to 5.6.2.1-3, corresponding to zero Doppler and constant delay for the type of satellite under test, keeping such information constant throughout the duration of each measurement in the test:

-	In case UE supports only NGSO satellites, ephemeris values in Table 5.6.2.1-3 shall be used except for test cases 8.2.1.2, where ephemeris values in Table 5.6.1.2.1-2 shall be used.

-	In case UE supports both GSO and NGSO satellites, ephemeris values in Table 5.6.1.2.1-1 shall be used except for test cases 8.2.1.2, where ephemeris values in Table 5.6.1.2.1-2 shall be used.

Table 5.6.2.1-1: SIB19- NR NTN Ephemeris information for GSO satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -16944523 |  |  |
| positionY-r17 | 27362209 |  |  |
| positionZ-r17 | 4019393 |  |  |
| velocityVX-r17 | -336 |  |  |
| velocityVY-r17 | -208 |  |  |
| velocityVZ-r17 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 35.04 degrees, one-way delay equal to 127.33 ms and Doppler equal to 4.89E-06 ppm |  |  |  |

Table 5.6.2.1-2: SIB19- NR NTN Ephemeris information for NGSO (LEO-600) satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2550081 |  |  |
| positionY-r17 | 4151097 |  |  |
| positionZ-r17 | 2263034 |  |  |
| velocityVX-r17 | 21970 |  |  |
| velocityVY-r17 | -48164 |  |  |
| velocityVZ-r17 | 114109 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.61 degrees, one-way delay equal to 2.03 ms and Doppler equal to 0.013 ppm |  |  |  |

Table 5.6.2.1-3: SIB19- NR NTN Ephemeris information for NGSO (LEO1200) satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2771542 |  |  |
| positionY-r17 | 4509243 |  |  |
| positionZ-r17 | 2455572 |  |  |
| velocityVX-r17 | 20837 |  |  |
| velocityVY-r17 | -46176 |  |  |
| velocityVZ-r17 | 109528 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.54 degrees, one-way delay equal to 4.04 ms and Doppler equal to 3.6E-3 ppm |  |  |  |

### 5.6.3	NR NTN message contents for RF Tx/Rx, Demodulation tests

#### 5.6.3.1	SIB19 IE values

The ntn-config-r17 values including the EphemerisInfo-r17 values in SIB19 shall be periodically signalled to the UE during test based on the Ephemeris information in clause 5.6.2 (kept constant throughout the duration of each measurement in the test).

Table 5.6.3.1-1: SIB19 for RF tests

| Derivation Path: Table 4.6.2-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17::= SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ntn-UlSyncValidityDuration-r17 | s240 |  |  |
| cellSpecificKoffset-r17 | 258 |  | GSO |
|  | 14 |  | NGSO |
| kmac-r17 | Not present |  |  |
| ta-Info-r17 SEQUENCE { |  |  |  |
| ta-Common-r17 | 0 |  |  |
| ta-CommonDrift-r17 | 0 |  |  |
| ta-CommonDriftVariant-r17 | 0 |  |  |
| } |  |  |  |
| ntn-PolarizationDL-r17 | Not present |  |  |
| ntn-PolarizationUL-r17 | Not present |  |  |
| ephemerisInfo-r17 | See cl 5.6.2 |  |  |
| } |  |  |  |
| ta-Report-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

### 5.6.4	Satellite trajectory prediction

Unless otherwise specified, UE prediction of satellite trajectory shall be deactivated by any preconfigured means in the pre-test condition.

NOTE:	As constant ephemeris information in SIB19 is used in the test, the frequency doppler and transmission delay values remain same during the test execution. Therefore, prediction of satellite trajectory needs to be deactivated in the UE, so that the UE shall not update the doppler and delay pre-compensation with the assumption that the satellite is moving before the expiry of ntn-UlSyncValidityDuration-r17.

<<several sections skipped>>

### 6.3.4	Satellite assistance information for NTN

#### 6.3.4.0	Satellite trajectory prediction

Unless otherwise specified, for NGSO scenarios UE prediction of satellite trajectory shall be deactivated by any preconfigured means in the pre-test condition.

NOTE:	As constant ephemeris information in SIB19 is used in the test, the frequency doppler and transmission delay values remain same during the test execution. Therefore, prediction of satellite trajectory needs to be deactivated in the UE, so that the UE shall not update the doppler and delay pre-compensation with the assumption that the satellite is moving before the expiry of ntn-UlSyncValidityDuration-r17.

#### 6.3.4.1	GSO scenario

The position of GSO satellite is defined in Table 4.6.3-56C for NTN with below assumption:

-	Inclination: 0

-	Eccentricity: 0

-	Latitude: 0

-	Longitude: 121.56076999

-	Altitude: 35786 km

-	Speed: 0 km/s

In case of multi-cell NTN GSO scenario, a maximum of 3 NTN cells can be used. The relevant assistance information for satellite positioning and timing adjustment calculations is defined in Table 6.3.4.1-1 unless otherwise specified in a test case.

Table 6.3.4.1-1: Satellite assistance information in multi-cell NTN GSO scenario

| Cell type (Note 1) | positionX | positionY | positionZ | velocityVX/Y/Z | nta-Common | cellSpecificKoffset |
| --- | --- | --- | --- | --- | --- | --- |
| First | -16976014 | 27636499 | 0 | 0 | 65896378 | 550 |
| Second | -23550424 | 22301093 | 0 | 0 | 66485757 | 550 |
| Third | -14502736 | 29010891 | 0 | 0 | 66485757 | 550 |
| Note 1: First Cell is the cell defined as default serving cell in the test case at the end of the preamble. Second Cell and third Cell are used in multi-cell test cases with Second Cell being defined as the 1st neighbour cell and Third Cell being defined as the 2nd neighbour cell. For example, in TS 38.523-1[12] TC 9.4.1.1 running in GSO scenario, NGC Cell A is the serving cell in the preamble, so it uses the satellite assistance information of First Cell, while NGC Cell G uses the satellite assistance information of Second Cell. |  |  |  |  |  |  |

#### 6.3.4.2	NGSO scenario

The position of NGSO satellite is defined in Table 4.6.3-56C for NTN with below assumption:

-	Inclination: 90

-	Eccentricity: 0

-	Longitude: 121.56076999

-	Altitude: 600 km

-	Speed: 7.56 km/s

The UE position defined in clause 4.5.C is the projection position from one of the satellite information to the ground.

All NTN test cases defined in TS 38.523-1 [12], unless otherwise stated, shall use the first row of Table 6.3.4.2-1 as the assistance information for configuration of the serving cell with a specific location, corresponding to zero Doppler and constant delay for NGSO scenario, keeping such information constant throughout the duration of each measurement in the test.

In case of multi-cell NTN NGSO scenario, a maximum of 3 NTN cells can be used. The satellite assistance information for satellite positioning and timing adjustement calculations is defined in Table 6.3.4.2-1 unless otherwise specified in a test case, keeping such information constant throughout the duration of each measurement in the test.

Table 6.3.4.2-1: Satellite assistance information in multi-cell NTN NGSO scenario

| Cell type (Note 1) | positionX | positionY | positionZ | velocityVX | velocityVY | velocityVZ | kMac | cellSpecifickOffset | Fd (Note 2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| First | -2550081 | 4151097 | 2263034 | 21970 | -48164 | 114109 | 3 | 5 | 0.013 |
| Second | -2654549 | 4386991 | 1594205 | 14581 | -34487 | 120182 | 4 | 8 | 19.83 |
| Third | -2401021 | 3830217 | 2907041 | 28879 | -61410 | 105774 | 4 | 8 | -19.95 |
| Note 1: First Cell is the cell defined as default serving cell in the test case at the end of the preamble. Second Cell and Third Cell are used in multi-cell test cases with Second Cell being defined as the 1st neighbour cell and Third Cell being defined as the 2nd neighbour cell. For example, in TS 38.523-1[12] TC 9.4.1.1 running in NGSO scenario, NGC Cell A is the serving cell in the preamble, so it uses the satellite assistance information of First Cell, while NGC Cell G uses the satellite assistance information of Second Cell. Note 2: Fd is the frequency Doppler in the service link, unit is ppm. The values are not broadcasted but resemble those that are calculated at the UE based on the corresponding cell satellite assistance information. They need to be considered for UL & DL transmission in SS side. |  |  |  |  |  |  |  |  |  |

<<several sections skipped>>

## 7.6	Test environment for NTN testing

This section defines the test environment which applies to all RRM test cases executed for NR NTN UEs, unless otherwise specified.

### 7.6.1	UE location

Same as in section 5.6.1.

### 7.6.2	Ephemeris Information

Ephemeris information in this section has been generated as described in clause 5 in TR38.905 [69].

#### 7.6.2.0	Assumptions for Ephemeris generation

Same as in section 5.6.2.0.

#### 7.6.2.1	Ephemeris for elevation angle 30º

All NR NTN test cases defined in in section 14 in TS 38.533 [18], unless otherwise stated, shall use ephemeris configuration in tables 7.6.2.1-1a to 7.6.2.1-3b, corresponding to an elevation angle of 30º for the type of satellite under test, keeping such information constant throughout the duration of the test:

-	In case UE supports only NGSO satellites, ephemeris values in Table 7.6.2.1-3a or 7.6.2.1-3b shall be used.

Table 7.6.2.1-1a: SIB19- NR NTN serving cell Ephemeris information (stateVectors format) for GSO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -17129898 |  |  |
| positionY-r17 | 27505094 |  |  |
| positionZ-r17 | 1413939 |  |  |
| velocityVX-r17 | 116 |  |  |
| velocityVY-r17 | 378 |  |  |
| velocityVZ-r17 | -5944 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to -0.15 ppm. NOTE 2: stateVectors ephemeris format shall be used in NR NTN RRM test cases listed in Table A.3.36.3-1 in TS38.133 [13]. |  |  |  |

Table 7.6.2.1-1b: SIB19- NR NTN serving cell Ephemeris information (orbitalParameters format) for GSO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| orbital-r17 SEQUENCE { |  |  |  |
| semiMajorAxis-r17 | 8393544277 |  |  |
| eccentricity-r17 | 0 |  |  |
| periapsis-r17 | 266532360 |  |  |
| longitude-r17 | 224998499 |  |  |
| inclination-r17 | 2125399 |  |  |
| meanAnomaly-r17 | 118496066 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to -0.15 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in NR NTN RRM test cases listed in Table A.3.36.3-2 in TS38.133 [13]. |  |  |  |

Table 7.6.2.1-1c: SIB19- NR NTN neighbour cells Ephemeris information (stateVectors format) for GSO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-NeighCellConfig-r17 SEQUENCE { (SIZE(1..maxCellNTN-r17)) OF NTN-NeighCellConfig-r17{ | 1 entry for test cases requiring only one neighbour cell 2 entries in test cases requiring two neighbour cells |  | 2CELLS OR 3CELLS |
| NTN-NeighCellConfig-r17 [1] SEQUENCE { |  | entry 1 | 2CELLS OR 3CELLS |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -25235934 |  |  |
| positionY-r17 | 20178903 |  |  |
| positionZ-r17 | 2814664 |  |  |
| velocityVX-r17 | 298 |  |  |
| velocityVY-r17 | -241 |  |  |
| velocityVZ-r17 | 4398 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NTN-NeighCellConfig-r17 [2] SEQUENCE { |  | entry 2 | 3CELLS |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -6457459 |  |  |
| positionY-r17 | 31531426 |  |  |
| positionZ-r17 | 4004156 |  |  |
| velocityVX-r17 | -387 |  |  |
| velocityVY-r17 | -66 |  |  |
| velocityVZ-r17 | -104 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Neighbour cell 1 Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to +0.11 ppm. Neighbour cell 2 Satellite-UE elevation angle equal to 32.17 degrees, one-way delay equal to 128.14 ms and Doppler equal to 0 ppm. NOTE 2: stateVectors ephemeris format shall be used in NR NTN RRM test cases listed in Table A.3.36.3-1 in TS38.133 [13]. |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2CELLS | Test cases where one serving cell and one neighbour cell are required |
| 3CELLS | Test cases where one serving cell and two neighbour cells are required |

Table 7.6.2.1-1d: SIB19- NR NTN neighbour cells Ephemeris information (orbitalParameters format) for GSO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-NeighCellConfig-r17 SEQUENCE { (SIZE(1..maxCellNTN-r17)) OF NTN-NeighCellConfig-r17 { | 1 entry for test cases requiring only one neighbour cell 2 entries in test cases requiring two neighbour cells |  | 2CELLS OR 3CELLS |
| NTN-NeighCellConfig-r17 [1] SEQUENCE { |  | entry 1 | 2CELLS OR 3CELLS |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| orbital-r17 SEQUENCE { |  |  |  |
| semiMajorAxis-r17 | 8393544277 |  |  |
| eccentricity-r17 | 0 |  |  |
| periapsis-r17 | 137910436 |  |  |
| longitude-r17 | 105383807 |  |  |
| inclination-r17 | 67072560 |  |  |
| meanAnomaly-r17 | 32981819 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NTN-NeighCellConfig-r17 [2] SEQUENCE { |  | entry 2 | 3CELLS |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| orbital-r17 SEQUENCE { |  |  |  |
| semiMajorAxis-r17 | 8393544277 |  |  |
| eccentricity-r17 | 0 |  |  |
| periapsis-r17 | 249232221 |  |  |
| longitude-r17 | 228396393 |  |  |
| inclination-r17 | 12328441 |  |  |
| meanAnomaly-r17 | 67231401 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Neighbour cell 1 Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to +0.11 ppm. Neighbour cell 2 Satellite-UE elevation angle equal to 32.17 degrees, one-way delay equal to 128.14 ms and Doppler equal to 0 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in NR NTN RRM test cases listed in Table A.3.36.3-2 in TS38.133 [13]. |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2CELLS | Test cases where one serving cell and one neighbour cell are required |
| 3CELLS | Test cases where one serving cell and two neighbour cells are required |

Table 7.6.2.1-2a: Void

Table 7.6.2.1-2b: Void

Table 7.6.2.1-3a: SIB19- NR NTN serving cell Ephemeris information (stateVectors format) for NGSO (LEO-1200) satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2935552 |  |  |
| positionY-r17 | 4895460 |  |  |
| positionZ-r17 | 1188458 |  |  |
| velocityVX-r17 | 8506 |  |  |
| velocityVY-r17 | -23314 |  |  |
| velocityVZ-r17 | 118253 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.03 degrees, one-way delay equal to 6.67 ms and Doppler equal to 17.55 ppm. NOTE 2: stateVectors ephemeris format shall be used in NR NTN RRM test cases listed in Table A.3.36.3-1 in TS38.133 [13]. |  |  |  |

Table 7.6.2.1-3b: SIB19- NR NTN serving cell Ephemeris information (orbitalParameters format) for NGSO (LEO-1200) satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| orbital-r17 SEQUENCE { |  |  |  |
| semiMajorAxis-r17 | 273626880 |  |  |
| eccentricity-r17 | 769300 |  |  |
| periapsis-r17 | 198830479 |  |  |
| longitude-r17 | 89834718 |  |  |
| inclination-r17 | 65473992 |  |  |
| meanAnomaly-r17 | 7822790 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.03 degrees, one-way delay equal to 6.67 ms and Doppler equal to 17.55 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in NR NTN RRM test cases listed in Table A.3.36.3-2 in TS38.133 [13]. |  |  |  |

Table 7.6.2.1-3c: SIB19- NR NTN neighbour cells Ephemeris information (stateVectors format) for NGSO (LEO-1200) satellites (serving cell 30º elevation angle conditions)

| Derivation Path: TS 38.508-1 [12], table 4.6.2-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-NeighCellConfig-r17 SEQUENCE { (SIZE(1..maxCellNTN-r17)) OF NTN-NeighCellConfig-r17{ | 1 entry for test cases requiring only one neighbour cell 2 entries in test cases requiring two neighbour cells |  | 2CELLS OR 3CELLS |
| NTN-NeighCellConfig-r17 [1] SEQUENCE { |  | entry 1 | 2CELLS or 3CELLS |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2470752 |  |  |
| positionY-r17 | 3858366 |  |  |
| positionZ-r17 | 3624634 |  |  |
| velocityVX-r17 | 31542 |  |  |
| velocityVY-r17 | -67516 |  |  |
| velocityVZ-r17 | 94597 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NTN-NeighCellConfig-r17 [2] SEQUENCE { |  | entry 2 | 3CELLS |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2771542 |  |  |
| positionY-r17 | 4509243 |  |  |
| positionZ-r17 | 2455572 |  |  |
| velocityVX-r17 | 20837 |  |  |
| velocityVY-r17 | -46176 |  |  |
| velocityVZ-r17 | 109528 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Neighbour cell 1 Satellite-UE elevation angle equal to 150.02 degrees, one-way delay equal to 6.79 ms and Doppler equal to -17.65 ppm. Neighbour cell 2 Satellite-UE elevation angle equal to 89.54 degrees, one-way delay equal to 4.04 ms and Doppler equal to 0 ppm. NOTE 2: stateVectors ephemeris format shall be used in NR NTN RRM test cases listed in Table A.3.36.3-1 in TS38.133 [13]. |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2CELLS | Test cases where one serving cell and one neighbour cell are required |
| 3CELLS | Test cases where one serving cell and two neighbour cells are required |

Table 7.6.2.1-3d: SIB19- NR NTN neighbour cells Ephemeris information (orbitalParameters format) for NGSO (LEO-1200) satellites (serving cell 30º elevation angle conditions)

| Derivation Path: TS 38.508-1 [12], table 4.6.2-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-NeighCellConfig-r17 SEQUENCE { (SIZE(1..maxCellNTN-r17)) OF NTN-NeighCellConfig-r17{ | 1 entry for test cases requiring only one neighbour cell 2 entries in test cases requiring two neighbour cells |  | 2CELLS OR 3CELLS |
| NTN-NeighCellConfig-r17 [1] SEQUENCE { |  | entry 1 | 2CELLS OR 3CELLS |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| orbital-r17 SEQUENCE { |  |  |  |
| semiMajorAxis-r17 | 272189533 |  |  |
| eccentricity-r17 | 715030 |  |  |
| periapsis-r17 | 199909097 |  |  |
| longitude-r17 | 89257064 |  |  |
| inclination-r17 | 64358188 |  |  |
| meanAnomaly-r17 | 27947120 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NTN-NeighCellConfig-r17 [2] SEQUENCE { |  | entry 2 | 3CELLS |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 ::= CHOICE { |  |  |  |
| orbital-r17 SEQUENCE { |  |  |  |
| semiMajorAxis-r17 | 273057241 |  |  |
| eccentricity-r17 | 744972 |  |  |
| periapsis-r17 | 197978669 |  |  |
| longitude-r17 | 89682433 |  |  |
| inclination-r17 | 65034798 |  |  |
| meanAnomaly-r17 | 17169725 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Neighbour cell 1 Satellite-UE elevation angle equal to 150.02 degrees, one-way delay equal to 6.79 ms and Doppler equal to -17.65 ppm. Neighbour cell 2 Satellite-UE elevation angle equal to 89.54 degrees, one-way delay equal to 4.04 ms and Doppler equal to 0 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in NR NTN RRM test cases listed in Table A.3.36.3-2 in TS38.133 [13]. |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2CELLS | Test cases where one serving cell and one neighbour cell are required |
| 3CELLS | Test cases where one serving cell and two neighbour cells are required |

#### 7.6.2.2	Ephemeris for NR NTN RRM UL timing accuracy test cases

Table 7.6.2.2-1: SIB19- NR NTN serving cell Ephemeris information for GSO satellites (minimum Doppler)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -16944523 |  |  |
| positionY-r17 | 27362209 |  |  |
| positionZ-r17 | 4019393 |  |  |
| velocityVX-r17 | -336 |  |  |
| velocityVY-r17 | -208 |  |  |
| velocityVZ-r17 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 35.04 degrees, one-way delay equal to 127.33 ms and Doppler equal to 4.89E-06 ppm |  |  |  |

Table 7.6.2.2-2: SIB19- NR NTN serving cell Ephemeris information for NGSO (LEO-1200) satellites (minimum Doppler)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2771542 |  |  |
| positionY-r17 | 4509243 |  |  |
| positionZ-r17 | 2455572 |  |  |
| velocityVX-r17 | 20837 |  |  |
| velocityVY-r17 | -46176 |  |  |
| velocityVZ-r17 | 109528 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.54 degrees, one-way delay equal to 4.04 ms and Doppler equal to 3.6E-3 ppm. |  |  |  |

Table 7.6.2.2-3: SIB19- NR NTN serving cell Ephemeris information for GSO satellites (maximum positive Doppler)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SIB19-r17 ::= SEQUENCE { |  |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |  |
| epochTime-r17 | Not present |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |  |
| positionX-r17 | -16989970 |  |  |  |
| positionY-r17 | 27591785 |  |  |  |
| positionZ-r17 | 1413211 |  |  |  |
| velocityVX-r17 | 390 |  |  |  |
| velocityVY-r17 | -65 |  |  |  |
| velocityVZ-r17 | 5945 |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to 0.15 ppm |  |  |  |  |

Table 7.6.2.2-4: SIB19- NR NTN serving cell Ephemeris information for NGSO (LEO-600) satellites (maximum positive Doppler)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SIB19-r17 ::= SEQUENCE { |  |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |  |
| epochTime-r17 | Not present |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |  |
| positionX-r17 | -2654549 |  |  |  |
| positionY-r17 | 4386991 |  |  |  |
| positionZ-r17 | 1594205 |  |  |  |
| velocityVX-r17 | 14581 |  |  |  |
| velocityVY-r17 | -34487 |  |  |  |
| velocityVZ-r17 | 120182 |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.11 degrees, one-way delay equal to 3.60 ms and Doppler equal to 19.83 ppm. NOTE 2: This configuration can be skipped as it is the same as default 30º elevation angle |  |  |  |  |

Table 7.6.2.2-5: SIB19- NR NTN serving cell Ephemeris information for GSO satellites (maximum negative Doppler)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SIB19-r17 ::= SEQUENCE { |  |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |  |
| epochTime-r17 | Not present |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |  |
| positionX-r17 | -17129898 |  |  |  |
| positionY-r17 | 27505094 |  |  |  |
| positionZ-r17 | 1413939 |  |  |  |
| velocityVX-r17 | 116 |  |  |  |
| velocityVY-r17 | 378 |  |  |  |
| velocityVZ-r17 | -5944 |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to -0.15 ppm. NOTE 2: This configuration can be skipped as it is the same as default 30º elevation angle. |  |  |  |  |

Table 7.6.2.2-6: SIB19- NR NTN serving cell Ephemeris information for NGSO (LEO-600) satellites (maximum negative Doppler)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SIB19-r17 ::= SEQUENCE { |  |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |  |
| epochTime-r17 | Not present |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |  |
| positionX-r17 | -2401021 |  |  |  |
| positionY-r17 | 3830217 |  |  |  |
| positionZ-r17 | 2907041 |  |  |  |
| velocityVX-r17 | 28879 |  |  |  |
| velocityVY-r17 | -61410 |  |  |  |
| velocityVZ-r17 | 105774 |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 149.91 degrees, one-way delay equal to 3.66 ms and Doppler equal to -19.95 ppm |  |  |  |  |

### 7.6.3	NR NTN message contents for RRM tests

#### 7.6.3.1	SIB19 IE values

The ntn-config-r17 values including the EphemerisInfo-r17 values in SIB19 shall be periodically signalled to the UE during test based on the Ephemeris information in clause 7.6.2 (kept constant throughout the duration of each the test).

Table 7.6.3.1-1: SIB19 for RRM tests

| Derivation Path: Table 4.6.2-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17::= SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ntn-UlSyncValidityDuration-r17 | s900 |  | GSO |
|  | s5 |  | NGSO |
| cellSpecificKoffset-r17 | 258 |  | GSO |
|  | 14 |  | NGSO |
| kmac-r17 | Not present |  |  |
| ta-Info-r17 SEQUENCE { |  |  |  |
| ta-Common-r17 | 0 |  |  |
| ta-CommonDrift-r17 | 0 |  |  |
| ta-CommonDriftVariant-r17 | 0 |  |  |
| } |  |  |  |
| ntn-PolarizationDL-r17 | linear |  |  |
| ntn-PolarizationUL-r17 | linear |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  | TC_EF_SV |
| positionX-r17 | See cl. 7.6.2 |  |  |
| positionY-r17 | See cl. 7.6.2 |  |  |
| positionZ-r17 | See cl. 7.6.2 |  |  |
| velocityVX-r17 | See cl. 7.6.2 |  |  |
| velocityVY-r17 | See cl. 7.6.2 |  |  |
| velocityVZ-r17 | See cl. 7.6.2 |  |  |
| } |  |  |  |
| orbital-r17 SEQUENCE { |  |  | TC_EF_OP |
| semiMajorAxis-r17 | See cl. 7.6.2 |  |  |
| eccentricity-r17 | See cl. 7.6.2 |  |  |
| periapsis-r17 | See cl. 7.6.2 |  |  |
| longitude-r17 | See cl. 7.6.2 |  |  |
| inclination-r17 | See cl. 7.6.2 |  |  |
| meanAnomaly-r17 | See cl. 7.6.2 |  |  |
| } |  |  |  |
| } |  |  |  |
| ta-Report-r17 | Not present |  |  |
| } |  |  |  |
| ntn-NeighCellConfig-r17 | Not present |  | 1CELL |
|  | See cl. 7.6.2 |  | 2CELLS |
|  | See cl. 7.6.2 |  | 3CELLS |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TC_EF_SV | Test cases using ephemeris with Position Velocity format according to section 7.6.2 |
| TC_EF_OP | Test cases using ephemeris with Orbital format according to section 7.6.2 |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |
| 1CELL | Test cases where only one cell (serving cell) is required |
| 2CELLS | Test cases where one serving cell and one neighbour cell are required |
| 3CELLS | Test cases where one serving cell and two neighbour cells are required |

### 7.6.4	Satellite trajectory prediction

Unless otherwise specified, UE prediction of satellite trajectory shall be deactivated by any preconfigured means in the pre-test condition.

NOTE:	As constant ephemeris information in SIB19 is used in the test, the frequency doppler and transmission delay values remain same during the test execution. Therefore, prediction of satellite trajectory needs to be deactivated in the UE, so that the UE shall not update the doppler and delay pre-compensation with the assumption that the satellite is moving before the expiry of ntn-UlSyncValidityDuration-r17.