3GPP TSG-RAN5 Meeting #109	R5-256095r1

Dallas, USA, November 17-21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.579-6 | CR | 0018 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 6.3.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung R&D Institute UK, Ericsson, FirstNet, Element |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc17_enh3MCPTT_eMCData3_MCOver5GS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test case ‘6.3.4 On-network / Emergency Alert / Emergency alert area’ included in the Work Plan, is not implemented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new test case ‘6.3.4 On-network / Emergency Alert / Emergency alert area’ according to the Work Plan. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case ‘6.3.4 On-network / Emergency Alert / Emergency alert area’ included in the Work Plan will not be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.579-4 CR 0007 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1 - Added missing '}' in the test purpose. - In the Main Behaviour "a emergency" is replaced by "an emergency" - At steps 2a1 and 5a1 pc_MCX_DisplayInfoEmergencyAlertArea check has been added - Updated step "10a" to be "5a1" - In Table 6.3.4.3.3-2 "Emergency-alert-area-ind" has been replaced by "emergency-alert-area-ind" - Modified step 3 to do procedure 'MCX SIP MESSAGE Request - Accept CO' to request an emergency alert providing location information. - Added test purposes in clause 6.3.4.1 for.steps 3-5a1 where pass verdicts for TP 2 and 3 assigned -Updated step numbers -Updated Table 6.3.4.3.3-3 to Table 6.3.4.3.3-7 to align with test procedure providing location information -Added Tables 6.3.4.3.3-8 and Table 6.3.4.3.3-9 to align with test procedure for step 4 |  |  |  |  |  |  |  |  |

Table 6.3.3.3.3-10: PIDF in SIP PUBLISH (Table 6.3.3.3.3-8)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.5.1-2, condition AFFILIATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  |  |  |
| tuple |  |  |  |  |
| status | Not present |  |  |  |
| p-id | any allowed value | p-id shall be present acc. to TS 24.281 [26] clause 8.2.2.2.5 and clause 8.2.2.3.5 |  |  |

### 6.3.4	On-network / Emergency Alert / Emergency alert area

6.3.4.1	Test Purpose (TP)

(1)

with { UE (MCVideo Client) registered and authorized for MCVideo Service, including authorized to notify of entry into or exit from an emergency alert }

ensure that {
  when { UE (MCVideo Client) receives an emergency alert via s SIP MESSAGE message with the

<emergency-alert alert-ind> element in the application/vnd.3gpp.mcvideo-info+xml MIME body set to a value of either "true" or "false" }

then { UE (MCVideo Client) responds by sending a SIP 200 (OK) message }

}

(2)

with { UE (MCVideo Client) having been previously notified of entry into an emergency alert area }

ensure that {

when { the MCVideo User requests to send an emergency alert with the location of emergency }

then { UE (MCVideo Client) sends a SIP MESSAGE initiating an emergency alert and reporting location information }

}

(3)

with { UE (MCVideo Client) in the “MVEA 3: emergency-alert-initiated” state}

ensure that {

when { the MCVideo Server notifies the UE (MCVideo Client) of exit from an emergency alert area by sending the UE (MCVideo Client) a SIP MESSAGE }

then { UE (MCVideo Client) acknowledges the exit from emergency alert area notification by sending a SIP 200 (OK) response and notifies the user of the exit from the emergency alert area }

}

6.3.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.281 clauses 11.2.1.1 and 11.2.1.5. Unless otherwise stated these are Rel-17 requirements.

[TS 24.281 clause 11.2.1.1]

Upon receiving a request from the MCVideo user to send an MCVideo emergency alert to the indicated MCVideo group shall determine whether the group document contains a <list-service> element that contains a <preconfigured-group-use-only> element. If a <preconfigured-group-use-only> element exists and is set to the value "true", then the MCVideo client:

1)	should indicate to the MCVideo user that alerts are not allowed on the indicated group; and

2)	shall skip the remainder of this procedure.

If this is an authorised request for an MCVideo emergency alert as determined by clause 6.2.8.1.6, the MCVideo client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [11] and IETF RFC 3428 [17] with the clarifications given below.

NOTE 1:	this SIP MESSAGE request is assumed to be sent out-of-dialog.

The MCVideo client:

1)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcvideo" (coded as specified in 3GPP TS 24.229 [11]), in a P-Preferred-Service header field according to IETF RFC 6050 [14] in the SIP MESSAGE request;

2)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcvideo" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [20];

3)	may include a P-Preferred-Identity header field in the SIP MESSAGE request containing a public user identity as specified in 3GPP TS 24.229 [11];

4)	shall include an application/vnd.3gpp.mcvideo-info+xml MIME body as specified in clause F.1 with the <mcvideoinfo> element containing the <mcvideo-Params> element with:

a)	the <mcvideo-request-uri> element set to the group identity;

b)	the <alert-ind> element set to a value of "true";

c)	the <mcvideo-client-id> element set to the MCVideo client ID of the originating MCVideo client;

d)	if the MCVideo client needs to include an active functional alias in the SIP MESSAGE request, the <anyExt> element with the <functional-alias-URI> element set to the URI of the used functional alias; and

f)	if the MCVideo user has requested an application priority, the <anyExt> element with the <user-requested-priority> element set to the user provided value.

NOTE 1A:	The MCVideo client learns the functional aliases that are activated for an MCVideo ID from procedures specified in clause 20.2.1.3.

5)	shall include an application/vnd.3gpp.mcvideo-location-info+xml MIME body as specified in clause F.3 with a <Report> element included in the <location-info> root element;

6)	shall include in the <Report> element the specific location information configured for the MCVideo emergency alert location trigger;

7)	shall set the MCVideo emergency state if not already set;

8)	shall set the MCVideo emergency alert state to "MVEA 2: emergency-alert-confirm-pending";

9)	shall set the Request-URI to the public service identity identifying the participating MCVideo function serving the group identity; and

10)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [11].

On receiving a SIP 2xx response to the SIP MESSAGE request, the MCVideo client shall set the MCVideo emergency alert state to "MVEA 3: emergency-alert-initiated".

On receiving a SIP 4xx response a SIP 5xx response or a SIP 6xx response to the SIP MESSAGE request, the MCVideo client shall set the MCVideo emergency alert state to "MVEA 1: no-alert".

NOTE 2:	the MCVideo emergency state is left set in this case as the MCVideo user presumably is in the best position to determine whether or not they are in a life-threatening condition. The assumption is that the MCVideo user can clear the MCVideo emergency state manually if need be.

[TS 24.281 clause 11.2.1.5]

Upon receipt of a "SIP MESSAGE request for notification of entry into or exit from an emergency alert area", the MCVideo client:

1)	if the received SIP MESSAGE request contains an application/vnd.3gpp.mcvideo-info+xml MIME body with the <emergency-alert-area-ind> element of the value:

a)	set to "true":

i)	may display to the MCVideo user an indication that MCVideo client has entered a pre-defined emergency alert area; and

ii)	if the MCVideo user is not in emergency state, shall initiate the emergency alert origination procedure as specified in clause 12.1.1.1; or

b)	set to "false":

i)	may display to the MCVideo user an indication that MCVideo client has exited a pre-defined emergency alert area.

NOTE:	In this case, the MCVideo emergency state remains set, as the MCVideo user is in the best position to determine whether or not they are in a life-threatening condition. The MCVideo user can clear the MCVideo emergency state manually, if needed.

2)	shall generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [11]; and

3)	shall send the SIP 200 (OK) response towards the MCVideo server according to rules and procedures of 3GPP TS 24.229 [11].

6.3.4.3	Test description

6.3.4.3.1	Pre-test conditions

Test configuration:

-	Single cell configuration according to TS 37.579-1 [2] clause 5.2.2.2.1.

Preamble:

-	The UE has performed procedure 'Initial registration' as described in TS 37.579-1 [2] clause 5.4.2.

-	UE States at the end of the preamble:

-	The UE is in RRC_IDLE state.

-	The client is registered for MCVideo.

6.3.4.3.2	Test procedure sequence

Table 6.3.4.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCVideo client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 37.579-1 [2] Table 5.3.33.3-1 to receive notification of entry into an emergency alert area? | - | - | 1 | P |
| - | EXCEPTION: Step 2a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE displays an indication that an emergency alert area has been entered. | - | - | - | - |
| 2a1 | IF pc_MCX_DisplayInfoEmergencyAlertArea THEN Check: Does the UE (MCVideo client) notify the user that an emergency alert area has been entered? (NOTE 1) | - | - | 1 | P |
|  | Check: Does the UE (MCVideo client) correctly perform procedure 'MCX SIP MESSAGE Request - Accept CO' as described in TS 37.579-1 [2] Table 5.3.30.3-1 to request an emergency alert providing location information? | - | - | 2 | P |
| 4 | Check: Does the UE (MCVideo client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 37.579-1 [2] Table 5.3.33.3-1 to receive notification of exit from an emergency alert area? | - | - | 3 | P |
| - | EXCEPTION: Step 5a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE displays an indication that an emergency alert area has been exited. | - | - | - | - |
| 5a1 | IF pc_MCX_DisplayInfoEmergencyAlertArea THEN Check: Does the UE (MCVideo client) notify the user that an emergency alert area has been exited? (NOTE 1) | - | - | 3 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.3.4.3.3	Specific message contents

Table 6.3.4.3.3-1: SIP MESSAGE from the SS (step 1, Table 6.3.4.3.2-1;
step 2, TS 37.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.7.2-1, condition LOCATION-INFO, ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCVideo-Info |  |  |
| MIME-part-body | MCVideo-Info as described in Table 6.3.4.3.3-2 |  |  |  |

Table 6.3.4.3.3-2: MCVideo-Info in SIP MESSAGE (Table 6.3.4.3.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.2-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcvideoinfo |  |  |  |  |
| mcvideo-Params |  |  |  |  |
| associated-group-id | px_MCVideo_Group_A_ID |  |  |  |
| anyExt |  |  |  |  |
| emergency-alert-area-ind | true |  |  |  |

Table 6.3.4.3.3-3: SIP MESSAGE from the UE (step 3, Table 6.3.4.3.2-1;
step 2, TS 37.579-1 [2] Table 5.3.30.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.7.1-1, condition LOCATION-INFO |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCVideo-Info |  |  |
| MIME-part-body | MCVideo-Info as described in Table 6.3.4.3.3-4 |  |  |  |
| MIME body part |  | Location-info |  |  |
| MIME-part-body | Location-info as described in Table 6.3.4.3.3-5 |  |  |  |

Table 6.3.4.3.3-4: MCVideo-Info in SIP MESSAGE (Table 6.3.4.3.3-3)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.1-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcvideoinfo |  |  |  |  |
| mcvideo-Params |  |  |  |  |
| mcvideo-request-uri | Encrypted <mcvideo-request-uri> with mcvideoURI set to px_MCVideo_Group_A_ID | Encryption according to NOTE 2 in TS 37.579-1 [2] Table 5.5.3.2.1-2 |  |  |
| alert-ind | Encrypted <alert-ind> with mcvideoBoolean set to true | Encryption according to NOTE 2 in TS 37.579-1 [2] Table 5.5.3.2.1-2 |  |  |
| mcvideo-client-id | encrypted <mcvideo-client-id> with mcvideoString set to the UE’s UUID URN | Encryption according to NOTE 2 in TS 37.579-1 [2] Table 5.5.3.2.1-2 |  |  |

Table 6.3.4.3.3-5: Location-Info in SIP MESSAGE (Table 6.3.4.3.3-3)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.4.1-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Report |  |  |  |  |
| CurrentLocation |  |  |  |  |
| CurrentCoordinate |  |  |  |  |
| longitude | Encrypted <longitude> with any content | Encryption according to NOTE 1 in TS 37.579-1 [2] Table 5.5.3.4.1-2 |  |  |
| latitude | Encrypted <latitude> with any content | Encryption according to NOTE 1 in TS 37.579-1 [2] Table 5.5.3.4.1-2 |  |  |

Table 6.3.4.3.3-6: SIP MESSAGE from the SS (step 3, Table 6.3.4.3.2-1;
step 4, TS 37.579-1 [2] Table 5.3.30.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.7.2-2, condition ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCVideo-Info |  |  |
| MIME-part-body | MCVideo-Info as described in Table 6.3.4.3.3-7 |  |  |  |

Table 6.3.4.3.3-7: MCVideo-Info in SIP MESSAGE (Table 6.3.4.3.3-6)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.2-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcvideoinfo |  |  |  |  |
| mcvideo-Params |  |  |  |  |
| mcvideo-calling-user-id | not present |  |  |  |
| alert-ind | Encrypted <alert-ind> with mcvideoBoolean set to true | Encryption according to NOTE 1 in TS 37.579-1 [2] Table 5.5.3.2.2-2 |  |  |
| mcvideo-client-id | Encrypted <mcvideo-client-id> with mcvideoString set to the client-id as received from the UE | Encryption according to NOTE 1 in TS 37.579-1 [2] Table 5.5.3.2.2-2 |  |  |
| alert-ind-rcvd | true |  |  |  |

Table 6.3.4.3.3-8: SIP MESSAGE from the SS (step 4, Table 6.3.4.3.2-1;
step 2, TS 37.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.7.2-2, condition LOCATION-INFO, ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCVideo-Info |  |  |
| MIME-part-body | MCVideo-Info as described in Table 6.3.4.3.3-9 |  |  |  |

Table 6.3.4.3.3-9: MCVideo-Info in SIP MESSAGE (Table 6.3.4.3.3-8)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.2-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcvideoinfo |  |  |  |  |
| mcvideo-Params |  |  |  |  |
| associated-group-id | px_MCVideo_Group_A_ID |  |  |  |
| anyExt |  |  |  |  |
| emergency-alert-area-ind | false |  |  |  |