3GPP TSG RAN Meeting #109					RP-251944

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.1

Source:	Ericsson

Title:	On deployment scenarios

Document for:	Approval

# Introduction

In this paper we propose scenarios for evaluation of the 6G Radio (6GR). As the 6GR is intended to fulfill the IMT-2030 requirements, the IMT-2030 test environments need to be a subset of the evaluation scenarios used in 3GPP. Beyond these, 3GPP should study additional scenarios of interest. These 3GPP related scenarios are the focus of this paper.

A note on terminology: In ITU, a test environment is a combination of a service type, a geographical environment with associated propagation models, and system configurations. In 3GPP, a deployment scenario is a combination of a geographical environment with associated propagation models, and system configurations. Although some deployment scenarios are defined with a service in mind, the selection of which deployment scenarios a service is evaluated in is typically done outside the definition of the deployment scenario.

# 2	Discussion

The proposals for 6G deployment scenarios are based on multiple inputs; those described for 3GPP NR in 38.913, what is discussed for IMT-2030 in ITU, and additional aspects that Ericsson have identified as relevant for 6G.

In short, the main additions since NR and IMT-2020 are:

- Addition of scenarios for evaluation of sensing

- Addition of a new service: Fixed Wireless Access (FWA) to be evaluated

- Addition of a suburban macro scenario

- Addition of a new frequency band around 7GHz

- Modification of parameters for the 30GHz band

- Simultaneous use of multiple frequencies/carriers

The text below summarizes the proposed changes in the existing NR scenarios:

Indoor Hotspot (InH)

- Remove the 70 GHz carrier frequency.

- Propose additional candidate site numbers of 3 and 6 to represent less dense deployments.

- Propose a static UE speed option to capture stationary users (e.g., seated people, desktop laptops).

Dense Urban (DU)

- Propose an outdoor traffic split of 80% Outdoor / 20% Indoor.

  - To differentiate from the UMa split; otherwise, the only distinction is ISD, which could lead to merging.

  - To cover scenarios with heavy outdoor traffic, such as plazas (e.g., Times Square) and avenues.

Urban Macro (UMa)

- Propose a two-layer deployment option to study small cells on a less dense grid compared with DU. This also reflects real deployments, where small cells are deployed on grids with higher ISDs.

- Propose splitting eMBB outdoor traffic between pedestrian and in-car users.

- Propose a static MTC user speed option to capture fixed sensors.

Rural

- Propose a new traffic mix compared to the original 50/50 split, with a higher share of indoor traffic.

High-Speed

- Propose to remove the relay node–based deployment option.

New added scenarios:

Indoor Factory (InF)

- For HRLLC and positioning, propose an additional UE speed of 0 km/h to represent non-moving machines (e.g., production belts).

Suburban Macro (SMa)

- Propose a sub-1 km ISD to model more dense deployments based on real life examples. The combination of 850m and 1732m of ISD gives a more representative span of ISDs found in suburban areas.

- For eMBB traffic follow the Urban Macro UE split.

- For FWA, propose two CPE options targeting a flexible indoor/outdoor CPE installation.

The details of these additions are provided as part of the text proposals in the next Chapter.

For sensing, we propose a minimum set of scenarios in the discussion below, while we are open that the study item adds additional scenarios for this service, if identified to be necessary (e.g., sensing of humans in Rural). In addition, for UEs performing sensing, we assume this is not a handheld device, but a roadside unit (RSU) device that is more conformant with the CPE form factor and assumptions on antennas etc. This has also been reflected below.

In addition, for coverage limited scenarios, solutions related to multi-band operation (e.g. uplink in a lower band) are important for 3GPP to evaluate and to study to understand the possible needed standard support. Hence, we are supportive of e.g., 4GHz + 7GHz combinations in Urban scenario for example, as will be seen in the tables below.

When it comes to the number of BS antenna elements (except in FR2), we see it would be good to be able to compare different antennas with the same number of antenna elements but with different subarray implementation (1x2, 1x3, 1x4, 1x6, 1x8, 1x12 are common ones). Hence, choosing max number of BS antenna elements from the set N=2n, n≥1 e.g, N=1024 doesn’t allow the use of all 1024 antenna elements for 1x3, 1x6, 1x12 subarray implementation. So, we suggest the max number is instead taken from the set  N=2n*3m, n,m≥1, hence we suggest  N=144, 288, 1152 is used instead of N=128, 256, 1024.

When it comes to the number of UE antenna elements in FR2, often multi-panel implementation is assumed, where some panels are inactive while others are transmitting/receiving, here we assume that the stated numbers are the max number of simultaneously active antenna elements.

When it comes to system bandwidths in the tables below, we identify these as the upper limit, while limitations in simulation environments may require the use of smaller bandwidths. This can be discussed in the corresponding WGs for each feature.

As discussed in the introduction we have scenarios and services/use cases. In 6GR, we will have many more deployment scenarios compared to earlier. The ITU has already expanded the number of scenarios, increasing from five in ITU-2020 to ten in ITU-2030 (or thirteen, if the three positioning scenarios are also considered). 3GPP shall include the ITU scenarios, but has in addition defined new scenarios, such as Suburban macro, and intends to study additional services such as FWA, NTN, Air to Ground, high-speed trains and highway.

If we follow the monolithic approach, where each scenario corresponds to one service, the result would be several pages of scenario descriptions with a high degree of redundancy, as the same deployment assumptions would need to be repeated in multiple scenarios.

Hence, we propose to define multiple use cases/services that share the same deployment scenario assumptions within a single scenario, we can achieve a compact and efficient way of capturing the expanding need for new cases while minimizing redundancy in number of deployment scenarios.

Define multiple use cases/services that share the same deployment scenario assumptions within a single scenario

A deployment scenario is basically defined by:

- whether the BS is indoor/outdoor/sky

- the ISD of the grid and

- the propagation model (InH, InF, UMi, UMa, RMa, +their derivatives for UAV and SMa -no UAV for SMa).

The approach in the proposal above is not entirely new. For example, the UMa scenario includes two services—eMBB and URLLC as defined in TR 38.802. So overall, we have the magnificent-6 scenarios (InH, InF, Dense Urban, Urban, Suburban, Rural), along with some special structure scenarios (NTN, High-speed train, Air-to-Ground, Urban grid, Highway) and for each of these deployment scenarios a number of services or use cases can be defined.

With the modest approach of mapping services to deployment scenarios that is captured in the table below, the number of scenarios can increase from twelve for 5G to twenty for 6G.

For example, in the Urban Macro table in the next section, we can have eMBB, HRLLC, Positioning, MTC and Sensing of UAV types of use cases.

This also has the advantage that we can simulate each service independently or a combination of two or more services in a certain deployment scenario (for example eMBB and sensing services in the Urban deployment scenario). Example of the latter is described in Table A.2.4-1 of 38.802, where eMBB is multiplexed with URLLC traffic.

Note however, combining services in a single simulation (simulating different slices) can be advanced, therefore we suggest this is optional.

This table gives an overview of our proposals, where the combinations of scenarios and service types to be assumed is indicated. The yellow highlighted ones are supported by IMT-2030 while the rest are additional combinations for 3GPP evaluations to be added in 38.914. See details in the TP section below.

|  | Indoor hotspot | Indoor factory | Dense urban | Urban | Suburban | Rural | NTN | Air to Ground | High Speed train |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| eMBB | X |  | X | X | X | X | X |  |  |
| FWA |  |  |  |  | X | X | X |  |  |
| HRLLC |  | X |  | X |  |  |  |  | X |
| ISAC |  | X |  | X |  | X |  |  |  |
| Positioning |  | X |  | X |  |  |  |  |  |
| MTC |  |  |  | X |  |  | X |  |  |
| Commercial Air |  |  |  |  |  |  |  | X |  |

# 3 Text proposals

Based on the discussion above we propose the following text proposal for TR 38.914.

# 4. Deployment scenarios Editor note: Sensing related aspects will be included in section 4 High-level descriptions on deployment scenarios including carrier frequency, aggregated system bandwidth, network layout / ISD/ Orbit, BS / UE antenna elements, UE distribution / speed and service profile are proposed in this TR. It is assumed that more detailed attributes and simulation parameters, for example, the channel model, BS / UE Tx power, number of antenna ports, etc. should be defined in the new RAT study item. ## 4.1 Indoor hotspot The indoor hotspot deployment scenario focuses on small coverage per site/TRxP (transmission and reception point) and high user throughput or user density in buildings. The key characteristics of this deployment scenario are high capacity, high user density and consistent user experience indoor. Some of its attributes are listed in Table 4.1. Table 4.1: Attributes for indoor hotspot Attributes; Values or assumptions. Carrier Frequency NOTE1; Around 4 GHz Around 7 GHz Around 30 GHz. Aggregated system bandwidth NOTE2; Around 4 GHz: Up to 200 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: Up to 800 MHz (DL+UL). Layout; Single layer: - Indoor floor (Open office), 120m x 50m. ISD; 20 m (Equivalent to 12 sites per 120m x 50m) Other candidate site numbers: 3, 6. BS antenna elements; Around 4 GHz: Up to 32 Tx and Rx antenna elements Around 7 GHz: Up to 144 Tx and Rx antenna elements Around 30GHz: Up to 64 Tx and Rx antenna elements. UE antenna elements; eMBB UE “handheld” device (used in eMBB/Positioning): - Around 4 GHz: Up to 4 Tx and 4 Rx antenna elements - Around 7 GHz: Up to 4 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 8 Tx and 8 Rx antenna elements Optional eMBB UE “non-handheld” device (used in eMBB/Positioning): - Around 4 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 7 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 32 Tx and Rx antenna elements Note: For around 30 GHz, with multi-panel UE, it is the number of simultaneously used antenna elements.. Service deployment and speed options; eMBB - Profile 1 100% Indoor: 3km/h - Profile 2 (Optional, for seated people, desktop stations) 100% Indoor: 0 km/h,. Service profile; NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI.. ## 4.1X Indoor factory The indoor factory deployment scenario focuses on small coverage per site/TRxP (transmission and reception point) and high user throughput or user density in buildings. The key characteristics of this deployment scenario are high capacity, high user density and consistent user experience indoor. Some of its attributes are listed in Table 4.1X. Table 4.1X: Attributes for indoor factory Attributes; Values or assumptions. Carrier Frequency NOTE1; Around 4 GHz Around 7 GHz Around 4GHz+Around 7GHz Around 30 GHz. Aggregated system bandwidth NOTE2; Around 4 GHz: Up to 200 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: Up to 800 MHz (DL+UL). Layout; Single layer: - Indoor floor (Factory), 300m x 150m. ISD; 50m (Equivalent to 18 sites per 300m x 150m). BS antenna elements; Around 4 GHz: Up to 32 Tx and Rx antenna elements Around 7 GHz: Up to 144 Tx and Rx antenna elements Around 30GHz: Up to 64 Tx and Rx antenna elements. UE antenna elements; eMBB UE “handheld” device (used in HRLLC, Positioning): - Around 4 GHz: Up to 4 Tx and 4 Rx antenna elements - Around 7 GHz: Up to 4 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 8 Tx and 8 Rx antenna elements Optional eMBB UE “non-handheld” device (used in HRLLC, Sensing TxRx ): - Around 4 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 7 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 32 Tx and 32 Rx antenna elements Note: For around 30 GHz, with multi-panel UE, it is the number of simultaneously used antenna elements.. User distribution and UE speed; HRLLC: - Profile 1 100% Indoor: 3km/h - Profile 2 (Optional, stationary machine) 100% Indoor: 0 km/h Positioning: 100% Indoor: 3km/h Human sensing: - 100% Indoor - Speed: 0 km/h or 3 km/h or ~U(0, 3) km/h Sensing target assumptions as in Table 7.9.1-3 (TR. 38.901) AGV sensing: - 100% Indoor - Speed: 3 km/h Sensing target assumptions as in Table 7.9.1-4 (TR. 38.901). Service profile; NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI.. ## 4.2 Dense urban The dense urban microcellular deployment scenario focuses on macro TRxPs with or without micro TRxPs and high user densities and traffic loads in city centres and dense urban areas. The key characteristics of this deployment scenario are high traffic loads, outdoor and outdoor-to-indoor coverage. This scenario will be interference-limited, using macro TRxPs with or without micro TRxPs. A continuous cellular layout and the associated interference shall be assumed. Some of its attributes are listed in Table 4.2. Table 4.2: Attributes for dense urban Attributes; Values or assumptions. Carrier Frequency NOTE1; Macro layer Around 2 GHz Around 4 GHz Around 7 GHz Around 30 GHz Around 4 GHz + Around 30GHz Around 4 GHz + Around 7 GHz Around 2 GHz + Around 4 GHz (Other combinations are not precluded) Micro layer Around 30 GHz Around 7 GHz. Aggregated system bandwidth NOTE2; Around 2 GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 200 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: Up to 800 MHz (DL+UL). Layout; Layout option 1. Single layer: - Hex. Grid Layout option 2. Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop, all outdoor, around 7 GHz and/or around 30 GHz. ISD; Macro: 200m Micro: 3 micro TRxPs per macro TRxP. BS antenna elements; Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 288 Tx and Rx antenna elements Around 7 GHz: Up to 1152 Tx and Rx antenna elements Around 30GHz: Up to 1152 Tx and Rx antenna elements. UE antenna elements; eMBB UE “handheld” device (used in eMBB): - Around 2 GHz: Up to 2 Tx and 4 Rx antenna elements - Around 4 GHz: Up to 4 Tx and 4 Rx antenna elements - Around 7 GHz: Up to 4 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 8 Tx and 8 Rx antenna elements Note: For around 30 GHz, with multi-panel UE, it is the number of simultaneously used antenna elements.. Service deployment and speed options; Option 1. Uniform/Macro TRxP Option 2. Uniform/Macro TRxP and clustered/Micro TRxP eMBB: - Profile 1 (Majority indoor) 20% Outdoor in cars: 30 km/h 80% Indoor: 3 km/h - Profile 2 (Majority outdoor, “Times Square” scenario) 60% Outdoor: 3 km/h, 20% Outdoor in cars: 30 km/h 20% Indoor: 3 km/h. Service profile; NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI.. ## 4.3 Rural The rural deployment scenario focuses on larger and continuous coverage. The key characteristics of this scenario are continuous wide area coverage supporting high speed vehicles. This scenario will be noise-limited and/or interference-limited, using macro TRxPs. Some of its attributes are listed in Table 4.3 Table 4.3: Attributes for rural scenario Attributes; Values or assumptions. Carrier Frequency; Around 700 MHz (for ISD 1 or ISD 2) Around 4 GHz (for ISD 1) Around 700 MHz + Around 2 GHz (for ISD 2) Around 7GHz (ISD 1) Around 700MHz + Around 7GHz (for ISD 1). Aggregated system bandwidth; Around 700 MHz: Up to [60] MHz (DL+UL) Around 2GHz: Up to [200 MHz (DL+UL) Around 4 GHz: Up to 200 MHz (DL+UL) Around 7GHz: Up to 400 MHz (DL+UL). Layout; Single layer: Layout Option 1: - Hex. Grid Layout Option 2 (for extreme long-distance coverage): - Single site, around 700 MHz. ISD; ISD 1: 1732m ISD 2: 5000m [ISD 3: 7500m assuming 700MHz]. BS antenna elements; Around 700 MHz: Up to 32 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 288 Tx and Rx antenna elements Around 7 GHz: Up to 1152 Tx and Rx antenna elements. UE antenna elements; eMBB UE “handheld” device (used in eMBB) - Around 700 MHz: 1 Tx and 2 Rx antenna elements (ITU up to 4Tx/Rx) - Around 2 GHz: Up to 2 Tx and 4 Rx antenna elements - Around 4 GHz: Up to 4 Tx and Rx antenna elements - Around 7 GHz: Up to 4 Tx and 8 Rx antenna elements FWA CPE or Tx/Rx Sensing device (used in FWA, Sensing): - Around 700 MHz: Up to 2 Tx and 4 Rx antenna elements - Around 2 GHz: Up to 2 Tx and 4 Rx antenna elements - Around 4 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 7 GHz: Up to 8 Tx and 8 Rx antenna elements. Service deployment and speed options (NOTE: each service is set to be evaluated individually. However, evaluation of combination of services is not precluded); eMBB: Profile 1 - 15% Outdoor: 3 km/h, 15% Outdoor in cars: 120 km/h 70% Indoor: 3 km/h Profile 2 - 50% Outdoor in cars: 120 km/h, 50% Indoor: 3 km/h FWA: - Rooftop - 100% Outdoor: 0 km/h, UAV sensing: - 100% Outdoor - Horizontal speed: 0 km/h or ~U(0, 180) km/h - Vertical speed: 0 km/h, optional {20, 40} km/h Sensing target assumptions as in Table 7.9.1-1 (TR. 38.901) Automotive sensing: - 100% Outdoor - Speed: 30 km/h Sensing target assumptions as in Table 7.9.1-2 (TR. 38.901). Service profile; NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI.. ## 4.4 Urban macro The urban macro deployment scenario focuses on large cells and continuous coverage. The key characteristics of this scenario are continuous and ubiquitous coverage in urban areas. This scenario will be interference-limited, using macro TRxPs (i.e. radio access points above rooftop level). Some of its attributes are listed in Table 4.4. Table 4.4: Attributes for urban macro Attributes; Values or assumptions. Carrier Frequency; Macro layer Around 2 GHz Around 4 GHz Around 7 GHz Around 2 GHz + Around 30GHz Around 2 GHz + Around 7 GHz Around 2 GHz + Around 4 GHz + Around 7 GHz (Other combinations not precluded) Micro layer Around 30 GHz Around 4 GHz + Around 30 GHz Around 2 GHz + Around 30 GHz. Aggregated system bandwidth; Around 700 MHz: Up to 60 MHz (DL+UL) Around 2 GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 200 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: Up to 800 MHz (DL+UL). Layout; Layout Option 1. Single layer: - Hex. Grid Layout Option 2. Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop, all outdoor, around 7 GHz and/or around 30 GHz. ISD; Macro layer: 500 m Micro layer: 6 [3] Micro TRxPs per Macro TRxP. BS antenna elements; Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 288 Tx and Rx antenna elements Around 7 GHz: Up to 1152 Tx and Rx antenna elements Around 30GHz: Up to1152 Tx and Rx antenna elements. UE antenna elements; eMBB UE “handheld” device (used in eMBB, HRLLC, Positioning): - Around 2 GHz: Up to 2 Tx and 4 Rx antenna elements - Around 4 GHz: Up to 4 Tx and 4 Rx antenna elements - Around 7 GHz: Up to 4 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 8 Tx and 8 Rx antenna elements Sensing TxRx “non-handheld” device (used in Sensing): - Around 4 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 7 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 32 Tx and 32 Rx antenna elements MTC device (used in MTC): - Around 2 GHz: 1 Tx/Rx antenna elements - Around 4 GHz: 1 Tx and up to 2 Rx antenna elements - Around 7 GHz: 1 Tx and up to 2 Rx antenna elements. Service deployment and speed options (NOTE: each service is set to be evaluated individually. However, evaluation of combination of services is not precluded); Option 1. Uniform/Macro TRxP Option 2. Uniform/Macro TRxP and clustered/Micro TRxP eMBB: - 10% Outdoor: 3 km/h, 10% Outdoor in cars: 30 km/h 80% Indoor: 3 km/h HRLLC: - 20% Outdoor: 3 km/h, 80% Indoor: 3 km/h Positioning: - 25% [10%] Outdoor: 3 km/h 25% [10%] Outdoor in cars: 30 km/h 50% [80%] Indoor: 3 km/h MTC: - 20% Outdoor in cars: 30 km/h, or Outdoor: 3 km/h (optional 0 km/h) 80% Indoor: 3 km/h, (optional 0 km/h for utility meters, sensors etc) UAV sensing: - 100% Outdoor - Horizontal speed: 0 km/h or ~U(0, 180) km/h - Vertical speed: 0 km/h, optional {20, 40} km/h Sensing target assumptions as in Table 7.9.1-1 (TR. 38.901) Automotive sensing: - 100% Outdoor - Speed: 30 km/h Sensing target assumptions as in Table 7.9.1-2 (TR. 38.901) Human sensing: - 100% Outdoor - Speed: 0 km/h or 3 km/h or ~U(0, 3) km/h Sensing target assumptions as in Table 7.9.1-3 (TR. 38.901). Service profile; NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI.. ## 4.5 Sub-Urban macro The sub urban deployment scenario focuses on large and continuous coverage. [The key characteristics of this scenario are continuous wide area coverage supporting pedestrian and vehicles, energy saving, positioning performance.] This scenario will be noise-limited and/or interference-limited, using macro TRxPs. Therefore this scenario deployment can be considered for 6G as well and the values or assumptions are described as Table 4.5. Table 4.5: Attributes for Sub-Urban macro scenario Attributes; Values or assumptions. Carrier Frequency; Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 700 MHz + Around 7 GHz Around 7GHz +Around 4GHz Around 2GHz+ Around 7 GHz + Around 30 GHz Around 7GHz +Around 4GHz + Around 2GHz + Around 700MHz. System bandwidth; Around 700 MHz: Up to 60 MHz (DL+UL) Around 2 GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 200 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: Up to 800 MHz (DL+UL). Layout; Single layer - Hex. Grid. ISD; ISD Profile 1: 850m ISD Profile 2: 1732m. TRP antenna elements; Around 700 MHz: Up to 32 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 288 Tx and Rx antenna elements Around 7 GHz: Up to 1152 Tx and Rx antenna elements Around 30GHz: Up to 1152 Tx and Rx antenna elements. UE antenna elements; eMBB UE “handheld” device (used in eMBB) - Around 700 MHz: 1 Tx and 2 Rx antenna elements (ITU up to 4Tx/Rx) - Around 2 GHz: Up to 2 Tx and 4 Rx antenna elements - Around 4 GHz: Up to 4 Tx and 4 Rx antenna elements - Around 7 GHz: Up to 4 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 8 Tx and 8 Rx antenna elements FWA CPE device (used in FWA): - Around 700 MHz: Up to 2 Tx and 4 Rx antenna elements - Around 2 GHz: Up to 2 Tx and 4 Rx antenna elements - Around 4 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 7 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 32 Tx and 32 Rx antenna elements. Service deployment and speed options (NOTE: each service is set to be evaluated individually. However, evaluation of combination of services is not precluded); eMBB: - 10% Outdoor: 3 km/h, 10% Outdoor in cars: 40 km/h 80% Indoor: 3 km/h FWA: Profile 1 (Indoor mounted CPE) - Behind wall or window - 100% Indoor: 0 km/h, Profile 2 (Exterior mounted CPE) - Exterior wall mounted or rooftop - 100% Outdoor: 0 km/h,. Service profile; NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI.. ## 4.6 High speed Editor note: Highway scenario will be considered and whether highway scenario is merged to section 4.6 can be further discussed Editor note: airborne will not be captured in section 4.6. Table 4.6: Attributes for high speed scenario Attributes; Values or assumptions. Carrier Frequency; Around 4 GHz Around 7GHz. Aggregated system bandwidth; Around 4 GHz: Up to [200] MHz (DL+UL) Around 7GHz: Up to [400] MHz (DL+UL). Layout; Single layer: - Linear Grid along railway line. ISD; 1732m. BS antenna elements; Around 4 GHz: Up to 288 Tx and Rx antenna elements Around 7 GHz: Up to 1152 Tx and Rx antenna elements Note: The numbers N= 288 and 1152 allows evenly division of the antenna elements into subarrays of size 1x2,1x3,1x4,1x6,1x8,1x9,1x12,... UE antenna elements; eMBB-HRLLC UE “handheld” device - Around 4 GHz: Up to 4 Tx and 4 Rx antenna elements - Around 7 GHz: Up to 4 Tx and 8 Rx antenna elements Optional eMBB-HRLLC UE “non-handheld” device - Around 4 GHz: Up to 4 Tx and 8 Rx antenna elements - Around 7 GHz: Up to 4 Tx and 8 Rx antenna elements. Service deployment and speed options (NOTE: each service is set to be evaluated individually. However, evaluation of combination of services is not precluded); eMBB with high reliability: - 100% Outdoor in train: Up to 500 km/h. Service profile; NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI.. ## 4.7X Air-to-Ground Scenario The commercial Air to Ground deployment scenario is defined to allow for the provision of services for commercial aircraft to enable both humans and machines aboard the aircraft to initiate and receive mobile services. It is not for the establishment of airborne based base stations. The key characteristics of this scenario are upward pointed Macro cells with very large area coverage supporting basic data and voice services, with moderate user throughput that are optimized for high altitude users that are travelling at very high speeds. The commercial airlines aircrafts are likely equipped with an aggregation point (e.g. Relay). Some of the characteristics of this deployment scenario are listed in Table 4.7X. Table 4.7X: Attributes for commercial Air to Ground Scenario Attributes; Values or assumptions. Carrier Frequency; Below 7 GHz. Aggregated system bandwidth; Up to 100 MHz (DL+UL). Layout; Single layer: - Hex. Grid. Cell range; 100 km. BS antenna elements; Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 288 Tx and Rx antenna elements Around 7 GHz: Up to 1152 Tx and Rx antenna elements. UE antenna elements; Belly mounted relay antenna. Service deployment and speed options; eMBB with high reliability: - 100% Outdoor in aircraft: Up to 1000 km/h - Altitude up to 15 km. Service profile; NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI.. ## 4.10 Non-Terrestrial Network Editor note: orbit type and payload type will be captured in section 4.10 Table 4.10: Attributes for Non-Terrestrial scenario Attributes; Values or assumptions. Carrier Frequency; Around 2 GHz Around 15 GHz Around 30 GHz (UL) and 20 GHz (DL). Aggregated system bandwidth; Around 2 GHz: Up to 30 MHz (DL+UL) Around 15 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: Up to 500 MHz (DL+UL). Layout; Single layer: - Hex. Pattern (at least 19 spot beams). Satellite Orbit; LEO 600 km (optional 1200 km) altitude GEO 35786 km altitude. BS antenna elements; Bessel function as in Section 6.4.1 in TR 38.811. UE antenna elements; eMBB UE “handheld” device (used in eMBB, MTC) - Around 2 GHz: Up to 2 Tx and 4 Rx antenna elements - Around 15 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 8 Tx and 8 Rx antenna elements FWA CPE device (used in FWA): - Around 2 GHz: Up to 2 Tx and 4 Rx antenna elements - Around 15 GHz: Up to 32 Tx and 32 Rx antenna elements - Around 30 GHz: Up to 32 Tx and 32 Rx antenna elements. Service deployment and speed options (NOTE: each service is set to be evaluated individually. However, evaluation of combination of services is not precluded); eMBB: Profile 1 - 100% Outdoor: 0 km/h Profile 2 - 100% Outdoor in car: 120 km/h FWA: - Rooftop mounted - 100% Outdoor: 0 km/h, MTC: - 100% Outdoor: 0 km/h,. Service profile; NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI..

# 4	Conclusion

In this contribution we propose adding the text proposal in section 3 to TR 38.914.