| 3GPP TSG RAN Meeting #109 | RP-251964 |
| --- | --- |
| Beijing, China, September 15-18, 2025 |  |

| Agenda Item: | 8.2.1.1 |
| --- | --- |
| Source: | IIT Hyderabad (IITH) and WiSig |
| Title: | Low-Cost Macro Base Station Options for Upper Mid-Band (6.425–7.125 GHz) |
| Document for: | Discussion |

Abstract

This contribution proposes a study on standardized, low-cost macro base-station architectures for the upper mid-band (6.425–7.125 GHz). The approach combines (i) simplified RU/DU implementations and (ii) dual-waveform PHY (low-PAPR + CP-OFDM on demand) to recover PA back-off, improve energy efficiency, and reduce total cost of ownership—while preserving macro-class EIRP and coverage. The study targets 4×–10× cost reduction vs. conventional gNBs at comparable coverage/capacity and enables scalable deployments, particularly for FWA-heavy markets.

1. Motivation and Context

- India is expected to make 6.425–7.125 GHz available as a dedicated 6G block (aligned with WRC outcomes). With modest fibre penetration, Fixed Wireless Access (FWA) will remain a primary broadband mode in both urban and rural areas.

- India’s existing 5G mid-band (3.3–3.67 GHz) is increasingly congested. Stop-gap alternatives (unlicensed Wi-Fi; Wi-Fi Baseband + FR2 mm Wave backhaul with CPE dishes) offer limited standardization, weaker long-term interoperability, and challenging economics at scale.

- A standardized, macro-class, lower-cost upper-midband solution is needed for rapid, large-scale rollouts.

2. Objectives

Define macro-class systems for 6.425–7.125 GHz that:

- Achieve comparable EIRP and coverage to today’s macro gNBs (3–4 sectors per site).

- Deliver ¼–¹⁄₁₀ the cost (per coverage/capacity) of current macro solutions.

- Use simplified RU/DU designs and a dual-waveform PHY to minimize PA back-off while preserving MU-MIMO and spectral efficiency when needed.

3. Scope: What “Low-Cost Macro” Means

This work item targets macro/micro class equipment, not small cells:

- Per-sector RF power: ~100–400 Watts; site: 3–4 sectors (including virtual/beam sectors).

- RU: simplified PAs (reduced back-off), improved thermal/mechanical design; a single RU may feed 2–4 sectors/beams to lower per-sector cost.

- DU: Use a single baseband (PHY) and stack (L2/L3) where possible with simplified scheduler/PHY controls; one DU may drive one RU for 180°–360° azimuth coverage utilizing multiple beams/sectors.

- One cost lever is the PA, which in CP-OFDM typically requires ≥8 dB back-off; recovering this margin is important.

4. Waveform Strategy and PA Efficiency (Dual-Waveform PHY)

We propose a dual-waveform PHY that defaults to low-PAPR for sync/control/coverage-critical data, and selectively enables CP-OFDM for high-throughput/MU-MIMO:

- Low-PAPR family (e.g., DFT-s-OFDM including OTFDM variants):

  - Enables near-saturation PA operation for π/2-BPSK, QPSK, low-MCS data, improving link budget and energy efficiency.

  - Flexible DMRS placement, robustness to Doppler, and support for pre-DFT time-domain multiplexing.

- CP-OFDM on demand (e.g., PDSCH bursts for MU-MIMO/high MCS):

  - Accepts requisite back-off only when needed to deliver higher spectral efficiency.

Synchronization channels: Map PSS/SSS/PBCH to low-PAPR by default to maximize coverage.
Alternative (not preferred): Keep sync on CP-OFDM; then coverage must be regained via repetitions (NB-IoT-like), which is less energy/cost efficient.

Scheduling model with OTFDM-style multiplexing:

- In a TTI with 14 symbols, time-multiplex 2–4 UEs per symbol, enabling up to 28–56 scheduled UEs/TTI (illustrative).

- A default 2-port DMRS (H/V) supports single-layer or 2-layer SU-MIMO without cross-polarization precoding (ports are polarization-separated).

5. Architecture Option A — Low-Cost Split RU (O-RAN 7.2A Compatible)

A representative low-cost macro architecture (aligned with O-RAN FH split 7.2A):

- Single RU with ≥8 antenna ports (dual-pol H/V) feeding 2–4 sectors/beams via external panels.

- PHY: Dual-waveform baseband; low-PAPR by default; CP-OFDM enabled for capacity bursts.

- Modes:

  - SU-MIMO: 1–2 layers via H/V polarization and low PAPR Waveform.

  - MU-MIMO: activated with CP-OFDM as needed.

  - CSI: CSI-RS/SRS with explicit CSI feedback and/or calibration.

6. Architecture Option B — Upper-Midband Massive MIMO (e.g., 64T/128T)

For high-port-count systems (64T/64R to 128T/128R), the same dual-waveform principles apply:

- For low-order modulations, dimension PAs so saturation ≈ average power → zero/low back-off.

- For higher-order modulations, selective back-off only for capacity-critical traffic.

- CP-OFDM is used tactically (MU-MIMO, high spectral-efficiency bursts); low-PAPR maximizes coverage/energy efficiency otherwise.

This yields a tunable coverage–capacity–cost trade-off while preserving macro-class performance.

7. Architecture Option C — Single-RU, 360° Coverage Massive MIMO

To avoid the cost of multiple sectorized RUs, deploy a single RU with integrated 360° antenna (e.g., cylindrical array or azimuthal panels within the RU chassis):

- Re-purposes available antenna ports for site-wide coverage; TDD reciprocity calibration and massive-MIMO processing are built-in.

- Combined with low-PAPR, recovers PA back-off, increasing effective EIRP; a single DU–single RU can cover 360° with sufficient control-channel capacity for the target load.

8. Spectrum Efficiency and Uplink Options

- Dynamic waveform switching per service/latency/throughput need.

- Feature selectivity: DMRS/CSI-RS/PDSCH activation tuned to coverage and load.

- Uplink: 1Tx/2Tx UE options extend reach for rooftop/pole-mounted CPEs; dual-waveform UL (as already in 5G) can be optimized for 6G to balance SU/MU-MIMO against robustness.

9. Multi-RAT Spectrum Sharing

Where 6G coexists with 5G in the same band:

- Use CP-OFDM to maintain full spectrum-sharing compatibility with legacy 5G.

- Dual-mode 5G/6G RUs: extend 6G coverage with low-PAPR for static/FWA users while serving legacy 5G traffic with CP-OFDM.

Proposal: We propose that RAN Plenary initiate an SI on “Low-Cost Macro for Upper Mid-Band (6.425–7.125 GHz)”