3GPP TSG RAN Meeting #109		                                		  RP-251972

Beijing, China, September 15-18, 2025

Agenda Item:	6.4

Source:	OPPO

Title:	Discussion on 6G fronthaul

Document for:	Discussion

# Introduction

In RAN#108, a summary of the joint workshop between 3GPP and O-RAN on 6G coordination was presented [1] and it triggered extensive and intensive discussions during the meeting [2][3].  Based on the summary and the discussions, a revised way forward was proposed for the relationship between 3GPP 6G air interface specification and 6G fronthaul design/specification [4]. This way forward was relatively stable with limited controversial points.

Take the following as a basis for further discussions aiming at a resolution at RAN#109 to aid 6G architecture studies in RAN3: - 3GPP specifies the 6G air interface [without restrictions from fronthaul design] - [It is expected that 6G fronthaul design in O-RAN Alliance will be based on the 6G air interface designed by 3GPP] - 3GPP to capture the following in a normative Annex of TSxx.401 (RAN3) or TSxx.300 (RAN2) - “6G NodeB architecture includes a distinct logical Radio Unit (RU). - The functions hosted by the RU as well as the interface between RU and other parts of 6G NodeB are not specified in 3GPP. - [Note: Implementation of this interface can be according to proprietary solution(s), or standardized solutions as specified in O-RAN ALLIANCE”]

In this contribution, we continue to discuss some controversial points within the brackets in the above-mentioned way forward and provide our views on these discussion points.

# Discussion

The first discussion point in the afore-mentioned way forward is whether “without restrictions from fronthaul design” is needed or not for the 6G air interface design/specification in 3GPP.  To be specific, the question is whether 3GPP work for 6GR design needs to consider some specific implementation of 6G fronthaul or not.

Firstly, 3GPP specified previous generations of wireless communications (e.g., LTE, NR) without restrictions from any specific implementation/specification of fronthaul. These systems have been deployed worldwide and are working well for various commercial services. Thus, 3GPP design of air interface “without restrictions from fronthaul design” has been proved to be a good practice for new cellular communication systems.

Secondly, all the current fronthaul specification(s)/implementation(s) are focusing on other systems (i.e., NR, LTE) rather than 6GR. To be specific, this is no fronthaul design for 6GR that have achieved the common support of the telecommunication industry so far. Thus, 3GPP delivery of 6G air interface specification is very likely to be delayed if the 3GPP activities need to wait for the progress of 6G fronthaul design/specification.

Thirdly, the 6RG SID clearly states the requirement for the coverage of around-7GHz deployment (highlighted by Yellow) as below

The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

The most critical technology to meet the afore-mentioned coverage requirement is to utilize more antennas (super-massive MIMO) and more advanced MIMO schemes, thereby leading to tight collaboration of antennas and the baseband processing units. If some unnecessary restriction(s) were introduced due to the non-optimized fronthaul, the performance of more advanced schemes with super-massive MIMO will be limited and may not ensure the coverage of 6GR deployment at around 7 GHz.

In summary, we suggest 3GPP to specify 6GR without restrictions from fronthaul design:

Proposal 1:  3GPP specifies the 6G air interface without restrictions from fronthaul design.

Once the 6G air interface is specified by 3GPP, the industry may implement different designs of 6G fronthaul by considering different tradeoff between cost/complexity and performance. Different companies and/or different organization(s) are free to choose their own designs of 6G fronthaul.  Thus, we have the following proposal:

Proposal 2:  6G fronthaul design(s)/implementation(s) will be based on the 6G air interface specified by 3GPP

# Conclusion

In this contribution, we discuss some controversial aspects of the way forward on the relationship between 3GPP air interface design and 6G fronthaul design [4]. Based on the analysis, we suggest the following proposals:

Proposal 1:  3GPP specifies the 6G air interface without restrictions from fronthaul design.

Proposal 2:  6G fronthaul design(s)/implementation(s) will be based on the 6G air interface specified by 3GPP

# References

- RP-251415	Open fronthaul for 6G, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

- Meeting Report of 3GPP TSG RAN Meeting #108

- RP-251833       Way forward on 6G fronthaul, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

- RP-251876       Way forward on 6G fronthaul, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

- RP-251881        New SID: Study on 6G Radio, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025