3GPP TSG RAN Meeting #109								RP-251990

Beijing, China, September 15-18, 2025

Agenda Item:		8.2.1.3

Source:			T-Mobile USA Inc., Ericsson, Nokia

Title:				6G requirements for architecture and migration

Document for:		Approval

### 1	Introduction

This document discusses and proposes 6G requirements for architecture and migration. The document also includes a text proposal to be captured into the TR 38.914 v0.0.1 for Study on 6G Scenarios and requirements.

The study item in [1] defines number of important objectives, which need to be considered when defining 6G requirements for architecture and migration. A multi-company contribution on 6G design principles in RAN#108 [2] discusses the importance of a single stand-alone 6G system with streamlined and lean architecture including optimized spectrum sharing (MRSS) with 5G and enabling easy macro 6G deployments using the existing macro sites on both existing and new spectrum with efficient aggregation of carriers. The document also proposes key technical principles and requirements for 6G.

Aligned with the key technical principles and requirements proposed in [2] in this contribution we discuss further motivation and details for design principles and requirements for 6G standalone architecture with 5G-6G Multi-Radio Spectrum Sharing (MRSS) and 6G Carrier Aggregation, which enable smooth migration and intelligent use of the existing and new spectrum.

### 2	Discussion

6G standalone system enables unlocking of full 6G potential and allows all 6G benefits to become universally available on day one. For achieving lean and implementation friendly 6G system it is important to avoid different architecture options, which fragment the market.  6G radio on a new spectrum is an engine for capacity growth and enables wider channel bandwidths for elevated radio performance and improved spectral efficiency.

5G-6G MRSS enables best possible 5G to 6G migration for 5G NR FR1 FDD and TDD bands by dynamically sharing a carrier with 5G and 6G. 5G-6G MRSS needs to be highly efficient on all the existing FR1 TDD and FDD bands. It also needs to be easily deployable on all the existing 5G NR frequency bands on 6G day one. To achieve this MRSS needs to be transparent to the UE. MRSS is an integral part of the 6G radio design and day one network products and deployments.

By sharing 5G carrier with 6G using MRSS we’ll enable significant 6G radio performance gains like significantly improved spectral efficiency and higher throughputs on the existing FR1 FDD and TDD bands. 6G Carrier Aggregation is important for connecting a new 6G high-capacity band with the existing lower frequency FR1 coverage band, which is shared between 5G and 6G using MRSS. Figure 1 illustrates standalone 6G system with 5G-6G MRSS and 6G carrier aggregation.

Figure 1: 6G Standalone system with 5G-6G Multi-Radio Spectrum Sharing and 6G Carrier Aggregation

### 3	Text Proposal to TR 38.914

We propose the following text to the 6G Scenarios and Requirements TR 38.914.

--- Text Proposal starts ---

## 5.2	Requirements for architecture and migration

6G Radio Access Technology shall be designed to fulfil the following requirements and design principles:

- A single 6G standalone architecture

- Spectrum sharing between 5G and 6G, which enables dynamic and agile sharing of the 5G NR FDD and TDD bands between 5G and 6G radio access technologies with high efficiency. This 5G-6G dynamic multi-radio spectrum sharing (5G-6G MRSS) shall be transparent to the UE.

- 6G carrier aggregation for downlink and uplink.

--- Text proposal ends ---

### 4	References

[1] RP-251395 Study on 6G Scenarios and requirements, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom

[2] RP-250878 6G Design Principles T-Mobile USA, Apple, AT&T, BT, Ericsson, KT, MediaTek, Nokia, Qualcomm, Rakuten Mobile, Samsung, SK Telecom, Telstra, Verizon