3GPP TSG-RAN Meeting #109			RP-251993

Beijing, China, September 15-18, 2025

Agenda item:	8.2.1.1

Title:	Views on 6G deployment scenarios

Source:	NTT DOCOMO, INC.

Document for:	Discussion

# 1	Introduction

In [1], it is mentioned to identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors. Accordingly, the 6G deployment scenarios were discussed in RAN#108 and agreements were captured in TR 38.914 [2]. There are still missing key aspects of 6G deployment scenarios in the TR, and this contribution provides NTT DOCOMO’s view on 6G deployment scenarios.

# 2	Discussion

First, from an operator perspective, we believe that it would be important to consider realistic values that can be implemented/deployed when determining the assumptions for 6G deployment scenarios to truly deliver value to customers both in existing and new spectrum. In fact, looking back at 5G NR, some assumptions were far from practical setting, which has created a gap between theoretical performance and commercial performance, resulting in a difficulty to meet customer expectations. On the other hand, it would be also important to include advanced values to clearly show the gains from 5G NR. Therefore, we think it would be important to ensure that the baseline values for the assumptions are practical while including advanced settings.

Observation 1

- To make truly valuable 6G, it would be important to ensure that the baseline values for the deployment scenarios are practical while including advanced settings in both existing and new spectrum.

On top of that, FR3-related parameters need to be considered for 6G deployment scenarios. Currently, towards WRC27, around 7 GHz and around 15 GHz are being studied as potential FR3 bands. Considering this situation, we believe it would be important to consider both frequency bands as candidates to evaluate each frequency characteristic.

Proposal 1

- As carrier frequency of FR3 band, both 7 GHz and 15 GHz should be evaluated.

To decide FR3-related parameters, it would be necessary to consider appropriate values for parameters that vary depending on the frequency band, as the differences in deployment scenarios from the 5G NR frequency band. To consider this, it would be important to consider ensuring coverage equivalent to 5G NR mid-band (e.g., around 3.5 GHz) as a minimum requirement for 7 GHz, as described in the SID. On the other hand, it would be necessary to first consider how 15 GHz should be handled.

Proposal 2

- For 7 GHz, parameter values should be decided to meet ensuring coverage equivalent to 5G NR mid-band (e.g., 3.5 GHz) as a minimum requirement.

- For 15 GHz, how the band is treated (e.g., FR2-like) should be discussed.

At the RAN#108 meeting, the TR for scenarios and requirements was updated with new deployments scenarios and key performance indicators. In the updated TR, more than 10 candidate deployment scenarios are captured. To enable early evaluation at RAN1, we should begin discussing the assumptions for at least the general deployment scenarios (e.g., Indoor hotspot, Dense urban, Rural, and Urban macro). As mentioned above, the evaluation of FR3 would be one of the key points for the 6G deployment scenarios. In addition, the FR3 band would be expected to be used for the same purposes as the 5G NR mid band. Therefore, it would be necessary to evaluate whether FR3 can meet the requirements of existing scenarios for 5G NR mid band.

Proposal 3

- For general scenarios (e.g., Indoor hotspot, Dense urban, Urban macro and Rural), FR3 should be considered in addition to existing carrier frequencies.

6GR is expected to utilize the FR3 band, including 7 GHz and 15 GHz. In addition, the suburban macro scenario is recognized as a scenario in which the FR3 band can be effectively utilized. Based on the above, considering FR3 evaluation, suburban macro could be one of the important scenarios. Also, considering suburban macro would be scenarios for FR3 evaluation, FR3 should be considered for the carrier frequency.

Proposal 4

- Suburban macro should be introduced as general scenario in addition to Indoor hotspot, Dense urban, Urban macro, and Rural.

  - FR3 should be considered as carrier frequency for suburban macro scenario.

Another important assumption that depends on the carrier frequency is the configuration of the BS/UE antenna. In 5G NR, the number of antenna elements for FR2 (e.g., 30 GHz, 70 GHz) was not practical. Thus, the practical number of antenna elements (e.g., 512 BS antenna elements) should be evaluated in addition to the larger number (e.g., 1024 BS antenna elements) for FR3 (e.g., 7 GHz, 15 GHz).

Proposal 5

- Practical number of BS/UE antenna elements (e.g., 512 BS antenna elements) for FR3 should be used for evaluation as the baseline.

Accordingly, we propose the TP in the next section for 6G deployment scenarios.

# 3	Text proposals on deployment scenarios in TR38.914

Start of TP

## 4.1	Indoor hotspot

The indoor hotspot deployment scenario focuses on small coverage per site/TRxP (transmission and reception point) and high user throughput or user density in buildings. The key characteristics of this deployment scenario are high capacity, high user density and consistent user experience indoor.

Some of its attributes are listed in Table 4.1.

Table 4.1: Attributes for indoor hotspot

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 4 GHz Around 7 GHz Around 4GHz+Around 7GHz Around 15 GHz Around 30 GHz |
| Aggregated system bandwidth NOTE2 | Around 4 GHz: Up to [200] MHz (DL+UL) NOTE3 Around 7 GHz and around 15 GHz: Up to [400] MHz (DL+UL) NOTE3 Around 30 GHz: [1]GHz (DL+UL) NOTE3 |
| Layout | Single layer: - Indoor floor (Open office) |
| ISD | 20m for around 30GHz TBD on other carrier frequencies (Equivalent to [12]TRxPs per 120m x 50m) |
| BS antenna elements | Around 4 GHz: Up to 32 Tx and Rx antenna elements Around 7 GHz and around 15 GHz: Up to 128 Tx and Rx antenna elements Around 30 GHz: Up to 64 Tx and Rx antenna elements |
| UE antenna elements | Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz and around 15 GHz: Up to [8] Tx and Rx antenna elements Around 30 GHz: Up to 32 Tx and Rx antenna elements |
| User distribution and UE speed | 100% Indoor, 3km/h, [10] users per TRxP NOTE4 |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

## 4.2	Dense urban

The dense urban microcellular deployment scenario focuses on macro TRxPs with or without micro TRxPs and high user densities and traffic loads in city centres and dense urban areas. The key characteristics of this deployment scenario are high traffic loads, outdoor and outdoor-to-indoor coverage. This scenario will be interference-limited, using macro TRxPs with or without micro TRxPs. A continuous cellular layout and the associated interference shall be assumed.

Some of its attributes are listed in Table 4.2.

Table 4.2: Attributes for dense urban

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 2GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 4 GHz + Around 30GHz Around 4 GHz + Around 7 GHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] |
| Aggregated system bandwidth NOTE2 | Around 700 MHz: Up to [60] MHz (DL+UL) Around 2GHz: Up to [120] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7 GHz and around 15 GHz: Up to [400]MHz (DL+UL) Around 30 GHz: [1]GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid Two layers for Around 4 GHz + Around 30 GHz: - Macro layer: Hex. Grid, 4GHz - Micro layer: Random drop, 30GHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz ([single layer or two layers])]:TBD |
| ISD | Macro: 200m Micro: 3 micro TRxPs per macro TRxP Micro: [100]m All micro TRxPs are all outdoor |
| BS antenna elements | Around 2 GHz: Up to 64 Tx and Rx antenna elements Around 4 GHz: Up to 128 Tx and Rx antenna elements Around 7 GHz and around 15 GHz: Up to 512 Tx and Rx antenna elements Around 30 GHz: Up to 256 Tx and Rx antenna elements |
| UE antenna elements | Around 2 GHz: Up to 4 Tx and Rx antenna elements Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz and around 15 GHz: Up to [8] Tx and Rx antenna elements Around 30 GHz: Up to 32 Tx and Rx antenna elements |
| User distribution and UE speed | Step1: Uniform/macro TRxP, 10 users per TRxP Step2: Uniform/macro TRxP + Clustered/micro TRxP, 10 users per TRxP [10] users per TRxP with single-layer only 80% indoor (3km/h), 20% outdoor (30km/h) |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

## 4.3	Rural

The rural deployment scenario focuses on larger and continuous coverage. The key characteristics of this scenario are continuous wide area coverage supporting high speed vehicles. This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

Some of its attributes are listed in Table 4.3.

Table 4.3: Attributes for rural scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 700 MHz (for ISD 1 or ISD 2) Around 4 GHz (for ISD 1) Around 700 MHz + Around 2 GHz (for ISD 2) Around 7GHz (ISD 1) Around 15GHz (ISD 1) Around 700MHz + Around 7GHz (for ISD 1) |
| Aggregated system bandwidth | Around 700 MHz: Up to [60] MHz (DL+UL) Around 2GHz: Up to [120] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7GHz and around 15 GHz: Up to [400] MHz (DL+UL) |
| Layout | Single layer: - Hex. Grid |
| ISD | ISD 1: 1732m ISD 2: 5000m [ISD 3: 7500m assuming 700MHz] |
| BS antenna elements | Around 700 MHz and around 2 GHz: Up to 64 Tx and Rx antenna elements Around 4 GHz: Up to 256 Tx and Rx antenna elements Around 7 GHz and around 15 GHz: Up to 1024 Tx and Rx antenna elements |
| UE antenna elements | Around 700 MHz and around 2 GHz: Up to 4 Tx and Rx antenna elements Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz and around 15 GHz: Up to [8] Tx and Rx antenna elements |
| User distribution and UE speed | [15% outdoor vehicles (120km/h), 15% outdoor (3 km/h) and 70% indoor (3 km/h), or 50% outdoor vehicles (120km/h), 50% indoor (3 km/h)] [10] users per TRxP |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

## 4.4	Urban macro

The urban macro deployment scenario focuses on large cells and continuous coverage. The key characteristics of this scenario are continuous and ubiquitous coverage in urban areas. This scenario will be interference-limited, using macro TRxPs (i.e. radio access points above rooftop level).

Some of its attributes are listed in Table 4.4.

Table 4.4: Attributes for urban macro

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 2GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 4 GHz + Around 30GHz Around 4 GHz + Around 7 GHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] |
| Aggregated system bandwidth | Around 700 MHz: Up to [60] MHz (DL+UL) Around 2GHz: Up to [120] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7 GHz and around 15 GHz: Up to [400]MHz (DL+UL) Around 30 GHz: [1]GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid Two layers for Around 4 GHz + Around 30 GHz: - Macro layer: Hex. Grid, 4GHz - Micro layer: Random drop, 30GHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz ([single layer or two layers])]:TBD |
| ISD | Macro: 500m [TBD on micro layout] |
| BS antenna elements | Around 2 GHz: Up to 64 Tx and Rx antenna elements Around 4 GHz: Up to 128 Tx and Rx antenna elements Around 7 GHz and around 15 GHz: Up to 512 Tx and Rx antenna elements Around 30 GHz: Up to 256 Tx and Rx antenna elements |
| UE antenna elements | Around 2 GHz: Up to 4 Tx and Rx antenna elements Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz and around 15 GHz: Up to [8] Tx and Rx antenna elements Around 30 GHz: Up to 32 Tx and Rx antenna elements |
| User distribution and UE speed | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h [10] users per TRxP |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

## 4.5	Sub-Urban macro

The sub urban deployment scenario focuses on large and continuous coverage. [The key characteristics of this scenario are continuous wide area coverage supporting pedestrian and vehicles, energy saving, positioning performance.] This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

Therefore this scenario deployment can be considered for 6G as well and the values or assumptions are described as Table 4.5.

Table 4.5: Attributes for Sub-Urban macro scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 7GHz +Around 4GHz Around 2GHz+ Around 700 MHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] |
| System bandwidth | Around 700 MHz: Up to [60] MHz (DL+UL) Around 2GHz: Up to [120] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7 GHz and around 15 GHz: Up to [400]MHz (DL+UL) |
| Layout | Single layer - Hex. Grid [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz ([single layer or two layers])]:TBD |
| ISD | ISD 1: 1299m ISD 2: 1732m |
| TRP antenna elements | TBDAround 700 MHz and around 2 GHz: Up to 64 Tx and Rx antenna elements Around 4 GHz: Up to 256 Tx and Rx antenna elements Around 7 GHz and around 15 GHz: Up to 1024 Tx and Rx antenna elements |
| UE antenna elements | Around 700 MHz and around 2 GHz: Up to 4 Tx and Rx antenna elements Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz and around 15 GHz: Up to [8] Tx and Rx antenna elements |
| User distribution and UE speed | [10% Outdoor pedestrian: 3km/h, 10% Outdoor in cars: 40km/h, 80% Indoor in houses: 3km/h] [10] users per TRxP |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

End of TP

# 4	Conclusion

In this contribution, the following observations and proposals are provided.

Observation 1

- To make truly valuable 6G, it would be important to ensure that the baseline values for the deployment scenarios are practical while including advanced settings in both existing and new spectrum.

Proposal 1

- As carrier frequency of FR3 band, both 7 GHz and 15 GHz should be evaluated.

Proposal 2

- For 7 GHz, parameter values should be decided to meet ensuring coverage equivalent to 5G NR mid-band (e.g., 3.5 GHz) as a minimum requirement.

- For 15 GHz, how the band is treated (e.g., FR2-like) should be discussed.

Proposal 3

- For general scenarios (e.g., Indoor hotspot, Dense urban, Urban macro and Rural), FR3 should be considered in addition to existing carrier frequencies.

Proposal 4

- Suburban macro should be introduced as general scenario in addition to Indoor hotspot, Dense urban, Urban macro, and Rural.

  - FR3 should be considered as carrier frequency for suburban macro scenario.

Proposal 5

- Practical number of BS/UE antenna elements (e.g., 512 BS antenna elements) for FR3 should be used for evaluation as the baseline.

Proposal 6

- Adopt the TP in Section 3 for 6G deployment scenarios in TR38.914.

# 5	References

[1] RP-251395, “Revised SID: Study on 6G Scenarios and requirements” CMCC, Verizon, NTT DOCOMO, Deutsche Telekom

[2] RP-251873, “3GPP TR 38.914 V0.1.0: Study on 6G Scenarios and Requirements”