3GPP TSG RAN Meeting #109									      RP-252008

Beijing, China, September 15-18, 2025

Status Report to TSG

Agenda item:			9.6.1.4

| WI / SI Name | Low-power wake-up signal and receiver for NR (LP-WUS/WUR) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_LPWUS |  |  |  |  |
| Unique ID | 1020094 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-241824 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2025 | Performance part: 03/2026 | Testing part: N/A |  |
| Overall Completion level | Study Item: n/a | Core part: 100% | Performance Part: 25% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN 1 |
| --- | --- | --- |
| Rapporteur (primary) | Name | Xueming Pan |
|  | Company | vivo |
|  | Email | panxueming@vivo.com |
| Rapporteur | Name | Shinya Kumagai |
|  | Company | NTT DOCOMO |
|  | Email | shinya.kumagai.yw@nttdocomo.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

RAN1#122

Maintenance on LP-WUS and LP-SS design

Conclusion

No consensus to specify OOK waveform generation for LP-SS ON chip if overlaid OFDM sequence is not configured for LP-SS with M=1.

Agreement

Adopt the following TP to Section 7.4.5, TS38.211 in principle:

----------Start of Text Proposal on 3GPP TS 38.211 V19.0.0----------------------- <Unchanged part is omitted> The sequence is defined by Tables 7.4.5.1.1-1 to 7.4.5.1.1-3 with the quantity given by the higher-layer parameter XXX. The value of configuration defined in Tables 7.4.5.1.1-1 to 7.4.5.1.1-3 can be indicated by the higher-layer parameter lpss-BinarySeqIndex; <Unchanged part is omitted>

Agreement

Adopt the following TP to Section 7.4.4.1.2 and Section 7.4.4.1.3, TS 38.211 in principle.

<Unchanged part is omitted> 7.4.4.1.2 Generation of rWUS(n) The block of complex-valued symbols is defined by NscWUS=11NscRB where The quantity is given by the higher-layer parameter LP-WUS_Mvalue_IDLE/INACTIVE or LP-WUS_Mvalue_CONNECTED. The bit sequence and the number of bits Mbit corresponds to and G0, respectively, in clause 7.4.3 of [4, 38.212]. <Unchanged part is omitted> 7.4.5.1.3 Generation of rLPSS(n) The block of complex-valued symbols is defined by NscWUS=11NscRB where <Unchanged part is omitted>

Agreement

Parameters ‘LP-SS_Binary_Seq’, ‘LP-SS_Binary_Seq_Length’, ‘LP-SS_periodicityoffset’ and ‘LP-SS_Start_Symbol’ shall be configured when OOK-based LP-WUS operation is configured in IDLE/INACTIVE.

| Parameter name in the spec | Description | Value range |
| --- | --- | --- |
| LP-SS_Binary_Seq | Provides the LP-SS binary sequence index in the cell for IDLE/INACTIVE. | Sequence index {1,2,3,4} |
| LP-SS_Binary_Seq_Length | Provides the LP-SS binary sequence length in the cell for IDLE/INACTIVE. | [Length1, Length2] |
| LP-SS_periodicityoffset | Porvides the configuration of LP-SS periodicity and time offset.The time offset is configured for the first LP-SS occasion with reference to SFN0. | Periodicity: [320ms, 160ms] Offset: For 320ms periodicity: {0, 1, …, 319}ms For 160ms peirodiicty: {0, 1, …, 159}ms |
| LP-SS_Start_Symbol | To configure one or two start symbol locations within a slot for LP-SS, where the slot is determined from the periodicity/offset configuration for LP-SS | A list of one or two starting symbols, with each starting symbol in range of {{0, 1, …, 10}} |

Note: How to configure OOK-based LP-WUS operation in IDLE/INACTIVE is up to RAN2.

Conclusion

No consensus on the FFS part.

| Parameter name in the spec | Description | Value range |
| --- | --- | --- |
| LP-WUS_overlaidSeq_roots_IDLE/INACTIVE | Configuration of one or two roots for the overlaid OFDM sequence(s) for LP-WUS in IDLE/INACTIVE FFS: This parameter shall be configured for OFDM-based LP-WUR for LP-WUS operation in IDLE/INACTIVE Two roots shall be configured for the case when the value of parameter LP-WUS_num_overlaidseq_IDLE/INACTIVE is configured as 16/8/4 for M=1/2/4, respectively, otherwise, one or two roots can be configured. | a list of up to two roots can be configured For each root - For M=1, the value range is 1~131 - For M=2, the value range is1~61 - For M=4, the value range is 1~31 |

Agreement

Adopt the following TP to Section 7.4.4.1.1, TS38.211 for alignment.

---------------------------------Start of Text Proposal on 3GPP TS 38.211 V19.0.0----------------------- <Unchanged part is omitted> ##### 7.4.4.1.1 Generation of rZC,m(n) <Unchanged part is omitted> The sequence number cm=0 if LNseq=1, otherwise is given by δ=log2Nseq where - L is given by the higher-layer parameter XXX - f1i and E1 are given by clause 7.4.2.2 of [4, 38.212] <Unchanged part is omitted> --------------------------------------End of Text Proposal on 3GPP TS 38.211 V19.0.0 ------------------

Maintenance on LP-WUS operation in IDLE/INACTIVE modes

Agreement

The 3 capabilities for UE’s wake-up delay in idle/inactive mode are defined as follows:

| SSB periodicity (ms) | Wake-up delay (ms) UE capability 1 | Wake-up delay (ms) UE capability 2 | Wake-up delay (ms) UE capability 3 |
| --- | --- | --- | --- |
| 5/10/20 | 70 | 500 | 900 |
| 40 | 130 | 600 | 1000 |
| 80 | 250 | 800 | 1200 |
| 160 | 490 | 1200 | 1600 |

Agreement

Following TP to Section 7.4, TS 38.212 is endorsed in principle.

## 7.4 Wake-up information The wake-up information is carried by a wake-up signal as defined in clause 7.4.4 of [4, TS 38.211]. - For a UE configured with higher layer parameter LP-WUS_LP-SS_startRB_IDLE_INACTIVE and operating in the RRC_IDLE or RRC_INACTIVE state, the wake-up information bit sequence is the binary sequence of the codepoint as defined by Clause 10.4C of [5, TS38.213], where is the most significant bit and K is - max(, 1) if , where is configured by higher layer parameter PO-to-LO association and is configured by higher layer parameter subgroupNumber_PO_LPWUS; - if , where is configured by higher layer parameter subgroupNumber_PO_LPWUS; ===== unchanged text omitted =====

| Reason for change: | The number of bits for the wake-up information for the case of NSGPO=1 and NPOLO=1 is 0, which is incorrect. |
| --- | --- |
| Summary of change: | The number of bits for the wake-up information for the case of NSGPO=1 and NPOLO=1 is set to 1. |
| Consequences if not approved: | Cannot support wake-up information for the case of NSGPO=1 and NPOLO=1. |

Agreement

The value range for offset_firstMO_withinLO is:

- 15kHz SCS: 0-139 OFDM symbols

- 30kHz SCS: 0-279 OFDM symbols

- 120kHz SCS: 0-1119 OFDM symbols

Agreement

The value range for WUS_Actual_WUS_duration_IDLE/INACTIVE is:

- M=1: 2*[1-32] OFDM symbols

- M=2: 1-32 OFDM symbols

- M=4: 1-16 OFDM symbols

Agreement

10.4C PDCCH monitoring Following TP to TS 38.213 Clause 10.4C is endorsed in principle. activation by WUS in RRC_IDLE/RRC_INACTIVE ===== unchanged text omitted ===== A UE receives an LPSS in consecutive symbols within a slot. The UE can be provided one or two first symbols for respective one or two LPSS reception occasions in the slot by lpss-StartSymbol. The UE determines slots for LPSS reception occasions based on a periodicity and a time offset, relative to the start of a system frame with SFN 0, provided by lpss-periodicityoffset. Within a period of LPSS reception occasions, LPSS reception occasions are in a set of consecutive slots that have all symbols indicated as downlink by tdd-UL-DL-ConfigurationCommon, if provided, and start from the first slot provided by the time offset in the period, where K is the number of transmitted SS/PBCH blocks indicated by ssb-PositionsInBurst in SIB1 and L is the number of LPSS reception occasions in a slot. ===== unchanged text omitted =====

| Reason for change: | The reference point for LP-SS reception occasions is unclear. |
| --- | --- |
| Summary of change: | The reference point for LP-SS reception occasions is defined as the start of SFN0. |
| Consequences if not approved: | The reference point for LP-SS reception occasions is unclear. |

Agreement

Following TP to TS 38.213 Clause 10.4C is endorsed in principle.

10.4C PDCCH monitoring activation by WUS in RRC_IDLE/RRC_INACTIVE ===== unchanged text omitted ===== A UE can be provided, by WUS_available_slot_IDLE/INACTIVE, a bitmap that corresponds to a set of time units that repeats continuously and indicates a subset of time units from the set of time units that is available for the UE to monitor WUS [12, TS 38.331]. A time unit includes one slot or two slots. A set of time units can be {10, 20 or 40} time units, but maximum of 40 msec. The first symbol of a set of time units every 40 msec/P periods is a first symbol in frame mod 4 = 0, where P is the duration of a set of time units in msec. The UE can be additionally provided, by WUS_available_symbol_IDLE/INACTIVE, an indication of symbols in each time unit from the subset of time units that is available for the UE to monitor WUS. If the UE is not provided WUS_available_slot_IDLE/INACTIVE, the UE assumes that all time units are available for the UE to monitor WUS. If the UE is not provided WUS_available_symbol_IDLE/INACTIVE, the UE assumes that, for a time unit that is available for the UE to monitor WUS, all symbols in the time unit are available for the UE to monitor WUS. The UE assumes that a symbol is not available to monitor WUS when - the symbol is indicated as uplink, by tdd-UL-DL-configurationCommon - the symbol is indicated for an SS/PBCH block transmission, by ssb-PositionsInBurst in SIB1, and the SS/PBCH block transmission would overlap in frequency with the WUS transmission - the symbol is indicated for PDCCH transmissions, by pdcch-ConfigSIB1, and CORESET 0 for the PDCCH transmissions would overlap in frequency with the WUS transmission ===== unchanged text omitted =====

| Reason for change: | The reference point for applying unit-level bitmap is unclear. |
| --- | --- |
| Summary of change: | The reference point for applying unit-level bitmap is defined as SFN0. |
| Consequences if not approved: | The reference point for applying unit-level bitmap is unclear. |

Agreement

The value range for LP-WUS_NominalMO_duration_IDLE/INACTIVE is:

FR1/FR2: [1-98] OFDM symbols (value of 1 is not applicable to M=1)

Maintenance on LP-WUS operation in CONNECTED modes

Agreement

- For the RRC parameter LP-WUS_Mvalue_CONNECTED,

  - Type: Per cell group on Pcell/PScell

Agreement

- For the RRC parameter LPWUS_PDCCHMonitoringTimer, from RAN1 perspective,

  - Separate parameter is defined/configured for each DRX group

  - Value range

    - subMilliSeconds: INTEGER (1..31)

    - milliSeconds: ENUMERATED {1, 2, 3, 4, 5, 6, 8, 10, 20, 30, 40, 50, 60}

      - It is up to RAN2 whether to include candidate values beyond 60

Agreement

- For the RRC parameters NumOfMO_Monitoring_Option 1-1 and NumOfMO_Monitoring_Option 1-2,

  - Value range: INTEGER {1..4}

Agreement

- For the RRC parameter LP-WUS_TCI_state(s)_CONNECTED,

  - The parameter is configured per BWP on Pcell/Pscell and the configured CORESET#ID shall be within the set of CORESETs configured for the BWP.

  - Value range: Integer (0 .. 15)

Agreement

- For the RRC parameters LP-WUS_available_slot_CONNECTED and LP-WUS_available_symbol_CONNECTED,

  - Type: Per cell group on Pcell/PScell

Agreement

- For the RRC parameter LP-WUS_NominalMO_duration_CONNECTED,

  - Type: Per cell group on Pcell/PScell

  - Value range:

    - FR1/FR2: [1-98] OFDM symbols (value of 1 is not applicable to M=1)

Agreement

- For the RRC parameter LP-WUS_Actual_WUS_duration_CONNECTED,

  - Type: Per cell group on Pcell/PScell

  - Value range:

    - M=1: 2*[1-32] OFDM symbols

    - M=2: 1-32 OFDM symbols

    - M=4: 1-16 OFDM symbols

Agreement

- For the RRC parameter LP-WUS_startRB_CONNECTED,

  - Type: Per cell group on Pcell/Pscell

Agreement

- For the RRC parameters Time_offset_CONNECTED_Option1-1 and Time_offset_CONNECTED_Option1-2,

  - Value range: in multiples of 0.125ms (milliseconds), INTEGER (41, …,  592)

Agreement

- For the RRC parameter LPWUS_MO_CONNECTED_Option1-1,

  - Periodicity/offset parameter

    - sl1, NULL,

    - sl2, INTEGER {0..1},

    - sl4, INTEGER {0..3},

    - sl5, INTEGER {0..4},

    - sl8 INTEGER {0..7}

    - sl10, INTEGER {0..9}

    - sl16, INTEGER {0..15}

    - sl20, INTEGER {0..19}

    - sl40, INTEGER {0..39}

    - sl80, INTEGER {0..79}

    - sl160, INTEGER {0..159}

    - sl320, INTEGER {0..319}

    - sl640, INTEGER {0..639}

    - sl1280, INTEGER {0..1279}

    - sl2560, INTEGER {0..2559}

    - sl5120, INTEGER {0..5119}

    - sl10240, INTEGER {0..10239}

- For the RRC parameter LPWUS_MO_CONNECTED_Option1-2,

  - Periodicity/offset parameter

    - sl1, NULL,

    - sl2, INTEGER {0..1},

    - sl4, INTEGER {0..3},

    - sl5, INTEGER {0..4},

    - sl8 INTEGER {0..7}

    - sl10, INTEGER {0..9}

    - sl16, INTEGER {0..15}

    - sl20, INTEGER {0..19}

    - sl40, INTEGER {0..39}

    - sl80, INTEGER {0..79}

Conclusion

- For the RRC parameter LP-WUS_unifiedTCI_CONNECTED,

  - This parameter is not necessary, and legacy RRC parameter dl-OrJointTCI-StateList is used to apply the unified TCI for LP-WUS reception

2.1.2	Remaining Open issues

None

2.2	RAN2

2.2.1	Agreements

RAN2#131

From RAN2 point of view, the Rel-19 work item NR_LPWUS-Core is completed.

In the running CR, ‘UE supporting LP-WUS’ is used instead of ‘LP-WUS UE’.

Nothing is needed on high priority frequency for serving cell measurement offloading or measurement relaxation with LP-WUS in 38.304 running CR if the corresponding higher priority frequency relaxation has been captured in RAN4 specification.

Same as LP-WUS monitoring, it is up to UE implementation to choose whether SSB measurement based or OOK LP-SS measurement based are used for RRM relaxation/offloading conditions if UE supports both measurement types.

RAN2 assumes the following value range of ThresholdPLP and ThresholdQLP for LR measurement based threshold for entry/exit condition for LP-WUS monitoring and RRM relaxation/offloading

- The IE ThresholdLP is used to indicate a measured RSRP threshold for LP-WUS. Actual value of threshold = field value * 2 [dBm].

- ThresholdP-LP ::=               INTEGER (-80..0)

- The IE ThresholdQ-LP is used to indicate a measured RSRQ threshold for LP-WUS. Actual value of threshold = field value [dB].

- ThresholdQ-LP ::=               INTEGER (-34..0)

LR measurement based RX level and cell quality value should be derived by UE implementation in multi-beam operations. We assume this conclusion does not impact the cell reselection procedure.

RAN2 will keep the current terminologies in RAN2 specification, i.e. LP-WUS, LP-SS, LO (LP-WUS Occasion), LR, and MR.

Confirm the following working assumption to support LP-WUS with dual DRX group.

- Working assumption: LP-WUS can be configured on the PCell with secondary DRX. LP-WUS with secondary DRX is supported with option 1-1 and 1-2, i.e. the UE monitors LP-WUS before the on-duration occasion or periodically outside ActiveTime.  When LP-WUS is detected, then UE starts the drx-onDurationTimer (with option 1-1) or the lpwus-PDCCHMonitoringTimer (with option 1-2) in both DRX groups.

If secondary DRX group is configured, the lpwus-PDCCH-MonitoringTimer configuration for secondary DRX group is different from that for the default DRX group.

If secondary DRX group is configured, UE monitors LP-WUS only when both DRX groups are not in DRX active time.

RAN2 understand that the RAN1 agreement on not supporting simultaneous LR and MR operation is only applicable within one cell group (with or without secondary DRX group configuration). Send LS to RAN1 for confirmation.

The lpwus-PDCCH-MonitoringTimer configuration for secondary DRX group is smaller than or equal to that for the default DRX group.

Confirm the following RAN2#129bis working assumption for Option 1-1:

- In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected).

For Option 1-2, UE does not start the lpwus-PDCCH-MonitoringTimer in collision cases, i.e. when the UE is not able to monitor the LP-WUS occasion(s). Can discuss if critical issue identified with this mechanism.

Agree the addition of the MUSIM gap case), for the UE operation in Option 1-1 for the collision and timing issue.

There is no MAC spec impact to reflect the LP-WUS operation in Cell DTX operation. Can further check in maintenance phase.

RAN2 confirm that the available UL occasions (e.g. SR occasion, RACH occasion, CG occasion) are MR-ready. Can further check whether any spec change is needed.

RAN2 assumes NAS signalling is introduced to support enabling/disabling LP-WUS per UE. Inform SA2, CT1 and RAN3 about this conclusion.

RAN2 assumes that NAS signalling needs to be extended to enable/disable LP-WUS for a UE in IDLE and INACTIVE. Detail signalling is up to SA2, CT1.

RAN2 assumes that CN needs to inform gNB that LP-WUS is enabled/disabled for a UE in IDLE and INACTIVE. Details signalling is up to SA2, CT1, RAN3.

RAN2 assumes that without such NAS singalling, UE is allowed to use LPWUS in IDLE and INACTIVE.

The LS is approved unseen in R2-2506261

LP-WUS can be used in any cell, i.e., don’t introduce lastUsedCellOnly for LP-WUS.

In RRC_INACTIVE state, for LP-WUS, when the UE uses the same i_s as for RRC_IDLE state, the UE shall use the same iPO LP-WUS as for RRC_IDLE state. Otherwise, the UE determines the iPO for LP-WUS for based on the i_s for RRC_INACTIVE state.

In RRC_INACTIVE state with CN configured PTW, the SubgroupID for LP-WUS used outside CN PTW is the same as the SubgroupID used inside CN PTW.

Confirm that SDT can be initiated while UE is monitoring LP-WUS, and there is no impact to the SDT procedure. Can check if any spec change is needed.

For the RRC-IDLE and RRC-INACTIVE, if the UE is not able to monitor the LP-WUS in all MO then the UE is required to monitor the following PEI and/or PO. Detailed changes to the spec can be further checked.

RAN2 assumes the entry/exit thresholds for LP-WUS monitoring for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. Network is allowed to provide either OOK based threshold or OFDM based WUR measuring SSB threshold or both.

The capability for supporting UE_ID based subgrouping is defined as one of the components of LP-WUS operation basic features based on OOK signal (62-1) and OFDM overlaid sequence (62-1a) in IDLE/INACTIVE.

The exit condition for RRM relaxation is defined as ‘not fulfilling the entry condition’.

RAN2 assumes UE low mobility criterion is not included for Rel-19 LP-WUS RRM relaxation/offloading mode.

LR measurement is not used for R16/R17 RRM relaxation condition.

RAN2 assume UE does not start or re-start the bwp-InactivityTimer when receiving the LP-WUS.

2.2.2	Remaining Open issues

None

2.3	RAN3

2.3.1	Agreements

RAN3#129

The range of CN assigned subgroup ID in NG/Xn/F1 message is 0.. 30.

The solution for missing UE capability for paging features will be introduced from rel 17 for initial context setup.

The anchor gNB provides the new LP-WUS paging subgrouping assistance information IE into the Xn RAN paging message including CN assigned subgroup ID.

2.3.2	Remaining Open issues

None

2.4	RAN4

2.4.1	Agreements

LP-WUS RRM

RAN4#116

The Topic summary for LP-WUS RRM is recorded in [1];

The ad-hoc meeting minutes for [116][222] NR_LPWUS is approved in [145];

The WF on LP-WUS RRM is approved in [146] with the following agreements:

Sub-topic 1-1 General aspects

Issue 1-1-14: LP-WUR operation with eDRX

Agreement: No RAN4 RRM requirements for LP-WUR operation with eDRX with PTW window in Rel-19.

For LP-WUR operation with eDRX without PTW (i.e., for eDRX cycles <=10.24s)

Agreement: No RAN4 RRM requirements

Issue 1-1-15 LP-WUR operation with RedCap

Specify LP-WUR related idle/inactive requirements including requirement on serving cell offloading, RRM relaxation and higher priority frequency layer search for Redcap UE.

  - Existing requirements for MR offloading, RRM relaxation and higher priority frequency layer search will be reused for Redcap UE

    - Confirm the MR wake up delay will apply for 2 Rx Redcap

Issue 1-1-16: LP-WUR operation with EMR

Agreement:

When both Rel-16 EMR and Rel-19 LP-WUR are configured:

When T331 is running

  - When Case#1 (RRM offloading) conditions are met:

    - The UE shall search for NR inter-frequency layers configured for idle mode CA/DC measurements by following the higher priority frequency measurement requirements (i.e., based on 60s).

    - Higher priority NR inter-frequency layers follow the higher priority frequency measurement requirements (i.e., based on 60s).

  - When Case#3 (RRM relaxation) conditions are met:

    - If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ

      - The UE shall search for NR inter-frequency layers configured for idle mode CA/DC measurements by following the higher priority frequency measurement requirements (i.e., based on 60s).

      - Higher priority NR inter-frequency layers follow the higher priority frequency measurement requirements (i.e., based on 60s).

    - if Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ

      - the UE shall search for NR inter-frequency layers configured and not configured for idle mode CA/DC measurements by following the legacy measurement requirements (no relaxation)

when T331 is not running,

  - Serving cell, intra-frequency and NR inter-frequency layers not configured for idle mode CA/DC measurements and NR inter-frequency layers configured for idle mode CA/DC measurements and mobility measurement follow corresponding agreed requirements defined in case 1 and case 3 when they are satisfied.

Sub-topic 1-2 Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

Issue 1-2-1-1: Detail on LR accuracy and side conditions requirements

Use 2.5 dB as the RF impairment margin for LP-RSRP accuracy requirements.

Remove [] in agreements in RAN4 114bis and the final agreements are as:

For FR1:

    - 3.5 dB is used for core requirements for LP-RSRQ accuracy and 6 dB is used for core requirements for LP-RSRP accuracy, under the side conditions Ês/Iot = -3 dB

    - 3.5 dB is used for core requirements of SSB based RSRQ accuracy and 6 dB is used for core requirements for SSB based RSRP accuracy, under the side conditions Ês/Iot = -3 dB

Issue 1-2-4-2-3: On how to define LR evaluation requirements

Agreement:

Using x1=2*x and y1=2*y for the evaluation requirement.;

y = 2;

For x, x= 3;

Issue 1-2-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case)

Agreement:

  - No LR measurement and evaluation requirements apply at the legacy state, i.e., for the following cases: from legacy case to LP-WUR monitoring, from legacy case to RRM measurement fully offloading (case 1), and from legacy case to RRM measurement relaxation (case 3).

  - At legacy state, when both MR and LR entry thresholds are configured, the UE shall evaluate both MR and LR thresholds at least once before entering relaxation / offloading

Issue 1-2-11: RRM requirements for FR2

On the SSB based LR FR2 requirement

| LO periodicity [s] Note 1 |
| --- |
|  | FR2 (Scaling factor N1) |
| 0.32 | 8 |
| 0.64 | 5 |
| 1.28 | 4 |
| 2.56 | 3 |

No requirement for the FR2 LP-SS based LR

For SSB based FR2 LR,

- the target measurement accuracy for FR2 SS-RSRP is 7.5 dB.

- the target measurement accuracy for FR2 SS-RSRQ is 3.5dB.

For MR-based relaxed serving and neighbour cell measurements in FR2, reuse the measurement relaxation factor of 16 as agreed for FR1.

Note: No RF margin for RSRQ accuracy requirement, and 4dB RF margin for FR2 RSRP requirement.

Issue 1-2-12: UE behaviro when both Rel-16 relaxation and Rel-19 LP-WUR offloading/relaxation are satisfied; or when both Rel-17 relaxation for Redcap and Rel-19 LP-WUR offloading/relaxation are satisfied

Agreement:

UE is allowed to follow the most relaxation requirements when only Rel-16 not at cell edge is configured, and Rel-16 not at cell edge and Rel-19 LP-WUR offloading/relaxation are both satisfied and Rel-16 EMR is not configured or Rel-16 EMR is configured and T331 is not running.

Sub-topic 1-3 MR RRM relaxation

Sub-topic 1-4 LP-WUR CONNECTED mode

Issue 1-4-1: LP-WUR at CONNECTED mode

Agreement: No RRM impact for connected mode in this WI.

Note: This can be revisited if any issue is identified later

Sub-topic 1-5 Others

Issue 1-5-3: MR wake up delay after exiting case 1 due to offloading conditions cannot be met

Agreement: When exiting case 1 due to exiting conditions of offloading is met, MR reuse the existing wake up delay.

LP-WUS UE RF

RAN4#116

The new version of TR 38.774 v0.5.0 is approved in [52];

The Topic summary for LP-WUS RF is recorded in [2];

The big CR for LP-WUS/WUR UE RF is agreed in [142];

The WF on UE RF testability is approved with the following detailed agreements:

Issue 4-1-1 test parameters for Side condition of Rx requirements

Agreement:

The following setting will be used for Rx requirements related testing. RAN4 will inform it to RAN5 next meeting via LS.

    - LP-WUS, LPSS and SSB all share the same EPRE.

    - LPSS shares the same M and same length as LP-WUS

    - LPSS shares the same length in OFDM symbols as LP-WUS

    - LPSS is transmitted at lowest periodicity (160 ms), or equivalently at highest rate of repetition

    - SSB is transmitted at 20 ms periodicity

Issue 4-1-2: Whether all NR CBW requirements should be tested

Agreement:

  - RAN5 can decide.

Issue 4-1-3: Whether both OOK and OFDM should be tested, if UE support both

Agreement:

  - Make suggestion to RAN5: For the case when LR supporting both OOK and OFDM waveforms, only test with OFDM waveform.

Issue 4-1-3: Whether spherical coverage can be skipped, if UE pass REFSENS

Agreement:

  - FFS whether need to verify to confirm the spherical coverage performance

Issue 4-1-7: RAN4 Input on Test mode

Agreement:

  - Leave test mode decision to RAN5.

Issue 4-1-8: Testing time reduction consideration for LP-WUS

Agreement:

  - Make suggestion to RAN5: Testing time reduction of LP-WUS RF test cases can be considered.

Issue 4-1-9: LS to RAN5 on Testability

Agreements:

  - Send a LS to RAN5 on RF testability considerations to RAN5 next meeting.

LP-WUS BS RF

RAN4#116

The Topic summary for LP-WUS BS RF is recorded in [133];

The big CR for LP-WUS/WUR BS RF is agreed in [161];

LP-WUS Demodulation

RAN4#116

The Topic summary for LP-WUS Demod is recorded in R4-2511476;

The ad-hoc meeting minutes for [116][330] NR_LPWUS_demod is captured in R4-2512621;

The WF on LP-WUS demod is agreed R4-2512526 with the following detailed agreements:

LP-WUS Demodulation testability

Issue 1-1-1: Whether to define a test mode for LP-WUS/WUR

- Agreement: Interested companies can provide an analysis of the expected test durations for the different modes next meeting. Further discuss if a test mode is required based on the provided analysis.

Issue 1-1-2: Whether to consider shortening the test duration

- Proposals

  - Option 1: Yes, if FAR test is introduced, introduce new test mode to reduce the FAR test run time.

  - Option 2: Based on the test mode, and the necessity to make sure the test confidence, it seems no need to add additional assumptions in order to shortening the test duration.

Issue 1-1-3: Whether to define separate test cases for idle/inactive and connected modes

- Proposals

  - Option 1: Define separate test cases procedure for the modulator in idle and connected modes.

Demodulation requirements

Issue 2-1-1: Assumption of FAR for the MDR test

- Agreement: 1% FAR

Issue 2-1-2: Whether to introduce FAR test

- Proposals

  - Option 1: Yes

  - Option 2: No

Issue 2-1-3: Test metric for FAR test (if introduced)

- Proposals

  - Option 1: 1% FAR

  - Option 2: 0.1% FAR

Issue 2-1-4: LP-WUS waveform for FR1

- Agreement: Consider OOK-4 only

Issue 2-1-5: LP-WUS waveform for FR2

- Agreement: Consider OOK-1 only

Issue 2-1-6: Receiver type

- Agreement: Consider both OOK and OFDM receivers

Issue 2-1-7: Frequency range

- Agreement: Consider both FR1 and FR2. Finalize the FR1 parameters firstly and then consider the FR2 parameters.

Issue 2-1-8: Duplex mode

  - Agreement: Consider both TDD and FDD.

Issue 2-1-9: Number of RBs

  - Agreement:  11RB for all channel bandwidth and SCS combinations

Issue 2-1-10: SCS

- Agreement:

  - 15kHz for FDD FR1 and 30kHz for TDD FR1

  - 120KHz for TDD FR2

Issue 2-1-11: Bandwidth

- Agreement:

  - 10MHz CHBW with SCS 15KHz for FR1 FDD and 40MHz CHBW with SCS 30KHz for FR1 TDD

  - 100MHz CHBW with SCS 120KHz for FR2 TDD

Issue 2-1-12: Carrier frequency

- Proposals

- Option 1: Use fc=900MHz for FR1 FDD and fc=2.6GHz for FR1 TDD as starting point

- FFS for FR2 TDD

Issue 2-1-13: Antenna configuration

- Agreement:

  - 1T1R for FR1

  - FFS: 1T1R for FR2

Issue 2-1-14: Channel model

- Agreement:

  - Consider both TDLA30-10 and TDLC300-100 for FR1 evaluation. Further discuss down-selection based on the simulation results.

  - Consider TDLA30-10 only for FR2

Issue 2-1-15: Whether to consider ideal time and frequency offsets for demodulation requirements

- Agreement: RAN4 should not configure the reception timing error and frequency error in TS 38.101-4 (only for specification)

Issue 2-1-16: Calculation of timing error

- Proposals

  - Option 1: Time error= Residual time error + time drift (frequency offset*320ms)

Issue 2-1-17: Time offsets

- Agreement:

  - T= 5us for OOK-1, T= 2us for OOK-4 with M=2, T= 1us for OOK-4 with M=4

  - Note: The selection of time offsets will depend on the agreed LP-WUS waveform.

Issue 2-1-18: Frequency offsets

- Agreement: For OFDM based receiver 5 ppm and for OOK based receiver 10 ppm.

Issue 2-1-19: Whether to define performance requirements for LP-SS

- Agreement: Do not define performance requirements for LP-SS

Issue 2-1-20: Whether to configure LP-SS for WUS detection requirements

- Agreement:  RAN4 should configure LP-SS for the WUS reception requirements (only for specification)

Issue 2-1-21: Whether to define single requirement for Type 1 and Type 2 receivers

- Proposals

  - Option 1: RAN4 to study the performance for both receiver types (Type 1: OOK and Type 2: OFDM) and further discuss based on simulation results if one or two sets of requirements are to be defined.

  - Option 2: Do not define separate demodulation requirements for Type-1 and Type-2 LP-WUR capable UEs if those are not tied to Envelope vs Sequence Detection.

Issue 2-1-22: Whether to define capability rules for Type 1 and Type 2 receivers

- Proposals

  - Option 1: Don’t define any capability rules related to Type1/2 receivers for demodulation requirements.

Issue 2-1-23: Whether to re-use the definition from RAN4 RF for the receiver types (if separate requirements are introduced)

- Proposals

  - Option 1: If agreed to introduce two sets of requirements, then re-use the definition from RAN4 RF for the receiver types.

Issue 2-1-24: Coding/decoding algorithms for LP-WUS

- Agreement: small block coding for 2 info bits and RM coding for 5 info bits as a starting point

Issue 2-1-25: Number of coded bits after rate-matching

- Agreement:

  - [12, 16] for 2 info bits

  - 32 for 5 info bits

  - Other options are not precluded

Issue 2-1-26: Choice of M for ORFM receiver

- Agreement:

  - M=1 and M=4 as a starting point

  - Interested companies can also bring results with M=2

Issue 2-1-27: Inclusion of requirements in the specification

- Proposals

  - Option 1:

    - RAN4 to add a new subclause ‘5.7 Wake-up signal (WUS) demodulation requirements’ in TS 38.101-4 for FR1.

    - RAN4 to add a new subclause ‘7.7 Wake-up signal (WUS) demodulation requirements’ in TS 38.101-4 for FR2.

2.4.2	Remaining Open issues

Core part:

- none

Performance part:

- RRM performance requirements and test cases

- Demodulation performance requirements and test cases

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

### RAN1#122

- R1-2506524	Summary #1 of discussion on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2506525	Summary #2 of discussion on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2505226	Maintenance on LP-WUS and LP-SS design	Huawei, HiSilicon

- R1-2505330	Remaining issues on LP-WUS and LP-SS design	CATT

- R1-2505381	Maintenance on LP-WUS and LP-SS design	vivo

- R1-2505545	Remaining issues on LP-WUS and LP-SS design	Samsung

- R1-2505598	Discussion on LP-WUS design	ZTE Corporation, Sanechips

- R1-2505742	Signal design for LP-WUS and LP-SS	OPPO

- R1-2505773	Remaining issues relating to LP-WUS and LP-SS design	Nokia

- R1-2505845	Remaining issues on LP-WUS and LP-SS design	LG Electronics

- R1-2505880	Maintenance on LP-WUS and LP-SS design	Apple

- R1-2505951	Maintenance on LP-WUS and LP-SS design	Ericsson

- R1-2506183	Maintenance on LP-WUS and LP-SS design	Qualcomm Incorporated

- R1-2506464	Summary #1 on maintenance for LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2505227	Maintenance on LP-WUS operation in IDLE/INACTIVE mode	Huawei, HiSilicon

- R1-2505331	Remaining issues on LP-WUS operation in IDLE/Inactive modes	CATT

- R1-2505382	Maintenance on LP-WUS operation in IDLE/INACTIVE modes	vivo

- R1-2505546	Remaining issues on LP-WUS operation in IDLE/INACTIVE modes	Samsung

- R1-2505599	Discussion on LP-WUS operation in IDLE/INACTIVE mode	ZTE Corporation, Sanechips

- R1-2505743	Remaining issues on LP-WUS operation in RRC_IDLE/INACTIVE modes	OPPO

- R1-2505774	Remaining issues relating to LP-WUS in Idle/Inactive Mode	Nokia

- R1-2505846	Remaining issues on LP-WUS operation in IDLE/INACTIVE modes	LG Electronics

- R1-2505881	Maintenance on LP-WUS operation in IDLE/INACTIVE modes	Apple

- R1-2505952	Maintenance on LP-WUS operation in IDLE/INACTIVE modes	Ericsson

- R1-2506010	Remaining issues on LP-WUS operation in IDLE/INACTIVE modes	Sharp

- R1-2506081	Maintenance of LP-WUS operation in IDLE/INACTIVE mode	CMCC

- R1-2506184	Maintenance on LP-WUR operation in idle and inactive modes	Qualcomm Incorporated

- R1-2506278	Maintenance on LP-WUS operation in IDLE/INACTIVE modes	NTT DOCOMO, INC.

- R1-2506517   FL summary#1 on maintenance on LP-WUS operation in CONNECTED mode 	Moderator (NTT DOCOMO)

- R1-2506546   FL summary#2 on maintenance on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

- R1-2505228	Maintenance on LP-WUS in CONNECTED mode	Huawei, HiSilicon

- R1-2505332	Remaining issues on LP-WUS operation in CONNECTED modes	CATT

- R1-2505383	Maintenance on LP-WUS operation in CONNECTED modes	vivo

- R1-2505435	Remaining issues on LP-WUS operation in connected mode	Xiaomi

- R1-2505547	Remaining issues on LP-WUS operation in CONNECTED modes	Samsung

- R1-2505600	Discussion on LP-WUS operation in CONNECTED mode	ZTE Corporation, Sanechips

- R1-2505664	Maintenance of LP-WUS Operation in CONNECTED mode	 Ofinno

- R1-2505744	Remaining issues on LP-WUS operation in connected mode	OPPO

- R1-2505775	Remaining issues relating to LP-WUS in Connected Mode 	Nokia

- R1-2505847	Remaining issues on LP-WUS operation in CONNECTED modes	LG Electronics

- R1-2505882	Maintenance on LP-WUS operation in CONNECTED modes	Apple

- R1-2505953	Maintenance on LP-WUS operation in CONNECTED modes	Ericsson

- R1-2506011	Remaining issues on LP-WUS operation in CONNECTED modes	Sharp

- R1-2506082	Maintenance of LP-WUS operation in CONNECTED mode	CMCC

- R1-2506185	LP-WUR operation in connected mode	Qualcomm Incorporated

- R1-2506279	Maintenance on LP-WUS operation in CONNECTED mode	NTT DOCOMO, INC.

### RAN2#131

- R2-2505020	Reply LS on LP-WUS in RRC_CONNECTED (R1-2504888; contact: NTT DOCOMO)	RAN1	LS

- R2-2505025	Reply LS on LP-WUS UE RF (R1-2504943; contact: vivo)	RAN1	LS

- R2-2505028	LS on TP to TS38.300 for Rel-19 LP-WUS/WUR (R1-2505070; contact: vivo)	RAN1	LS in

- R2-2505035	Reply LS on LP-WUS subgrouping progress (R3-253846; contact: NEC)	RAN3	LS in

- R2-2505234	Introduction of LP-WUS in TS 38.304	CATT

- R2-2505392	Introduction of LP-WUS/WUR in RRC	vivo (Rapporteur)	CR

- R2-2505469	Introduction of LP-WUS in TS 37.340	ZTE Corporation, Sanechips

- R2-2505476	Running MAC CR for LP-WUS	Apple (Rapporteur)

- R2-2505863	Introduction of Low-Power Wake-Up Signal and Receiver for NR	Ericsson

- R2-2505670	Introduction of R19 LP-WUS UE Capabilities	Huawei, HiSilicon

- R2-2505235	Discussion of [Post130][211][LPWUS] Running CR for 38.304 (CATT)	CATT

- R2-2505393	Discussion summary and list of RRC open issue for LP-WUS WUR	vivo

- R2-2505477	Report of [Post130][213][LPWUS] Running CR for TS 38.321 (Apple)	Apple(Rapporteur)

- R2-2505478	Draft Reply LS on LP-WUS in RRC_CONNECTED	Apple	LS out

- R2-2505381	Summary of [Post130][222][LPWUS] Potential solution to support enabling/disabling LP-WUS monitoring in IDLEI/NACTVE per UE (Huawei)	Huawei, HiSilicon

- R2-2506038	IDLE/Inactive LP-WUS disabling and enabling	Qualcomm Incorporated

- R2-2505605	Discussion on LP-WUS procedure and configuration	OPPO

- R2-2505856	LP-WUS in Idle and Inactive	Ericsson

- R2-2505605	Discussion on LP-WUS procedure and configuration	OPPO

- R2-2505479	Remaining issues of LP-WUS in RRC_IDLE/INACTIVE	Apple

- R2-2505976	Remaining issues on LP-WUS in RRC IDLE or INACTIVE	LG Electronics Inc.

- R2-2505856	LP-WUS in Idle and Inactive	Ericsson

- R2-2505394	Discussion on LP-WUS WUR in RRC_IDLE INACTIVE	vivo

- R2-2505236	Open issues on LP-WUS in IDLE and INACTIVE	CATT

- R2-2505280	Remaining issues on LP-WUS paging monitoring	Xiaomi Communications

- R2-2505336	Discussion on LP-WUS in RRC_IDLE INACTIVE 	NEC

- R2-2505379	Further discussion on LP-WUS in RRC_IDLE/INACTIVE	Huawei, HiSilicon

- R2-2505381	Summary of [Post130][222][LPWUS] Potential solution to support enabling/disabling LP-WUS monitoring in IDLEI/NACTVE per UE (Huawei)	Huawei, HiSilicon

- R2-2505394	Discussion on LP-WUS WUR in RRC_IDLE INACTIVE	vivo

- R2-2505479	Remaining issues of LP-WUS in RRC_IDLE/INACTIVE	Apple

- R2-2505529	Procedure and Configuration of LP-WUS in RRC Idle Inactive Mode	Samsung

- R2-2505588	Remaining issues on LP-WUS in IDLE and INACTIVE	NTT DOCOMO INC.

- R2-2505605	Discussion on LP-WUS procedure and configuration	OPPO

- R2-2505629	Discussion on prioritizing the frequencies supporting LP-WUS	Huawei, HiSilicon, vivo, Nokia, Samsung, LG Electronics Inc., Apple, Ericsson, OPPO, Sharp, NEC

- R2-2505655	Disabling/enabling LP-WUS in RRC Idle/Inactive mode	Sony

- R2-2505682	Open issues on LP-WUS in RRC_IDLE/INACTIVE mode	Lenovo

- R2-2505752	LP-WUS in IDLE and INACTIVE	Nokia

- R2-2505779	Remaining issues in IDLE/INACTIVE procedure for LP-WUS	Tejas Network Limited

- R2-2505856	LP-WUS in Idle and Inactive	Ericsson

- R2-2505906	Remaining issues on LP-WUS operation in RRC_IDLE/INACTIVE modes	InterDigital, Inc.

- R2-2505968	Remaining issues of LP-WUS operation in IDLE/INACTIVE	CMCC

- R2-2505976	Remaining issues on LP-WUS in RRC IDLE or INACTIVE	LG Electronics Inc.

- R2-2505992	Procedure and configuration of LP-WUS for IDLE and INACTIVE mode ZTE Corporation, Sanechips

- R2-2506038	IDLE/Inactive LP-WUS disabling and enabling	Qualcomm Incorporated

- R2-2505967	Remaining issues of RRM measurement relaxation and offloading in RRC_IDLE INACTIVE	CMCC

- R2-2505907	Remaining issues on RRM measurement relaxation and offloading	InterDigital, Inc.

- R2-2506040	Open issues on LP-WUS RRM measurement relaxation and offloading	Qualcomm Incorporated

- R2-2505530	RRM measurement relaxation and offloading in RRC Idle Inactive Mode	Samsung

- R2-2505395	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE 	vivo

- R2-2505289	Remaining issues on RRM measurement relaxation for RRC_IDLE_INACTIVE Xiaomi Communications

- R2-2505237	Open issues on RRM Relaxation and Offloading in IDLE and INACTIVE	CATT

- R2-2505237	Open issues on RRM Relaxation and Offloading in IDLE and INACTIVE	CATT

- R2-2505289	Remaining issues on RRM measurement relaxation for RRC_IDLE_INACTIVE Xiaomi Communications

- R2-2505395	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	 vivo

- R2-2505480	Remaining issues of LP-WUS RRM Measurement	Apple

- R2-2505530	RRM measurement relaxation and offloading in RRC Idle Inactive Mode	Samsung

- R2-2505596	Remaining issues on RRM measurement relaxation and offloading	NTT DOCOMO INC.

- R2-2505606	Discussion on the remaining issues on RRM measurement	OPPO

- R2-2505683	Open issues on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Lenovo

- R2-2505737	Further discussion on the criteria for RRM measurement relaxation and offloading	Huawei, HiSilicon

- R2-2505753	RRM measurement relaxation in RRC_IDLE/INACTIVE	Nokia

- R2-2505780	Remaining issues in LP-WUS based RRM relaxation and offloading 	Tejas Network Limited

- R2-2505803	Remaining issues for LP-WUS RRM	ZTE Corporation, Sanechips

- R2-2505857	LP-WUS and RRM measurements	Ericsson

- R2-2505907	Remaining issues on RRM measurement relaxation and offloading	InterDigital, Inc.

- R2-2505967	Remaining issues of RRM measurement relaxation and offloading in RRC_IDLE INACTIVE	CMCC

- R2-2505977	Remaining issues on measurement offloading and relaxation	LG Electronics Inc.

- R2-2506040	Open issues on LP-WUS RRM measurement relaxation and offloading	Qualcomm Incorporated

- R2-2505630	Further discussion on LP-WUS for RRC_CONNECTED mode	Huawei, HiSilicon

- R2-2505684	Open issues on LP-WUS in RRC Connected mode	Lenovo

- R2-2505396	Discussion on LP-WUS WUR in RRC_Connected	vivo

- R2-2505993	Procedure for LP-WUS in RRC_Connected state	ZTE Corporation, Sanechips

- R2-2505630	Further discussion on LP-WUS for RRC_CONNECTED mode	Huawei, HiSilicon

- R2-2505108	Discussing on LP-WUS monitoring in Connected mode	Xiaomi

- R2-2505238	Analysis on LP-WUS for RRC_CONNECTED	CATT

- R2-2505396	Discussion on LP-WUS WUR in RRC_Connected	vivo

- R2-2505463	Remaining issues on LP-WUS in RRC_CONNECTED	LG Electronics Inc.

- R2-2505481	Remaining issues of LP-WUS in RRC_CONNECTED	Apple

- R2-2505531	Procedures for LP-WUS in RRC Connected Mode	Samsung

- R2-2505581	LP-WUS in RRC_CONNECTED	Nokia, Nokia Shanghai Bell

- R2-2505597	Remaining issues on LP-WUS in RRC_CONNECTED	NTT DOCOMO INC.

- R2-2505607	Discussion on LP-WUS in RRC_CONNECTED	OPPO

- R2-2505630	Further discussion on LP-WUS for RRC_CONNECTED mode	Huawei, HiSilicon

- R2-2505645	LP-WUS in CONNECTED mode	InterDigital

- R2-2505684	Open issues on LP-WUS in RRC Connected mode	Lenovo

- R2-2505782	Remaining issues in CONNECTED procedure for LP-WUS	Tejas Network Limited

- R2-2505858	LP-WUS in Connected	Ericsson

- R2-2505942	Discussion on LP-WUS operation in CONNECTED mode	CMCC

- R2-2505993	Procedure for LP-WUS in RRC_Connected state	 ZTE Corporation, Sanechips

- R2-2506039	Open issues on LP-WUS operation in CONNECTED state	Qualcomm Incorporated

### RAN3#129

- R3-255082	(BL CR to 38.300) Introduction of low-power wake-up signal and receiver for NR (vivo, NTT DOCOMO INC., Nokia, Samsung, Ericsson, ZTE, CATT, Huawei)

- R3-255083	(BL CR to 38.470) Introduction of low-power wake-up signal and receiver for NR (Nokia, Samsung, ZTE, Ericsson, Huawei, CATT)

- R3-255084	(BL CR to 38.410) Introduction of LP-WUS (CATT, Nokia, Ericsson, Huawei, Vivo, ZTE, Qualcomm, NTT Docomo)

- R3-255085	(BL CR to 38.413) Introduction of low-power wake-up signal and receiver for NR (ZTE Corporation, Nokia, Samsung, Ericsson, Huawei, CATT, NTT DOCOMO)

- R3-255086	(BL CR to 38.420) Introduction of LP-WUS (Qualcomm, ZTE Corporation, Nokia, Ericsson, CATT, Huawei, Samsung)

- R3-255087	Introduction of low-power wake-up signal and receiver for NR (Ericsson, Vivo, NTT Docomo, Huawei, ZTE, Nokia, Samsung, CATT)

- R3-255088	(BL CR to 38.473) Introduction of low-power wake-up signal and receiver for NR (Huawei, Nokia, Samsung, Ericsson, CATT, ZTE, Vivo, NTT DOCOMO INC.)

- R3-255026	Response to Reply LS on paging capability loss issue (SA2(Vodafone))

- R3-255528	[Draft] Reply LS on paging capability loss issue (VODAFONE Group Plc)

- R3-255420	Further discussion on LP-WUS (NTT DOCOMO INC..)

- R3-255237	(TP to BLCR for TS 38.413, TS 38.423 and TS 38.300) Discussion on Low-power wake-up signal and receiver for NR (Huawei)

- R3-255220	[TP for LP-WUS BL CR TS 38.423] Conclusions for LP-WUS open points (Nokia)

- R3-255256	Support LP-WUS subgrouping in RRC_IDLE/RRC_INACTIVE (Qualcomm Inc.)

- R3-255567	Discussion on forwarding of paging capabilities (Ericsson)

- R3-255527	Missing UE radio paging capabilities (VODAFONE Group Plc)

- R3-255230	Remaining issues on LP-WUS (ZTE Corporation)

- R3-255250	(TP to BLCR of TS 38.300) On remaining issues of LP-WUS (CATT)

- R3-255706	Discussion on paging capability loss issue (CMCC)

- R3-255652	Further discussion on paging capability loss issue (Huawei, China Unicom, China Telecom)

- R3-255221	[TP for LP-WUS BL CR TS 38.300] Conclusion of Paging Loss (Nokia)

- R3-255231	(TP to TS 38.413) on support of LP-WUS (ZTE Corporation)

- R3-255238	(TP to BLCR for TS 38.473) Discussion on Low-power wake-up signal and receiver for NR (Huawei)

- R3-255251	(TP to BLCR for TS 38.413, 38.423, 38.473) On CN Subgroup ID range for LP-WUS (CATT)

- R3-255529	Paging capability loss in RAN (VODAFONE Group Plc)

- R3-255530	Paging capability loss in RAN (VODAFONE Group Plc)

- R3-255531	Paging capability loss in RAN (VODAFONE Group Plc)

- R3-255568	(TP for LP-WUS BL CR for TS38.423) Removal of FFS (Ericsson)

- R3-255653	Avoiding paging capability loss for CN paging (Huawei, China Unicom, China Telecom)

- R3-255654	Avoiding paging capability loss for CN paging (Huawei, China Unicom, China Telecom)

- R3-255655	Avoiding paging capability loss for CN paging (Huawei, China Unicom, China Telecom)

- R3-255656	Avoiding paging capability loss for CN paging (Huawei, China Unicom, China Telecom)

- R3-255657	Avoiding paging capability loss for CN paging (Huawei, China Unicom, China Telecom)

- R3-255658	Avoiding paging capability loss for CN paging (Huawei, China Unicom, China Telecom)

### RAN4#116

- R4-2509065	Topic summary for [116][222] NR_LPWUS	Moderator (vivo)

- R4-2509103	Topic summary for [116][136] NR_LPWUS_UERF	Moderator (Vivo)

- R4-2509118	LP WUR UE RX REFSENS	Murata Manufacturing Co Ltd.

- R4-2509119	LP WUR UE RX ACS	Murata Manufacturing Co Ltd.

- R4-2509170	Discussion on demodulation requirements for R19 LP-WUS/WUR	MediaTek inc.

- R4-2509192	BS RF requirements for low-power wake-up signal for NR	Nokia

- R4-2509285	Draft CR to TS 38.133 on definitions, symbols and abbreviations	CATT

- R4-2509286	Draft CR to TS 38.133 on measurements of inter-RAT E-UTRAN cells for UE with LP-WUR	CATT

- R4-2509287	Discussion on RRM performance requirements for Rel-19 LP-WUS	CATT

- R4-2509346	Further discussion on LP-WUS demodulation requirements	CATT

- R4-2509374	Finalized consideration on testability for UE RF requirements for LP-WUS	CATT

- R4-2509378	TP to TR 38.774 on transmitted signal quality (clause 7.2.4) for LP-WUS	CATT

- R4-2509379	draftCR to TS 38.101-1 on UE channel bandwidth for LP-WUS (sub-clause 5.3M)	CATT

- R4-2509380	Finalized consideration on REFSENS, ACS and ASCS for LP-WUS	CATT

- R4-2509381	Finalized consideration on other RF requirements for LP-WUS	CATT

- R4-2509382	Finalized consideration on BS RF requirements for LP-WUS	CATT

- R4-2509383	CR to TS 38.141-1 on declaration of support for LP-WUS	CATT

- R4-2509384	CR to TS 38.141-2 on declaration of support for LP-WUS	CATT

- R4-2509500	On RRM core requirements for LP-WUR	Apple

- R4-2509545	On LP-WUR Rx requirements	Apple

- R4-2509574	Discussion of Simulation Assumptions and Results for LP-WUR RRM Requirements	Apple

- R4-2509575	Summary of Simulation Results for LP-WUR RSRP Requirements	Apple

- R4-2509576	On RRM Performance for LP-WUS/WUR	Apple

- R4-2509577	On the MDR Performance Requirements for LP-WUS	Apple

- R4-2509676	Simulation assumptions and results for LP-WUSWUR	OPPO

- R4-2509677	Discussion on RRM core requirements for LP-WUSWUR	OPPO

- R4-2509678	DraftCR on measurements of intra-frequency NR cells for UE with LP-WUR in IDLE and INACTIVE state	OPPO

- R4-2509679	Discussion on RRM performance requirements for LP-WUSWUR	OPPO

- R4-2509737	Simulation results for RRM LP-SS simulation campaign	LG Electronics Inc.

- R4-2509738	Discussion on simulation results for LP-WUS/WUR	LG Electronics Inc.

- R4-2509739	Draft CR for Measurements of inter-frequency NR cells for UE with LP-WUR	LG Electronics Inc.

- R4-2509774	Discussion on simulation assumptions and results for LP-WUS/WUR	Xiaomi

- R4-2509775	Discussion on RRM core requirements for LP-WUS/WUR	Xiaomi

- R4-2509776	(NR_LPWUS) draftCR on 5.X.2.3 MR serving cell measurement and evaluation requirements	Xiaomi

- R4-2509937	Views on requirements of the low-power wake-up receiver	Sony

- R4-2510030	Discussion on RRM impact of LP-WUR	CMCC

- R4-2510031	(NR_LPWUS-Core) draftCR on Introduce inter-frequency and inter-RAT cell re-selection requirements for UE with LP-WUR in RRC_Inactive	CMCC

- R4-2510032	Discussion on test cases for LP-WUS	CMCC

- R4-2510093	Discussion on RRM core requirements for LP-WUS/WUR	China Telecom

- R4-2510094	Discussion on RRM performance requirements for LP-WUS/WUR	China Telecom

- R4-2510134	Discussion on LP-WUS UE RF Rx requirements	CMCC

- R4-2510135	Discussion on LP-WUS BS RF requirements	CMCC

- R4-2510192	On remaining issues for simulation for LP-WUR	vivo

- R4-2510193	Further simulation results for LP-WUR	vivo

- R4-2510194	Further discussion on remaining RRM issues for LP-WUR	vivo

- R4-2510195	Consideration on performance part for LP-WUR	vivo

- R4-2510197	Draft CR for inactive state LP-WUR serving cell measurement and evaluation requirements	vivo

- R4-2510218	Discussion on FR2 UE RF requirements for LP-WUR	Samsung

- R4-2510219	TP to TR 38.774 on FR2 UE RF of LP-WUR	Samsung

- R4-2510220	Discussion on testability for LP-WUR	Samsung

- R4-2510221	TP to TR 38.774 on testability for LP-WUR	Samsung

- R4-2510292	TR 38.774 v0.5.0	vivo

- R4-2510293	Big CR for LP-WUS UE RF requirements in Rel-19	vivo

- R4-2510294	TP to TR 38.774 on LP-WUS UE RF General part	vivo

- R4-2510295	Discussions on LP-WUS REFSENS, ASCS, ACS and RMC	vivo

- R4-2510296	Draft CR to 38.101-1 on LP-WUS REFSENS ACS and ASCS	vivo

- R4-2510297	Proposals on concluding LP-WUS Testability	vivo

- R4-2510298	Discussions on LP-WUS UE RF feature list	vivo

- R4-2510299	System parameters for LP-WUR	Nokia

- R4-2510300	Fundamental RX requirements for LP-WUR	Nokia

- R4-2510301	Other RX requirements for LP-WUR	Nokia

- R4-2510302	Testability aspects of LP-WUR	Nokia

- R4-2510303	TP for TR 38.774 on LP-WUS	Nokia

- R4-2510304	Draft CR to TS 38.101-1 on REFSENS requirements for LP-WUR	Nokia

- R4-2510305	Draft CR to TS 38.101-1 on Blocking characteristics for LP-WUR	Nokia

- R4-2510399	Discussion on BS RF requirements for LP-WUS/WUR	ZTE Corporation, Sanechips

- R4-2510400	Discussion on REFSENS, ASCS, ACS for LP-WUR	ZTE Corporation, Sanechips

- R4-2510401	Discussion on receiver characteristics for LP-WUR	ZTE Corporation, Sanechips

- R4-2510402	Draft CR to Rel-19 TS38.101-1 on receiver characteristics for LP-WUR	ZTE Corporation, Sanechips, OPPO, CMCC

- R4-2510403	TP to TR 38.774 on other Rx requirements (Clause 7.1.9)	ZTE Corporation, Sanechips

- R4-2510404	Discussion on testability of LP-WUR	ZTE Corporation, Sanechips

- R4-2510414	draftCR to TS 38.104 on introduction of LP-WUS FR2-1 BS RF considerations	Qualcomm Germany

- R4-2510426	Discussion on UE demodulation performance requirements for LPWUS	Nokia

- R4-2510442	Discussion on UE demodulation requirements for NR LP-WUS and receiver	Samsung

- R4-2510656	Simulation assumption and additional simulation results for LP-WUR measurement	Huawei, HiSilicon

- R4-2510657	Discussion on RRM requirements for LP-WUR	Huawei, HiSilicon

- R4-2510658	draftCR on MR measurement requirements for serving cell in IDLE	Huawei, HiSilicon, vivo

- R4-2510659	Discussion on test case for LP-WUR	Huawei, HiSilicon

- R4-2510749	Discussion for LP-WUS/WUR REFSENS	LG Electronics France

- R4-2510839	UE demodulation requirements for LP-WUS/WUR	Ericsson

- R4-2510848	on LPWUS RX requirement	OPPO

- R4-2510893	On performance requirements for LP-WUS	Huawei,HiSilicon

- R4-2510909	Discussion on LP-WUS RRM requirement	Ericsson

- R4-2510910	Simulation on LP-WUS RRM measurement	Ericsson

- R4-2510911	draftCR on LP-WUS introduction	Ericsson

- R4-2510912	Discussion on performance requirements for LP-WUR	Ericsson

- R4-2510986	Discussion on UE demodulation performance requirements for LP-WUS ZTE Corporation, Sanechips

- R4-2510996	Further consideration on UE REFSENS, ACS requirements for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2510997	Further consideration on UE RF other Rx requirements for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2510998	Consideration on FR2 requirements for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2510999	Further consideration on UE RF testability issues for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2511000	Further consideration on BS RF for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2511001	draft CR to TS 38.101-1 on ACS for LP-WUR	Huawei, HiSilicon

- R4-2511002	draft CR to TS 38.101-1 on Blocking characteristics for LP-WUR	Huawei, HiSilicon

- R4-2511003	TP for 38.774: Blocking characteristics for LP-WUR	Huawei, HiSilicon

- R4-2511004	draft CR to TS 38.104 on clarification of transmitted signal quality for LP-WUS operation	Huawei, HiSilicon

- R4-2511005	draft CR to TS 38.141-1 on clarification of transmitted signal quality for LP-WUS operation	Huawei, HiSilicon

- R4-2511006	draft CR to TS 38.141-2 on clarification of transmitted signal quality for LP-WUS operation	Huawei, HiSilicon

- R4-2511026	Discussion on core parts on LP-WUS	ZTECorporation,Sanechips

- R4-2511027	Discussion on RRM performance requirements for LP-WUS	ZTECorporation,Sanechips

- R4-2511048	(NR_LPWUS-Core) LP-WUR and MR measurement capability in IDLE state	ZTECorporation,Sanechips

- R4-2511049	(NR_LPWUS-Core) LP-WUR and MR measurement capability in INACTIVE state	ZTECorporation,Sanechips

- R4-2511112	(NR_LPWUS-Core) LP-WUR and MR measurement capability in IDLE state	ZTECorporation,Sanechips

- R4-2511113	(NR_LPWUS-Core) LP-WUR and MR measurement capability in INACTIVE state	ZTECorporation,Sanechips

- R4-2511240	Discussion on LP-WUR Simulation assumptions and results	Nokia

- R4-2511241	LP-SS Simulation results	Nokia

- R4-2511242	Discussion on NR_LP-WUS RRM core requirements	Nokia

- R4-2511243	DraftCR to TS 38.133 on LP-WUR requirements	Nokia

- R4-2511244	Discussion on RRM performance requirements for NR_LP-WUS	Nokia

- R4-2511268	DraftCR to TS 38.101-1 on maximum input level for LP-WUR	CATT

- R4-2511330	System level considerations for UE RF LPWUS requirements	Qualcomm Incorporated

- R4-2511331	On FR1 LPWUS UE RF jammer requirements	Qualcomm Incorporated

- R4-2511332	On FR2 LPWUS UE RF requirements	Qualcomm Incorporated

- R4-2511333	TP to TR38.774  on ACS	Qualcomm Incorporated

- R4-2511334	TP to TR38.774  on FR2 refsens and spherical coverage	Qualcomm Incorporated

- R4-2511335	TP to TR38.774  on general aspects	Qualcomm Incorporated

- R4-2511336	TP to TR38.774  on FR2 IBB	Qualcomm Incorporated

- R4-2511337	TP to TR38.774  on max. power limit and ASCS	Qualcomm Incorporated

- R4-2511338	dCR to 38.101-1, LPWUS ACS	Qualcomm, Nokia

- R4-2511339	dCR to 38.101-1, LPWUS max. input level + ASCS	Qualcomm, Nokia

- R4-2511340	dCR to 38.101-1, LPWUS intermod	Qualcomm Incorporated

- R4-2511341	dCR to 38.101-1, LPSS and SSB side conditions	Qualcomm Incorporated

- R4-2511342	dCR to 38.101-1, LPWUS IBB, OBB	Qualcomm Incorporated

- R4-2511343	dCR to 38.101-2, LPWUS feature CR	Qualcomm Incorporated

- R4-2511425	CR to 38.104 for transmit signal quality of LP-WUS	Ericsson

- R4-2511426	TP to TR38.774: BS RF requirement overview for LP-WUS	Ericsson

- R4-2511427	On blocking performance for WUR	Ericsson

- R4-2511428	WUR RF requirement of IBB and OOB	Ericsson

- R4-2511429	TP for TR 38.774 WUR RF requirement of ACS	Ericsson

- R4-2511430	On WUR RF requirement testability	Ericsson

- R4-2511431	TP for TR 38.774 WUR RF requirement of Maximum input level	Ericsson

- R4-2511432	draftCR for 38.101-1 Maximum input power for WUR	Ericsson

- R4-2511454	Topic summary for [116][308] NR_LPWUS	Moderator (Huawei)

- R4-2511476	Topic summary for [116][330] NR_LPWUS_demod	Moderator (Qualcomm)

- R4-2511591	Discussions on demodulation performance requirements of LP-WUS_WUR for NR	QUALCOMM Europe Inc. - Spain

- R4-2511606	Simulation assumptions for R19 LP-WUS	MediaTek inc.

- R4-2511607	Discussion on the RRM core requirements for LP-WUS	MediaTek inc.

- R4-2511608	Discussion on the RRM performance requirements for LP-WUS	MediaTek inc.

- R4-2511632	RRM core requirements for LP-WUR	Qualcomm Incorporated

- R4-2511633	RRM performance requirements for LP-WUR	Qualcomm Incorporated

- R4-2511634	draftCR on LP-WUR requirements for FR2	Qualcomm Incorporated

- R4-2511852	Big CR for LP-WUS UE RF requirements in Rel-19	vivo

- R4-2511856	dCR to 38.101-1, LPWUS ACS	Qualcomm, Nokia

- R4-2511858	dCR to 38.101-2, LPWUS feature CR	Qualcomm Incorporated

- R4-2512138	Ad-hoc minutes for NR_LPWUS	Vivo

- R4-2512139	WF on RRM requirements for NR_LPWUS	Vivo

- R4-2512211	DraftCR on measurements of intra-frequency NR cells for UE with LP-WUR in IDLE and INACTIVE state	OPPO

- R4-2512234	Collection on the simulation results for LP-WUR	Apple

- R4-2512354	Big CR on RRM requirements for LP-WUS/WUR	vivo

- R4-2512526	Way Forward for [116][330] NR_LPWUS_demod	Qualcomm

- R4-2512596	BigCR to TS 38.141-1 on support for LP-WUS	CATT, ZTE

- R4-2512597	BigCR to TS 38.141-2 on support for LP-WUS	CATT, ZTE

- R4-2512598	CR to 38.104 for transmit signal quality of LP-WUS	Ericsson

- R4-2512599	TP to TR38.774: BS RF requirement overview for LP-WUS	Ericsson

- R4-2512600	Big CR to 38.104 on LPWUS	Huawei, HiSilicon, vivo, Nokia, ZTE, CATT, Qualcomm, Ericsson, CMCC

- R4-2512601	draftCR to TS 38.104 on introduction of LP-WUS FR2-1 BS RF considerations	Qualcomm Germany

- R4-2512621	Ad-hoc meeting minutes for [116][330] NR_LPWUS_demod	Qualcomm

- R4-2512667	TP to TR 38.774 on transmitted signal quality (clause 7.2.4) for LP-WUS	CATT, Ericsson

- R4-2512692	CR to 38.104 for transmit signal quality of LP-WUS	Ericsson

- R4-2511885   WF on LP-WUS UE RF	Vivo

- R4-2512699	Big CR to 38.104 on LPWUS	Huawei, HiSilicon, vivo, Nokia, ZTE, CATT, Qualcomm, Ericsson, CMCC

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template