3GPP TSG-RAN Meeting #109											RP-252056

Beijing, China, 15 – 18 September 2025

Agenda Item:	9.2.1

Source: 	NEC

Title:	Discussion and revision on the SID of NR ISAC

Document for:	Discussion and Decision

# Introduction

In RAN#108 meeting, the SID of Integrated Sensing And Communication (ISAC) for NR was approved in RP-251861[1]. The objectives of the SID are cited below:

This study item aims to study the following aspects for Integrated Sensing and Communication (ISAC): Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. - For other waveform and reference signals, companies are to share relevant information - No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. - FR1 frequency range is prioritized. Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

In this contribution, we discuss and show our views on the ambiguity issues of the SID.

# Discussion

  - The interpretation of "gNB bistatic sensing"

Regarding "gNB bistatic sensing" in the SID objectives, we think it is ambiguous whether it refers to "inter-gNB bistatic sensing" or "intra-gNB bistatic sensing".

People may interpret it as "inter-gNB bistatic sensing" where one gNB acts as a sensing transmitter and another gNB acts as a sensing receiver. (e.g., Fig. 1: Interpretation 1).

On the other hand, it may also be understood as "intra-gNB bistatic sensing" where one TRP of a gNB acts as a sensing transmitter and another TRP of the same gNB acts as a sensing receiver. (e.g., Fig. 2: Interpretation 2).

We also checked the moderator's summary[2] and observed different understandings of the wording "gNB bistatic sensing" based on some of the comments cited in the following table:

| Forward compatible design for gNB-gNB bi-static sensing mode is not precluded. | Interpretation 1 |
| --- | --- |
| As mentioned in our paper, from a RAN3 point of view bistatic sensing is not about specifying inter-gNB coordination but about ensuring that the CN-RAN logical protocol design allows addressing more than one gNB. So, I would propose removing “gNB-gNB”. | Interpretation 2 |
| One gNB can include multiple antenna/TRPs. The term of gNB-gNB bistatic mode is misleading and may give people impression that signal is transmitted from a group of antenna/TRPs within on gNB to a group of antenna/TRPs in another gNB. Suggest to use the term “gNB-based bistatic sensing” to align with the main bullet. | Interpretation 2 |

Therefore, we believe the current wording "gNB bistatic sensing" in the SID is not sufficiently clear regarding whether it refers to interpretation 1 (inter-gNB bistatic sensing) or interpretation 2 (intra-gNB bistatic sensing). A common understanding of this issue is deemed necessary for upcoming WG work.

"gNB bistatic sensing" in the SID is ambiguous regarding whether it refers to "inter-gNB bistatic sensing" or "intra-gNB bistatic sensing".

RANP is requested to provide guidance on the interpretation of "gNB bistatic sensing" in the SID.

From our perspective, we propose to clarify it as interpretation 2 (intra-gNB bistatic sensing) for the reason that interpretation 2 is more aligned with another objective stating “No inter-gNB coordination will be studied”. The possible revision to the SID could be as follow:

Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to intra-gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. Coordination with SA2 as necessary.

Adopt the above SID revision to clarify "gNB bistatic sensing" as "intra-gNB bistatic sensing"

  - Acronym correction

Currently, the acronym in SID of ISAC for NR in RP-251861[1] is recorded as “FS_Sensing_NR_Ph2”, which should be “FS_Sensing_NR_bis” according to the 3GPP WI details in WI #1080070.

Correct "FS_Sensing_NR_Ph2" to "FS_Sensing_NR_bis" for the acronym.

# Conclusion

In this contribution, we discussed issues related to the current SID, including the interpretation of "gNB bistatic sensing" and the correction of the relevant acronym.

Observation 1:	"gNB bistatic sensing" in the SID is ambiguous regarding whether it refers to "inter-gNB bistatic sensing" or "intra-gNB bistatic sensing".

Proposal 1:	RANP is requested to provide guidance on the interpretation of "gNB bistatic sensing" in the SID.

Proposal 2:	Adopt the above SID revision to clarify "gNB bistatic sensing" as "intra-gNB bistatic sensing"

Proposal 3:	Correct "FS_Sensing_NR_Ph2" to "FS_Sensing_NR_bis" for the acronym.

# Reference

- RP-251861, New SID: Study on Integrated Sensing And Communication (ISAC) for NR, AT&T (Moderator)

- RP-251798, Moderator's summary for Release 20 5G-Advanced Integrated Sensing and Communication, RAN VC Chair (AT&T)