3GPP TSG RAN Meeting #109                                 RP-252089

Beijing, China, Sep 15-18, 2025

Source: 	CMCC

Title:	Discussion on 6G RAN Architecture

Agenda item:	8.2.1.3

Document for:	Discussion

- Background

In TR 38.913[1], the requirements of architecture of 5G RAT are captured, including the functions to be supported by 5G RAN. Based on TR 38.913, we provide text proposals for the 3GPP TR 38.914 for the architecture of 6G RAN in this contribution, considering the potential new requirements/functions/scenarios in 6G.

- Discussion

Except for inheriting the general requirements of architecture of 5G RAT, more importantly, we need to identify the new requirements/functions/scenarios of 6G RAN. In our view, at least the following requirements should be addressed in 6GR.

- AI/ML function: It can be observed that AI/ML is a crucial component of 6G and RAN for AI is assumed as new service based on 6G SID [2]. The potential RAN for AI new services can include such as AI agent (including multi-agent collaboration), XR/Immersive communication and token communication. Furthermore, to satisfy the requirements on high computing demand, low-latency communication and high security, a centralized AI node within RAN is introduced to support 6G RAN for AI new service. For instance, the computing of AI applications/services can be offloaded to RAN to fulfill the real-time requirements of AI services. The centralized AI node, which serves multiple base stations, will support various AI functions, including AI computing resource management, online model training, model inference and service exposure. The details can be found in our contribution [3].

Figure.1 Overall architecture of centralized AI node in 6G RAN

Furthermore, the centralized AI node can also be used to better support AI for RAN use cases by the flexible and efficient AI computing resource management. In addition, the duplicated AI data collection and reporting will be avoided, since the centralized AI node can coordinate the duplicated requests for the same data, e.g. the L1 measurement results used for different use cases.

On the other hand, a RAN data plane should be considered to support the large volumes of AI data collection, especially for model training, and AI/ML model transfer/delivery.

- Sensing: Sensing is identified as an objective of 6GR. Considering the enormous deployment areas, 6G sensing should take data privacy, service latency and deployment flexibility into consideration. To achieve these requirements, Sensing Function Node (SF Node) deployed in RAN (illustrated in Figure.2) should be considered in 6G network architecture. To further improve sensing performance, the impact on RAN architecture of multiple Base Stations cooperation (including integrated AI and sensing) should be considered. Detailed thoughts are provided in [4].

Figure.2 Proposed Radio Access Network architecture for sensing in 6G RAN

- Cooperative Communication: To improve the spectral efficiency and achieve consistent user experience, 6G should support multi-node access, cooperative transmission, and flexible handover. To fulfill these requirements, the 6G RAN architecture should incorporate the following aspects:

- High-PHY/Low-PHY functional split, with High-PHY deployed at the DU to perform centralized decision-making, while Low-PHY carrying out distributed execution;

- CU/DU splitting, which should be retained to ensure scalability and flexibility and enable coordination for performance features, load management, and real-time performance optimization in cooperative communication;

- Inter-DU interface, which should be open to support inter-site coordination across DUs and enable real-time information exchange at L1/L2;

- Fusion processing unit, which should be introduced at layers such as High-PHY and MAC to support multi-point data fusion processing.

Figure.3 RAN architecture for cooperative communication in 6G

- Service-based interface (SBI): Replacing the pre-established peer-to-peer (p2p) signalling interface between the RAN and the CN with an SBI will help to improve interaction efficiency. On one hand, the RAN can more directly expose information to CN NFs or third-party applications. On the other hand, new capabilities of the RAN such as computing, AI, and sensing can adopt a unified interaction model with corresponding capabilities in the CN, enabling efficient coordination of these new features. For the RAN service definition, (some) NGAP procedures can be defined as services first before gradually considering the servitization of other interfaces such as XnAP, E1AP and F1AP, details can be found in [5].

Figure.4 Service-based interface

- Network Digital twin (NDT): NDT enables proactive network state prediction and policy pre-verification, thereby helping to achieve a higher level of network autonomy. It needs to be introduced into 6G RAN due to the following reasons:

  - NDT has been introduced in SA5 to support autonomous network management and operation. However, the NDT deployed in MnS(Management Service) cannot meet the near real-time predicting and pre-validation requirements of RAN.

  - Self-organizing network (SON) cannot pre-validate configurations, and therefore cannot guarantee that configuration, optimization, or fault-related parameters will not have negative impacts.

Therefore, NDT in 6G RAN needs to be supported from 6G Day 1, as illustrated in Figure.5. Based on dynamic network mapping and modeling, NDT enables near real-time network prediction and policy pre-validation (e.g., AI-driven policies) in RAN, thereby reducing fault risks, and lowering costs. Its performance elevation has been proven in several RAN senarios such as proactive link adaptation and AI-based resource allocation senarios. To fully unleash the capabilities of NDT, 6G RAN should standardize bi-directional interfaces between NDT and RAN network elements to support near real-time or real-time data collection and controls. It should also standardize the functions and invocation interfaces of various twin models from multi-vendor devices. Its specific analysis provided in our contribution [6].

Figure.5 Proposed Radio Access Network architecture for NDT in 6G RAN

- UE aggregation: In the future, there will be more and more applications with high UL bitrates. Considering the limitation of UL UE transmission power, UE aggregation is a flexible way to achieve the required UL bitrates. What’s more, UE aggregation can help to improve the stability and reduce delay of uplink services as well, that is, if the channel condition of a terminal is deteriorating, another terminal can be used to make up for the traffic performance unsteadiness caused by channel condition variation. Therefore, UE aggregation can be considered from 6G Day1.

- Text Proposal for TR 38.914

Based on the discussion in Section 2, we propose the following text proposal for TR 38.914 for the general requirements for architecture of 6G RAN.

# 5.2	Requirements for architecture and migration

### 5.2.X	Requirements for architecture of 6G RAN

The RAN architecture design for the 6G shall be designed to fulfil the following requirements:

-	The RAN architecture shall allow for C-plane/U-plane separation.

-	The RAN architecture shall allow for the RAN and the CN to evolve independently.

-	RAN-CN interfaces and RAN internal interfaces (both between 6G RAT logical nodes/functions and between 6G RAT and5G logical nodes/functions) shall be open for multi-vendor interoperability.

-	The design of the RAN architecture shall enable lower CAPEX/OPEX with respect to current networks to achieve the same level of services.

-	The RAN architecture shall allow for deployment flexibility e.g. to host relevant RAN, CN and application functions close together at the edges of the network, when needed, e.g. to enable low latency services, etc.

-	The RAN architecture shall allow AI function(s), and a unified RAN architecture for AI service both within the network (i.e., AI for RAN) and outside the network (i.e., RAN for AI), e.g., AI service for UE.

-	The RAN architecture shall allow for RAN data plane.

-	The RAN architecture shall allow integrated sensing and communication function(s).

-	The design of RAN-CN interfaces shall allow service-based interface.

-	The RAN architecture shall support UE aggregation.

-	The RAN architecture shall allow for network digital twin (NDT).

-	The RAN architecture shall support sharing of the RAN between multiple operators.

The 6G RAN architecture is illustrated in Figure 5.2-X below.

Figure 5.2-X: Overall Architecture

- Conclusion

In this contribution, we propose to add the text proposal in Section 3 to TR 38.914.

- References

- 3GPP TR 38.913 V18.0.0 (2024-03) Study on Scenarios and Requirements for Next Generation Access Technologies; (Release 18);

- RP-251881 New SID: Study on 6G Radio;

- RP-252092 Views on architecture of 6G RAN for AI, 3GPP TSG RAN Meeting #109, CMCC；

- RP-252091 Views on architecture of 6G RAN for Sensing, 3GPP TSG RAN Meeting #109, CMCC；

- RP-252093 Consideration on 6G service-based RAN, 3GPP TSG RAN Meeting #109, CMCC；

- RP-252102 Requirements for Network Digital Twin in 6G RAN, 3GPP TSG RAN Meeting #109, CMCC.