3GPP TSG RAN Meeting #109                          RP-252102

Beijing, China, Sep 15-18, 2025

Agenda Item:	8.2.1.4

Source:	CMCC

Title:	Requirements for Network Digital Twin in 6G RAN

Document for:	 Discussion

1	Introduction

In Release 19, 3GPP SA5 has been studying the management aspects of Network Digital Twins (NDT) [1]. NDT is the digital representation of an object that captures the characteristics and behaviors of the corresponding physical network, and supports network management and operations by providing a virtual representation of physical network processes. NDT enables proactive prediction of network states and policy pre-validation, which helps reduce the risk of failures caused by inappropriate network policies (e.g., AI-driven policies), thereby achieving higher levels of network autonomy. To further enhance 6G RAN real-time policy pre-validation, it is proposed that the NDT Function be introduced into the 6G RAN. Such an introduction is expected to reduce NDT processing latency and data overhead, thereby improving the efficiency and responsiveness of RAN operations.

- Discussion

  - Motivation for introducing NDT to RAN

2.1.1 Limitations of SON

A self-organizing network (SON) is an automated technology which is designed to enable network self-configuration and self-optimization [1]. However, it has some limitations:

- Due to lack of accurate modeling of the wireless environment, network equipments and terminals, SON cannot pre-validate configurations, and therefore cannot guarantee that configuration, optimization, or fault-related parameters will not have negative impacts [1][3].

- It can only predict networks state variations at coarse granularity，based on large statistic data collected over a long period, without being able to predict fine-granularity changes over a short time scale.

- It cannot predict sudden variations (e.g., channel variation caused by shadow fading in a factory), because it normally relies on prediction algorithms assuming continuous variation of predicted values over time. In other words, lacking of modeling of the physical environment makes it cannot comprehensively consider a series of influencing factors in the predictions [3].

Observation 1: SON cannot accurately pre-validate configuration before deployment to ensure performance. In addition, its prediction process cannot accurately forecast changes in a small time-scale. Moreover, it cannot predict sudden variations, due to lack of modeling the physical network.

2.1.2 Limitation of NDT Disccused in SA5

An NDT is a virtual replica of mobile network or part of one, that captures its attributes, behaviour and interactions. By modeling the physical network and environment, NDT can:

- Accurately pre-validate the operational performance of different network policies in the physical network. Therefore, NDT can be used to evaluate the configurations of the automation functionality (e.g., SON) [1].

- Accurately predict future network states, including sudden variations. Hence, NDT can be used to predict potential network faults and enable proactive intervention, as well as to forecast abrupt changes during normal network operation (e.g., channel variation caused by shadow fading in a factory [5]).

- Empower network AI strategies, such as data generation and policy pre-validation [1].

Observation 2: By modeling the attributes, behavior, and interactions of the physical network, NDT possesses key capabilities including pre-validation of network policies, prediction of sudden variations in network states, and empowerment of AI strategies.

NDT has been introduced into SA5, where its objective is to enable evaluation of high-risk network operations, verification of network configurations, and generation of ML training data in non-real-time scenarios [1]. Its limitation is below：

- NDT in SA5 relies on long-term collected data for statistical and offline predictions, optimization and pre-validations [1], which cannot meet the requirement for personalized and short time-scale optimizations RAN..

Observation 3: NDT in SA5 relies on long-term collected data for statistical and offline optimization, which cannot meet the requirements of many personalized and near real-time scenarios in RAN.

2.1.3 Introducing NDT in RAN

Introducing NDT in RAN can enable near real-time network state prediction and policy pre-validation, through accurate modeling of the wireless environment, physical network, and terminals, based on near real-time physical-digital interaction. Its key values and corresponding industry use cases are listed below:

- NDT in RAN can proactively predict network states, wireless channels, and user behaviors in near real-time, thereby providing reliable references for dynamic network policies and enhancing network performance.

- Use case (link adaptation and interference coordination): in factory scenarios with complex shadow fading, NDT can accurately predict large-scale fading variations by constructing a twin wireless environment. Based on this prediction, precise link adaptation and cell-level interference coordination can be performed, leading to an improvement in spectral efficiency (from 0.3 bits/s/Hz to 4.2 bits/s/Hz), a reduction in latency (the tail 1% latency reduced from 20.66 ms to 0.125 ms), and an enhancement in reliability (from 99.89% to 99.99%), as depicted in Figure 2.1.1 [5].

Figure 2.1.1. Performance comparison for conventional and digital-twin-based link adaptation algorithms

- NDT in RAN can comprehensively take into account complex factors such as dynamic network interference, 3D physical environment, user tragectory, etc., and provide near real-time pre-validation for dynamic policies to ensure their performance.

- Use case (UAV interference coordination): To mitigate UAV interference on ground users served by neighboring base stations [4], NDT is utilized to predict interference situations in near real-time and to pre-validate interference coordination strategies. Concretely, the NDT includes a wireless environment twin model of the UAV flight area (e.g., based on lightweight ray-tracing algorithms), a site twin model (including coordinates, antenna configurations, etc.), a UAV twin model (encompassing service models and trajectory models), and a ground service traffic twin model. Based on these, the NDT predicts the interference levels caused by each UAV to different neighboring sites over a future time period, generating inter-cell interference coordination policy based on time-frequency resource allocation. Additionally, using the wireless environment twin model and the UAV twin model, the NDT calculates performance metrics such as Signal-to-Interference-plus-Noise Ratio (SINR) and throughput under the interference coordination policy, enabling pre-validation and adjustment of the policy.

- For network AI strategies, in addition to generating sufficient datasets during the AI training phase, NDT in RAN can also, during the AI inferring phase, proactively predict potential performance degradation trends of AI models based on forecasts of the dynamically changing network environment, and iteratively update them in advance to continuously ensure their quality.

Use case (AI-based resource allocation)：In the case of dynamic variations of XR traffic and wireless environment, NDT can be used to pre-validate the performance of AI-based network resource allocation policies and to update their training. Specifically, first, the NDT collects historical data from a physical environment, and models the wireless environment, service traffic, and network performance. Based on the modeling, NDT generates extensive training data. The data supports offline training of network slicing resource allocation policy which is designed based on Deep Double Q-Network (DDQN). After the policy has been deployed, the NDT continuously updates the policy model. Concretely, the NDT uses real-time data to predict future traffic and wireless environment changes. It pre-validates the policy future performance based on these predictions. If performance degradation is detected, NDT will generate large-scale AI datasets that conform to the distribution of future network states. The dataset is used to proactively updates the model to maintain performance. Figure 2.1.2 shows that the NDT-assisted policy reduces network latency from 13.8 ms to 12.6 ms.

Figure 2.1.2 NDT-enabled intelligent resource allocation

Observation 4: Recent industry explorations have shown that introducing NDT into RAN can address issues such as policy latency, unsatisfactory performance, and slow policy updates, thereby effectively improving network performance.

Proposal 1: NDT should be introduced into RAN to provide accurate near real-time prediction, pre-validation of network policies, and proactive policy iteration and updates, so as to effectively enhance network performance.

  - Consideration on NDT Function in RAN

NDT in RAN requires Low-latency digital-physical mapping. RAN shall support real-time or near-real-time digital-physical mapping between the physical network and its NDT, enabling timely simulation, monitoring, and decision-making.

To achieve this, first, it is necessary to design bidirectional standardized interfaces between NDT and RAN network elements for:

a) Data collection and status reporting: RAN-NDT bidirectional interface needs to efficiently collect and transmit performance metrics, measurements, and state information from physical network to NDT, with miminized data overhead while supporting timely updates.

Proposal 2: Standardized interfaces between NDT and RAN network elements shall be defined to support efficient data collection, enabling real-time synchronization from the physical network to NDT.

b) Configuration and control delivery: The interface needs to enable real-time or near-real-time configuration and control delivery to achieve dynamic network management, policy pre-validation, and AI-driven operations.

Proposal 3: Standardized interfaces between NDT and RAN network elements shall support real-time or near-real-time policy delivery to enable timely policy control from NDT to the physical network.

Second, NDT shall accurately model RAN components to ensure reliable pre-validation and AI-assisted decision support. To this end, vendors are required to provide various types of twin models for network elements and terminal devices (including functional models and performance models), together with standardized invocation interfaces to support efficient and flexible NDT model construction.

Proposal 4: 6G RAN should standardize the functions and invocation interfaces of various twin models.

6G RAN shall deploy a shared NDT across multiple geographically proximate RAN nodes (as depicted in Figure 2.2.1), to allow cross-vendor (e.g., different terminals and network elements) optimization.

Proposal 5: 6G RAN shall deploy a shared NDT across multiple geographically proximate RAN nodes, to allow cross-vendor optimization.

Figure 2.2.1 Proposed RAN architecture for NDT in 6G RAN

3	Conclusion

In summary, in this contribution we expressed our views on the deployment scenarios and requirements on NDT for 6G RAN, and have following proposals:

Observation 1: SON cannot accurately pre-validate configuration before deployment to ensure performance. In addition, its prediction process cannot accurately forecast changes in a  small time-scale. Moreover, it cannot predict sudden variations, due to lack of modeling the physical network.

Observation 2: By modeling the attributes, behavior, and interactions of the physical network, NDT possesses key capabilities including pre-validation of network policies, prediction of sudden variations in network states, and empowerment of AI strategies.

Observation 3: NDT in SA5 relies on long-term collected data for statistical and offline optimization, which cannot meet the requirements of many personalized and near real-time scenarios in RAN.

Observation 4: Recent industry explorations have shown that introducing NDT into RAN can address issues such as policy latency, unsatisfactory performance, and slow policy updates, thereby effectively improving network performance.

Proposal 1: NDT should be introduced into RAN to provide accurate near real-time prediction, pre-validation of network policies, and proactive policy iteration and updates, so as to effectively enhance network performance.

Proposal 2: Standardized interfaces between NDT and RAN network elements shall be defined to support efficient data collection, enabling real-time synchronization from the physical network to NDT.

Proposal 3: Standardized interfaces between NDT and RAN network elements shall support real-time or near-real-time policy delivery to enable timely policy control from NDT to the physical network.

Proposal 4: 6G RAN should standardize the functions and invocation interfaces of various twin models.

Proposal 5: 6G RAN shall deploy a shared NDT across multiple geographically proximate RAN nodes, to allow cross-vendor optimization.

References

- TS 28.561, Management aspects of Network Digital Twins;

- 3GPP, Self-Organising Networks(SON), https://www.3gpp.org/technologies/son#:~:text=By%20Juha%20Korhonen%2C%203GPP%20MCC%20A%20self-organizing%20network,mobile%20networks.%20SON%20enables%20network%20self-configuration%20and%20self-optimization;

- TR 28.861, Study on the Self-Organizing Networks (SON) for 5G networks;

- TR 36.777, Study on Enhanced LTE Support for Aerial Vehicles;

- Ericsson, Network digital twins–outlook and opportunities, https://www.ericsson.com/en/reports-and-papers/ericsson-technology-review/articles/network-digital-twins-outlook-and-opportunities.