3GPP TSG RAN Meeting #109												RP-252132

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.4

Source:	Huawei, HiSilicon, CAICT, CMCC, China Unicom

Title:	Consideration on supporting new AI services for 6G

Document for:	Discussion and decision

Introduction

A RAN-level study on “6G Scenarios and requirements” was approved in RAN#107[1]. Regarding the services in the scenario, the objective of this SID includes:

This study item aims to

- investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs.

  - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN

- Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors.

- Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services.

- Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate.

- …..

In RAN#108, one of the objectives of the approved SID about “study on 6G Radio” is to support various services and functionalities, where AI/ML related services serve as examples[2]:

The detailed objectives of the study are:

…

(3)Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

…

e)	Data transfer design to support various types of data (e.g., AI/ML, Sensing, etc.) [RAN2, RAN3]

…

(6) Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g., AI/ML, sensing, etc.). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

…

(8)	AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc.…

Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc.…)

NOTE: lead WG depends on the use case.

b)	AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

i.	LCM procedures [RAN2, RAN1, RAN3, RAN4]

ii.	Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Note: NW for AI is assumed to be covered by new services

6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

3GPP SA1 has begun the study on AI/ML related use cases for 6G [3],  which cover aspects related to NET4AI. The requirements of these use cases are identified for the whole 6G system and RAN shall also support these service requirements. This paper is to analyze RAN requirements to support AI services.

Consideration on 6G Radio for new AI services

Token communication

The AI services powered by large AI/ML models such as large language models (LLM), vision language models (VLM), multi-modal language models (MLM), visual language action models (VLA), etc., are utilized widely for main-stream AI services, e.g., AI agent, chatbot, personal assistant, autonomous driving, AI phone/wearables, and embodied AI robots.

In these AI services, instead of transmitting raw input data (e.g., text, images), transforming them into tokens becomes popular for efficient transmission and processing as shown in [4]. In [5], it shows that by 2027, the daily token consumption of Doubao (A famous mobile AI application in China) is projected to exceed 100 trillion tokens per day. This may bring new business opportunities for operators by providing services adapting to such an application, considering such a rapid development and huge consumption of tokens.

Token is seen as a compact data carrying the context information, and can be generated and processed by AI models as shown in Figure 2.1-1. A tokenizer converts the raw data of multiple modalities (e.g., text, image, audio, etc.) to tokens at the transmitter side, and a corresponding de-tokenizer converts the tokens back to the raw data. The tokens generated by/for AI/ML models have lots of special characteristics, which are different from traditional traffics.

Each token plays a central role in enabling AI agents (as described in SA1 use cases in TR 22.870) to understand and respond to natural language instructions, interpret environmental data, make decisions for robotic systems, communicate with other AI agents, etc.

Figure 2.1-1 Illustration of 6G token communication service for Agentic AI

Token communication refers to the transmission of tokens over the air interface. One important characteristic of token transmission is error tolerance capability. The below Figure 2.1-2 shows the difference between the cases with and without token prediction for image reconstruction, considering different token error rate. Without token prediction, when the error rate is larger than 5%, the image reconstruction quality is degraded significantly; and with token prediction via generative or predictive capabilities from AI models, even when the error rate is up to 50%, the image reconstruction quality seems still acceptable.

Figure 2.1-2 Image generation with/without token prediction (image from ImageNet[6])

Such resilience of error tolerance can bring new opportunities for operators to utilize spectrum resources in an efficient way. To be more specific, the existing design of transmission over radio is on a per bit basis to ensure every bit is correct, consequently reliability design is introduced from physical layer to higher layers, including PHY retransmission, L2 HARQ/ARQ etc. If the transmission is considered to be per token basis and only part of these functions is selected according to the error tolerance capability, the resources scheduled for token communication can be significantly saved, which will be beneficial for operators to support higher number of users, or provide more resources for edge users to improve coverage and cell edge performance.

Tokens may also have different priorities or importance [7][8]. Firstly, tokens serve for different modality may take different importance to the error tolerance capability. For example, as shown in Figure 2.1-3(left), the image tokens can be predicted more easily and more tolerant to error rate, while text tokens are more sensitive to error rate and cause accuracy degradation significantly once error rate is larger than 1%. Secondly, the tokens of the same modality data may also have different importance weights. For example, as shown in Figure 2.1-3(right), the importance of tokens may have a decreasing order with their token positions for an image, where the loss of the preceding tokens would cause higher reconstruction error than the subsequent tokens. With the preceding tokens being transmitted correctly, the importance of the subsequent tokens will decrease. Hence, based on the token importance, different transmission priority of tokens can be utilized to improve the transmission efficiency.

Figure 2.1-3 Impact of different tokens on the performance.

(Left: tokens of different modality data for text Visual Question Answering task [9]. Right: different tokens of image data for reconstruction.)

According to the above analysis, it is seen useful to study mechanism considering token characteristics instead of original per bit basis, to better serve token communication and enhance resource utilization efficiency for operators.

To support the above mechanism, RAN needs to be aware about the token pattern, e.g., allowed error rate for a (group) of tokens, importance of the token etc. This may bring new QoS type and parameters for token communication which needs E2E design.  One example is shown in Figure 2.1-4, which include specific QoS types (related to error tolerance and traffic patterns), enabling partial reliability to token traffic (related to error tolerance).

Figure 2.1-4 Example of transmission mechanism to facilitate token communication

Proposal 1: 6G RAN shall support efficient transmission for traffics relevant for AI applications, by taking into account of specific characteristics of AI traffics, e.g., token communication, error tolerance, etc.

AI/ML-enabled RAN Digital Twin

As a real-time virtual replica of a physical RAN, the RAN digital twin can provide comprehensive information of physical RAN, including environmental surroundings (e.g., buildings and vehicles), radio signal propagation information in the environment (e.g., channel multi-path information), and RAN operational data. This information provided by RAN digital twin can not only serve as prior information to improve RAN communication performance, but also have the potential to better serve other services.

Several digital twin related use cases are introduced in TR 22.870 [3] for 6G, one example described in TR Section 7.13 is related to a new service about collaborative robots. In this case the collaborative robots will utilize the Digital Twin information to fully control of the action in the physical world without human interaction, and such information will be got via combination of sensing measurements and AI capabilities. There are also other cases for data collection and environment object reconstruction described in [3] in Section 5.9.3, 5.9.4, 7.5 etc. to facilitate utilization of Digital Twin information for different services. According to the above use cases, RAN Digital Twin is regarded important to serve more applications, e.g., collaborative robots, 3rd party AI agents etc. Therefore, Digital Twin use case shall be supported to support emerging applications, e.g., collaborative robots, 3rd party AI agents etc.

To construct the RAN digital twin, both UEs and base stations (BSs) need to be involved to get sufficient information. As Digital Twin needs to reflect the whole picture of the reality and requires timely update to track the changing of environment, a distributed and continuous way of collaboration among UEs and base stations needs to be considered, considering the balance of digital twin performance requirement and complexity/overhead from the system.

One example is to aggregate the local and distributed environment information reported from UEs to build global environmental information at BS. As shown in Figure 2.2-1, each single UE can collect local sensing data which reflects the local environmental information around the UE, conduct local inference through AI/ML models for environmental information prediction as required in [10]. The output can be compressed and sent to the BS, where multiple UEs’ inputs are collected to generate a more complete knowledge of the environment information within a larger area. Based on such environmental information, the AI/ML Model from applications can deduce the inference result and provide more accurate inference, e.g., to give specific instruction command to collaborative robots. Additionally operators can also utilize such information to do continuous learning for system performance improvement.

Figure 2.2-1	AI/ML-based environment construction

For the above reporting, different types of data need to be collected to construct the digital twin, e.g., channel measurement information, sensing information etc. This leads to the question on how to design the data collection mechanism for different types of data. In 5G, there already exists quite a few mechanisms on different data collection which are summarized as below:

- RAN channel measurement data, originating from either the base station or the user equipment (UE), based on the measurement of reference signals, including measurements based on uplink SRS (Sounding Reference Signal), downlink CSI-RS (Channel State Information Reference Signal), DMRS (Demodulation Reference Signal), and SSB (Synchronization Signal Block). Such measurement data is reported by UE and managed by BS.

- RAN RRM measurement data for mobility, including L3 measurement for SSB, CSI-RS for mobility management. Such measurement data is reported by UE and managed by BS.

- Data used by SON/MDT (Self-Organizing Networks/Minimization of Drive Test), which supports RAN automation functionalities. Such measurement data is reported by UE and managed at RAN or OAM side.

- QoE (Quality of Experience) measurement collection enables collection of application layer measurements from the UE. The measurements are collected for different services, e.g., DASH (Dynamic Adaptive Streaming over HTTP), MTSI (Multimedia Telephony Service over IMS), and VR (Virtual Reality) measurements. Such measurement data is reported by UE and managed by the OAM side.

- Positioning data. Measurement results for UE positioning are collected from UE/RAN to core network. Such measurement data is reported by the UE and managed by the core network side.

- …..

It can be seen in existing mechanism, different types of data are collected and utilized independently using different mechanism. However, in reality, the data consumer may require multiple types of data. For instance, in XR services in [11] and environment object reconstruction services in [10], the service may require the collection of both sensing data and channel measurement data, and also utilize the reported data for AI training or inference. As the current data is reported and managed by independent mechanisms, these data cannot be well integrated. There might be overlapping, duplicating, mixed classification of data, which are not efficient to be utilized by the network, and also difficult to be provided as a service friendly to the data consumers.

Looking ahead to future 6G systems, there is a clear opportunity to establish an end-to-end, unified data management framework, to enable joint collection, management and distribution of data within 3GPP system, as below example shown in figure 2.2-2. This requirement has also been captured in SA1 TR22.870 [3] Section 5.9.3.2 on “Efficient data collection and consumption for 6G system”.

This unified approach could support two major targets:

- Enable data-driven service: E2E transmission scheme and utilization of diverse data types to support new service and applications.

- RAN specific enhancements: Leveraging the integrated data to improve network operation, such as continuous learning and improvement of RAN performance.

Figure 2.2-2	Example of end-to-end and unified data management

Proposal 2: 6G RAN shall enable an E2E data collection and management framework to enable the data-driven services for operators as described in TR 22.870.

Proposal 3: 6G RAN shall enable efficient utilization of local measurements for the support of new services and applications, e.g., by distributed and continuous AI/ML learning.

Summary

Based on the above example and analysis, we propose

Proposal 1: 6G RAN shall support efficient transmission for traffics relevant for AI applications, by taking into account of specific characteristics of AI traffics, e.g., token communication, error tolerance, etc.

Proposal 2: 6G RAN shall enable an E2E data collection and management framework for operators to enable the data-driven services as described in TR 22.870.

Proposal 3: 6G RAN shall enable efficient utilization of local measurements for the support of new services and applications, e.g., by distributed and continuous AI/ML learning.

Proposal 4: Adopt the text proposal for AI/ML services requirements in TR38.914 as shown in Section 5.

References

- RP-250810, “New SID: Study on 6G Scenarios and requirements”, RAN#107, March 2025.

- RP-251881, “New SID: Study on 6G Radio”, RAN#108, June 2025.

- 3GPP TR 22.870: “Study on 6G Use Cases and Service Requirements”.

- Bondcap, “Trends - Artificial Intelligence”, May 2025

- https://xueqiu.com/9332969756/321418828

- J. Deng, et al, ImageNet: A Large-Scale Hierarchical Image Database, IEEE Conference on Computer Vision and Pattern Recognition (CVPR), 2009.

- Liu Z, Ma Y, Tafazolli R. ResiTok: A Resilient Tokenization-Enabled Framework for Ultra-Low-Rate and Robust Image Transmission[J]. arXiv preprint arXiv:2505.01870, 2025.

- K. Miwa, et al. One-D-Piece: Image Tokenizer Meets Quality-Controllable Compression, ICML Tokenization Workshop, 2025.

- A. Singh, et al, Towards VQA Models That Can Read, Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition (CVPR), 2019.

- S1-253507, “Update of clause 7.5 use case on environment object reconstruction”, SA1#111, August 2025

- S1-252323, “Use case on data service for efficient collection and distribution of diversified data”, SA1#110, May 2025

TP for 38.914

In section 5.4 Requirements of New and Existing Services in TR38.914, the text proposal is provided as follow:

-------------------------------------------------Start the text proposal ------------------------------------------------

- 5.4.x	AI/ML Services

6G RAN shall support efficient transmission for traffics relevant for AI applications, by taking into account of specific characteristics of AI traffics, e.g., token communication, error tolerance, etc.

6G RAN shall enable an E2E data collection and management framework to for operators to enable the data-driven services as described in TR 22.870.

6G RAN shall enable efficient utilization of local measurements for the support of new services and applications, e.g., by distributed and continuous AI/ML learning.

-------------------------------------------------End the text proposal ------------------------------------------------