3GPP TSG RAN Meeting #109												RP-252133

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.4

Source:	Huawei, HiSilicon, CAICT, China Telcom, China Unicom, KT Corp., LG Uplus，SK Telecom

Title:	Consideration on supporting ISAC services for 6G

Document for:	Discussion and decision

Introduction

A RAN-level study on “6G Scenarios and requirements” was approved in RAN#107[1]. Regarding the services in the scenario, the objective of this SID includes:

This study item aims to

- Investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs.

  - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN

- Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors.

- Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services.

- Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate.

- …..

Also sensing studies are mentioned in in RAN#108 [2]. The detailed objectives of the study are demonstrated as following:

- PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4]

- Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1]

- Aspects of integration with communication services [RAN1]

- higher layer procedures and protocol aspects [RAN2]

- RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4]

The new radio emphasizes requirements for the Integrated Sensing and Communication (ISAC) performance. The new 6G RAT will support vast range multi-dimensional sensing scenarios and use cases [TR 22.870], whose performance requirements also vary considerably. ISAC studies would be based on these use cases and associated requirements.

In this contribution, the considerations and views on 6G ISAC services are discussed.

Consideration on 6G Radio for ISAC services

In recommendation ITU-R M.2160-0, integrated sensing and communication (ISAC) is defined as one of the key usage scenarios of IMT-2030. Its typical use cases include navigation, activity detection and movement tracking, environmental monitoring, and provision of sensing data/information of the surroundings for AI, XR, and digital twin applications [3]. The study [4] has identified multiple sensing scenarios, including UAV supervision, motion monitoring (e.g., respiration rate estimation), that demonstrate significant potential for enhancing network performance and enabling new service paradigms which can be further studied in 6G.

In 6G, the following services should also be considered for study and meeting their requirements, considering the potential for huge markets.

Target detection and tracking

Target detection and tracking, such as UAV to enable the low-altitude economy, has already been under discussion in 5G-A, which requires around 10m horizontal accuracy with 95% confidence level of outdoor detection. In 6G more use cases are expected and have already been identified in SA1 6G study, such as road digitization as described in Sec 7.6 in TR 22.870.

The use case can be utilized for applications like smart transportation, automated driving, intelligent navigation, which can bring new business opportunities for operators. According to [5], the required horizontal localization accuracy is specified as 0.5 and 1 meter for target which is static or moving up to 5km/h, with density less than 750 vehicles per 1000 m x 24 m and less than 100 pedestrians per 1000 m x 24 m. For higher speed target up to 120km/h, the required horizontal localization accuracy is 3m. The horizontal velocity accuracy is 0.5 m/s and 3 m/s for low and high-speed respectively for dual three-lane carriageway. Considering other more complicated lane setting at outdoor Marco scenario, the relevant metrics may need further relaxation and could potentially be set to around 5m/s.

Figure 1 Illustration of Road digitalization

Proposal 1: 6G RAN shall support use case of target detection and tracking, e.g. road digitization, with the below performance target:

Static target or moving target up to 5km/h:

- Localization accuracy: horizontal 1m

- Velocity accuracy: 1m/s

Moving target up to 120km/h:

- Localization accuracy: horizontal 3m

- Velocity accuracy: 5m/s

Digital twin services enabled by Environment Object Reconstruction

According to Recommendation ITU-R M.2160, IMT-2030 is expected to be used to replicate the physical world into a digital virtual world as precise real-time representations or Digital Twin (DT).

In the current TR 22.870, several use cases have mentioned DT. As the section 7.5 of use case on environment object reconstruction, for static environmental objects, such as outdoor building/city construction and indoor machinery/wall, 3GPP wireless sensing is effective to provide a wealth of low-cost environmental information for both wide areas and regional areas. Additionally, for monitoring dynamic targets of interest, such as vehicles, it requires environmental perception enhancement, especially in scenarios where non-line-of-sight (NLOS) conditions are met, or poor visibility limited by other conventional sensing methods.  This also facilitates smart transportation, smart city and smart home as depicted in Figure 2.

Moreover, 6G system could obtain the sensing measurement data by RAN sensing, and then utilize AI capability to generate local digital twin including spatial information. The ISAC use case of collaborative robots using digital twinning as described in section 7.13 TR 22.870 also elaborated the new 6G service, where the DT application takes full control of the actions in the physical world without human interactions. Eventually, the global DT can be constructed by distributed local DT, and then delivered to a 3rd party as a real-time network service.

On the other hand, digital twins can also enhance the communication performance. Once the digital twin of a physical environment is available, the wireless propagation characteristics in the physical environment can be easily predicted. This can be used to assist to improve performance e.g. energy saving, handover successful rate, MIMO operation with reduced overhead. In addition, by utilizing this environmental object information, non-line-of-sight (NLOS) positioning issues can be resolved by using the EO as an anchor in Figure 3. This can solve the pain points of supporting positioning services in 5G and facilitate the commercialization.

Figure 2 Physical world and digital twin

Figure 3 Enhanced NLOS positioning

To construct digital twin and support the other services based on digital twin, 6G needs the sensing capability of [sub meter to meter-level] reconstruction accuracy as illustrated in [6]. Besides, to construct a complete picture of digital twin information, both static and dynamic targets need to be measured as the essential components.

Proposal 2: 6G RAN shall be able to provide sensing service considering characteristics of individual environmental object [6] as described:

- Measure and reconstruct a relatively large Environment Object which can be static and moving

- [sub meter to meter-level] reconstruction accuracy

Proposal 3: 6G RAN shall study mechanism of utilizing digital twin information for performance improvement, e.g. for optimizing communication and positioning services.

General Consideration on RAN design principles for ISAC

To achieve the above high-performance target for ISAC, it is important to consider the characteristics of wireless communication. In scenarios of the above use cases, non-line-of-sight (NLOS) phenomena occur frequently. As illustrated in Figure 1 and Figure 3, even when line-of-sight (LOS) paths are obstructed, the detection and tracking of NLOS targets can still be achieved.

Furthermore, bi-static sensing between BSs and UE can enlarge the sensing range significantly since UEs’ locations are flexibly distributed. Taking digital twin as one example, if there is only one sensing link between BS and one UE, the reconstruction can only do a limited part. With multiple BSs and multiple UEs, the full view of the reconstruction can be achieved to satisfy the target performance requirements, by expanding the range of sensing observation angles. Additionally, it can also reduce interference between sensing links through resource coordination.

Therefore, cooperative sensing involving multiple base stations and multiple user devices need to be considered to ensure that target performance is achieved.

Proposal 4: 6G RAN shall study cooperative sensing mechanism involving multiple UEs and base stations to ensure target sensing performance.

In addition, in practical deployment it will be cost-effective to utilize communication network to provide ISAC services. In this case, it requires the RAN design to not only achieve the target performance for sensing, but also minimize the impact on communication services, and potential software/hardware sharing for sensing and communication could also be beneficial for operator’s investment.

Following these principles, sensing and communication design shall have joint consideration to ensure a trade-off between sensing performance and communication performance, e.g. whether to have common radio design like waveform and reference signals needs to have a thorough study. In this case, it is expected that these technical aspects in RAN WG study shall take both communication and sensing into account, to avoid the case that one design for communication is concluded without considering the impact from sensing perspective.

Proposal 5: 6G RAN shall enable efficient resource utilization by considering trade-off between sensing and communication performance.

- Corresponding technical aspects shall be studied thoroughly in RAN WG study, with performance evaluation provided from both communication and sensing perspective before reaching the conclusion.

Another general consideration for sensing is about measurement and feedback. On one hand, sensing requires measuring and reporting resources to meet versatile service requirements. On the other hand, this measurement and reporting mechanism need to consider the overall overhead of the system. Such a measurement and feedback design needs also to be taken into account to the overall E2E data collection and management framework. In addition, the efficient utilization of such measurement data is also important: such measurement data from sensing cannot only be utilized by itself, but also used by communication for potential performance improvement, e.g. utilized by positioning, dynamically switch-off TRPs for energy saving.

Proposal 6:  6G RAN shall enable efficient measurement feedback and usage of the measurements obtained by sensing, to improve communication performance and/or enable new ISAC services/applications.

Summary

We have discussed the new and extended services and the following are proposals for agreements:

For sensing provided services:

Proposal 1: 6G RAN shall support use case of target detection and tracking, e.g. road digitization, with the below performance target:

Static target or moving target up to 5km/h:

- Localization accuracy: horizontal 1m

- Velocity accuracy: 1m/s

Moving target up to 120km/h:

- Localization accuracy: horizontal 3m

- Velocity accuracy: 5m/s

Proposal 2: The 6G system shall be able to provide sensing service by detecting and tracking the characteristics of individual environmental object [6] as described:

- Measure and reconstruct a relatively large Environment Object which can be static and moving

- [sub meter to meter-level] reconstruction accuracy

Proposal 3: 6G RAN shall study mechanism of utilizing digital twin information for performance improvement, e.g. for optimizing communication and positioning services.

Proposal 4: 6G RAN shall study cooperative sensing mechanism involving multiple UEs and base stations to ensure target sensing performance.

Proposal 5: 6G RAN shall enable efficient resource utilization by considering trade-off between sensing and communication performance.

- Corresponding technical aspects shall be studied thoroughly in RAN WG study, with performance evaluation provided from both communication and sensing perspective before reaching the conclusion.

Proposal 6: 6G RAN shall enable efficient measurement feedback and usage of the measurements obtained by sensing, to improve communication performance and/or enable new ISAC services/applications.

Proposal 7: Adopt the text proposal in Sec 6 for ISAC services requirements in TR 38.914.

References

- RP-250810, “New SID: Study on 6G Scenarios and requirements”, RAN#107, March 2025.

- RP-251881, “New SID: Study on 6G Radio”, RAN#108, June 2025.

- Recommendation ITU-R M.2160-0: "Framework and overall objectives of the future development of IMT for 2030 and beyond".

- 3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1".

- 3GPP TR 22.870: “Study on 6G Use Cases and Service Requirements”.

- S1-253507, “Update of clause 7.5 use case on environment object reconstruction”, SA WG 1 Meeting #111, August 2025.

Text proposal for 38.914

In section 5 Requirements and key technical principles in TR38.914, the text proposal for section 5.4 are provided as follow:

-------------------------------------------------Start the text proposal ------------------------------------------------

5.4.x ISAC services

6G RAN shall support use cases and corresponding performance requirements as defined in TR 22.870

- use case of target detection and tracking, e.g. road digitization

- use case of digital twin and environment reconstruction

6G RAN shall study cooperative sensing mechanism involving multiple UEs and base stations to ensure target sensing performance.

6G RAN shall enable efficient measurement feedback and usage of the measurements obtained by sensing, to improve communication performance and/or enable new ISAC services/applications.

6G RAN shall enable efficient resource utilization by considering trade-off between sensing and communication performance.

-------------------------------------------------End the text proposal ------------------------------------------------