3GPP TSG RAN Meeting #109												RP-252138

Beijing, China, September 15-18, 2025

Agenda Item:	8.3

Source:	Huawei, HiSilicon

Title:	Consideration on UE management

Document for:	Discussion and decision

Introduction

User experience is always one important metric for a generation, including data rate, latency, energy saving and so on. These performance requirements will reflect into one particular aspect design, which is about UE management including UE states and UE mobility. These aspects are quite fundamental and will impact the fundamental performance for UEs, like initial access delay, state transition delay and processing complexity, etc. Therefore, aspects related to UE management will be introduced for a new generation in Day One design. This paper is to discuss potential requirements on UE management for 6G Day One design.

Discussion

RRC states management

In NR, three RRC states have been defined: RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED, among which RRC_INACTIVE is the additional state introduced compared with LTE. The intention for introducing three states is to balance the performance between UE power consumption and latency to access to the network. In particular, it was expected that RRC_INACTIVE state can have equivalent power consumption as RRC_IDLE state, and have fast access to the network for data transmission with latency close to RRC_CONNECTED mode. Figure 1 below shows the current RRC states clearly [1]:

Figure 1 NR RRC state transition

However, in reality RRC_INACTIVE was not widely commercialized. The reasons are analyzed as below:

- Transition from RRC_INACTIVE to RRC_CONNECTED does not provide better access latency, compared with transition from RRC_IDLE to RRC_CONNECTED

- RRC_INACTIVE does not obtain better performance of UE power consumption compared with RRC_IDLE

A UE in RRC_INACTIVE state still needs to trigger random access procedures to access to the network, the procedure has been simplified by keeping UE context and thus no need to reconfigure everything. Compared with the transition from RRC_IDLE to RRC_CONNECTED state, the access latency is similar between the two as shown in Figure 2a and Figure 2b (i.e. both requires 3 steps over radio interface for access). These procedures contribute the major latency for state transition, while both the UE side and network side have to additionally store the context of the UE for RRC_INACTIVE.

Access latency

Figure 2a NR RRC state transition from IDLE to CONNECTED

Access latency

Figure 2b NR RRC state transition from INACTIVE to CONNECTED

In addition, in case of supporting mobile-terminated services, the network has to support additional mechanism like RAN paging, which results in periodic signaling overhead. Due to the paging monitoring and downlink signal measurements, the power consumption for the UE in RRC_INACTIVE state does not get better compared with RRC_IDLE. Consequently, RRC_INACTIVE state becomes not attractive in real deployment.

In this case, RRC states in 6G can be potentially simplified, to avoid unnecessary state transition, signaling overhead and management cost for maintaining three RRC states per UE. On the other hand, the RRC states management shall still consider how to balance UE power consumption and access latency, to maximize user experience.

Proposal 1: Simplification of RRC state management shall be studied to avoid unnecessary state transition and signaling overhead, and to further improve balance of performance between UE power consumption and access latency.

The above balanced performance basically consists of two aspects: 1) whether more energy saving gains for UE can be achieved for different RRC states; 2) whether we can have faster access for data transmission and RRC state transition.

For 1), it is one important consideration for overall system design in 6G Day One. For 2), this mainly refers to random access procedures for data transmission and RRC state transition. Existing mechanism to support these procedures usually uses contention based random access (CBRA) as the default mechanism for access, where 4-step and 2-step random access procedures have been introduced in 5G, as shown in Figure 3.

Figure 3 4-step CBRA and 2-step CBRA procedures

For 4-step CBRA procedure, the access latency is around 20~30ms if there is no collision, including the RAR reception time and subsequent MSG3/MSG4 transmission. If the collision happens, the overall delay will become even longer, e.g., more than 40 ms, considering multiple times of re-transmission of random access requests. Such a long delay does not provide good user experience for new emerging applications, such as immersive communications with composite requirement in terms of 10-15ms latency and 20-100Mbps (UL) as described in [2]. In case the UE is in RRC_IDLE/RRC_INACTIVE state it can be observed that the access latency cannot afford the user experience.

For 2-step CBRA procedure, although the latency is reduced to 10~20 ms, if the collision occurs, additional latency similar as the above has to be taken into account.

Contention free random access (CFRA) has also been introduced in 5G to reduce the latency, which uses a dedicated preamble for a certain UE. However, this mechanism is only used for some specific scenarios, e.g. in handover scenario for RRC_CONNECTED state, beam failure recovery on the SpCell. The major reason that CFRA cannot be widely used is due to the limitation of PRACH resources. Therefore, ways of expanding PRACH resources can be considered to reduce the collision probability and reduce the access latency.

Proposal 2: Mechanism of reducing random access delay and collision probability shall be studied, to improve the access latency for data transmission and RRC state transition.

UE mobility management

The UE mobility management is to ensure the UE is reachable, and every time when the UE moves from one cell to another, either the cell reselection procedure or network-controlled procedures (such as e.g. handover, redirection) will be triggered according to different RRC states.

When UE is in RRC_CONNECTED state, 5G has introduced multiple schemes, including L3 handover procedure, conditional handover, DAPS handover, RACH-less handover and LTM handover, etc. Such a diversity of handover schemes leads to difficulty for commercialization, as it is challenging for operators to figure out what solution should be applied to various cases. For example, DAPS handover can achieve 0ms interruption but requires UE to support dual uplink stack; conditional handover can execute timely handover but causes the network additional complexity to set proper conditions and resource pre-allocation in advance. Therefore, in 6G, it is better to minimize the options and aim at having a unified mechanism for mobility management, considering both handover performance and system complexity.

Proposal 3: Mobility management mechanism shall be studied for a unified design, considering both handover performance and system complexity.

In all above options to support mobility, there is a common issue on signaling overhead to reconfigure the UE when it tries to access to a target cell: every time, the network needs to assign a new tentative ID for the UE for subsequent resource allocation and scheduling, along with other parameters for access. Such procedure is not only applicable to inter-cell mobility case, but also to intra-cell mobility case. This means even if other configurations are not modified, the network still has to send the reconfiguration message to assign the ID(s) to identify the UE, which may result in signaling overhead, interruption latency and management cost of UE ID(s). Therefore, simplification of UE ID management needs to be taken into account to further enhance mobility performance. This direction can also be beneficial for the case where the UE stays within the same cell,  e.g. to reduce latency when the UE needs to have a re-entry to the same cell for data transmission.

Proposal 4: Mechanism to simplify UE ID management shall be studied for the purpose of better performance for mobility management and fast re-entry to the network.

Summary

It is suggested to adopt the below requirements into TR 38.914 operational requirements

Proposal 1: Simplification of RRC state management shall be studied to avoid unnecessary state transition and signaling overhead, and to further improve balance of performance between UE power consumption and access latency.

Proposal 2: Mechanism of reducing random access delay and collision probability shall be studied, to improve the access latency for data transmission and RRC state transition.

Proposal 3: Mobility management mechanism shall be studied for a unified design, considering both handover performance and system complexity.

Proposal 4: Mechanism to simplify UE ID management shall be studied for the purpose of better performance for mobility management and fast re-entry to the network.

References

- 3GPP TS 38.331, NR Radio Resource Control (RRC) Protocol Specification

- RP-252131, Consideration on supporting immersive communication services for 6G, Huawei, HiSilicon