3GPP TSG RAN meeting #109		      RP-252188

Beijing, China, 15th – 18th Sep 2025

Source:	    vivo, Ericsson, CATT, T-Mobile, CMCC, Qualcomm incorporated, NTT DOCOCMO

Title:	Views on L3 beam-level prediction in Rel-20 AI/ML mobility WI

Agenda Item:	9.3.2.1

Document for:	Discussion and Decision

- Introduction

In the baseline WID of AI/ML for mobility[1] endorsed at the RANP#108, whether L3 beam-level prediction is included in the WI is still pending.

- Specify support for RRM measurement prediction for both UE and network sided models [RAN2]: - Specify signalling and protocol aspect to enable reporting inference outcome of UE sided model for RRM measurement prediction - Specify signalling and protocol aspect to enable reporting information from UE for inference operation of network sided model for RRM measurement prediction - At least the use cases studied so far in RAN2 SI are supported: Temporal domain prediction case A and case B, and Frequency domain prediction for collocated cases. Cell level predication is supported Note 1: For UE sided model, spatial domain is not supported. Can check in RAN#109 whether for network sided model spatial domain is supported or not, based on the final outcome of the SI. Note 2: Check in RAN#109 whether L3 beam level prediction is also supported, based on the final outcome of the SI.

At the RAN2#131 meeting, RAN2 reached the following agreements on L3 beam-level prediction:

Agreements on L3 beam-level prediction

- For RRM measurement prediction, L3 beam-level prediction is feasible for network sided model.  CB after spec impact/overhead etc on the different cases.

- For RRM measurement prediction, L3 beam-level prediction is feasible for UE sided model.  However, there are concerns on complexity and other WGs.

In this contribution, we will discuss the necessity and importance of L3 beam-level prediction, UE complexity and the potential RAN4 workload, and then propose to support L3 beam-level prediction in R20 WI.

- Discussion

## Necessity and importance of L3 beam-level prediction

In the existing non-AI handover procedure, L3 beam-level measurement results are exchanged between the source and target nodes to configure RACH resources, thereby reducing handover latency and ensuring robustness. For AI-based mobility, the absence of L3 beam-level prediction may reduce the performance gains of AI/ML-based approach over legacy mobility. For instance:

- For temporal domain Case A prediction, if the PW length is large, the UE can predict measurement events that will be fulfilled in advance and report them early. In this case, the time gap between the handover command and the measurement report becomes significant. If the target gNB configures the RACH resources based on beam-level measurements at the reporting time, the CFRA procedure during handover may fail due to wrong beam selection. Alternatively, CBRA would have to be used, but this could increase handover delay due to potential preamble collisions and raise the likelihood of handover failure.

- For frequency domain prediction, if L3 beam-level prediction is unavailable, only CBRA can be employed. As a result, an AI/ML-based approach may have poorer performance than legacy HO.

Consequently, for UE-sided model, it becomes critical to include L3 beam-level prediction in the measurement prediction report. Otherwise, the network may be unable to make an informed mobility decision, e.g., allocating RACH resources on the target cell for the upcoming UE. Therefore, relying solely on cell-level prediction is not sufficient and may lead to suboptimal or even failed mobility procedures.

Observation 1: For UE-sided model, relying solely on cell-level prediction may reduce the performance gains of AI/ML-based approach over legacy non-AI approach.

## UE complexity

For the L3 cell-level and beam-level prediction, the following sub-use cases are considered in the SI phase:

Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) Sub-use case 4: L1 filtered beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated Sub-use case 5: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s) Sub-use case 6: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

The UE-side complexity may rely on the selection of sub-use case, e.g., if sub-use case 1 is selected for cell-level prediction and sub-use case 4 is selected for beam-level prediction, the UE can utilize one single model to generate both cell-level and beam-level results. In this case, limit extra complexity is introduced to support beam-level prediction.

Additionally, even if a separate model is utilized for beam-level prediction, it can be implemented as an optional UE capability. An UE vendor can make the selection of support or not, e.g., based on whether an UE vendor cares about the mobility performance more, or UE complexity more.

Observation 2: The extra complexity for beam-level prediction of UE-sided model relies on the selection of sub-use case. Whether to support L3 beam-level prediction can be an optional UE capability.

## Potential R4 workload for L3 beam-level prediction

During the past RAN4 meetings, regarding RRM prediction, RAN4 primarily discussed and analyzed the potential RRM impacts. This mainly included candidate KPIs and their definitions, factors that may affect prediction performance, as well as the impact of measurement errors.

First, concerning the definition of KPIs, RAN4 has already defined the absolute and relative accuracy of predicted L3-RSRP as the candidate metrics for RRM requirements in RRM measurement prediction.

The absolute accuracy of predicted L3-RSRP is defined as: - Absolute accuracy of predicted L3-RSRP = reported predicted L3-RSRP – ground truth of L3-RSRP. The relative accuracy of predicted L3-RSRP for intra-frequency prediction is defined as: - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), - cell 1 and cell 2 are on the same frequency - the reported RSRP of cell 2 can be measured or predicted. - Editor Note: The relative accuracy of predicted L3-RSRP for beam level measurements may be further discussed during WI phase depending upon RAN2 progress. The relative accuracy of predicted L3-RSRP for inter-frequency prediction is defined as: - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported L3-RSRP of cell 2) – (ground truth of L3-RSRP of cell 1 – ground truth of L3-RSRP of cell 2), - cell 2 is on a different frequency than cell 1 but in the same FR as cell 1 - the reported L3-RSRP of cell 2 can be measured or predicted. Note: It is not precluded to update the definition based on further RAN2 progress in WI phase.

Regarding the applicability of the definition to L3 beam-level prediction, the current situation is:

- For the absolute RSRP accuracy definition, as shown below, this definition is common to both cell-level and beam-level prediction.

The absolute accuracy of predicted L3-RSRP is defined as: - Absolute accuracy of predicted L3-RSRP = reported predicted L3-RSRP – ground truth of L3-RSRP.

- For the relative RSRP accuracy definition, beam-level prediction was also involved in the detailed discussions on the definition in previous RAN4 meetings, and the following definition was agreed upon during the RAN4#114bis meeting, as highlighted in the section below.

Agreement in RAN4#114bis Issue 2-1-1: Relative RSRP accuracy definition Agreement: - For intra-frequency L3-RSRP, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency, or - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell. - FFS if this definition will be needed.

It can be observed that the above RAN4 agreements already refer explicitly to beam-level prediction, but are dependent on whether beam-level prediction is supported from signalling perspective (which is pending RAN2 conclusion). If RAN2 agrees to support L3 beam-level prediction in Rel-20, it is straightforward that the definitions agreed upon during the study item phase can be used as the baseline.

Observation 3: Some of the RAN4 agreements already refer explicitly to beam-level prediction, but are dependent on whether beam-level prediction is supported from signalling perspective.

Secondly, the potential factors currently identified by RAN4 that affect prediction performance should be applicable to both cell-level and beam-level prediction. For example, for intra-frequency temporal domain prediction, RAN4 has considered factors such as UE speed, MRRT, OW/PW length, and measurement error. The difference lies in the fact that the extent of their impact on prediction performance may vary between cell-level and beam-level prediction, and thus may require separate quantitative evaluations. However, since the platform, simulation assumptions, and dataset for cell-level prediction can be largely reused for beam-level prediction, the extra workload for beam-level prediction can be acceptable.

Additionally, regarding the performance monitoring mechanism not covered by RAN4 during the SI phase, in the Rel-19 WI "AI/ML for NR Air Interface," a basic version of this mechanism has already been designed for the BM use case, introducing the report quantity of accurate reference signal prediction instance(s) that meets the condition. Specifically, this condition refers to a certain prediction accuracy ratio of the top beam(s), i.e., whether the Top 1 or 2 beam(s) is among the top-K predicted beam(s).

Agreement For UE-sided AI/ML model for beam management, for Option 2 (UE-assisted performance monitoring), the performance metric of Top 1 or Top K beam prediction accuracy is defined as: - At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A (e.g., linked to at least one of the Top-K predicted beam(s) of Set A based on certain rule or signalling) - Where K is the number of predicted beam(s) in the corresponding inference report per time instance - Where Top M beam(s) is the best M beam(s) based on L1-RSRP measurements of the resource set(s) for monitoring - M is configured by NW in CSI report configuration for monitoring - M= 1, 2 - FFS: detailed rule or signalling Agreement For beam prediction accuracy report for monitoring, the report quantity RS-PAI is Np (0 ≤ Np≤ N) - Where Np is the total count of accurate reference signal prediction instance(s) that meets the condition, among N latest transmission occasion(s) of monitoring resources that no later than CSI reference resource corresponding to the CSI report for monitoring - condition: - for the transmission occasion of monitoring resources, it has a linked inference report - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources mapped to one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s), of the linked report of the CSI Reporting Setting for inference - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance. - Where N = 1, 3, 7, 15 is configured in CSI-ReportConfig - the size of CSI field associated with the RS-PAI is log2(N+1)

Currently, for beam-level measurement, if the network configures reportQuantityRS-Indexes and maxNrofRS-IndexesToReport, the UE will include beam measurement information in the triggered measurement report, which at least includes the index of the best beam(s). If the network further sets includeBeamMeasurements to true, the UE will additionally report beam-level measurement results. This is because the beam-level measurement results are mainly used to allocate RACH resources associated with the best beam(s). Therefore, similar to AI-based beam management, L3 beam-level prediction should also place great importance on the accuracy of best beam prediction, which can be considered as a starting point for discussing the performance metric for monitoring L3 beam-level prediction.

From the above analysis, it can be seen that from the RAN4 perspective, the support of L3 beam-level prediction can refer as much as possible to the outcomes discussed in L3 cell-level prediction and/or AI-based beam management. The main workload actually lies in performance evaluation and requirement definition. However, we understand that RAN2 has already conducted much constructive preliminary work on performance evaluation during the SI phase. Based on this, RAN4 can further proceed with the specific requirement definition.

Observation 4: With L3 cell-level prediction and/or AI-based BM as reference, the workload of RAN4 on L3 beam-level prediction is expected to be acceptable.

Based on the above discussion, we propose:

Proposal: Support L3 beam-level prediction of both UE-side and NW-side models in Rel-20 WI.

- Conclusion

Based on the above analysis, we make the following observations and proposals:

Observation 1: For UE-sided model, relying solely on cell-level prediction may reduce the performance gains of AI/ML-based approach over legacy non-AI approach.

Observation 2: The extra complexity for beam-level prediction of UE-sided model relies on the selection of sub-use case. Whether to support L3 beam-level prediction can be an optional UE capability.

Observation 3: Some of the RAN4 agreements already refer explicitly to beam-level prediction, but are dependent on whether beam-level prediction is supported from signalling perspective.

Observation 4: With L3 cell-level prediction and/or AI-based BM as reference, the workload of RAN4 on L3 beam-level prediction is expected to be acceptable.

Proposal: Support L3 beam-level prediction of both UE-side and NW-side models in Rel-20 WI.

- Reference

- RP-251864	New WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

- R2-2506468	Summary of [AT131][033][AI Mob] Conclusions for TR (OPPO)