3GPP TSG RAN Meeting #109															RP-252198

Beijing, China, September 15-18, 2025

Agenda item:	8.2.2

Source:	Nokia

Title:	On 6GR minimum requirement for UE channel bandwidth

Document for:	Discussion and Decision

# Introduction

RAN1#122 kick-off discussions on 6GR study item reached to following agreement that defers the discussion and confirmation on the smallest maximum channel BW that a UE is allowed to be limited to RAN:

Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different

# Discussion

When assessing what is the smallest maximum channel BW that a UE is allowed to be limited to, it is important to keep in mind that all the communication between the network and all the UEs need to take place within this BW up until the UEs’ BW capabilities are known to the network for such UEs to exist in the system. That is:

At least all of the following need to be designed to fit within this minimum channel BW requirement:

- The cell synchronization signals

- System information broadcast

- Random access channel

- Control and data channels during random access procedure

As the channel BW requirements are not independent from the cell’s supported subcarrier spacing (SCS), and thus not from the cell’s band, the minimum channel BW requirement needs to be considered in context with the SCS being considered.

As per the RAN1 agreement, 15 kHz SCS should be considered as the main-stream deployment for the FDD bands

Agreement - 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH. - For sub 6GHz - The following subcarrier spacing is at least supported - 15kHz SCS for FDD, 30kHz SCS for TDD - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz - FFS: 7.5kHz SCS for sub1GHz (FDD) - Whether to discuss the FFS will be subject to RANP decision. - For around 7GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz - FFS: For around 15GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz, 120kHz - Whether to discuss it will be subject to RANP decision - For between 24.25GHz - 52.6GHz - Subcarrier spacing 120kHz is supported - FFS whether to allow using additional subcarrier spacing for SSB - FFS subcarrier spacing for PRACH and up to initial access discussion.

It is clear, that 6GR must support 3 MHz CBW for specific scenarios in FDD bands, but at the same time it would not be desirable for the bands or deployments where 3 MHz support is not supported or needed to have to inherit the initial access design that was constrained by the 3 MHz channel BW. In 5G NR, the initial access channels in DL (SS block and CORESET#0) define the bottleneck scenario for the minimum channel bandwidth (CBW) size. Until Release 17 the minimum CBW size was 5 MHz for 15 kHz SCS, and it turned out to be challenging to build support for the 3 MHz CBW in Release 18.

It is reasonable to assume that the initial access design will be a bottleneck for the minimum CBW​ also in 6G. Hence, we need to ensure availability of the second (narrowband) SSB/CORESET0 design for channel bandwidths below 
5 MHz such as 3 MHz without puncturing for predefined bands in Rel-21. This should be applied for predefined scenarios with specific sync raster points in specific bands and FDD with 15 kHz SCS​.

The requirements of 5G NR should be reused for 15kHz SCS FDD and 30 kHz SCS TDD for sub 6 GHz and 30 / 60 kHz around 7 GHz, i.e. defined by 25 RBs. For 120 kHz SCS for higher bands (supported at least for 24.25 – 52.6 GHz, i.e. FR 2-1), we suggest increasing the minimum CBW to 100 MHz both, to allow for a large number of SSB beams by supporting 240 kHz SCS SSB, and as we don’t see the necessity to impair the FR2-1 deployments with the minimum CBW support that may not be otherwise needed.

Table 1 below shows our proposal for minimum CBW (Channel Bandwidth) for different SCS both for the system deployment, as well as the smallest maximum channel BW that a UE is allowed to be limited to.

Proposal: The minimum 6GR channel bandwidth all UEs are at least required to support is as listed in Table 1 below:

Table 1: Minimum channel bandwidth size for 6GR; all UEs are required to support at least this BW

| SCS (kHz) | Min CBW (MHz) |
| --- | --- |
| 15 | 5 (3*) |
| 30 | 10 |
| 60 (if supported) | 20 |
| 120 | 100 |

*	Supported for certain bands and sync raster points only in deployments,
3 MHz as the maximum CBW is allowed only for UEs that can only access 3 MHz cells

# Conclusions

Triggered by RAN1#122 agreement that defers the decision making to RAN plenary, in this contribution we discuss the minimum 6GR channel bandwidth all UEs are at least required to support, and how it is derivative of the smallest channel bandwidths the system should be designed to support. In this regard we propose the following:

Proposal: The minimum 6GR channel bandwidth all UEs are at least required to support is as listed in Table 1 below:

Table 1: Minimum channel bandwidth size for 6GR; all UEs are required to support at least this BW

| SCS (kHz) | Min CBW (MHz) |
| --- | --- |
| 15 | 5 (3*) |
| 30 | 10 |
| 60 (if supported) | 20 |
| 120 | 100 |

*	Supported for certain bands and sync raster points only in deployments,
3 MHz as the maximum CBW is allowed only for UEs that can only access 3 MHz cells

# References

[1] 	RP-251881, “Study on 6G Radio,” NTT Docomo, June 2025.