3GPP TSG RAN Meeting #109									RP-252202

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.1

Source:	Xiaomi

Title:	Deployment scenarios for ISAC

Document for:	Discussion and Decision

- Introduction

In RAN #108 meeting, the text proposal for TR 38.914 on deployment scenarios was agreed, but it is currently focusing on communication aspects [1]. There is a general sentence on the sensing aspects in the beginning of section 4 in TR 38.914.

Editor note: Sensing related aspects will be included in section 4

In this contribution, our views on 6G deployment scenarios for Integrated Sensing And Communication (ISAC) study are provided.

- Discussions

ISAC is a key usage scenario for 6G. It extends communication oriented 3GPP system with the new sensing capabilities for broader business opportunities. Various use cases are identified in SA1 TR 22.837 and TR 22.870. Considering the commercial request in the time being and near feature and existing work on ISAC in RAN, it is preferred to prioritize object detection/tracking use cases for 6G studies in Rel-20. All bistatic/monostatic sensing modes can be considered in the 6G studies, as well as cooperative sensing and/or multi-static sensing.

As described in our companion paper [2], from this wide range of use cases a careful selection should be done with focus on the most urgent commercial needs and taking into account what RAN already has developed during Rel-19, i.e., the ISAC channel model for object detection and tracking. Therefore, it is proposed in [2] to prioritize the use cases for UAV, human and automotive. The related deployment scenarios for the use cases are discussed in the following.

In the current TR 38.914, twelve deployment scenarios are defined mainly from the communication point of view. The sensing related attributes should be introduced based on the communication scenarios, which is motivated by joint design of sensing and communication. On the other hand, it is possible that not all assumptions of communication are necessary for sensing. Therefore, it is preferred to make separate sections on deployment scenarios for ISAC in TR 38.914. The attributes of deployment scenarios for ISAC should be aligned with communication scenarios whenever possible.

Proposal 1: Separate sections and Tables on attributes in TR 38.914 are used to capture the deployment scenarios for ISAC.

The number of combinations for deployment scenario, sensing mode and target type would become too large for Rel-20. It is therefore necessary to consider a reasonable sub-set for the most urgent applications:

- UAV detection and tracking: Urban macro (UMa-AV in NR). This is also the scenario for which the ISAC Rel-19 channel model has been calibrated and it is also the baseline scenario for performance evaluation for ISAC in Rel-20 5GA.

- Human intruder detection or health care are also important for sensing. Indoor hotspot is preferred since there are more applications for indoor users than for outdoor and RAN should therefore prioritize this scenario.

- A third important category is automotive. For these use cases, we prefer urban grid over highway, since more advanced automotive scenarios can be modelled, e.g., the crossroad or the blocking/shadowing of building, etc.

Proposal 2: For the different use cases of sensing, the following deployment scenarios should be considered,

- UAV detection and tracking: Urban macro (UMa-AV in NR)

- Human intruder detection or health care: Indoor hotspot

- Automotive: Urban grid

In the following we discuss the parameters that can be used for the different deployment scenarios for ISAC.

  - Urban macro for ISAC

TRP monostatic sensing mode for UAV use case is studied in 5GA Rel-20, other sensing modes or cooperative sensing of all sensing modes should be further considered in 6G. Compared with Table 4.1 in TR 38.914, the carrier frequency 700MHz and 2GHz are removed since they are not typical for sensing operations. Minimum UE-UE distance is not necessary for communication evaluation, but is required for UE-UE bistatic sensing. Further, the sensing targets related parameters are provided. Joint operation of sensing and communication can be supported in Urban macro for ISAC.

-------------------- TP #1 on Urban macro for ISAC ----------------------

4.4A	Urban macro for ISAC

The urban macro deployment scenario for ISAC focuses on UAV detection/tracking with a large 3D coverage. The key characteristics of this deployment scenario are continuous and ubiquitous coverage in urban areas. The sensing Tx/Rx can be BS and/or UE. Joint operation of sensing and communication can be considered.

Some of its attributes are listed in Table 4.4A.

Table 4.4A: Attributes for urban macro for ISAC

| Attributes |  | Values or assumptions |
| --- | --- | --- |
| Carrier Frequency |  | Around 4 GHz Around 7 GHz Around 30GHz Around 4 GHz + Around 7GHz Around 4 GHz + Around 30GHz |
| Aggregated system bandwidth |  | Around 4 GHz: Up to [200] MHz (DL+UL) Around 7 GHz: Up to [400]MHz (DL+UL) Around 30 GHz: [1]GHz (DL+UL) |
| Layout |  | Single layer: - Hex. Grid Two layers for Around 4 GHz + Around 30 GHz: - Macro layer: Hex. Grid, 4GHz - Micro layer: Random drop, 30GHz |
| ISD |  | Macro: 500m [TBD on micro layout] |
| BS antenna elements |  | TBD |
| UE antenna elements |  | TBD |
| User distribution and UE speed |  | TBD [10] users per TRxP |
| Service profile |  | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |
| Minimum UE-UE distance |  | TBD |
| Sensing target | Type | UAV |
|  | LOS/NLOS | LOS and NLOS |
|  | Outdoor/indoor | Outdoor |
|  | Mobility | Horizontal: {0 km/h, Uniform distribution between 0 and 180km/h} Vertical: {0, 20, 40} km/h Trajectory: straight-line |
|  | Distribution | N targets per sector within the center cell, N = 5 Horizontal: uniformly distributed within a sector Vertical: Uniform between 1.5m and 300m. |
|  | Orientation | Random in horizontal domain |
|  | Physical characteristics | Size: UAV with {small size, large size} defined in Rel-19 |

-------------------- End of TP #1 ----------------------

Proposal 3: TP #1 for a new deployment scenario Urban macro for ISAC can be added to TR 38.914 for UAV sensing scenarios.

  - Indoor hotspot for ISAC

For human intruder detection or health care, indoor hotspot can be a typical scenario. The sensing Tx/Rx can be BS or UE. The attributes of Indoor hotspot for ISAC can reuse those from indoor hotspot. The sensing targets related parameters need to be added.

-------------------- TP #2 on Indoor hotspot for ISAC ----------------------

4.1A	Indoor hotspot for ISAC

The indoor hotspot deployment scenario for ISAC focuses on human as sensing target in small coverage. The key characteristics of this deployment scenario are the large LOS probability and the large density of BSs/UEs. The sensing Tx/Rx can be BS and/or UE. Joint operation of sensing and communication can be considered.

Some of its attributes are listed in Table 4.1A.

Table 4.1A: Attributes for indoor hotspot for ISAC

| Attributes |  | Values or assumptions |
| --- | --- | --- |
| Carrier Frequency NOTE1 |  | Around 4 GHz Around 7 GHz Around 30GHz Around 4 GHz + Around 7GHz Around 4 GHz + Around 30GHz |
| Aggregated system bandwidth NOTE2 |  | Around 4 GHz: Up to [200] MHz (DL+UL) NOTE3 Around 7 GHz: Up to [400] MHz (DL+UL) NOTE3 Around 30 GHz: [1]GHz (DL+UL) NOTE3 |
| Layout |  | Single layer: - Indoor floor (Open office) |
| ISD |  | 20m for around 30GHz TBD on other carrier frequencies (Equivalent to [12]TRxPs per 120m x 50m) |
| BS antenna elements |  | TBD |
| UE antenna elements |  | TBD |
| User distribution and UE speed |  | 100% Indoor, 3km/h, [10] users per TRxP NOTE4 |
| Minimum UE-UE distance |  | TBD |
| Service profile |  | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |
| Sensing target | Type | Human |
|  | LOS/NLOS | LOS and NLOS |
|  | Outdoor/indoor | Indoor |
|  | Mobility | Uniform distribution between 0km/h and 3km/h Trajectory: straight-line |
|  | Distribution | N targets uniformly distributed over the horizontal area of the convex hull of the TRP deployment, N = 5 |
|  | Orientation | Random over the horizontal area |
|  | Physical characteristics | Adult: 0.5m x 0.5m x 1.75m |

-------------------- End of TP #2 ----------------------

Proposal 4: TP #2 for a new deployment scenario Indoor hotspot for ISAC can be added to TR 38.914 for human sensing scenarios.

  - Urban grid for ISAC

Urban grid deployment scenario was introduced for V2X use cases in 5G. It is expected that Urban grid deployment scenario in 6G is introduced mainly for sensing. All sensing modes including cooperative sensing can be considered. The sensing transmitter/receiver can be TRP, vehicle UE, pedestrian UE and/or RSU type UE. On the other hand, to verify the proper design of the ISAC operation, it is also important to check the communication performance in Urban grid deployment scenario. The attributes for Urban grid for ISAC are provided below. The attributes except for those for sensing target are obtained by referring to the attributes of Urban macro. The existing section [4.11 Urban grid] can be renamed as “4.11 Urban grid for ISAC”.

-------------------- TP #3 on Urban grid for ISAC ----------------------

4.11	Urban grid for ISAC

The Urban grid deployment scenario focus on vehicle/human detection/tracking in the dense urban area. The key characteristics of this deployment scenario are rich of scatterers in the environment. Certain environment object may be deterministically modeled. The sensing Tx/Rx can be BS, vehicle UE, pedestrian UE and/or RSU type UE. In Urban grid for ISAC, evaluation of communication performance can be jointly considered too.

Some of its attributes are listed in Table 4.11.

Table 4.11: Attributes for Urban grid for ISAC

| Attributes |  | Values or assumptions |  |
| --- | --- | --- | --- |
| Carrier Frequency |  | Around 4 GHz Around 7 GHz Around 30GHz Around 4 GHz + Around 7GHz Around 4 GHz + Around 30GHz |  |
| System bandwidth |  | Around 4 GHz: Up to [200] MHz (DL+UL) Around 7 GHz: Up to [400]MHz (DL+UL) Around 30 GHz: [1]GHz (DL+UL) |  |
| Layout |  | Single layer: - Hex. Grid Two layers for Around 4 GHz + Around 30 GHz: - Macro layer: Hex. Grid, 4GHz - Micro layer: Random drop, 30GHz |  |
| ISD |  | Macro: 500m [TBD on micro layout] |  |
| BS antenna elements |  | TBD |  |
| UE antenna elements |  | TBD |  |
| User distribution and UE speed |  | Based on TR37.885 |  |
| Minimum UE-UE distance |  | TBD |  |
| Service profile |  | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |  |
| Sensing target, i.e., UAV | Type | Vehicle | Human |
|  | LOS/NLOS | LOS and NLOS (including NLOSv) | LOS and NLOS |
|  | Outdoor/indoor | Outdoor | Outdoor |
|  | Mobility | Based on TR37.885 | Uniform distribution between 0km/h and 10km/h Trajectory: straight-line |
|  | Distribution | Based on dropping in TR37.885 | Based on dropping in TR37.885 |
|  | Orientation | Lane direction in horizontal plane | Random over the horizontal area |
|  | Physical characteristics | Type 1/2 (passenger vehicle) Vehicle type distribution per TR 37.885 | Adult Pedestrian: 0.5m x 0.5m x 1.75m |
| Environment Objects |  | Optional: Type-2 EO - 4 walls modelled as Type-2 EO, per building of size 413m x 230m x 20m. |  |

-------------------- End of TP #3 ----------------------

Proposal 5: TP #3 for a new deployment scenario Urban grid for ISAC can be added to TR 38.914 for automotive sensing scenarios.

- Conclusion

In this contribution, we discuss the deployment scenarios for ISAC and make the following proposals.

Proposal 1: Separate sections and Tables on attributes in TR 38.914 are used to capture the deployment scenarios for ISAC.

Proposal 2: For the different use cases of ISAC, the following deployment scenarios should be considered,

- UAV detection and tracking: Urban macro (UMa-AV in NR)

- Human intruder detection or health care: Indoor hotspot

- Automotive: Urban grid

Proposal 3: TP #1 for a new deployment scenario Urban macro for ISAC can be added to TR 38.914 for UAV sensing scenarios.

Proposal 4: TP #2 for a new deployment scenario Indoor hotspot for ISAC can be added to TR 38.914 for human sensing scenarios.

Proposal 5: TP #3 for a new deployment scenario Urban grid for ISAC can be added to TR 38.914 for automotive sensing scenarios.

- References

- 3GPP TR 38.914, Study on 6G Scenarios and Requirements

- RP-252203, Requirements of Integrated Sensing And Communication (ISAC), Xiaomi