3GPP TSG RAN # 109											           RP-252208

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.1

Source: 	CSCN, CATT

Title:           Views on 6G NTN Scenarios

Document for:  Discussion/Decision

- Introduction

In RAN#108 the “TR 38.914 v0.1.0 for Study on 6G Scenarios and requirements” was approved in RP-251873 [1].

- Discussion

This contribution presents our views on 6G NTN Scenarios.

=======================Start of the changes======================

# 4.	Deployment scenarios

## 4.10	Non-Terrestrial Network

These deployment scenarios are defined to allow for the provision of services for those areas including vertical space (e.g., several kilometers) where terrestrial network service is not available (on a permanent or temporary basis), and also for those services that can be more efficiently supported by Non-Terrestrial Networks. Non-Terrestrial Networks can overcome coverage limitations especially in low-density populated areas and maritime areas. As such, NTN will mainly contribute to:

- Achieve ubiquitous connectivity and ensure the availability of services in remote and sparsely populated areas.

- Improve the overall 6G network resiliency, especially in situations where disasters disrupt terrestrial networks or damage undersea communication cables. Maintain critical communications during disasters when terrestrial networks fail.

Non-Terrestrial Network may provide messaging, voice, video, delay-tolerant communications, machine-type communications and other data services. It also has the new capabilities in 6G, such as AI, computing, and sensing.

Some of its attributes are listed in Table 4.10-1.

Table 4.10-1: Examples for Satellite Deployment

| Attributes | Deployment-1 | Deployment-2 | Deployment-3 |
| --- | --- | --- | --- |
| Carrier Frequency NOTE1 | Around 1.5 or 2 GHz for both DL and UL | Sub-8GHz for both DL and UL | Around 20 GHz for DL Around 30 GHz for UL |
| Duplexing | FDD /HD-FDD | TDD NOTE2 | FDD/HD-FDD |
| Subcarrier spacing | 15k/30kHz | 30kHz | 120kHz |
| Satellite architecture | Bent-pipe, On-Board Processing NOTE3 | Bent-pipe, On-Board Processing | Bent-pipe, On-Board Processing |
| Cell Type | Earth-fixed cell, Quasi-Earth-fixed cell, Earth-moving cell | Quasi-Earth-fixed cell, Earth-moving cell | Earth-fixed cell, Quasi-Earth-fixed cell, Earth-moving cell |
| Feeder link type | Bent-pipe: 6G radio interface On-Board Processing: 3GPP satellite backhaul/ Non-3GPP | Bent-pipe: 6G radio interface On-Board Processing: 3GPP satellite backhaul/ Non-3GPP | Bent-pipe: 6G radio interface On-Board Processing: 3GPP satellite backhaul/ Non-3GPP |
| Typical satellite system positioning in the 6G architecture | Access network | Access network | Access network, Backhaul network |
| System Bandwidth (DL + UL) | 40M | 100M | Up to 400M without CA/ 800M with CA |
| Satellite Orbit | LEO, MEO, GEO | LEO | LEO, MEO, GEO |
| UE Distribution | 100% Outdoors | 100% Outdoors | 100% Outdoors |
| UE Mobility | Fixed, Portable, Mobile NOTE4 | Fixed, Portable, Mobile | Fixed, Portable, Mobile |

NOTE1:	The carrier frequencies noted here are for evaluation purpose only, satellites are deployed in wide range of frequency bands including L band (1-2GHz), S band (2-4GHz), C band (3.4-6.725 GHz), Ku band (10.7-14.8 GHz), Ka band (17.3-21.2 GHz, 27.0-31.0 GHz) and Q/V bands (37.5-43.5 GHz, 47.2-50.2 GHz and 50.4-51.4 GHz) and more. Here the around xGHz is used to denote the carrier frequency close to xGHz rather than used as band proxy to denote a band range.

NOTE2:   TDD is not precluded in 6G. TDD is only applicable to LEO.

NOTE3:	Bent pipe refers to the architecture where the satellite transponders are transparent—only amplify and change frequency but preserve the waveform. On Board Processing satellite transponders incorporate regeneration — including modulating and coding the waveform.

NOTE4:	Mobile include of both hand-helds and other moving platform receivers such as automobiles, ships, planes etc. Currently the hand-helds are limited to L and S bands.

## 4.10.1 Cell deployment

One-beam cell: The cell consists of only one logical beam as in Figure 4.10.1-1.

Figure 4.10.1-1: One-beam cell

Multi-beam cell: The cell consists of multiple logical beams with beam sweeping within a cell as in Figure 4.10.1-2. Logical beams may be SSB beams inherited from 5G or newly defined synchronization signals in 6G. In order to achieve refined management, more than 8 logical beams in FR1 can be considered.

Figure 4.10.1-2: Multi-beam cell

## 4.10.2 Cell type

NTN can be deployed to provide coverage with Earth-fixed cell, quasi-Earth-fixed cell and Earth-moving cell which are supported respectively by the following three types of service link:

-	Earth-fixed cell: provisioned by beam(s) continuously covering the same geographical areas all the time (e.g., the case of GSO satellites);

-	Quasi-Earth-fixed cell: provisioned by beam(s) covering one geographic area for a limited period and a different geographic area during another period (e.g., the case of NGSO satellites generating steerable beams);

Case 1: The areas of cell coverage in two consecutive periods are completely different, as shown in Figure 4.10.2-1.

Figure 4.10.2-1: Quasi-Earth-fixed cell case 1

Case 2: The areas of cell coverage in two consecutive periods partly overlap, as shown in Figure 4.10.2-2.

Figure 4.10.2-2: Quasi-Earth-fixed cell case 2

Note: The time (t-Service_new) may vary across different areas within the cell.

-	Earth-moving cell: provisioned by beam(s) whose coverage area slides over the Earth surface (e.g., the case of NGSO satellites generating fixed or non-steerable beams).

=======================End of the changes=======================

- Reference

- RP-251873, TR 38.914 v0.1.0 for Study on 6G Scenarios and requirements, 3GPP TSG#108, Prague, Czech Republic, June 9-13, 2025