3GPP TSG-RAN5 Meeting #108	R5-254150

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5478 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT testcase 22.4.23 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the current Prose, in table 22.4.23.3.2-2, at Step 8C a timer of 30s is started so that the UE goes into Idle mode, and camps on Ncell 1. However, some UEs, which have multiple bands enabled take much longer than 30s to detect Ncell 1 and camp on it. In such a scenario the UE wont respond to paging in Step 9-13, and can fail the testcase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to increase the timer at Step 8C to 120s. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may unfairly fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.4.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Associated with R5s250294 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 22.4.23	NB-IoT / Radio link failure / T310 expiry / Dedicated RLF timer (CP CIoT)

22.4.23.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {
  when { UE receives an RRCConnectionSetup-NB message containing an rlf-TimersAndConstants-r13 set to setup }

then { UE uses timer value received in the RRCConnectionSetup-NB  message }

}

(2)

with { UE in E-UTRA RRC_IDLE state }

ensure that {
  when { UE receives an RRCConnectionSetup-NB message without IE rlf-TimersAndConstants-r13 }

then { UE uses timer value received in SystemInformationBlockType2-NB }

}

22.4.23.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 clause 5.2.2.9, 5.3.10.0, 5.3.10.7 and 5.3.11.3.

[TS 36.331 clause 5.2.2.9]

Upon receiving SystemInformationBlockType2-NB, the UE shall:

1>	apply the configuration included in the radioResourceConfigCommon;

1>	apply the defaultPagingCycle included in the radioResourceConfigCommon;

1>	apply the specified PCCH configuration defined in 9.1.1.3.

[TS 36.331 clause 5.3.10.0]

The UE shall:

…

1>	if the received radioResourceConfigDedicated includes the rlf-TimersAndConstants:

2>	reconfigure the values of timers and constants as specified in 5.3.10.7;

[TS 36.331 clause 5.3.10.7]

The UE shall:

1>	if the received rlf-TimersAndConstants is set to release:

2>	use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SystemInformationBlockType2 (or SystemInformationBlockType2-NB in NB-IoT);

1>	else:

2>	reconfigure the value of timers and constants in accordance with received rlf-TimersAndConstants;

[TS 36.331 clause 5.3.11.3]

The UE shall:

1>	upon T310 expiry; or

…

2>	consider radio link failure to be detected for the MCG i.e. RLF;

…

2>	if AS security has not been activated:

3>	if the UE is a NB-IoT UE:

4>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

[TS 36.331, clause 5.3.12]

Upon leaving RRC_CONNECTED, the UE shall:

1>	reset MAC;

1>	stop all timers that are running except T320, T322, T325, T330;

1>	if leaving RRC_CONNECTED was triggered by suspension of the RRC:

…

1>	else:

2>	release all radio resources, including release of the RLC entity, the MAC configuration and the associated PDCP entity for all established RBs;

2>	indicate the release of the RRC connection to upper layers together with the release cause;

1>	if leaving RRC_CONNECTED was triggered neither by reception of the MobilityFromEUTRACommand message nor by selecting an inter-RAT cell while T311 was running:

…

2>	enter RRC_IDLE and perform procedures as specified in TS 36.304 [4, 5.2.7];

22.4.23.3	Test description

22.4.23.3.1	Pre-test conditions

System Simulator:

-	Ncell 1.

UE:

None.

Preamble:

-	The UE is in registered Idle Mode (State 3-NB) on Ncell 1 according to Table 8.1.5.3.3 [18]

22.4.23.3.2	Test procedure sequence

Table 22.4.23.3.2-1 illustrates the downlink power levels to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 22.4.23.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Ncell 1 | Remark |
| --- | --- | --- | --- | --- |
| T1 | Cell-specific NRS EPRE | dBm/15kHz | ”Off” | No Ncells are available. (NOTE 1). |
| T2 | Cell-specific NRS EPRE | dBm/15kHz | -85 | Only Ncell 1 is available. (NOTE 1). |
| NOTE 1: Power level “Off” is ≤ -150 dBm |  |  |  |  |

Table 22.4.23.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The generic test procedure in TS 36.508 subclause 8.1.5A.2.3 is performed on Ncell 1. | - | - | - | - |
| 1A | The SS changes Ncell 1 levels according to row "T1" in Table 22.4.23.3.2-1 in order that UE considers the radio link of Ncell 1 is out-of-sync. | - | - | - | - |
| 1B | Wait for 5s. |  |  |  |  |
| 1C | The SS changes Ncell 1 levels according to row "T2" in Table 22.4.23.3.2-1 in order that UE considers the radio link of Ncell 1 is in-sync. | - | - | - | - |
| 1D | The SS transmits a UECapabilityEnquiry-NB message to request UE radio access capability information. | <-- | UECapabilityEnquiry-NB | - | - |
| 1E | Check: Does the UE transmit a UECapabilityInformation-NB message? | --> | UECapabilityInformation-NB | 1 | P |
| 1F | The SS transmits an RRCConnectionRelease-NB message. | <-- | RRCConnectionRelease-NB | - | - |
| 1G | 'Generic Test Procedure NB-IoT Control Plane CIoT MT user data transfer non-SMS transport' as described in TS 36.508 [18], clause 8.1.5A.2.3 are performed | - | - | - | - |
| 2 | Void | - | - | - | - |
| 3-7 | Void | - | - |  |  |
| 8 | The SS changes Ncell1 power levels according to row "T1" in Table 22.4.23.3.2-1. | - | - | - | - |
| 8A | Wait for 3s. |  |  | - | - |
| 8B | The SS changes Ncell1 power levels according to row "T2" in Table 22.4.23.3.2-1. | - | - | - | - |
| 8C | Wait for 120s to ensure UE enters RRC_IDLE. | - | - | - | - |
| 9-13 | 'Generic Test Procedure NB-IoT Control Plane CIoT MT user data transfer non-SMS transport' as described in TS 36.508 [18], clause 8.1.5A.2.3 are performed |  |  | 2 | P |
| 14 | The SS transmits an RRCConnectionRelease-NB message. | <-- | RRCConnectionRelease-NB | - | - |

22.4.23.3.3	Specific message contents

Table 22.4.23.3.3-1: SystemInformationBlockType2-NB for Ncell 1 (all steps, Table 22.4.23.3.2-2)

| Derivation Path: 36.508, Table 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2-NB-r13 ::= SEQUENCE { |  |  |  |
| ue-TimersAndConstants-r13 SEQUENCE { |  |  |  |
| t310-r13 | ms1000 |  |  |
| n310-r13 | n1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.4.23.3.3-2: RRCConnectionSetup-NB (step 1, Table 22.4.23.3.2-2)

| Derivation Path: 36.508 Table 8.1.6.1-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetup-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionSetup-r13 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated-r13 SEQUENCE { |  |  |  |
| rlf-TimersAndConstants-r13 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| t301-r13 | ms10000 |  |  |
| t310-r13 | ms8000 |  |  |
| n310-r13 | n1 |  |  |
| t311-r13 | ms1000 |  |  |
| n311-r13 | n1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.4.23.3.3-3: Void