3GPP TSG-RAN Meeting #109	RP-252315

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.3

Source:	Intel Corporation

Title:	Views on requirements for 6G architecture and migration

Document for:	Discussion and decision

# Introduction

In this contribution, we provide views on requirements for 6G architecture and migration for input to [1]. We take requirements on 5G architecture and migration [2] as the starting points and discuss new requirements for 6G architecture and migration.

# Discussion

Requirements on 5G architecture and migration as defined in [2] are a good starting point for defining architecture and migration requirements for 6G. We expect many requirements defined in 5G will continue to be important for 6G, such as network virtualization, CAPEX/OPEX efficiency, CP/UP separation, RAN sharing, deployment flexibility, network slicing, multi-vendor interoperability.

On the other hand, 6G architecture and migration design should take into account key lessons learned from 5G, technology advancements, new use case challenges and shifting market demands, notably,

- From EN-DC deployment in 5G, we learned that the non-standalone architecture (NSA) option faces challenges including - not being able to offer 5G new services other than radio access, implementation and deployment complexities due to additional RAN-level interworking between technologies, challenges in multi-vendor interoperability, complexity in migration to 5G standalone operation.

- 6G core network is expected to be an extension of 5G core network. 5G core network is designed with a service-based architecture (SBA). Network functions (NF) are connected via a service bus. New functionality can be added to an existing NF or be defined as a new NF. The core network for 6G can be an extension of 5G core network by adding new functionalities and service capabilities. This differs from the evolution from 4G to 5G where the 5G core network was a completely new design.

- 6G radio can share existing spectrum with 5G NR via MRSS. Furthermore, through the use of 6G-6G CA, 6G radio will be able to aggregate existing spectrum (shared with 5G NR) with new spectrum in order to meet IMT-2030 throughput requirements. 5G NR was designed with the goal to minimize always-on signals and this should make dynamic spectrum sharing between 6G radio and 5G radio easy and efficient. This differs from 4G-5G dynamic spectrum sharing where 5G cannot easily and efficiently use 4G spectrum due to the presence of always on signals in 4G radio such as CRS. 5G used inter-RAT dual connectivity between 4G and 5G for as the primary means to aggregate existing 4G spectrum and new 5G spectrum. However, inter-RAT dual connectivity has been found to lose its traffic aggregation effectiveness in busty traffic due to the slow Scell addition and activation process.

- Inter-RAT mobility with single registration has been the default approach to address the issue of incomplete coverage of a new generation, particularly during the initial migration phase. However, this approach can result in service interruption at coverage boundaries. A value proposition of tight interworking between 4G and 5G was that it relied on 4G as the coverage layer for 5G at the initial deployment phase offering better user experience than inter-RAT mobility. However, with tools such as inter-RAT mobility with dual registration, the same goal of improved user experience at coverage boundaries can be achieved with much less implementation and deployment complexity.

- 6G system is expected to enable new services beyond communications. Notable examples include sharing network computing infrastructure with user applications for localized processing, enabling sensing capabilities in the radio and sensing data management/processing capabilities in the network. Functionalities for these new services may residue in the RAN or in the core network. If they reside in the RAN, service-based architecture and interfaces can be considered for these non-communication centric RAN functions to allow flexible and scalable deployment. If reside in the core network, service-based interface (SBI) can be considered to allow efficient service discovery and data transfer between RAN and these non-communication centric core network functions. Similar improvement can be considered for the positioning service. SBI can be used to connect RAN and LMF for direct service discovery and data transfer without AMF relaying.

Considering the above, the following requirements can be defined for 6G architecture and migration:

- The RAN architecture shall support standalone operation and support MRSS between 5G and 6G

- The RAN architecture shall support new services beyond communication services

  - The RAN architecture and RAN-CN interface shall support flexible and scalable deployment of the new functions

It is envisaged that more study and discussion will be required on the support of 5G-6G Dual Connectivity so no proposal on these points is made at this stage.

Proposal 1: The following requirements on 5G architecture and migration can continue to be applied for 6G architecture and migration:

- The RAN architecture shall support connectivity through multiple transmission points, either collocated or non-collocated.

  - The RAN architecture shall enable a separation of control plane signalling and user plane data from different sites.

  - The RAN architecture shall support interfaces supporting effective inter-site scheduling coordination.

- Different options and flexibility for splitting the RAN architecture shall be allowed.

- The RAN architecture shall allow for deployment flexibility e.g. to host relevant RAN, CN and application functions close together at the edges of the network, when needed, e.g. to enable context aware service delivery, low latency services, etc...

- The RAN architecture shall allow for C-plane/U-plane separation.

- The RAN architecture shall allow deployments using Network Function Virtualization.

- The RAN architecture shall allow for the RAN and the CN to evolve independently.

- The RAN architecture shall allow for the operation of Network Slicing.

- The RAN architecture shall support sharing of the RAN between multiple operators.

- The design of the RAN architecture shall allow the deployment of new services rapidly and efficiently.

- The design of the RAN architecture shall allow the support of 3GPP defined service classes (e.g. interactive, background, streaming and conversational).

- The design of the RAN architecture shall enable lower CAPEX/OPEX with respect to current networks to achieve the same level of services.

- RAN-CN interfaces and RAN internal interfaces (both between new RAT logical nodes/functions and between new RAT and LTE logical nodes/functions) shall be open for multi-vendor interoperability.

Proposal 2: The following new requirements can be defined for 6G architecture and migration:

- The RAN architecture shall support standalone operation and support MRSS between 5G and 6G.

- The RAN architecture shall support new services beyond communication services.

  - The RAN architecture and RAN-CN interface shall support flexible and scalable deployment of the new functions.

# Conclusions

In this contribution, we put forward the following proposals on requirements for 6G architecture and migration:

Proposal 1: The following requirements on 5G architecture and migration can continue to be applied for 6G architecture and migration:

- The RAN architecture shall support connectivity through multiple transmission points, either collocated or non-collocated.

  - The RAN architecture shall enable a separation of control plane signalling and user plane data from different sites.

  - The RAN architecture shall support interfaces supporting effective inter-site scheduling coordination.

- Different options and flexibility for splitting the RAN architecture shall be allowed.

- The RAN architecture shall allow for deployment flexibility e.g. to host relevant RAN, CN and application functions close together at the edges of the network, when needed, e.g. to enable context aware service delivery, low latency services, etc...

- The RAN architecture shall allow for C-plane/U-plane separation.

- The RAN architecture shall allow deployments using Network Function Virtualization.

- The RAN architecture shall allow for the RAN and the CN to evolve independently.

- The RAN architecture shall allow for the operation of Network Slicing.

- The RAN architecture shall support sharing of the RAN between multiple operators.

- The design of the RAN architecture shall allow the deployment of new services rapidly and efficiently.

- The design of the RAN architecture shall allow the support of 3GPP defined service classes (e.g. interactive, background, streaming and conversational).

- The design of the RAN architecture shall enable lower CAPEX/OPEX with respect to current networks to achieve the same level of services.

- RAN-CN interfaces and RAN internal interfaces (both between new RAT logical nodes/functions and between new RAT and LTE logical nodes/functions) shall be open for multi-vendor interoperability.

Proposal 2: The following new requirements can be defined for 6G architecture and migration:

- The RAN architecture shall support standalone operation and support MRSS between 5G and 6G.

- The RAN architecture shall support new services beyond communication services.

  - The RAN architecture and RAN-CN interface shall support flexible and scalable deployment of the new functions.

# References

- TR 38.914, “Study on 6G Scenarios and requirements”, 3GPP RAN

- TR 38.913, “Study on scenarios and requirements for next generation access technologies”, 3GPP RAN