3GPP TSG RAN Meeting #109	RP-252343

# Beijing, China, September 15-18, 2025

Agenda item:	8.2.2

Source:	Samsung

Title:	6G Device Types

Document for:	Discussion and Decision

1 Introduction

The evolution toward 6G requires not only new radio capabilities but also a clear classification of device types as concrete product categories. In 5G, device diversity was mainly addressed through smartphones, fixed wireless terminals, IoT modules, and reduced-capability (RedCap) devices. However, 6G is expected to broaden the device types further, and multiple distinct device groups are anticipated to emerge in commercial deployments.

From a consumer perspective, smartphones will remain the dominant product category. Future models will likely integrate advanced multi-band multi-antenna features, enhanced energy efficiency, and AI-driven connectivity management. From an industrial and vertical perspective, specialized devices such as factory automation terminals, autonomous vehicle units, and smart infrastructure modules will form a separate class, requiring high reliability, deterministic latency, and robust mobility support. At the low-cost and ultra-low-power end, mass-deployed IoT devices will demand extremely simplified transceiver structures, extended coverage, and tailored energy-saving mechanisms.

Establishing such product-oriented device classifications at an early stage is crucial for 6G standardization. By anchoring device requirements to real-world product groups, rather than abstract use cases, 6GR can ensure that physical layer, signaling, and architectural designs remain both practical and scalable. This contribution emphasizes the need to define 6G device types along product-category lines and discusses the implications of such categorization for future system design. In this context, it is important for 3GPP to establish a common understanding and classification of 6G device types early in the standardization process.

2 Discussion

For 6G, it is important to classify device types according to concrete product groups expected in commercial deployments. This section highlights four representative device categories, with NTN devices considered as an additional add-on example.

- Type I: This type spans from cost-sensitive industrial modules to ultra-low-power modules for IoT services. These devices will be deployed in massive scale and must operate under extremely tight cost and energy budgets. 6GR should provide simplified PHY/MAC designs, scalable addressing and identification mechanisms, and highly efficient paging/wake-up signaling.

- Type II: Type II is to support wearables (e.g., smartwatches). Wearable devices represent a distinct category with unique constraints in terms of battery size, smallest form factor, and connectivity requirements. Unlike smartphones, wearables typically operate with reduced complexity (e.g., limited bandwidth, fewer antennas) while requiring robust coverage and efficient signaling to support continuous operation under low power. Wearable devices such as smartwatches face unique RF challenges due to their small form factor, including limited number of antenna, constrained PA/filter design, and strict power/thermal budgets. These factors typically result in higher error vector magnitude (EVM) compared to smartphones or IoT devices. Consequently, while generic IoT devices can rely on similar requirements as normal UE, wearable devices require a separate classification in 6G with mid-level modulation capability (e.g., up to 64 QAM) but with more relaxed EVM requirements than smartphones. This distinction justifies treating wearables as a device type separate from IoT in 6GR.

- Type III: Smartphones will continue to be the most widely adopted 6G device type, serving as the baseline for feature-rich capabilities. Smartphones must support high spectrum flexibility, multi-antenna operation, and advanced AI-assisted resource management while maintaining strict energy efficiency and form-factor constraints. However, in practical smartphone form factors, achieving flexible multi-band frequency support is fundamentally constrained since adding more antennas and transmit chains is extremely difficult. The compact device architecture must already accommodate displays, batteries, cameras, and processors, leaving very limited space for RF front-end components. Each additional antenna requires its own matching network, filters, and often dedicated power amplifiers or switches, which not only increases board area but also raises power consumption and thermal challenges. As a result, the physical and electrical limitations of smartphone design make it unrealistic to scale antenna counts and RF chains in the same way as larger form-factor devices such as laptops or FWA.

- Type IV: This is to support high capable UE for fixed Wireless Access (FWA) service. FWA devices will remain an essential category in 6G, providing broadband connectivity to households and enterprises. Compared to smartphones, FWA UEs typically allow less stringent size and power constraints, enabling support for high-gain antennas, multi-band reception, and advanced beam management. For FWA UEs, 6G standardization must ensure efficient support for wideband carrier aggregation, optimized mobility handling for semi-static installations, and seamless integration with wired backhaul networks.

Add-on Example: NTN Devices

In addition to the above main categories, a separate consideration arises for devices supporting non-terrestrial networks (NTN). These devices may belong to one of the four primary categories (e.g., smartphones with NTN capability, or IoT modules for satellite communication), while also requiring support of additional features such as extended timing advance, enhanced Doppler compensation, and specific mobility procedures. NTN capability is therefore best treated as an add-on feature rather than a standalone one.

3 Proposal

We propose following proposals for 6G device types.

- Device types

  - Type I: e.g., IoT

  - Type II: e.g., Wearable with limited form factor (e.g., smartwatch)

  - Type III: e.g., Smartphone

  - Type IV: e.g., FWA

- Add-on use cases: NTN, [FFS for others]

- Key parameters for each device types (NTN can be applicable in addition to device types)

| Parameter list | Type IV (FWA) | Type III (Smartphone) | Type II (Wearable) | Type I (IoT) | NTN (if applicable) |
| --- | --- | --- | --- | --- | --- |
| TX antenna | 1-4Tx (depending on FRs) (Note: 2TX can be baseline) | 1Tx baseline (for all FRs), up to 2Tx (optional) | 1Tx | 1Tx | 1Tx to 4 Tx (depending on PC). |
| RX antenna | 4, 6 or 8 RX (optional) | Below 1 GHz: 2RX, up to 4 RX (optional) Around 3.5 GHz: 4 RX, Around 7 GHz: 4RX, up to 6 RX (optional) Around 24 GHz : 2 RX, up to 4 RX (optional) | 1Rx | 1Rx and up to 2RX (optional) | Depending on device type |
| MIMO layer | Up to 4, 6 or 8 Layer | Up to 2 or 4 layer (depending on FR) | 1 layer | 1 or 2 layer (optional) | 1 layer |
| RF BW (max) | 400 MHz | 200 or [400] MHz | 20 MHz | 20 MHz | Depending on device type |
| Maximum Power class | 29 dBm [or 31 dBm] | FDD 23 dBm, TDD 26 dBm | FDD 23 dBm, TDD 26 dBm | FDD 23 dBm, TDD 26 dBm | [Depending on outcome of 5G work] |
| DL max modulation order | 1024QAM or [4096QAM] | 1024QAM or [4096QAM] | 64 QAM | 256 QAM | Depending on device type |
| UL max modulation order | 256QAM or [1024QAM] | 256QAM or [1024QAM] | 64 QAM | 64 QAM | Depending on device type |