3GPP TSG-WG4 Meeting #116	 R4-2512634

# Bengaluru, India, 25th of August – 29th of August, 2025                 (revision of R4-2511370)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | 0211 | rev | 1 | Current version: | 18.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_enh-Core) Maintenance CR to TS 38.101-5 – Min Peak EIRP value for NTN UE in Ka-band – Cat F CR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | THALES |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clarification for Min peak EIRP for Ka-band. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of a Note to explain the applicability of min peak EIRP value. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for NTN UE in Ka-band not fully understood. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Change>

## 9.2	Transmitter power

### 9.2.1	NTN VSAT maximum output power

#### 9.2.1.0	General

The NTN VSAT classes are specified based on the assumptions of certain NTN VSAT types with specific device architectures including antenna beam steering types. The requirements are specified for different NTN VSAT types. For the hybrid beam steering capable NTN VSAT, which can adjust its antenna(s) or beam(s) in both electronic steering and mechanical steering ways, the applicable requirements shall follow either electronic or mechanical beam steering requirements depending on the NTN VSAT type it declared. The NTN VSAT types are specified in Table 9.2.1.0-1 below.

Table 9.2.1.0-1: The definitions of NTN VSAT Types

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT communicating with GSO with mechanical steering antenna. |
|  | 52 | Mobile VSAT communicating with GSO with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. |  |  |

#### 9.2.1.1	Minimum requirements for Fixed VSAT

The following requirements define the maximum output power radiated by the Fixed VSAT for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The minimum output power values for EIRP are specified in Table 9.2.1.1-1. The requirement should be verified with test metrics of EIRP (Link=Tx beam peak direction, Meas=Link angle).The peak EIRP of Tx beam peak direction should be verified within the declared minimum elevation angle supported for transmitting. The steered beam peak directions can be achieved by mechanical steering and/or electronic steering according to VSAT Type. Where the supported minimum elevation angle shall be declared by manufacturer and within the range of 3°≤minimum elevation angle≤75°, and it can be expressed as (90-θ) if the coordinate systems in Figure 9.2.1.1-1 below is taken as an example.

Figure 9.2.1.1-1 Example measurement grid for min peak EIRP with the declared supported minimum elevation angle

Table 9.2.1.1-1: Minimum peak EIRP for Fixed VSAT

| Operating band | UE Type |  | Min peak EIRP (dBm) |  |
| --- | --- | --- | --- | --- |
| n512, n511, n510 | 1 |  | 70 (NOTE 1) |  |
|  | 2 |  | 70 (NOTE 1) |  |
|  | 3 |  | 61 (NOTE 1) |  |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance. The value corresponds to a 100 MHz CBW configuration with 66 RBs and 120 kHz SCS. |  |  |  |  |

The maximum output power values for TRP and EIRP for fixed VSAT are specified in Table 9.2.1.1-2 below.

Table 9.2.1.1-2: Maximum output power limits for Fixed VSAT

| Operating band | UE Type | TRPMAX (dBm) | EIRPmax (dBm) |
| --- | --- | --- | --- |
| n512, n511, n510 | 1 | 35 | 76.2 |
|  | 2, 3 | 43 | 76.2 |
| NOTE: Maximum EIRP is defined using 13RBs allocation with 120kHz SCS. |  |  |  |

#### 9.2.1.2	Minimum requirements for Mobile VSAT

The following requirements define the maximum output power radiated by the Mobile VSAT for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The minimum output power values for EIRP are specified in Table 9.2.1.2-1. The requirement should be verified with test metrics of EIRP (Link=Tx beam peak direction, Meas=Link angle).

The peak EIRP of Tx beam peak direction should be verified within the declared minimum elevation angle supported for transmitting. The steered beam peak directions can be achieved by mechanical steering and/or electronic steering according to VSAT Type. Where the supported minimum elevation angle shall be declared by manufacturer and within the range of 3°≤minimum elevation angle≤75°, and it can be expressed as (90-θ) if the coordinate systems in Figure 9.2.1.2-1 below is taken as an example.

Figure 9.2.1.2-1 Example measurement grid for min peak EIRP with the declared supported minimum elevation angle

Table 9.2.1.2-1: Minimum peak EIRP for Mobile VSAT

| Operating band | UE Type | Min peak EIRP (dBm) |
| --- | --- | --- |
| n512, n511 | 4 | 70 (NOTE 1) |
|  | 5 | 70 (NOTE 1) |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance. The value corresponds to a 100 MHz CBW configuration with 66 RBs and 120 kHz SCS. |  |  |

The maximum output power values for TRP and EIRP for mobile VSAT are specified in Table 9.2.1.2-2 below.

Table 9.2.1.2-2: Maximum output power limits for Mobile VSAT

| Operating band | UE Type | TRPMAX (dBm) | EIRPmax (dBm) |
| --- | --- | --- | --- |
| n512, n511, n510 | 4 | 35 | 76.2 |
|  | 5 | 43 | 76.2 |
| NOTE: Maximum EIRP is defined using 13RBs allocation with 120kHz SCS. |  |  |  |

## <End of Change>