3GPP TSG-RAN4 Meeting #116	R4-2509912

# Bengaluru, India, 25th - 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.870 | CR | 0029 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to add min measurement distance for 50cm QZ |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd, Huawei, Ericsson, Rohde&Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRP_TRS_MIMO_OTA_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-08-12 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 50cm QZ has been defined in the TR but the minimum measurement distance has not been defined yet |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the table and calculations for the minimum measurement distance for 50cm QZ |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Measurements based on 50cm QC cannot be performed properly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Cat-F CR in R4-2505842 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Skip Unchanged Sections >>>

<<< START OF CHANGES >>>

## 7.7	Minimum Range Length

Editor’s Note: Performing measurements in NR FR1 TRP-TRS OTA systems with 50cm quiet zone size at shorter range lengths than those defined in Table 7.7.1-2 with an additional measurement uncertainty for inadequate measurement distance is FFS.

This sub-section specifies the minimum range lengths for NR FR1 TRP-TRS OTA systems. The range length is defined as the distance from the centre of the quiet zone to the aperture of the measurement probes/antennas, as illustrated in Figure 7.7.1-1.

Figure 7.7.1-1: Illustration of range length definition

The minimum range length shall be the maximum of the following three limits:

-	The phase uncertainty limit: RQZ+2Drad2/

-	The amplitude uncertainty limit: 3D

-	The reactive Near-Field limit: RQZ+2

where RQZ is defined as the radius of the quiet zone, i.e., RQZ=D/2, and Drad is the diameter of the effective radiating aperture. The minimum range length calculations for D=30cm quiet zone size TRP-TRS OTA test systems shall assume that Drad is 30cm below 1GHz and decrease linearly from 30cm to 5cm from 1 GHz to 7.125 GHz, respectively. For the D=50cm quiet zone size, TRP-TRS OTA test systems shall assume that Drad is 50cm below 1GHz and decrease linearly from 50cm to 5cm from 1 GHz to 7.125 GHz, respectively. The last column of Table 7.7.1-1 shall be considered the minimum range length for NR FR1 TRP-TRS OTA systems with 30cm quiet zone size. Table 7.7.1-2 tabulates the minimum range length for NR FR1 TRP-TRS OTA systems with 50cm quiet zone size.

Table 7.7.1-1: Minimum Range Length for NR FR1 TRP-TRS OTA systems with 30cm quiet zone size.

| f [GHz] | Drad [m] | RQZ+2Drad²/λ | 3D = 6RQZ | RQZ+2λ | max(RQZ+2λ,3D,RQZ+2D²/λ) |
| --- | --- | --- | --- | --- | --- |
| 0.41 | 0.30 | 0.40 | 0.9 | 1.61 | 1.61 |
| 0.6 | 0.30 | 0.51 | 0.9 | 1.15 | 1.15 |
| 0.7 | 0.30 | 0.57 | 0.9 | 1.01 | 1.01 |
| 0.8 | 0.30 | 0.63 | 0.9 | 0.90 | 0.90 |
| 1 | 0.30 | 0.75 | 0.9 | 0.75 | 0.90 |
| 1.2 | 0.29 | 0.83 | 0.9 | 0.65 | 0.90 |
| 1.4 | 0.28 | 0.90 | 0.9 | 0.58 | 0.90 |
| 1.6 | 0.28 | 0.96 | 0.9 | 0.52 | 0.96 |
| 1.8 | 0.27 | 1.01 | 0.9 | 0.48 | 1.01 |
| 2 | 0.26 | 1.05 | 0.9 | 0.45 | 1.05 |
| 2.2 | 0.25 | 1.07 | 0.9 | 0.42 | 1.07 |
| 2.4 | 0.24 | 1.09 | 0.9 | 0.40 | 1.09 |
| 2.6 | 0.23 | 1.11 | 0.9 | 0.38 | 1.11 |
| 2.8 | 0.23 | 1.11 | 0.9 | 0.36 | 1.11 |
| 3 | 0.22 | 1.10 | 0.9 | 0.35 | 1.10 |
| 4 | 0.18 | 0.99 | 0.9 | 0.30 | 0.99 |
| 5 | 0.14 | 0.77 | 0.9 | 0.27 | 0.90 |
| 6 | 0.10 | 0.52 | 0.9 | 0.25 | 0.90 |
| 7 | 0.06 | 0.29 | 0.9 | 0.24 | 0.90 |
| 7.125 | 0.05 | 0.27 | 0.9 | 0.23 | 0.90 |

Table 7.7.1-2: Minimum Range Length for NR FR1 TRP-TRS OTA systems with 50cm quiet zone size.

| f [GHz] | Drad [m] | RQZ+2Drad²/λ | 3D = 6RQZ | RQZ+2λ | max(RQZ+2λ,3D,RQZ+2D²/λ) |
| --- | --- | --- | --- | --- | --- |
| 0.41 | 0.50 | 0.93 | 1.5 | 1.71 | 1.71 |
| 0.6 | 0.50 | 1.25 | 1.5 | 1.25 | 1.50 |
| 0.7 | 0.50 | 1.42 | 1.5 | 1.11 | 1.50 |
| 0.8 | 0.50 | 1.58 | 1.5 | 1.00 | 1.58 |
| 1 | 0.50 | 1.92 | 1.5 | 0.85 | 1.92 |
| 1.2 | 0.49 | 2.14 | 1.5 | 0.75 | 2.14 |
| 1.4 | 0.47 | 2.32 | 1.5 | 0.68 | 2.32 |
| 1.6 | 0.46 | 2.47 | 1.5 | 0.62 | 2.47 |
| 1.8 | 0.44 | 2.59 | 1.5 | 0.58 | 2.59 |
| 2 | 0.43 | 2.68 | 1.5 | 0.55 | 2.68 |
| 2.2 | 0.41 | 2.74 | 1.5 | 0.52 | 2.74 |
| 2.4 | 0.40 | 2.78 | 1.5 | 0.50 | 2.78 |
| 2.6 | 0.38 | 2.79 | 1.5 | 0.48 | 2.79 |
| 2.8 | 0.37 | 2.78 | 1.5 | 0.46 | 2.78 |
| 3 | 0.35 | 2.74 | 1.5 | 0.45 | 2.74 |
| 4 | 0.28 | 2.34 | 1.5 | 0.40 | 2.34 |
| 5 | 0.21 | 1.67 | 1.5 | 0.37 | 1.67 |
| 6 | 0.13 | 0.95 | 1.5 | 0.35 | 1.50 |
| 7 | 0.06 | 0.41 | 1.5 | 0.34 | 1.50 |
| 7.125 | 0.05 | 0.37 | 1.5 | 0.33 | 1.50 |

<<< END OF CHANGES >>>