3GPP TSG RAN meeting #109								     	RP-252582

Beijing, China, September 15th-18th, 2025

## Status Report to TSG

Agenda item:			9.7.1.2

| WI / SI Name | Introduction of Ku Bands for NR NTN |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_NTN_Ku_bands |  |  |  |  |
| Unique ID | 1040135 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP‑250799 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2025 (updated from 09/2025) | Performance part: 03/2026 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 95% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Manook Soghomonian |
|  | Company | SES |
|  | Email | Manook.soghomonian@intelsat.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

The group has completed the specifications work for the Priority 1 bands of the WI. However, it was previously agreed that Priority 2 bands would not be addressed until the completion level of Priority 1 bands is achieved. Therefore, an additional quarter is required to complete the Priority 2 bands.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN4

Summary

The group decided to introduce a new Mobile VSAT type for Ku bands, Type 6, which is a Mobile VSAT communicating with LEO only using an electronically steered antenna. The group recommended reflecting the above agreement in the Ku WID revision during RAN#109.

The group then discussed and agreed on the remaining UE and SAN RF requirements for the priority 1 bands, including the EIRP requirements for VSAT Type 6, the EIS and maximum input level requirements for the Ku band VSAT UE, as well as the EIRP/TRP accuracy, OTA reference sensitivity level, and other requirements for the Ku band SAN.

The group also completed the coexistence studies and provided the required ACIR values for each scenario. In addition, the group agreed on the resulting ACS and ACLR requirements for GSO SAN, LEO SAN, and the VSAT UE.

RAN4 has completed the core part of the RRM work and further agreed on the satellite switching delay requirements for both hard and soft switchover in quasi-earth-fixed scenarios in RAN4#116.

In summary, the group has completed the core part of the priority 1 bands requirements in this WI, and big CRs to 38.101-5, 38.108, and 38.133 were agreed in [47], [70] and [100].

#### 2.1.1	Agreements

##### Core part:

###### RAN WG4 Meeting #116, Bengaluru, India, 25th – 29th August, 2025:

The following agreements were approved:

Way Forward for [116][312] NR_NTN_Ku_Band_General [75]

NTN Ku-bands applicability

The following table capturing which regulations have been studied for each of the new NTN Ku-bands bands, clarifying their applicability.

| NTN Ku-bands | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| n247 / n508 | CEPT countries: No ECC Decision and no Harmonised Standard for ESIM/ESOMP | FCC: Not allocated to ESIM | Taiwan (NS_207N, NS_208N) |
| n248 / n509 | CEPT countries (NS_205/NS_206) | FCC (NS_203N, NS_204N) | Taiwan (NS_203N, NS_204N) Japan (NS_205/NS_206) |

The following agreements documented in the Way Forward for NR_NTN_Ku_Band_Coexistence in [81] were approved:

Way Forward on NTN Ku band co-existence

#1 Simulation assumptions

Subtopic 1-1: Clutter loss assumptions in Path loss for Calibration and Co-existence analysis

Agreement: Refer from TR 38.811 Table 6.6.2-2 for clutter loss and shadow fading in Ka band for 35 degrees elevation angle for both calibration and co-existence simulations

Subtopic 1-2: ISD assumption

Agreement: Update Ku band co-existence simulation assumptions to use ISD = 450m for TN calibration.

#2 Co-existence results suggested required ACIR values

Agreement:

Table 1: Co-existence results suggested required ACIR values

| Co-existence results suggested ACIR values |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | ACIR values |  |  |
| Scenario | Contributing | NTN | Parabolic | Phased array - 45x45 | Phased array - 78x78 |
| 1 | VSAT ACLR | GEO | 20,06 | 21,18 | 21,08 |
|  |  | LEO600 | 17,26 | 12,45 |  |
| 4 | SAN ACS | GEO | 15 | 17 | 15 |
|  |  | LEO600 | 30 | 21,83 |  |
| 5 | VSAT ACS | GEO | 36,39 | 38,84 | 33,12 |
|  |  | LEO600 | 38,25 | 39,92 |  |
| 6 | SAN ACLR | GEO | 0 | 0 | 0 |
|  |  | LEO600 | 0 | 0 |  |

#4 Proposed SAN VSAT ACLR ACS results from TN/NTN co-existence simulation

Agreement:

Table 4: Proposed ACLR and ACS of NR-NTN Ku band

|  | SAN |  | VSAT |  |  |
| --- | --- | --- | --- | --- | --- |
|  | GEO | LEO 600 | Parabolic | Phased array 45x45 | Phased array 78x78 |
| ACLR (dBc) | Note 1 | Note 1 | GEO: 20.52 LEO 17.49 | GEO: 21.79 LEO: 12.52 | GEO: 21.67 |
| ACS (dBc) | 17.22 | 22.54 | GEO: 45.21 LEO: Note 2 | Note 2 | GEO: 35.4 |
| Note 1: NTN/TN coexistence simulations indicate that there is no interference condition. Note 2: The current simulation assumptions do not allow the required ACIR obtained from simulation. |  |  |  |  |  |

#5 CR Work split to include Ku band sections in TR 38.863

Agreement:

| Item | Company |
| --- | --- |
| Big CR to TR 38.863 | ZTE |
| Addition of simulation assumptions as Clause 6b.1, 6b.2 and 6b.3 (Based on TR 38.863 Clause 6.1, 6.2 and 6.3) | Ericsson |
| Collection of simulation results in excel and addition of simulation results as Clause 6b.4 and 6b.5 (Based on TR 38.863 Clause 6.4 and 6.5) | Qualcomm |
| Addition of Calibration results in Annex | SES |

WF for NTN Ku band UE and SAN RF [99]

Sub-topic 1-1: General issue of Ku band UE RF requirements

Issue 1-1-1: Introduction of new VSAT type communicate to LEO only

Agreement:

Introduce VSAT type 6 for Rel.19 NTN Ku bands

VSAT Type 6: Mobile VSAT communicating with LEO only with electronic steering antenna.

Issue 1-1-2: LS on UE capability for NTN Ku bands

RAN4 suggest to extend the ntn-VSAT-AntennaType-r18 and ntn-VSAT-MobilityType-r18 to FR1 in Rel.19 for the NTN Ku bands, while the detail signaling design is up to RAN2

Send an LS to RAN2

Issue 1-1-3: UE feature list

Agree the UE feature listed provided in the annex 1 of [99].

Sub-topic 1-2: Remaining Ku band UE Tx RF requirements

Issue 1-2-1: EIRP requirements for VSAT type 6

Agreement:

Agree on the following EIRP requirements for VSAT type 6

Max TRP = 40 dBm

Max EIRP = 76.6 dBm

Min peak EIRP = 60 dBm

Issue 1-2-2: Confirm EIRP requirements for VSAT type 4 & 5

Agreement:

Confirm proposal 1 & 2 for VSAT type 4 & 5

Issue 1-2-3: RB configuration for EIRP requirement

Agreement

For FR2-NTN Ku bands

Maximum EIRP is defined using 6RBs allocation with 120kHz SCS

For FR1-NTN Ku bands

Maximum EIRP is defined using 27RBs allocation with 30kHz SCS or using 54RBs allocation with 15kHz SCS

Issue 1-2-6: PFD limits for the Ku band

Suggest WF:

- FFS on how to capture the PFD requirement to the spec, as normative requirement or informative requirement

Sub-topic 1-3: Remaining Ku band UE Rx RF requirements

Issue 1-3-1: EIS requirement for FR2-NTN Ku bands VSAT

Agreement:

For Ku band VSAT type 4 & 5

Only consider one EIS value based on GSO operation

EISREFSENS_50MHz ≤ -126

Based on the G/T assumption ~5.82 [value after beam gain reduction@35 degree] and additional 2 dB IM

For Ku band VSAT type 6

EISREFSENS_50MHz ≤ -122

Based on the G/T assumption ~1.75 [value after beam gain reduction@35 degree] and additional 2 dB IM

Issue 1-3-2: Handling EIS requirement for FR1-NTN Ku bands VSAT

Agreements:

For EIS requirement for FR1-NTN Ku bands VSAT

Option 1: directly apply scaling based on EISREFSENS_50MHz

Issue 1-3-3: Maximum input level requirement

Agreement:

The maximum input level requirement will be calculated based on DL link budget analysis on 90 degree, Fc = 11GHz, Atm loss =  2dB with following consideration

Taking channel bandwidth 50MHz as a reference

For VSAT type 4, 5 the following scenarios are considered

GSO scenario: EIRP density 43dBW/MHz

LEO600 scenario: EIRP density 7, 18 dBW/MHz

For VSAT type 6 the following scenario is considered

LEO600 scenario: EIRP density 7, 18 dBW/MHz

The result of maximum input level  = -106 dBm (consider the largest value from LEO 600 scenario with EIRP density 18 dBW/MHz)

Only consider MCS target 16QAM

Sub-topic 2-1: Remaining Ku band SAN Tx RF requirements

Agreement:

Option 1: +-2.5 dB EIRP accuracy,  +-2.2 dB TRP accuracy

Issue 2-1-2:  Spurious frequency range for the Ku band SAN

Agreement:

- The upper limit of spurious frequency range as 26 GHz for both FR1-NTN Ku bands and FR2-NTN Ku bands

Sub-topic 2-1: Remaining Ku band SAN Rx RF requirements

Issue 2-2-1:  OTA reference sensitivity level for the Ku band SAN

Agreement:

EISREFSENS_50M for LEO class SAN: [-120 ~ -129] dBm

EISREFSENS_50M for GEO class SAN

Less than [-129] dBm (Vendor can declare any value lower than this value.)

Issue 2-2-2:  OTA out-of-band blocking for the Ku band SAN

Agreement

- ΔfOOB = 100 MHz for both FR1-NTN Ku bands and FR2-NTN Ku bands

- Operating band characteristics as 100 MHz ≤ FUL,high – FUL,low ≤ 900 MHz for both FR1-NTN Ku bands and FR2-NTN Ku bands

- Define upper limit of frequency range of interfering signal for FR1-NTN Ku-band as 2nd harmonic of the upper frequency edge of the operating band

Issue 2-2-6:  OTA ICS for the Ku band SAN

Agreement:

- 14dB ICS (5dB IoT +9dB C/I) defined for existing SAN type 2-O GEO and LEO can be reused for Ku-band GEO and LEO

Way Forward on VSAT type for LEO only [72]

RAN4 agreed to introduce a new Mobile VSAT type for Ku bands as follows:

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Mobile VSAT | 6 | Mobile VSAT communicating with LEO only with electronic steering antenna. |

RAN4 recommends to reflect the above agreement in the Ku WID revision during RAN#109 (September 2025).

Way Forward for Ku band ACLR and ACS specification band [98]

Agreement:

|  | SAN |  | VSAT |
| --- | --- | --- | --- |
|  | GEO | LEO 600 |  |
| ACLR (dBc) | 16 | 16 | 18.5 |
| ACS (dBc) | 20 | 26 | 26 |

The following agreements documented in the Way Forward for NR_NTN_Ku_Band_UE_SAN_RF in [90] were approved:

WF on RRM requirements for NR_NTN_Ku_bands [65]

Topic #1: RRM impact of NR NTN on Ku bands

Issue 1-2-1: Hard satellite switching with re-sync

Agreement:

The components in hard satellite switching delay requirements refer to legacy hard satellite switching delay and inter satellite handover delay.

Issue 1-2-2: Soft satellite switching with re-sync

Agreement:

Agree to introduce RRM requirements for soft satellite switching with re-sync for the VSAT UE with electronic beam steering:

Tsoft_switch = max(t-service-t-serviceStart, Tsat_beam + Tsearch + T∆ + Tmargin) + Tprocessing

The requirements in this clause do not apply if the time span from the last slot of SI transmission within SI modification period where the broadcasting of the last updated value for t-ServiceStart-r18 [and ssb-TimeOffset-r18] is acquired by the UE for the first time to the first slot corresponding to t-ServiceStart-r18 is is less than [1] s.

Further refinement on the wording is not precluded.

Further discussion on the value of [1] is not precluded.

Extend the period of the scheduling restriction by two sided TRX_switch, TRX_switch = 1 symbol.

Issue 1-3: inter-satellite cell reselection requirements (section 4.2C).

Agreement:

No consensus to define inter-satellite cell reselection requirements in Rel-19.

No consensus to define non-blind handover requirement in Rel-19.

Issue 1-4: RLM (section 8.1C)

Agreement:

LMax is equal to 8 for FR1-NTN numerolgoy, and 64 for FR2-NTN numerology.

Endorsed draft CRs in RAN4#116:

| [25] | R4-2510357 | DraftCR to TS38.101-5 Update NR NTN Ku band channel bandwidths section | ZTE Corporation, CHTTL | endorsed |
| --- | --- | --- | --- | --- |
| [34] | R4-2510504 | Draft CR to 38101-5 - Off axis EIRP emissions density | Ericsson | endorsed |
| [35] | R4-2510505 | Draft CR to 38101-5 - Receiver antenna off-axis performance | Ericsson | endorsed |
| [37] | R4-2510507 | Draft CR to TS 38.108 - Operating bands | Ericsson | endorsed |
| [66] | R4-2512309 | Draft CR on Ku-band NTN | LG Electronics Inc. | endorsed |
| [68] | R4-2512313 | CR to TS 38.133 for including measurement procedures for VSAT UEs operating in KU band | Nokia | endorsed |
| [69] | R4-2512349 | DraftCR on satellite switch delay for soft satellite switch with re-sync for Ku band | MediaTek inc., Eutelsat Group | endorsed |
| [73] | R4-2512543 | Draft CR to 38.101-5 on restriction of simultaneous operation of FR1-NTN and FR2-NTN numerology for Ku band | vivo | endorsed |
| [74] | R4-2512544 | Addition of Ku bands to section 4 applicability | Eutelsat Group | endorsed |
| [79] | R4-2512607 | Draft CR for TS 38.101-5: Introducing NTN VSAT Output power dynamics requirements on Ku band(s) | Xiaomi | endorsed |
| [85] | R4-2512652 | Draft CR for 38.108, Introduction of OTA reference sensitivity level for Ku-band for NR NTN | CATT | endorsed |
| [86] | R4-2512653 | DraftCR to TS38.101-5 Add 9.5.1, 9.5.2 and 9.7 requirements for NTN Ku bands | ZTE Corporation, CHTTL | endorsed |
| [87] | R4-2512654 | DraftCR to TS38.101-5 Add 9.5.3 requirement for NTN Ku bands | ZTE Corporation, Sanechips | endorsed |
| [88] | R4-2512655 | DraftCR to TS38.108 Add 10.4 and 10.5 requirements for NTN Ku bands | ZTE Corporation, Sanechips | endorsed |
| [89] | R4-2512656 | Draft CR to TS 38.101-5 - Antenna performance | Ericsson | endorsed |
| [92] | R4-2512659 | Draft CR to TS 38.108 to add 10.6 and 10.9 requirements for Ku-band NTN | Samsung, CHTTL | endorsed |
| [93] | R4-2512660 | Draft CR to TS 38.101-5 for adding 9.2.1 requirements for NTN Ku bands | SKY Perfect JSAT Corporation | endorsed |
| [94] | R4-2512661 | Draft CR for Ku-band VSAT OTA Reference Sensitivity and Maximum Input Level | LG Electronics Finland | endorsed |
| [95] | R4-2512662 | Draft CR to TS38.108 – for transmit power and output power in NTN Ku bands | THALES | endorsed |
| [96] | R4-2512663 | Draft CR to TS38.108 – for unwanted emission requirements for NTN Ku bands | THALES | endorsed |
| [97] | R4-2512664 | draftCR for TS 38.101-5 to introduce ACS and blocking characteristics for Ku band | Huawei, HiSilicon | endorsed |

The following big CRs are agreed during RAN4#116 post meeting:

| [47] | R4-2510831 | Big CR for 38.101-5 for the addition of Ku bands | Eutelsat Group | agreed |
| --- | --- | --- | --- | --- |
| [70] | R4-2512351 | Big CR on RRM requirements of Ku band for NR NTN | Apple | agreed |
| [100] | R4-2512903 | Big CR to TS 38.108 the SAN requirement of the NR NTN Ku band | CHTTL | agreed |

##### Performance part:

Not started.

#### 2.1.2	Remaining Open issues

##### Core part:

Define additional bands for Ku (discussion on the Priority 2 Ku band #2a and #2b in RP‑250799) and the related RF requirements

##### Performance part:

RRM performance requirements and test cases

UE demodulation and CSI reporting requirements

Satellite access node demodulation requirements

Satellite access node conformance tests

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

#### Not applicable.

## 4.	References

Contributions submitted to RAN4#116:

| Ref | Tdoc | Subject | Source | Status |
| --- | --- | --- | --- | --- |
| [1] | R4-2509052 | Topic summary for [116][209] NR_NTN_Ku_bands | Moderator (Apple) | noted |
| [2] | R4-2509121 | Summary of Ku Band Co-existence Analysis Results | SES S.A. | noted |
| [3] | R4-2509218 | Discussion on RRM requirements for Ku bands for NR NTN | MediaTek inc. | noted |
| [4] | R4-2509245 | Intra-SAN (own-system interference) ACLR analysis for Ku band | Eutelsat Group | noted |
| [5] | R4-2509335 | Further discussion on coexistence evaluations for Ku-band for NR NTN | CATT | noted |
| [6] | R4-2509336 | Co-existence calibration result for Ku-band for NR NTN | CATT | noted |
| [7] | R4-2509337 | Coexistence evaluation result for Ku-band for NR NTN | CATT | noted |
| [8] | R4-2509338 | Further discussion on SAN RF core requirements for Ku-band for NR NTN | CATT | noted |
| [9] | R4-2509339 | Draft CR for 38.108, Introduction of OTA reference sensitivity level for Ku-band for NR NTN | CATT | revised |
| [10] | R4-2509597 | DraftCR on satellite switch delay for soft satellite switch with re-sync for Ku band | MediaTek inc., Eutelsat Group | revised |
| [11] | R4-2509640 | NTN VSAT Type-6 Max TRP and EIRP values | Sharp | noted |
| [12] | R4-2509660 | Discussion on RRM requirements for Ku-band NTN | LG Electronics Inc. | noted |
| [13] | R4-2509661 | Draft CR on Ku-band NTN | LG Electronics Inc. | revised |
| [14] | R4-2509671 | Discussion on OTA reference sensitivity level for the Ku band SAN | LG Electronics Inc. | noted |
| [15] | R4-2509770 | Temporary calibration results and simulation results for the Ku-band NTN coexistence study | Xiaomi | noted |
| [16] | R4-2509817 | Draft CR for TS 38.101-5: Introducing NTN VSAT Output power dynamics requirements on Ku band(s) | Xiaomi | revised |
| [17] | R4-2509818 | View on supporting new VSAT type for Mobile VSAT communicating with LEO only | Xiaomi | noted |
| [18] | R4-2510181 | Discussion on NTN Ku band VSAT RF requirements | MediaTek (Wuhan) Inc. | noted |
| [19] | R4-2510182 | Discussion on NTN Ku band coexistence study | MediaTek (Wuhan) Inc. | noted |
| [20] | R4-2510185 | EIS requirement | Sharp | noted |
| [21] | R4-2510215 | On co-existence simulation results for NR NTN Ku band | Ericsson | revised |
| [22] | R4-2510240 | Coexistence simulation results between TN and NTN for Ku band | Qualcomm Incorporated | noted |
| [23] | R4-2510258 | Draft CR to 38.101-5 on restriction of simultaneous operation of FR1-NTN and FR2-NTN numerology for Ku band | vivo | revised |
| [24] | R4-2510356 | Simulation results of NR NTN Ku band co-existence study | ZTE Corporation, Sanechips | noted |
| [25] | R4-2510357 | DraftCR to TS38.101-5 Update NR NTN Ku band channel bandwidths section | ZTE Corporation, CHTTL | endorsed |
| [26] | R4-2510358 | Discussion on UE RF requirements for NR NTN Ku band | ZTE Corporation, Sanechips | noted |
| [27] | R4-2510359 | DraftCR to TS38.101-5 Add 9.5.1, 9.5.2 and 9.7 requirements for NTN Ku bands | ZTE Corporation, CHTTL | revised |
| [28] | R4-2510360 | DraftCR to TS38.101-5 Add 9.5.3 requirement for NTN Ku bands | ZTE Corporation, Sanechips | revised |
| [29] | R4-2510361 | Discussion on SAN RF requirements for NR NTN Ku band | ZTE Corporation, Sanechips | noted |
| [30] | R4-2510362 | DraftCR to TS38.108 Add 10.4 and 10.5 requirements for NTN Ku bands | ZTE Corporation, Sanechips | revised |
| [31] | R4-2510380 | Discussion on RRM requirements for Ku bands for NR NTN | Ericsson | noted |
| [32] | R4-2510502 | NTN Ku-band - bands and regulation | Ericsson | noted |
| [33] | R4-2510503 | NTN Ku-band - NTN VSAT PFD | Ericsson | noted |
| [34] | R4-2510504 | Draft CR to 38101-5 - Off axis EIRP emissions density | Ericsson | endorsed |
| [35] | R4-2510505 | Draft CR to 38101-5 - Receiver antenna off-axis performance | Ericsson | endorsed |
| [36] | R4-2510506 | Draft CR to TS 38.101-5 - Antenna performance | Ericsson | revised |
| [37] | R4-2510507 | Draft CR to TS 38.108 - Operating bands | Ericsson | endorsed |
| [38] | R4-2510508 | Draft CR to TS 38.101-5 - VSAT PFD limits | Ericsson | revised |
| [39] | R4-2510649 | Discussion on RRM requirements for NTN in Ku band | Huawei, HiSilicon | noted |
| [40] | R4-2510650 | draftCR on mobility requirements for Ku band NTN | Huawei, HiSilicon | merged |
| [41] | R4-2510666 | Offline discussion on the NTN Ku band draft CRs review | CHTTL | noted |
| [42] | R4-2510668 | LS on UE Capability for Rel.19 Ku band VSAT | CHTTL | revised |
| [43] | R4-2510670 | Discussion on the UE feature list for Rel.19 Ku band NTN | CHTTL | noted |
| [44] | R4-2510698 | Further discussion on remaining requirement for the Rel.19 NTN Ku band VSAT | CHTTL | noted |
| [45] | R4-2510766 | draft CR on the general update for 38.101-5 clause 4 for the NTN Ku band | CHTTL | noted |
| [46] | R4-2510818 | Addition of Ku bands to section 4 applicability | Eutelsat Group | revised |
| [47] | R4-2510831 | Big CR for 38.101-5 for the addition of Ku bands | Eutelsat Group | agreed |
| [48] | R4-2510843 | Discussion on UE RF requirements for NR NTN Ku band | SKY Perfect JSAT Corporation | noted |
| [49] | R4-2510853 | Draft CR to TS 38.108 to add 10.6 and 10.9 requirements for Ku-band NTN | Samsung, CHTTL | revised |
| [50] | R4-2510861 | Draft CR to TS 38.101-5 for adding 9.2.1 requirements for NTN Ku bands | SKY Perfect JSAT Corporation | revised |
| [51] | R4-2510898 | On Ku-band VSAT reference sensitivity and maximum input level | LG Electronics Finland | noted |
| [52] | R4-2510899 | Draft CR for Ku-band VSAT OTA Reference Sensitivity and Maximum Input Level | LG Electronics Finland | revised |
| [53] | R4-2511024 | Discussion on Ku bands for NR NTN | ZTECorporation,Sanechips | noted |
| [54] | R4-2511110 | (NR_NTN_Ku_bands-Core) Definitions,symbols and abbreviations | ZTECorporation,Sanechips | postponed |
| [55] | R4-2511148 | Draft CR to TS38.108 – for transmit power and output power in NTN Ku bands | THALES | revised |
| [56] | R4-2511154 | Draft CR to TS38.108 – for unwanted emission requirements for NTN Ku bands | THALES | revised |
| [57] | R4-2511277 | Requirements for NTN UEs on Ku band operation | Nokia | noted |
| [58] | R4-2511283 | CR to TS 38.133 for including measurement procedures for VSAT UEs operating in KU band | Nokia | revised |
| [59] | R4-2511422 | draftCR for TS 38.101-5 to introduce ACS and blocking characteristics for Ku band | Huawei, HiSilicon | revised |
| [60] | R4-2511457 | Topic summary for [116][312] NR_NTN_Ku_Band_General | Moderator (Eutelsat) | noted |
| [61] | R4-2511458 | Topic summary for [116][313] NR_NTN_Ku_Band_Coexistence | Moderator (Intelsat) | noted |
| [62] | R4-2511459 | Topic summary for [116][314] NR_NTN_Ku_Band_UE_SAN_RF | Moderator (CHTTL) | noted |
| [63] | R4-2511558 | Discussion on PFD limits consideration for NTN UE RF specifications | Huawei, HiSilicon | noted |
| [64] | R4-2511599 | Suggestions for improvement of VSAT UE RF requirements in Ku-band | THALES | noted |
| [65] | R4-2512121 | WF on RRM requirements for NR_NTN_Ku_bands | Apple | approved |
| [66] | R4-2512309 | Draft CR on Ku-band NTN | LG Electronics Inc. | endorsed |
| [67] | R4-2512310 | DraftCR on satellite switch delay for soft satellite switch with re-sync for Ku band | MediaTek inc., Eutelsat Group | revised |
| [68] | R4-2512313 | CR to TS 38.133 for including measurement procedures for VSAT UEs operating in KU band | Nokia | endorsed |
| [69] | R4-2512349 | DraftCR on satellite switch delay for soft satellite switch with re-sync for Ku band | MediaTek inc., Eutelsat Group | endorsed |
| [70] | R4-2512351 | Big CR on RRM requirements of Ku band for NR NTN | Apple | agreed |
| [71] | R4-2512540 | Ad-hoc meeting minutes for [116][314] NR_NTN_Ku_Band_UE_SAN_RF | CHTTL | revised |
| [72] | R4-2512542 | Way Forward on VSAT type for LEO only | Huawei | approved |
| [73] | R4-2512543 | Draft CR to 38.101-5 on restriction of simultaneous operation of FR1-NTN and FR2-NTN numerology for Ku band | vivo | endorsed |
| [74] | R4-2512544 | Addition of Ku bands to section 4 applicability | Eutelsat Group | endorsed |
| [75] | R4-2512545 | Way Forward for [116][312] NR_NTN_Ku_Band_General | Ericsson | approved |
| [76] | R4-2512546 | Way Forward for [116][313] NR_NTN_Ku_Band_Coexistence | Ericsson | revised |
| [77] | R4-2512547 | Ad-hoc meeting minutes for [116][313] NR_NTN_Ku_Band_Coexistence | SES | noted |
| [78] | R4-2512549 | Ku band simulation assumptions running document | Ericsson | approved |
| [79] | R4-2512607 | Draft CR for TS 38.101-5: Introducing NTN VSAT Output power dynamics requirements on Ku band(s) | Xiaomi | endorsed |
| [80] | R4-2512611 | Ad-hoc meeting minutes for [116][314] NR_NTN_Ku_Band_UE_SAN_RF | CHTTL | noted |
| [81] | R4-2512648 | Way Forward for [116][313] NR_NTN_Ku_Band_Coexistence | Ericsson | approved |
| [82] | R4-2512649 | Way Forward for Ku band ACLR and ACS specification | Eutelsat, SES, Thales | revised |
| [83] | R4-2512650 | Way Forward for [116][314] NR_NTN_Ku_Band_UE_SAN_RF | CHTTL | revised |
| [84] | R4-2512651 | Way Forward for [116][314] NR_NTN_Ku_Band_UE_SAN_RF | CHTTL | revised |
| [85] | R4-2512652 | Draft CR for 38.108, Introduction of OTA reference sensitivity level for Ku-band for NR NTN | CATT | endorsed |
| [86] | R4-2512653 | DraftCR to TS38.101-5 Add 9.5.1, 9.5.2 and 9.7 requirements for NTN Ku bands | ZTE Corporation, CHTTL | endorsed |
| [87] | R4-2512654 | DraftCR to TS38.101-5 Add 9.5.3 requirement for NTN Ku bands | ZTE Corporation, Sanechips | endorsed |
| [88] | R4-2512655 | DraftCR to TS38.108 Add 10.4 and 10.5 requirements for NTN Ku bands | ZTE Corporation, Sanechips | endorsed |
| [89] | R4-2512656 | Draft CR to TS 38.101-5 - Antenna performance | Ericsson | endorsed |
| [90] | R4-2512657 | Draft CR to TS 38.101-5 - VSAT PFD limits | Ericsson | not pursued |
| [91] | R4-2512658 | LS on UE Capability for Rel.19 Ku band VSAT | CHTTL | approved |
| [92] | R4-2512659 | Draft CR to TS 38.108 to add 10.6 and 10.9 requirements for Ku-band NTN | Samsung, CHTTL | endorsed |
| [93] | R4-2512660 | Draft CR to TS 38.101-5 for adding 9.2.1 requirements for NTN Ku bands | SKY Perfect JSAT Corporation | endorsed |
| [94] | R4-2512661 | Draft CR for Ku-band VSAT OTA Reference Sensitivity and Maximum Input Level | LG Electronics Finland | endorsed |
| [95] | R4-2512662 | Draft CR to TS38.108 – for transmit power and output power in NTN Ku bands | THALES | endorsed |
| [96] | R4-2512663 | Draft CR to TS38.108 – for unwanted emission requirements for NTN Ku bands | THALES | endorsed |
| [97] | R4-2512664 | draftCR for TS 38.101-5 to introduce ACS and blocking characteristics for Ku band | Huawei, HiSilicon | endorsed |
| [98] | R4-2512695 | Way Forward for Ku band ACLR and ACS specification | Eutelsat, SES, Thales | approved |
| [99] | R4-2512696 | Way Forward for [116][314] NR_NTN_Ku_Band_UE_SAN_RF | CHTTL | approved |
| [100] | R4-2512903 | Big CR to TS 38.108 the SAN requirement of the NR NTN Ku band | CHTTL | agreed |

12.08.2024		minor adaptations for RAN #105

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v01	13.11.2009		First version of the template